| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0478-v3 |
| --- | --- |

| Title: | CE1: Hadamard transform domain filter (CE1-2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sergey Ikonin Victor Stepin Alexander Karabutov Jianle Chen | Tel: Email: | +7-495-660-44-59 sergey.ikonin@huawei.com stepin.victor@huawei.com karabutov.alexander@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., LTD |  |  |

_____________________________

# Abstract

This contribution reports results of CE1 test 2 on Hadamard transform domain filter. According to CE restrictions the filter is applied only to inter blocks with minimum block size of 64 luma samples, and excluding CIIP and IBC blocks.

The simulations over VTM-4.0 show following results:

- Test 2.1. The filter is applied as post-reconstruction filter: -0.32%/-0.34% of the luma BD-rate change with 102%/101% of encoding time and 100%/100%/of decoding time for RA/LDB configurations correspondingly.

- Test 2.3. The filter is applied as conventional in-loop filter (without restrictions on block size and type): -0.18%/-0.45%/-0.34% of the luma BD-rate change with 108%/104%/103% of encoding time and 105%/100%/101% of decoding time for AI/RA/LDB configurations correspondingly.

- Test 2.4. The filter is applied as post-prediction filter based on block-level flag: -0.32%/-0.27% of the luma BD-rate change with 109%/112% of encoding time and 100%/100%/of decoding time for RA/LDB configurations correspondingly.

- Test 7.8. The filter is applied either as post-prediction or post-reconstruction filter (mutually exclusive): -0.44% of the luma BD-rate change with 111% of encoding time and 99% of decoding time for RA configuration.

This contribution also reports results on different restrictions on neighboring reconstructed samples access and combined tests with LIC and Uniform Luma Inter Prediction filter from CE1.

# Technology description

Hadamard transform domain filter was proposed in JVET-M0468. The filter parameters are explicitly derived from the coded information and filtering process is as follows.

For each pixel from reconstructed block pixel processing comprises the following steps:

- Scan for 4 neighboring pixels around processing pixel including current one according to scan pattern

- 4 point Hadamard transform of read pixels

Fi,σ=Ri2Ri2+m*σ2*R(i)

Spectrum filtering based on the following formula:

,wherein (i) is index of spectrum component in Hadamard spectrum, R(i)  is spectrum component of reconstructed pixels corresponding to index, m=4 is normalization constant equal to number of spectrum components, σ is filtering parameter deriving from codec quantization parameter QP using following equation:

σ=2.64*2(0.1269*QP-11)

The first spectrum component corresponding to DC value is bypassed without filtering.

- Inverse 4 point Hadamard transform of filtered spectrum

- After filtering step the filtered pixels are placed to its original positions into accumulation buffer.

- After completing filtering of pixels the accumulated values are normalized by number of processing groups used for each pixel filtering. Due to use of padding of one sample around the block number of processing groups is equal to 4 for each pixel in the block and normalization is performed by right shifting on 2 bits.

Scan

Hadamard transform

Filtering with 16 entries 
look-up table

Inverse Hadamard transform

To accumulation buffer

A – current pixel

BCD – neighboring pixels

The filtering process is schematically presented on Figure 1.

Figure 1 Filtering process

Equivalent filter shape is 3x3 pixels as depicted on Figure 2.

Figure 2 Equivalent filter shape

It can be seen that all pixels in the block can be processed independently in case of maximum parallelism is required. It should also be noted that results of 2x2 groups filtering are reused for spatial collocated samples. Generally each new pixel in the block requires one 2x2 filtering, rest three are reused.

More detailed description of the filter including hardware aspects analysis and results of RTL simulations can be found in (JVET-L0326). The RTL simulations shows that having multiplication free design the filter can be implemented in HW with 1 or 2 clock latency depending on frequency and manufacturing process.

# Test restrictions

According to the CE1 restrictions for all tests, the size of the smallest processed CU is 64 luma samples.

Processing is not applied on intra blocks, nor to combined intra/inter blocks (CIIP blocks) or intra block copy (IBC) blocks.

For all tests, the following three constraint conditions are tested:

- No restriction on using samples to the left and above the block.

- Samples to the left and above can be used, but only if they come from an inter block (i.e., not from an intra block or a CIIP block or an IBC/CPR block).

- No samples outside the block can be used.

For loop-filter tests, these constraints are relaxed:

- There is no restriction on the smallest processed block.

- There is no restriction to apply filtering on intra blocks, i.e., both intra and inter blocks can be filtered.

- There is no restriction on using samples to the left or above the block, as long as the used samples reside in the same CTU as the filtered pixels.

# Test description

The proposed filter was tested in three different positions:

- Test CE1-2.1. The filter is applied as post-reconstruction filter.

- Test CE1-2.3. The filter is applied as conventional in-loop filter.

- Test CE1-2.4. The filter is applied as post-prediction filter based on block-level flag. In this test encoder is able to select either one of two filters with different strength either do not filtering at all. The decision is signaled to bitstream. It should be noted that filter implementation is the same as in test CE1-2.1. One additional LUT (16 entries of 7 bit) corresponding to one more filtering strength is added in comparison to five existing in CE1-2.1.

Combined test (CE1-7.8) on using the proposed filter mutually exclusive either for post-prediction (CE1-2.4) or post-reconstruction (CE1-2.1) filtering was also conducted. Here and after mutually exclusive means that only one filter can be applied to the current block, not both simultaneously. Another algorithms from CE1 was also tested in combination with proposed filter.

Table 1. Test list.

| Test | Description |
| --- | --- |
| CE1-2.1a | Filter is applied as post-reconstruction filter, restriction a. |
| CE1-2.1b | Filter is applied as post-reconstruction filter, restriction b. |
| CE1-2.1c | Filter is applied as post-reconstruction filter, restriction c. |
| CE1-2.3 | Filter is applied as conventional in-loop filter (no restriction on block size or block type, no restriction on using samples to the left or above the block, as long as the used samples reside in the same CTU as the filtered pixels). |
| CE1-2.4a | Filter is applied as post-prediction filter based on block-level flag, restriction a. |
| CE1-2.4b | Filter is applied as post-prediction filter based on block-level flag, restriction b. |
| CE1-2.4c | Filter is applied as post-prediction filter based on block-level flag, restriction c. |

Table 2. Combined test list.

| Test | Description |
| --- | --- |
| CE1-7.3b | Uniform Luma Inter Prediction Filter (CE1-4.2) and Hadamard transform domain in-loop filter (CE1-2.3), restriction b |
| CE1-7.4b | Uniform Luma Inter Prediction Filter (CE1-4.2) and Hadamard transform domain post-reconstruction filter (CE1-2.1), mutually exclusive, restriction b |
| CE1-7.7.1a | Local Illumination Compensation filter (CE1-5.1) and Hadamard transform domain post-reconstruction filter (CE1-2.1), mutually exclusive, restriction a |
| CE1-7.7.1b | Local Illumination Compensation filter (CE1-5.1) and Hadamard transform domain post-reconstruction filter (CE1-2.1), mutually exclusive, restriction b |
| CE1-7.7.2a | Local Illumination Compensation filter (CE1-5.1) and Hadamard transform domain post-prediction filter (CE1-2.4), mutually exclusive, restriction a |
| CE1-7.7.2b | Local Illumination Compensation filter (CE1-5.1) and Hadamard transform domain post-prediction filter (CE1-2.4), mutually exclusive, restriction b |
| CE1-7.7.3a | Local Illumination Compensation filter (CE1-5.1), Hadamard transform domain post-reconstruction and post-prediction filter (CE1-2.1, CE1-2.4), mutually exclusive, only one of three filters can be applied to the same block, restriction a |
| CE1-7.7.3b | Local Illumination Compensation filter (CE1-5.1), Hadamard transform domain post-reconstruction and post-prediction filter (CE1-2.1, CE1-2.4), mutually exclusive, only one of three filters can be applied to the same block, restriction b |
| CE1-7.8a | Hadamard transform domain post-reconstruction (CE1-2.1) and post-prediction (CE1-2.4) filter, mutually exclusive, restriction a |
| CE1-7.8b | Hadamard transform domain post-reconstruction (CE1-2.1) and post-prediction (CE1-2.4) filter, mutually exclusive, restriction b |
| CE1-7.8c | Hadamard transform domain post-reconstruction (CE1-2.1) and post-prediction (CE1-2.4) filter, mutually exclusive, restriction c |

# Test results

Simulations were carried out on top of VTM-4.0 software under common test conditions described in JVET-M1010.

## CE1-2.1a

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.10% | -0.04% | 102% | 100% |
| Class A2 | -0.44% | -0.27% | -0.08% | 102% | 100% |
| Class B | -0.38% | -0.15% | -0.16% | 102% | 100% |
| Class C | -0.25% | -0.03% | 0.05% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.32% | -0.13% | -0.06% | 102% | 100% |
| Class D | -0.31% | 0.05% | 0.12% | 101% | 100% |
| Class F | -0.22% | 0.09% | 0.03% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.33% | 0.51% | 0.75% | 102% | 99% |
| Class C | -0.43% | 0.76% | 0.70% | 101% | 101% |
| Class E | -0.21% | 0.72% | 0.18% | 100% | 100% |
| Overall | -0.34% | 0.64% | 0.59% | 101% | 100% |
| Class D | -0.59% | 1.56% | 0.42% | 102% | 101% |
| Class F | -0.44% | 0.18% | 0.41% | 101% | 100% |

## CE1-2.1b

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.20% | 0.00% | 102% | 100% |
| Class A2 | -0.44% | -0.26% | -0.06% | 102% | 100% |
| Class B | -0.39% | -0.08% | -0.17% | 102% | 100% |
| Class C | -0.22% | 0.01% | -0.06% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.31% | -0.11% | -0.09% | 102% | 100% |
| Class D | -0.30% | 0.03% | 0.09% | 101% | 101% |
| Class F | -0.24% | 0.06% | 0.07% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.35% | 0.66% | 0.87% | 102% | 100% |
| Class C | -0.43% | 0.70% | 0.51% | 101% | 102% |
| Class E | -0.23% | 0.62% | 0.23% | 100% | 101% |
| Overall | -0.35% | 0.66% | 0.59% | 101% | 101% |
| Class D | -0.61% | 0.70% | 0.02% | 102% | 102% |
| Class F | -0.44% | 0.22% | 0.11% | 102% | 100% |

## CE1-2.1c

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.18% | -0.01% | 102% | 100% |
| Class A2 | -0.43% | -0.26% | -0.05% | 102% | 99% |
| Class B | -0.37% | -0.07% | -0.11% | 102% | 100% |
| Class C | -0.20% | 0.10% | -0.04% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.30% | -0.09% | -0.06% | 102% | 100% |
| Class D | -0.30% | 0.08% | 0.06% | 101% | 100% |
| Class F | -0.20% | 0.08% | 0.03% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.36% | 0.77% | 0.71% | 102% | 100% |
| Class C | -0.46% | 0.90% | 0.76% | 101% | 102% |
| Class E | -0.24% | 0.52% | 0.68% | 100% | 100% |
| Overall | -0.36% | 0.75% | 0.72% | 101% | 100% |
| Class D | -0.55% | 1.56% | 0.36% | 102% | 101% |
| Class F | -0.46% | -0.13% | 0.09% | 101% | 100% |

## CE1-2.3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.48% | 0.39% | 107% | 105% |
| Class A2 | -0.22% | 0.47% | 0.31% | 107% | 104% |
| Class B | -0.11% | 0.35% | 0.47% | 108% | 105% |
| Class C | -0.41% | 0.31% | 0.31% | 108% | 105% |
| Class E | -0.16% | 0.38% | 0.18% | 108% | 105% |
| Overall | -0.18% | 0.39% | 0.35% | 108% | 105% |
| Class D | -0.34% | 0.47% | 0.19% | 108% | 104% |
| Class F | -0.62% | 0.26% | 0.24% | 105% | 104% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.15% | 0.01% | 104% | 100% |
| Class A2 | -0.69% | -0.49% | -0.34% | 104% | 100% |
| Class B | -0.48% | 0.08% | -0.26% | 105% | 101% |
| Class C | -0.44% | 0.20% | -0.12% | 103% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.45% | -0.05% | -0.19% | 104% | 100% |
| Class D | -0.49% | -0.30% | -0.08% | 103% | 102% |
| Class F | -0.64% | -0.05% | -0.10% | 103% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.32% | 0.88% | 0.38% | 105% | 100% |
| Class C | -0.54% | 1.02% | 0.62% | 104% | 101% |
| Class E | -0.11% | 1.15% | 0.67% | 101% | 100% |
| Overall | -0.34% | 0.99% | 0.54% | 103% | 101% |
| Class D | -0.70% | 1.39% | 0.34% | 104% | 101% |
| Class F | -0.68% | 0.21% | -0.19% | 103% | 100% |

## CE1-2.4a

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.71% | -0.34% | 108% | 99% |
| Class A2 | -0.44% | -0.70% | -0.56% | 109% | 99% |
| Class B | -0.48% | -0.87% | -0.88% | 109% | 100% |
| Class C | -0.14% | -0.03% | -0.32% | 110% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.32% | -0.58% | -0.56% | 109% | 100% |
| Class D | -0.14% | -0.07% | -0.03% | 109% | 100% |
| Class F | -0.14% | 0.10% | 0.03% | 106% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.33% | 0.07% | 0.26% | 115% | 100% |
| Class C | -0.28% | 0.50% | 0.55% | 116% | 100% |
| Class E | -0.26% | 0.75% | 0.61% | 109% | 100% |
| Overall | -0.30% | 0.38% | 0.44% | 114% | 100% |
| Class D | -0.28% | 0.45% | 0.36% | 115% | 99% |
| Class F | -0.63% | 0.43% | -0.02% | 110% | 100% |
| (*) – LDB results are estimated for 1 missing point, will be updated shortly. |  |  |  |  |  |

## CE1-2.4b

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.72% | -0.35% | 109% | 99% |
| Class A2 | -0.40% | -0.64% | -0.44% | 110% | 99% |
| Class B | -0.45% | -0.67% | -0.76% | 110% | 99% |
| Class C | -0.09% | -0.05% | -0.18% | 111% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.29% | -0.51% | -0.46% | 110% | 99% |
| Class D | -0.12% | 0.06% | 0.06% | 110% | 100% |
| Class F | -0.13% | 0.13% | 0.16% | 107% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.30% | 0.16% | 0.03% | 116% | 99% |
| Class C | -0.26% | 0.68% | 0.56% | 117% | 101% |
| Class E | -0.33% | 0.82% | -0.14% | 110% | 100% |
| Overall | -0.29% | 0.50% | 0.17% | 115% | 100% |
| Class D | -0.19% | 0.81% | 0.38% | 116% | 100% |
| Class F | -0.72% | 0.62% | -0.57% | 111% | 101% |
| (*) – LDB results are estimated for 1 missing point, will be updated shortly. |  |  |  |  |  |

## CE1-2.4c

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.65% | -0.14% | 108% | 99% |
| Class A2 | -0.43% | -0.51% | -0.32% | 109% | 99% |
| Class B | -0.45% | -0.64% | -0.85% | 109% | 99% |
| Class C | -0.08% | 0.07% | -0.11% | 110% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.30% | -0.42% | -0.41% | 109% | 99% |
| Class D | -0.08% | 0.01% | 0.03% | 109% | 100% |
| Class F | -0.10% | 0.14% | 0.07% | 105% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.29% | 0.12% | 0.01% | 115% | 99% |
| Class C | -0.24% | 0.78% | 0.63% | 116% | 100% |
| Class E | -0.26% | 1.05% | 0.52% | 109% | 101% |
| Overall | -0.27% | 0.57% | 0.34% | 114% | 100% |
| Class D | -0.21% | 1.04% | 0.34% | 115% | 100% |
| Class F | -0.54% | 0.10% | -0.30% | 110% | 100% |

## CE1-7.3b

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.48% | 0.39% | 107% | 105% |
| Class A2 | -0.22% | 0.47% | 0.31% | 107% | 104% |
| Class B | -0.11% | 0.35% | 0.47% | 108% | 105% |
| Class C | -0.41% | 0.31% | 0.31% | 108% | 105% |
| Class E | -0.16% | 0.38% | 0.18% | 108% | 105% |
| Overall | -0.18% | 0.39% | 0.35% | 108% | 105% |
| Class D | -0.34% | 0.47% | 0.19% | 108% | 104% |
| Class F | -0.62% | 0.26% | 0.24% | 105% | 104% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.46% | -0.41% | -0.24% | 113% | 100% |
| Class A2 | -0.96% | -0.72% | -0.62% | 115% | 99% |
| Class B | -0.87% | -0.56% | -0.92% | 114% | 100% |
| Class C | -0.55% | 0.06% | -0.22% | 114% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.72% | -0.40% | -0.54% | 114% | 100% |
| Class D | -0.50% | -0.30% | -0.23% | 114% | 101% |
| Class F | -0.80% | -0.02% | -0.06% | 109% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.48% | 1.09% | 1.05% | 124% | 99% |
| Class C | -0.45% | 1.50% | 1.09% | 125% | 101% |
| Class E | -0.01% | 1.83% | 1.82% | 113% | 100% |
| Overall | -0.35% | 1.41% | 1.25% | 122% | 100% |
| Class D | -0.59% | 1.29% | 0.11% | 126% | 102% |
| Class F | -0.61% | 0.72% | 0.37% | 119% | 100% |

## CE1-7.4b

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.43% | -0.51% | -0.20% | 110% | 99% |
| Class A2 | -0.70% | -0.62% | -0.32% | 112% | 99% |
| Class B | -0.80% | -0.61% | -0.82% | 111% | 100% |
| Class C | -0.31% | -0.14% | -0.29% | 112% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.58% | -0.47% | -0.46% | 111% | 100% |
| Class D | -0.32% | -0.15% | -0.10% | 112% | 100% |
| Class F | -0.29% | -0.14% | -0.13% | 107% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.38% | 1.17% | 1.20% | 118% | 100% |
| Class C | -0.43% | 1.00% | 1.04% | 122% | 102% |
| Class E | -0.18% | 1.42% | 1.56% | 111% | 101% |
| Overall | -0.35% | 1.17% | 1.24% | 117% | 101% |
| Class D | -0.43% | 1.35% | 0.83% | 122% | 103% |
| Class F |  |  |  |  |  |

(*) – LDB results are estimated for 2 missing points, will be updated shortly.

## CE1-7.7.1a

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.82% | 0.36% | 0.29% | 118% | 99% |
| Class A2 | -0.74% | -0.48% | -0.38% | 111% | 100% |
| Class B | -0.85% | -0.31% | -0.22% | 113% | 100% |
| Class C | -0.58% | -0.18% | -0.23% | 114% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.75% | -0.17% | -0.15% | 114% | 100% |
| Class D | -0.43% | 0.13% | 0.08% | 113% | 101% |
| Class F | -1.09% | -0.48% | -0.52% | 108% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.95% | 0.35% | 0.07% | 114% | 99% |
| Class C | -0.96% | 0.50% | 0.18% | 117% | 100% |
| Class E | -0.18% | 1.02% | 0.55% | 100% | 101% |
| Overall | -0.76% | 0.56% | 0.23% | 111% | 100% |
| Class D | -0.71% | 0.65% | 0.03% | 115% | 100% |
| Class F | -3.09% | -1.70% | -1.79% | 112% | 100% |

## CE1-7.7.1b

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.72% | -0.40% | -0.41% | 120% | 100% |
| Class A2 | -0.67% | -0.51% | -0.38% | 112% | 100% |
| Class B | -0.71% | -0.28% | -0.26% | 115% | 101% |
| Class C | -0.54% | -0.23% | -0.30% | 116% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.66% | -0.34% | -0.32% | 116% | 101% |
| Class D | -0.43% | -0.11% | -0.11% | 115% | 101% |
| Class F | -0.95% | -0.36% | -0.49% | 110% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.87% | 0.46% | 0.45% | 117% | 100% |
| Class C | -0.89% | 0.48% | 0.03% | 121% | 103% |
| Class E | -0.20% | 0.95% | 0.18% | 101% | 102% |
| Overall | -0.71% | 0.59% | 0.24% | 114% | 101% |
| Class D | -0.72% | 0.90% | 0.12% | 119% | 103% |
| Class F | -2.61% | -1.63% | -0.69% | 115% | 100% |
| (*) LDB results are estimated for few missing points, will be updated shortly |  |  |  |  |  |

## CE1-7.7.2a

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |
| Class A1 | -0.83% | -0.36% | -0.02% | 126% | 98% |  |
| Class A2 | -0.63% | -0.47% | -0.54% | 117% | 99% |  |
| Class B | -0.92% | -0.98% | -1.07% | 121% | 99% |  |
| Class C | -0.44% | -0.29% | -0.36% | 125% | 100% |  |
| Class E |  |  |  |  |  |  |
| Overall | -0.74% | -0.67% | -063% | 123% | 99% |  |
| Class D | -0.27% | -0.18% | -0.24% | 123% | 100% |  |
| Class F | -0.99% | -0.47% | -0.56% | 114% | 100% |  |
|  | Low delay B Main10 |  |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.93% | -0.19% | -0.08% | 130% | 99% |  |
| Class C | -0.73% | 0.51% | 0.33% | 134% | 100% |  |
| Class E | -0.28% | 1.12% | 0.92% | 111% | 100% |  |
| Overall | -0.70% | 0.37% | 0.31% | 126% | 100% |  |
| Class D | -0.43% | 0.24% | -0.46% | 130% | 100% |  |
| Class F | -3.07% | -1.69% | -1.44% | 122% | 100% |  |
| (*) LDB results are estimated for 1 missing point, will be updated shortly. |  |  |  |  |  |  |

## CE1-7.7.2b

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |
| Class A1 | -0.74% | -0.87% | -0.66% | 128% | 99% |  |
| Class A2 | -0.64% | -0.90% | -0.80% | 121% | 99% |  |
| Class B | -0.79% | -0.89% | -0.98% | 123% | 99% |  |
| Class C | -0.41% | -0.36% | -0.42% | 127% | 100% |  |
| Class E |  |  |  |  |  |  |
| Overall | -0.65% | -0.74% | -0.73% | 125% | 99% |  |
| Class D | -0.22% | -0.29% | -0.17% | 124% | 100% |  |
| Class F | -0.86% | -0.40% | -0.42% | 116% | 100% |  |
|  | Low delay B Main10 |  |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.79% | -0.16% | -0.22% | 132% | 99% |  |
| Class C | -0.69% | 0.26% | 0.06% | 138% | 101% |  |
| Class E | -0.28% | 0.96% | 0.28% | 112% | 101% |  |
| Overall | -0.63% | 0.26% | 0.00% | 129% | 100% |  |
| Class D | -0.35% | 0.25% | 0.13% | 133% | 101% |  |
| Class F | -2.77% | -2.24% | -2.18% | 125% | 100% |  |
| (*) LDB results are estimated for few missing points, will be updated shortly |  |  |  |  |  |  |

## CE1-7.7.3a

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.88% | -0.21% | 0.06% | 128% | 99% |
| Class A2 | -0.92% | -1.03% | -0.81% | 121% | 99% |
| Class B | -1.06% | -0.84% | -0.98% | 123% | 100% |
| Class C | -0.60% | -0.38% | -0.49% | 127% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.87% | -0.63% | -0.61% | 125% | 100% |
| Class D | -0.39% | -0.13% | -0.16% | 125% | 101% |
| Class F | -1.11% | -0.43% | -0.52% | 115% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.03% | 0.46% | 0.03% | 131% | 99% |
| Class C | -0.90% | 0.54% | 0.22% | 135% | 101% |
| Class E | -0.09% | 1.93% | 0.97% | 111% | 101% |
| Overall | -0.75% | 0.85% | 0.33% | 127% | 100% |
| Class D | -0.58% | 1.33% | 0.58% | 133% | 102% |
| Class F | -3.16% | -1.57% | -1.46% | 123% | 100% |

## CE1-7.7.3b

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |
| Class A1 | -0.81% | -0.88% | -0.72% | 130% | 99% |  |
| Class A2 | -0.82% | -0.92% | -0.69% | 122% | 100% |  |
| Class B | -0.94% | -0.84% | -0.99% | 125% | 100% |  |
| Class C | -0.55% | -0.34% | -0.47% | 129% | 101% |  |
| Class E |  |  |  |  |  |  |
| Overall | -0.79% | -0.73% | -0.74% | 127% | 100% |  |
| Class D | -0.40% | -0.24% | -0.20% | 126% | 100% |  |
| Class F | -0.96% | -0.46% | -0.47% | 116% | 101% |  |
|  | Low delay B Main10 |  |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.82% | 0.66% | 0.43% | 126% | 100% |  |
| Class C | -0.82% | 0.75% | 0.24% | 139% | 103% |  |
| Class E | -0.08% | 0.95% | 0.97% | 112% | 102% |  |
| Overall | -0.63% | 0.77% | 0.50% | 126% | 101% |  |
| Class D | -0.54% | 0.86% | 0.11% | 136% | 103% |  |
| Class F |  |  |  |  |  |  |
| (*) – LBD results are estimated for 1 missing points, will be updated shortly |  |  |  |  |  |  |

## CE1-7.8a

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -0.58% | -0.33% | 110% | 99% |
| Class A2 | -0.61% | -0.66% | -0.46% | 111% | 99% |
| Class B | -0.59% | -0.79% | -0.83% | 111% | 99% |
| Class C | -0.26% | -0.09% | -0.23% | 111% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.44% | -0.53% | -0.50% | 111% | 99% |
| Class D | -0.27% | -0.09% | 0.06% | 110% | 99% |
| Class F | -0.28% | 0.07% | -0.01% | 107% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.41% | 0.68% | 0.52% | 117% | 99% |
| Class C | -0.40% | 0.84% | 0.72% | 117% | 101% |
| Class E | -0.16% | 1.81% | 0.93% | 109% | 101% |
| Overall | -0.34% | 1.01% | 0.69% | 115% | 100% |
| Class D | -0.46% | 1.71% | 1.39% | 117% | 100% |
| Class F | -0.52% | 0.39% | -0.05% | 111% | 99% |
| (*) LDB results are estimated for few missing points, will be updated shortly |  |  |  |  |  |

## CE1-7.8b

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -0.44% | -0.26% | 110% | 99% |
| Class A2 | -0.60% | -0.69% | -0.40% | 111% | 99% |
| Class B | -0.59% | -0.63% | -0.72% | 111% | 100% |
| Class C | -0.25% | -0.13% | -0.17% | 112% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.44% | -0.47% | -0.42% | 111% | 100% |
| Class D | -0.28% | 0.02% | 0.03% | 111% | 101% |
| Class F | -0.27% | 0.00% | 0.03% | 107% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.43% | 0.54% | 0.51% | 118% | 100% |
| Class C | -0.39% | 0.84% | 0.80% | 118% | 101% |
| Class E | -0.06% | 0.91% | 0.97% | 110% | 101% |
| Overall | -0.33% | 0.73% | 0.72% | 116% | 100% |
| Class D | -0.41% | 1.21% | 0.92% | 118% | 103% |
| Class F | -0.50% | 0.51% | 0.55% | 112% | 99% |

## CE1-7.8c

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | -0.59% | -0.25% | 109% | 99% |
| Class A2 | -0.63% | -0.48% | -0.29% | 111% | 99% |
| Class B | -0.58% | -0.51% | -0.75% | 111% | 99% |
| Class C | -0.20% | -0.03% | -0.10% | 111% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.43% | -0.40% | -0.38% | 111% | 99% |
| Class D | -0.20% | 0.05% | 0.17% | 110% | 98% |
| Class F | -0.23% | 0.04% | 0.02% | 107% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.39% | 0.71% | 0.60% | 117% | 99% |
| Class C | -0.37% | 0.90% | 0.72% | 117% | 101% |
| Class E | -0.08% | 1.09% | 1.07% | 109% | 100% |
| Overall | -0.31% | 0.87% | 0.76% | 115% | 100% |
| Class D | -0.44% | 1.10% | 0.86% | 117% | 102% |
| Class F | -0.43% | 0.02% | -0.29% | 112% | 99% |

# Complexity analysis

Detailed complexity analysis was conducted during previous study in CE and can be found in JVET-L0326. According to this analysis total amount of operations required per each sample is:

- Multiplications: 0,

- Additions: 20 + 4 of 1-bit additions for rounding,

- Shifts: 8,

- Checks: 6.

It can be further observed that 2 operations can be saved due to reusing results of forward Hadamard transform calculation in shared area of overlapping 2x2 blocks (Figure 3). To illustrate this let’s consider calculation of Hadamard transform for first 2x2 block:

x0x1x2x3=>t0t1t2t3, where t0=x0+x2+(x1+x3)t1=x0+x2-(x1+x3)t2=x0-x2+(x1-x3)t3=x0-x2-(x1-x3)

Then for second 2x2 block shifted on one pixel right:

x1x4x3x5=>t0t1t2t3, where t0=x1+x3+(x4+x5)t1=x1+x3-(x4+x5)t2=x1-x3+(x4-x5)t3=x1-x3-(x4-x5)

It can be seen that 2 components x1+x3 and x1-x3 are part of both transforms. Thus it can be shared between two overlapping blocks and 2 adding operations can be saved.

Figure 3 Sharing calculation results in overlapped area.

After taking into account shared calculation amount of operations required per each sample is:

- Multiplications: 0,

- Additions: 18 + 4 of 1-bit additions for rounding,

- Shifts: 8,

- Checks: 6.

| Test# | filter shape | Computational complexity (# of multiplications/ additions/shifts/checks*) per sample | Sequential operation to get one sample processed, e.g. S=A+B+C+D is 2 sequential additions | Operations before block processing | Precision of multiply | Parallel friendly (each sample can be filtered independently from other samples) | Number of clock cycles | memory requirement | How to derive filter coeffs | Min. and max. CU Size for applying filters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-2.1 | 3x3 | 0/18+4 l-bit add for round/8/6 | 5 add; 1 look-up table (14 bytes); 2 checks | Load 16*7bits per CU if the QP is switched to a different range | n/a | yes | 1 clock: @770MHz 16nm @450MHz 28nm 2 clocks: @770MHz 28nm | 70 bytes (16 7-bit values per qp group) | Precal-culated in LUT | min: 8x8 max: 16xN or Nx16 (N<128) |
| 1-2.3 | 3x3 | —”— | —”— | —”— | —”— | —”— | —”— | —”— | —”— | min: no restrictions max: intra: 64x64 inter: 16x64, 64x16 |
| 1-2.4 | 3x3 | —”— | —”— | —”— | —”— | —”— | —”— | 84 bytes (16 7-bit values per qp group) | —”— | min: 8x8 max: 16xN or Nx16 (N<128) |
| 1-7.8 | 3x3 | —”— | —”— | —”— | —”— | —”— | —”— | —”— | —”— | —”— |

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).