| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0482r1 |
| --- | --- |

| Title: | CE1 Related: Additional test results for CE1-7 tests |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy, Kevin Reuze, Marta Karczewicz 10160 Pacific Mesa Blvd San Diego, CA 92121 (USA) | Email: | dmytror@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document presents additional results for CE1-1.7 tests. An alternative, LUT-free implementation of the bilateral filter design of CE1-1.7 is proposed in this document. The CE1-1.7 method with proposed implementation (and a bug-fix) provides:

Constrains A: -0.38% BD-Rate for RA, -0.41% BD-Rate for LDB

Constrains C: -0.32% BD-Rate for RA, -0.39% BD-Rate for LDB

It is claimed, that reported results indicates that proposed parametrization (with higher accuracy approximation)

provides better bd-rate reduction then alternative BIF methods. It is also argued, that observed improvement to the BIF parametrization can be applicable for other BIF filtering methods, e.g. placed in the loop-filter chain.

# Introduction

Bilateral filter proposed tested in CE1-1.7 can be implemented either with LUT-based approach or with a LUT-free approach when weights are derived on the fly. Simulation results for the implementation with LUT are reported in the JVET-N0489. This document reports additional simulation results produced with LUT-free implementation and with a minor bug-fix introduced.

# Proposal

Test CE1-1.7 targeted more accurate PWL modeling of BIF parameters. It is proposed to parametrize weight function wi∆Ii  through a piece-wise linear model with 2 ranges, in comparison to the single range linear approximation employed by the proponent of CE1-1.

Similarly to the base design, CE1-1.1, bilateral filter is used as a post-reconstruction filter. To filter the intensity IC of the current sample (see Figure 1), the surrounding four-neighbors IA (above) IB (below) IL (left) IR (right) are used (further jointly referred as Ii). The filtered value IF is calculated as a weighted sum of the surrounding delta-intensity values as:

IF=IC+wAΔIA+wBΔIB+wLΔIL+wRΔIR+c1≫c2, 		1

|  | IA |  |
| --- | --- | --- |
| IL | IC | IR |
|  | IB |  |

Figure 1. Samples used in the weighted sum.

where the deltas are the difference between sample value Ii (sample at i position) against the center sample, ΔIi=Ii-IC etc. The delta-value dependent weights factors wi are determined by a piece-wise linear function W(ΔI), derived from parameters of bilateral filter, QP value and parameters of mode, as presented in JVET-L0406.

Side information (ROM) of 285 bytes of ROM data defines the weighting function W(ΔI):

shiftPWL[46] = [ 0, 1, 1, 2, 2, 2, 2, 3, 3, 3, 3, 3, 3, 3, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5 ];

scalePWL[2][2][46] =

[[250, 125, 83, 62, 50, 41, 35, 31, 27, 25, 22, 20, 19, 17, 18, 15, 14, 14, 13, 12, 12, 11, 10, 10, 10, 9,9, 9, 9, 9, 8, 7, 7, 7, 7, 7, 6, 6, 6, 6, 6,6,5,5,5,5],
[531, 278, 171, 139, 107, 86, 73, 69, 60, 53, 48, 43, 39, 36, 37, 35, 32, 30, 28, 26, 25, 24, 22, 21, 20, 19, 19, 18, 19, 18, 18, 17, 16, 16, 15, 15, 14, 14, 13, 13, 13, 12, 12, 12, 11, 11 ]],
[186, 93, 62, 46, 37, 31, 26, 23, 20, 18, 16, 15, 14, 13, 13, 11, 11, 10, 9, 9, 8, 8, 8, 7, 7, 7, 6, 6, 7, 6, 6, 5, 5, 5, 5, 5, 5, 4, 4, 4, 4, 4, 4, 4, 4, 4 },
[394, 206, 127, 103, 79, 64, 54, 51, 45, 39, 35, 32, 29, 27, 27, 25, 24, 22, 21, 19, 18, 17, 16, 16, 15, 14, 14, 13, 14, 13, 13, 13, 12, 12, 11, 11, 10, 10, 10, 9, 9, 9, 9, 8, 8, 8]]

offsetPWL0[2] = [2135, 1582]

offsetPWL1[2][46] =

[1884, 1884, 1968, 1884, 1934, 1968, 1992, 1885, 1912, 1934, 1953, 1968, 1981, 1992, 1845, 1885, 1898, 1911, 1923, 1934, 1943, 1952, 1960, 1967, 1974, 1980, 1986, 1991, 1823, 1845, 1865, 1885, 1891, 1897, 1905, 1911, 1917, 1923, 1929, 1933, 1939, 1943, 1947, 1951, 1955, 1959 ]

[1396, 1396, 1458, 1396, 1433, 1458, 1475, 1396, 1416, 1433, 1446, 1458, 1467, 1475, 1367, 1396, 1406, 1416, 1425, 1433, 1440, 1446, 1452, 1458, 1462, 1467, 1471, 1475, 1352, 1368, 1382, 1396, 1400, 1406, 1410, 1416, 1420, 1424, 1428, 1432, 1436, 1440, 1442, 1446, 1448, 1452 ]

5 BIF parameters are required for 1 block (48 bytes): scalePWL[2], offsetPWL[2], shiftPWL.

Filtering process:

Two implementation approaches are feasible for the proposed scheme.

First is LUT-based approach, where weights WΔI are tabulated for all non-zero weights and accessed by ΔI:

WΔI=sigmaDistTabWΔI. 					(2)

This approach may be beneficial for large block size processing due to reduction in number of operations or for memory non-constrained platforms.

Second is LUT-free approach fir weight derivation for given delta value ΔI:

T=1≪shiftPWL

indx=(ΔI<T) ? 1:0

n=ΔI<T? -ΔI : T-ΔI

k=scalePWLindx

m=offsetPWLindx

WΔI=max0, m-k*n .							(3)

This derivation is similar to the implementation of CE1-1, with addition of determining value of T, indx and n variables which adds 1 comparator, 1 shift and 1 add to complexity estimate of CE1-1.

In the current document, simulation results for LUT-based implementation is reported. Additional test results for proposed method with LUT-free weight derivation by Eq.2 (and a bug-fix) are reported in CE1-related document JVET-N0482.

### Hardware complexity

In this section, the complexity of the most critical component – the complexity of weight derivation and filtering process is assessed.

For LUT based implementation CE1-7 tests, reported in this document , the weight of the BIF is derived from the LUT as shown in Eq 2 and wi*ΔIi  from Eq. 1 is implemented as a multiplication of the two 16 bits number.

Taking into account parallel calculation of the Eq. 1 for neighbouring samples of the block, term wi*ΔIi is calculated only for the right and below samples, thus total number of operations for LUT-based implementation is assessed as:

- 2 mults       Eq.1

- 13 adds      7 for non-local sum + 5 Eq.1

- 1 shift

- 1 comp

The memory requirements for LUT-based design are assessed as:

ROM: 					120 bytes

RAM: 					5449 words for LUT W’(ΔI, QP,sizeID)

For LUT-free approach, weights are derived by group of calculations in Eq.2.

For the Eq.2 for any n > 172, the resulting weight is equal to 0, thus variable n can be represented with 8 bits. With max scale value k is 531, the Eq.2 can be implemented with 8-bit by 10-bit multiplication and one subtraction. Maximum value of the product in Eq.2 is below 2394, which can be accommodated by a 12 bit register following by 1 comp. Additionally, 1 comparator, 1 shift and 1 add operations are needed to derive k, m, n parameters. Another 8-bit by 12-bit multiplication is needed for term wi*ΔIi in Eq 1.

Multiplications: 2         8-bit by 10-bit mult + one 8-bit by 12 bit mult

Additions: 23                  (Eq.2)

Shifts: 1                        (Eq. 2)

Checks: 2                      (Eq. 2)

Taking into account parallel calculation of the Eq.1 for neighbouring samples of the block, term wi*ΔIi is calculated only for the right and below samples, thus total number of operations for on-fly approach is assessed as:

- 4 mults

- 9 adds

- 3 shifts

- 4 comps

The memory requirements for LUT-free approach is assessed as:

ROM: 									285 bytes

Params at block level: 					48 bytes (5 params)

## Results

Simulation results presented in this section are produced for bilateral filtering method of CE1-1.7 implemented with LUT-free implementation approach.

### CE1-1.7.a with LUT-free implementation

Random Access results for CE1-1.7a. No SIMD code has been used, reported encoded time est. are inaccurate.

|  |  |  | Over VTM-4.0 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -0.28% | -0.04% | 107% | 116% |
| Class A2 | -0.42% | -0.24% | -0.08% | 100% | 102% |
| Class B | -0.49% | -0.08% | -0.04% | 88% | 99% |
| Class C | -0.35% | -0.03% | -0.08% | 90% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.38% | -0.14% | -0.06% | 95% | 104% |
| Class D | -0.37% | 0.07% | 0.04% | 90% | 98% |
| Class F | -0.16% | 0.08% | 0.01% | 100% | 86% |

Lowdelay B results for CE1-1.7 a. No SIMD code has been used, reported encoding time est. are inaccurate.

|  |  |  | Over VTM-4.0 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.41% | 0.43% | 0.37% | 115% | 105% |
| Class C | -0.45% | 0.71% | 0.39% | 103% | 97% |
| Class E | -0.38% | 0.58% | 0.71% | 85% | 97% |
| Overall | -0.41% | 0.56% | 0.46% | 103% | 100% |
| Class D | -0.60% | 1.33% | -0.05% | 100% | 94% |
| Class F | -0.31% | 0.14% | 0.77% | 101% | 84% |

### CE1-1.7.c with LUT-free implementation

Random access results for CE1-1.7c. No SIMD code has been used, reported encoding time est. are inaccurate.

|  |  |  | Over VTM-4.0 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.08% | 0.03% | 112% | 119% |
| Class A2 | -0.39% | -0.23% | -0.12% | 103% | 106% |
| Class B | -0.41% | -0.05% | -0.06% | 85% | 101% |
| Class C | -0.25% | 0.12% | 0.01% | 84% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.32% | -0.09% | -0.06% | 93% | 105% |
| Class D | -0.34% | 0.22% | 0.13% | 83% | 99% |
| Class F | -0.13% | 0.03% | -0.04% | 101% | 102% |

Lowdelay B results for CE1-1.7 c. No SIMD code has been used, reported encoded time est. are inaccurate.

|  |  |  | Over VTM-4.0 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.34% | 0.49% | 0.58% | 105% | 120% |
| Class C | -0.39% | 0.89% | 0.65% | 98% | 109% |
| Class E | -0.45% | 0.50% | -0.05% | 84% | 105% |
| Overall | -0.39% | 0.63% | 0.45% | 97% | 112% |
| Class D | -0.60% | 0.80% | 0.29% | 82% | 96% |
| Class F | -0.26% | 0.06% | 0.09% | 102% | 103% |

# Conclusion

Current document presents additional test results for CE1-1.7 tests produced with a LUT-free implementation. Reported coding gain of 0.38% of BD-rate reduction for RA and 0.41% of BD-rate reduction for LDB for constrains A and around 0.32%/0.39% of BD-rate gain for constrains B. It is proposed to include presented in this document results in the CE1 report for CE1-1.7 test.

Constrains A: -0.38% BD-Rate for RA, -0.41% BD-Rate for LDB

Constrains C: -0.32% BD-Rate for RA, -0.39% BD-Rate for LDB

# Acknowledgement

We would like to thank Ateme for cross-checking of the current proposal.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).