| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 | Document: JVET-N0513 |
| --- | --- |

| Title: | CE13: Experimental results of CNN-based In-Loop Filter (USTC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yuanying Dai, Dong Liu, Ning Yan, Feng Wu | Email: | daiyy@mail.ustc.edu.cn dongeliu@ustc.edu.n nyan@mail.ustc.edu.cn fengwu@ustc.edu.cn |
| Source: | University of Science and Technology of China |  |  |

_____________________________

- Abstract

This document presents the simulation results of a convolutional network for loop filter which was proposed in document JVET-M0510.

Introduction

The experiment settings were based on the document JVET-M1033 as shown in Table 1 and Table 2. VTM4.0 with CPU was used.

Table 1 The summary of different test set

| Test part | # | Description For SubCE1 | Description For SubCE2 |
| --- | --- | --- | --- |
|  | Anchor | DBF + SAO + ALF, no NN filter, use CTC QP to test | DBF + SAO + ALF, no NN filter, use CTC QP to test |
| Filter position | 1a(Default) | DBF + SAO + ALF+ NN filter | DBF + NN filter + SAO + ALF |
|  | 2a | NN filter. | NN filter. |
|  | 3a | N/A | NN filter + ALF. |
| CTU switch on/off | 1b(Default) | NN filter CTU switch | NN filter CTU switch |
|  | 2b | N/A | always use NN filter |
| Generalization (QP) | 1c(Default) | use CTC QP | use CTC QP |
|  | 2c | N/A | use CTC QP + 2 to test |
|  | 3c | N/A | use CTC QP - 2 to test |

Table 2 Test configuration

| Purpose | # | CTC test (Full test) |  |  | First intra period test (Short version) |
| --- | --- | --- | --- | --- | --- |
|  |  | All Intra | Random Access | Low delay | Random Access |
|  | anchor | M (For SubCE2) | M | O | O,S |
|  | 1a+1b+1c | M (For SubCE2) | M | O | O,S |
| Filter position | 2a+1b+1c | N/A | N/A | N/A | O,S |
|  | 3a+1b+1c | N/A | N/A | N/A | O,S |
| CTU switch | 1a+2b+1c | N/A | N/A | N/A | O,S |
| Generalization (QP) | 1a+1b+2c | N/A | N/A | N/A | O,S |
|  | 1a+1b+3c | N/A | N/A | N/A | O,S |

Test Results

- 2.1 DBF + NN filter + SAO + ALF

CNN filter follows the description in JVET-M0510 was located between Deblocking Filter and SAO. The filters in VTM4.0 including DF, SAO and ALF were all enabled. The results are shown in Table 3.

Table 3 Experimental Results of 1a+1b+1c

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.20% | -0.20% | 111% | 1176% |
| Class A2 | -0.66% | -0.22% | -0.28% | 107% | 2026% |
| Class B | -0.58% | -0.02% | -0.02% | 103% | 1681% |
| Class C | -1.63% | -1.61% | -2.12% | 106% | 3480% |
| Class E | -1.32% | -0.02% | -0.02% | 105% | 1636% |
| Overall | -0.87% | -0.44% | -0.56% | 106% | 1912% |
| Class D | -2.15% | -1.44% | -1.72% | 123% | 4191% |
| Class F | -1.00% | -0.53% | -0.57% | 110% | 816% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | 0.00% | -0.06% | 125% | 315% |
| Class A2 | -0.63% | -0.11% | -0.10% | 121% | 517% |
| Class B | -0.43% | 0.01% | -0.10% | 131% | 419% |
| Class C | -0.76% | -0.80% | -0.98% | 119% | 670% |
| Class E |  |  |  |  |  |
| Overall | -0.49% | -0.23% | -0.33% | 124% | 468% |
| Class D | -0.79% | -0.91% | -0.99% | 109% | 610% |
| Class F | -0.35% | -0.33% | -0.31% | 120% | 222% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.19% | -0.18% | -0.24% | 102% | 223% |
| Class C | -0.59% | -0.24% | -0.46% | 112% | 639% |
| Class E | -0.60% | 0.20% | -0.60% | 109% | 122% |
| Overall | -0.43% | -0.10% | -0.41% | 107% | 272% |
| Class D | -0.89% | -1.08% | -1.81% | 102% | 666% |
| Class F | -0.16% | -0.07% | -0.24% | 101% | 124% |

- 2.2 First intra period test

In the following tests, first intra period in video were tested.

- 2.2.1 DBF + NN filter + SAO + ALF

CNN filter was located between Deblocking Filter and SAO. The filters in VTM4.0 including DF, SAO and ALF were all enabled. The results are shown in Table 4.

Table 4 Experimental Results of 1a+1b+1c (first intra period)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.09% | -0.09% | 90% | 182% |
| Class A2 | -0.74% | -0.25% | -0.01% | 84% | 510% |
| Class B | -0.40% | 0.06% | -0.10% | 92% | 245% |
| Class C | -0.89% | -0.68% | -1.41% | 91% | 669% |
| Class E |  |  |  |  |  |
| Overall | -0.53% | -0.23% | -0.43% | 90% | 349% |
| Class D | -0.92% | -0.82% | -1.72% | 111% | 546% |
| Class F | -0.29% | -0.15% | -0.15% | 111% | 191% |

- 2.2.2 Only NN filter

The default filters in VTM were all disabled. The results are shown in Table 5.

Table 5 Experimental Results of 2a+1b+1c

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 9.52% | 10.08% | 9.57% | 98% | 397% |
| Class A2 | 6.51% | 5.85% | 4.26% | 87% | 1118% |
| Class B | 5.64% | 7.38% | 7.44% | 98% | 381% |
| Class C | 2.42% | 1.68% | 3.09% | 96% | 959% |
| Class E |  |  |  |  |  |
| Overall | 5.73% | 6.09% | 6.07% | 95% | 610% |
| Class D | 1.87% | -0.50% | -0.68% | 123% | 696% |
| Class F | 1.44% | 0.96% | 0.42% | 117% | 244% |

- 2.2.3 NN filter+ALF

The default filters in VTM were disabled except ALF and NN filter was located before ALF. The results are shown in Table 6.

Table 6 Experimental Results of 3a+1b+1c

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 2.64% | 4.22% | 5.17% | 81% | 264% |
| Class A2 | 1.00% | 3.63% | 2.73% | 82% | 791% |
| Class B | 1.15% | 2.51% | 2.99% | 98% | 350% |
| Class C | 0.14% | 0.41% | 0.28% | 94% | 876% |
| Class E |  |  |  |  |  |
| Overall | 1.14% | 2.52% | 2.65% | 90% | 497% |
| Class D | -0.40% | 0.03% | -0.31% | 120% | 598% |
| Class F | 0.40% | 1.52% | 1.90% | 119% | 222% |

- 2.2.3 NO CTU switch

The filtering was based on the whole frame instead of the CTU-level. CNN filter was located between Deblocking Filter and SAO. The filters in VTM4.0 including DF, SAO and ALF are all enabled. The results are shown in Table 7.

Table 7 Experimental Results of 1a+2b+1c

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 8.18% | 42.71% | 38.42% | 100% | 17803% |
| Class A2 | 9.09% | 41.52% | 27.37% | 93% | 15965% |
| Class B | 12.18% | 38.42% | 48.10% | 108% | 18572% |
| Class C | 9.12% | 15.85% | 17.60% | 108% | 18121% |
| Class E |  |  |  |  |  |
| Overall | 9.94% | 33.88% | 33.89% | 103% | 17750% |
| Class D | 10.56% | 19.03% | 21.29% | 148% | 15509% |
| Class F | 27.87% | 36.70% | 40.99% | 124% | 23421% |

- 2.2.4 CTC QP+2 for testing

The NN filter were trained based on CTC QP. To demonstrate the generalization capability, different QP were used to test. Table 8 shows the results that test QP is larger than the CTC QP.

Table 8 Experimental Results of 1a+1b+2c

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.05% | -0.09% | 104% | 196% |
| Class A2 | -0.75% | -0.03% | -0.27% | 93% | 579% |
| Class B | -0.45% | 0.07% | 0.05% | 111% | 282% |
| Class C | -0.76% | -1.19% | -1.60% | 83% | 646% |
| Class E |  |  |  |  |  |
| Overall | -0.52% | -0.31% | -0.48% | 98% | 378% |
| Class D | -0.77% | -0.78% | -2.03% | 120% | 604% |
| Class F | -0.33% | -0.42% | -0.20% | 115% | 223% |

- 2.2.5 CTC QP-2 for testing

Table 9 shows the results that test QP is smaller than the CTC QP.

Table 9 Experimental Results of 1a+1b+3c

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.12% | -0.17% | 123% | 239% |
| Class A2 | -0.64% | -0.09% | -0.08% | 116% | 708% |
| Class B | -0.33% | 0.10% | 0.27% | 117% | 311% |
| Class C | -0.78% | -1.19% | -1.65% | 101% | 642% |
| Class E |  |  |  |  |  |
| Overall | -0.46% | -0.33% | -0.40% | 113% | 422% |
| Class D | -0.61% | -0.76% | -1.48% | 104% | 494% |
| Class F | -0.27% | -0.34% | -0.27% | 110% | 186% |

Conclusion

From the above results, it can be seen that the result with all filters enabled is the best, and the result without CTU switch is the worst. Even if the QPs of training and testing were different, the NN filter can achieve performance improvement.

Patent rights declaration(s)

University of Science and Technology of China may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).