| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0533-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-N0189 (Non-CE7: Unified last position coding for 32-point transforms) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xin Zhao | Tel: Email: | xinzzhao@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of JVET-N0189 which is related to a unification of the last position coding for 32-point transform. The software implementation as provided by the proponent of JVET-N0189 has been studied and confirmed to be aligned with the proposal description. Available simulation results as provided in this contribution are reportedly matching with the results provided by proponent of JVET-N0189.

# Crosscheck report

In current VTM-4.0 and VVC draft, for 32-point transform using DST-7 or DCT-8 basis as used in Multiple Transform Selection (MTS) and Sub-Block Transform (SBT), the binarizations for last position in range of 12 to 15 were modified accordingly to meet the valid block size of 16 instead of original 32. In JVET-N0189, it was proposed to keep the last position independent from the transform selection.

The SW implementation was provided by the proponent of JVET-N0189 and simulations are performed under common test condition (CTC) [1] and Low QP configuration as defined in CE7 description [2]. Below figure summarizes the results, the anchor used for the simulations is VTM-4.0.

Table 1: Coding performance of JVET-N0189 versus VTM-4.0 under CTC.

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | 0.00% | 100% | 97% | 0.00% | -0.04% | 0.00% | 101% | 101% |  |  |  |  |  |
| Class A2 | 0.01% | 0.15% | 0.03% | 101% | 100% | -0.01% | 0.09% | 0.04% | 100% | 100% |  |  |  |  |  |
| Class B | 0.02% | 0.04% | 0.04% | 100% | 99% | 0.02% | 0.03% | -0.06% | 100% | 99% | 0.00% | -0.14% | 0.07% | 101% | 103% |
| Class C | 0.01% | -0.04% | 0.01% | 100% | 102% | 0.02% | -0.11% | -0.13% | 101% | 101% | -0.02% | 0.35% | 0.03% | 99% | 99% |
| Class E | 0.04% | 0.02% | -0.05% | 102% | 101% |  |  |  |  |  | -0.01% | 0.42% | 0.30% | 102% | 103% |
| Overall | 0.02% | 0.03% | 0.01% | 100% | 100% | 0.01% | -0.01% | -0.05% | 101% | 100% | -0.01% | 0.17% | 0.11% | 100% | 102% |
| Class D | 0.01% | -0.09% | -0.04% | 101% | 103% | 0.01% | -0.08% | 0.15% | 102% | 103% | 0.02% | 0.05% | -0.22% | 101% | 102% |
| Class F | 0.00% | -0.01% | 0.00% | 101% | 100% | -0.01% | 0.00% | 0.08% | 102% | 102% | -0.05% | -0.33% | 0.86% | 100% | 102% |

Table 2: Coding performance of JVET-N0189 versus VTM-4.0 under low QP test condition.

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.03% | -0.01% | 103% | 102% | 0.01% | 0.00% | 0.02% | 99% | 99% |  |  |  |  |  |
| Class A2 | 0.00% | 0.01% | 0.02% | 99% | 100% |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.02% | 0.00% | 101% | 102% | 0.00% | 0.01% | 0.02% | 101% | 101% | 0.00% | 0.02% | 0.01% | 100% | 100% |
| Class C | 0.00% | 0.00% | -0.01% | 98% | 102% | 0.00% | -0.01% | 0.02% | 100% | 98% | -0.01% | 0.04% | 0.01% | 101% | 101% |
| Class E | 0.01% | 0.01% | 0.00% | 100% | 98% |  |  |  |  |  | 0.02% | -0.09% | -0.02% | 100% | 98% |
| Overall | 0.00% | 0.01% | 0.00% | 100% | 101% |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | -0.01% | 0.03% | 97% | 97% | 0.00% | 0.02% | -0.05% | 100% | 102% | -0.02% | -0.05% | -0.02% | 103% | 101% |
| Class F | 0.01% | 0.02% | -0.02% | 100% | 100% | 0.00% | 0.01% | 0.02% | 101% | 100% | 0.19% | 0.21% | 0.19% | 99% | 100% |

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, 13th meeting, Marrakesh, MA, 9–18 Jan. 2019.

- H. Schwarz, M. Coban, C. Auyeung, “Description of Core Experiment 7 (CE 7): Quantization and coefficient coding,” JVET-M1027, 13th meeting, Marrakesh, MA, 9–18 Jan. 2019.