| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0562-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-N0433 (CE3-related: Unification of MPM derivation for luma intra modes) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | M.G.Sarwer No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 mohammed.sarwer@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The document reports the results of crosschecking JVET-N0433 proposed by Qualcomm. JVET-N0433 proposes a combined MPM list derivation for luma intra mode coding. A set of six MPM candidates and two replacement modes are derived with starting point as the MPM list in VTM-4.0 for blocks predicted from reference line with index equal to 0. The verification task was done, and the BD-rate results match exactly with those in JVET-N0433.

# Verification

Qualcomm provided the source code of their proposed method described in JVET-N0433 [1]. Table 1 of JVET-N0433 is cross-checked. One verification was carried out by setting the following parameters in the provided code:

#define ALL_MPM_RD 0

#define DISABLE_MRL_MPM 1

#define DISABLE_ISP_MPM 1

#define MPM_UNIF 1

The test is conducted under the the common test conditions (CTC) defined in [2] and are carried out by using the VTM4.0 software [3]. Table 1 shows the results using VTM4.0 as the anchor and enabling the proposed method in the test. The BD-rates of the test from our experimental results match exactly those in JVET-N0433. Please note that the runtime comparison may be not accurate.

Table 1. The cross-check results of JVET-N0433 using VTM4.0 as anchor

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.07% | -0.08% | 100% | 98% |
| Class A2 | -0.09% | 0.09% | -0.07% | 100% | 98% |
| Class B | -0.10% | -0.07% | 0.02% | 100% | 105% |
| Class C | -0.03% | -0.12% | -0.02% | 101% | 106% |
| Class E | -0.16% | 0.07% | -0.07% | 100% | 104% |
| Overall | -0.11% | -0.03% | -0.03% | 100% | 103% |
| Class D | -0.09% | -0.14% | -0.19% | 98% | 104% |
| Class F | 0.12% | 0.16% | 0.15% | 102% | 103% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.04% | -0.20% | 100% | 103% |
| Class A2 | -0.03% | 0.21% | 0.04% | 100% | 101% |
| Class B | -0.03% | 0.04% | -0.21% | 100% | 102% |
| Class C | -0.02% | -0.14% | -0.22% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | 0.02% | -0.16% | 100% | 102% |
| Class D | -0.01% | 0.04% | -0.17% | 100% | 106% |
| Class F | 0.05% | 0.05% | 0.09% | 100% | 104% |

# References

- A. K. Ramasubramonian, G. Van der Auwera, L. Pham Van, M. Karczewicz, “CE3-related: Unification of MPM derivation for luma intra modes”, JVET-N0433

- F. Bossen, J. Boyce, K. Sühring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

- VTM4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).