| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0694-v2 |
| --- | --- |

| Title: | Crosscheck of JVET-N0524 (Non-CE3: Modified beta derivation in CCLM) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Kei Kawamura Kyohei Unno Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-70-4560-2752 yo-kidani@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This cross-check contribution reports that that the source code provided by SONY who proposes JVET-N0524 has been carefully analysed and has been found to be consistent with the description in JVET-N0524. In JVET-N0524, Modified beta derivation in CCLM is proposed and AI/RA conditions are tested. The cross checker could reproduce the results reported in JVET-N0524. The difference from measured encoder and decoder run time ratios to those reported in JVET-N0524 is acceptable.

Simulation Results

JVET-N0524 [1] has been implemented on top of VTM-4.0 [2]. The simulation was conducted under common test condition [3] . The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Table 1 and Table 2 show the cross-check results of JVET-N0524 in AI and RA respectively. The results were consistent with the results of JVET-N0158. The difference from measured encoder and decoder run time ratios to those reported in JVET-N0158 is acceptable.

Table 1 Summary of experimental results (Test 1)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | -0.51% | -0.41% | 100% | 100% |
| Class A2 | -0.12% | -0.34% | -0.33% | 100% | 101% |
| Class B | -0.08% | -0.45% | -0.51% | 100% | 100% |
| Class C | -0.04% | -0.42% | -0.26% | 101% | 100% |
| Class E | -0.01% | -0.24% | -0.39% | 100% | 101% |
| Overall | -0.11% | -0.40% | -0.39% | 100% | 100% |
| Class D | 0.01% | -0.35% | -0.34% | 100% | 101% |
| Class F | -0.04% | -0.13% | -0.13% | 100% | 100% |

Table 2 Summary of experimental results (Test 2)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Conclusion

In this contribution, the result of JVET-N0524 has been confirmed.

Reference

- M. Ikeda and T. Suzuki, “Non-CE3: Modified beta derivation in CCLM”, JVET-N0524, Geneva, CH, March 2019.

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 4 (VTM 4)”, JVET-M1002, Marrakech, MA, January 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Suehring, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, Marrakech, MA, January 2019.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).