| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0865-v1 |
| --- | --- |

| Title: | AHG17: Gradual Random Access |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Hendry Ye-Kui Wang Rickard Sjöberg Martin Pettersson Lulin Chen | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc., Huawei, Ericsson, MediaTek |  |  |

_____________________________

# Abstract

This document provides a proposed text related to gradual random access based on the discussions in HLS BoG. The proposal is a result from offline discussions at the Geneva meeting involving proponents of JVET-M0529, JVET-N0101, JVET-N0495, JVET-N0115 and JVET-N0072. The proposed text contains the following elements that the authors claim is a minimal set of specification text changes to enable a CVS to start with a gradual random access (GRA) picture that is not necessarily all Intra coded:

- One flag gra_enabled_flag in the SPS that specifies whether GRA pictures may be present or not

- One new VCL NAL unit type, GRA_NUT, that indicates a GRA picture

- One syntax element, recovery_poc_cnt, in the tile group header that is a copy of the recovery_poc_cnt syntax element from the recovery point SEI of HEVC.

- A process to start a CVS with a GRA picture similar to how a CVS is started with a CRA picture

# Introduction

The topic of gradual random access/ gradual decoder refresh high level syntax was discussed in HLS BoG. And following design goals were agreed as per JVEt-N0724-v5:

---

Suggested design goals:

- Clarifying use of recovery point indication, with syntax location other than in SEI

  - Should be applicable to use cases where GDR period is equal to 0

- Have GDR bitstreams without IRAP pictures not be non-conforming

  - A CVS could start with a picture corresponding to one of these GDR indicators

- No need to define conforming output during the recovery period, but should be conforming starting with the recovery point

- Minimum changes to the specification

Offline discussion to try to design a solution for these design goals, coordinated by S. Deshpande. (involving proponents of N0101, N0495, N0115, N0072)

----

The offline activity was conducted. The specification text in the attached document provides a starting point for enabling gradual random access functionality. It is asserted that the provided specification text satisfies most of the design goals.

# Conclusion

It is proposed to adopt the proposed text in the next version of VVC WD.

# Patent rights declaration(s)

Sharp Labs of America, Inc., Huawei, Ericsson, MediaTek, may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).