| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0003-v1 |
| --- | --- |

| Title: | AHG report: Test model software development (AHG3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Frank Bossen Xiang Li Karsten Suehring | Tel: Email: | frank@bossentech.com xlxiangli@tencent.com karsten.suehring@hhi.fraunhofer.de |
| Source: | AhG3 on Test model software development |  |  |

_____________________________

# Abstract

This report summarises the activities of the AhG3 on Test model software development that has taken place between the 15th and 16th JVET meetings.

# Introduction

The mandates given to the AHG are:

- Coordinate development of test model (VTM) software and associated configuration files.

- Produce documentation of software usage for distribution with the software.

- Discuss and make recommendations on the software development process.

- Propose improvements to the guideline document for developments of the test model software.

- Perform tests of VTM 6 behaviour relative to HEVC and VTM 5 using the VTM common test conditions.

- Coordinate with AHG on Draft text and test model algorithm description editing (AHG2) to identify any mismatches between software and text, and make further updates and cleanups to the software as appropriate.

- Coordinate with AHG6 for integration with 360lib software.

The software development continued on the GitLab server. VTM version 5.2 was tagged on Jul. 16., VTM 6.0 on Aug. 17, VTM 6.1 on Aug 28. VTM 6.2 is expected during the 16th JVET meeting.

For core experiments (CEs) the same development workflow was followed as for the last meeting.

# VTM software development

Development was continued on the GitLab server, which allows participants to register accounts and use a distributed development workflow based on git.

The server is located at:

https://vcgit.hhi.fraunhofer.de

The registration and development workflow is documented at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/VVC-Software-Development-Workflow

The VTM software can be found at

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/

VTM 5.2 was tagged on Jul. 16, 2019.

Changes related to VTM 5.1 include:

- Removal of JVET-N macros

After one release candidates, VTM 6.0 was tagged on Aug 17, 2019.

Changes related to VTM 5.2 include:

- JVET-O0164/JVET-O0587: remove spatial AMVP candidate scaling

- JVET-O0267: use the inter quant matrices for IBC mode

- JVET-O0429: CRS lambda rounding fix

- JEVT-O1109: Unified chroma residue scaling

- JVET-O0590: Reduce the DMVR cost of the original MV

- JVET-O0162/O0331/O0480/O0574: IBC mvp flag conditioned on MaxNumMergeCand>1

- JVET-O0425/O0427/O0669: remove prediction in ALF coefficients coding

- JVET-O0047/O0058/O0064/O0067/O0290/O0301/O0430: simplificed ALF clipping parameter coding

- JVET-O0280: SIMD implementation for weighted sample prediction process of triangle prediction mode

- JVET-O0428: LMCS cleanups

- JVET-O0288: remove slice type dependency in ALF

- JVET-O0364 part 4: align PDPC process for DC with the one for Planar

- JVET-O0247: not signal APS index when number APS is 2

- JVET-O0529/JVET-O0540: Harmonization of LFNST, MIP and implicit MTS

- JVET-O0541: Decouple the intra implicit transform selection from an inter MTS related SPS flag

- JVET-O0364 Part 5: simplify PDPC process for angular modes

- JVET-O0055: integer-distance DMVR cost to disable BDOF

- JVET-O0655: modify chroma intra angle mapping table for 4:2:2 format

- JVET-O1124: CCLM restriction to reduce luma-chroma latency for chroma separate tree

- JVET-O0637 Choose line0 and line3 for gradient computation when chroma is same size as luma

- JVET-O0294: Context modelling for MTS index

- JVET-O0078: Single HMVP table for all CUs inside the shared merge list region for IBC

- JVET-O0216/O0302/O0648: using 3rd order exponential-Golomb codes for ALF coeff

- JVET-O0272: CE2-related: Simplified inverse luma mapping

- JVET-O0277: DCT-IF interpolation filter is always used for 4x4, 4x8, and 8x4 luma CB

- JVET-O0126: align BPWA index coding with specification

- JVET-O0052: Method-1 TU-level context coded bin constraint

- JVET-O0256: Non-CE: Fast encoder with adjusted threshold in dependent quantization

- JVET-O0094: Zeroing of primary only coefficients for LFNST

- JVET-O0438: affine AMVR control flag conditioned on affine control flag in SPS

- JVET-O0284: condition sym_mvd_flag on mvd_l1_zero_flag

- JVET-O0592: on encoder ME improvement

- JVET-O0108: Disable DMVR and BDOF for CIIP

- JVET-O0315/O0200/O0205/O0296/O0342/O0463/O0542: Intra prediction mode alignment for BDPCM

- JVET-O1140: Add slice level BDOF DMVR disable flag

- JVET-O0065: CABAC Initialization

- JVET-N0288 proposal 1: infer num_tiles_per_tile_group to 1, if rect_tile_group_flag is 1 and the single_tile_per_group_flag is 0

- JVET-O0105 with JVET-O0543: joint chroma coding extension

- JVET-O0596: align CBF signaling with specification

- JVET-O0258: Remove Chroma IBC when dualtree is used

- JVET-O0414: long-term reference picture restriction for SMVD

- JVET-O0297: Simplification for DMVR padding process

- JVET-O0500: Different ctx models for inter affine flag and subblock merge flag

- JVET-O0106: 4xN prediction regions for 1xN and 2xN ISP subblocks.

- JVET-O0368/O0292/O0521/O0466: disabling LFNST for non-DCT2 MTS candidates normatively

- JVET-O0163/JVET-O0588: Remove switching between L0 and L1 for temporal MV

- JVET-O0193/O0375: remove transform depth in cbf ctx

- JVET-O1168: cu chroma QP offset

- JVET-O0366: BCW index derivation for affine constructed candidates

- JVET-O0594: BDOF reference sample padding using the nearest integer sample position

- JVET-O0426: MRL reference samples for DC intra mode prediction

- JVET-O0219: LFNST transform set selection for a CCLM coded block

- JVET-O0619/O0623 : Single pass coding of abs_level_gtx_flag[x] for TS residual coding

- JVET-O1136: Unified syntax for JVET-O0165/O0200/O0783 on TS and BDPCM signalling

- JVET-O0432: LMCS encoder improvement

- JVET-O0567: Constrain the signalled MVD value to the range of [-2^17, 2^17-1]

- JVET-O0122: Sign context, level mapping of TS residual coding

- JVET-O0090 test 2: CTB selection of ALF alternative chroma filters

- JVET-O0616:Various chroma format support in VVC

- JVET-O0265: Simplified motion field storage for TPM

- JVET-O0472: LFNST index signalling depends on the position of last significant coefficient

- JVET-O0046: Move delta-QP earlier for 64x64 VPDU processing

- JVET-O0428: (HLS part) Number of bits of referencing LMCS APS IDs in SH from 5 to 2

- JVET-O0159: 10bitTcTable Deblocking

- JVET-O0061: Halfpel MV threshold deblocking

- JVET-O0379: Speedup mode decision process for triangle prediction mode

- JVET-O0364 Part 2: clean up padding process in intra prediction

- JVET-O0409: exclude coded_subblock_flag from counting context-coded bins in transform skip

- JVET-O0220 method-1: Parse merge_subblock_flag conditioned on MaxNumSubblockMergeCand

- JVET-O0681: disable BCW for CIIP, method 2 inherit BCW index

- JVET-O0432: update LMCS encoder parameters in nonCTC cfg files

- JVET-O0249: merge syntax change

- JVET-O0057: Switchable interpolation filter

- JVET-O0263/O0220-method 2: Syntax cleanup on subblock merge

- JVET-O0060: 4x4 deblocking

- JVET-O0244: weighted prediction in SPS and delta POC for reference pictures

- JVET-O0213: Block size restriction of LFNST to maximum transform size

- JVET-O1153: intra chroma mode coding simplification

- JVET-O0545: Configurable maximum transform size

- JVET-O1170: IBC virtual buffer

- JVET-O0617: Significant flag context reduction of transform coefficients

- JVET-O0634: BDOF applied CU size align with DMVR

- JVET-O0538: Add SPS control for ISP and SBT transform

- JVET-O0570/JVET-O0211: SMID friendly spatial gradient calculation in BDOF and PROF

- JVET-O0919: Minimum QP for Transform Skip Mode

- JVET-O0070: Prediction refinement with optical flow for affine mode (PROF)

- JVET-O0640: Picture size constraint

- JVET-O0650: signal chroma qp mapping table

- JVET-O0455: Separate maximum number of IBC merge candidates from regular merge mode

- JVET-O0304: Reduction of number of multiplications in BDOF

- JVET-O0376: add sps_joint_cbcr_enabled_flag

- JVET-O0925: simplifications of MIP

- JVET-O0526: Setting the minimum CTU size to 32x32

- JVET-O0050: avoid small intra prediction with a local dual-tree technique

- JVET-O0119: Base palette mode for 4:4:4

- JVET-O0282: 4:4:4 low QP bug fix (under the macro of JVET-O0119)

- JVET-O0610: Enable presence of access unit delimiters

- JVET-O0756: HDRTools VTM software integration

- Reducing max number of ALF APSs from 32 to 8

- CABAC retraining

- various bug fixes and cleanups

VTM 6.1 was tagged on Aug 28, 2019. Changes include:

- JVET-O0525: remove PCM mode

- JVET-O0491: remove tb() and tu() codes

- JVET-O1164: reference picture resampling

- bug fixes, clean ups, and speed ups

VTM 6.2 is expected to be tagged during the 16th JVET meeting. Changes include:

- JVET-N0100 proposal 1: allow signaling long-term ref picture POC LSB in ref picture list

- JVET-O0238 Parameters in PPS or slice headers (merge request 2)

- JVET-O0299: Support for user defined Scaling List Matrices in the APS

- JVET-N0865: GDR syntax implementation

- JVET-N0353: Independent parsing of Buffering Period and Timing Information SEIs

- JVET-O0041: Frame Field Information SEI

- JVET-N0865: GDR Non-Signalling Part

- JVET-O0428: add decoder check that all slices in one picture shall use same LMCS aps id

- JVET-O0428: check lmcs aps id in multiple slices of same picture

- JVET-O0178: conditionally signal sps_sub_layer_ordering_info_present_flag

- JVET-O0179: Nal unit syntax

- JVET-N0494: Dependent RAP indication SEI message

- JVET-O0147: Leading picture constraints

- JVET-O0756 hdr tools vtm software integration (merge request 2)

- JVET-O0179: Update SEIRemovalApp and StreamMergeApp with the new nal unit syntax

- JVET-O0176 Proposal 3: condition for rect_slice_flag when brick_splitting_present_flag is equal to 1

- JVET-O0152: BufferingPeriod SEI with GDR

- JVET-O0148: Constraint on number of active ref pics for P and B picture

- JVET-O0173/O0176/O0338 : Replace num_brick_rows_minus1 with num_brick_rows_minus2

- JVET-O0145: Signalling of entry points

- JVET-O0189: Implementation of decoding unit (JVET_O0189_DU)

- JVET-O0610: Detect presence of access unit delimiters

- JVET-O0181 : non_reference_picture_flag

- JVET-O0143 Removal of top_left_brick_idx and fixes for uniform tiles, JVET-O0236 Removal of a parsing dependency

- JVET-N0867: Temporal Scalability Support in BP SEI and PT SEI

- JVET-O0625/O654/O0662: Unified padding method in ALF

- JVET-O0177 Proposal 1 : Allow signaling of HRD parameters for temporal sub-layers to be optionally controlled by a flag

- JVET-O0245: The value of TemporalId of VPS and DPS NAL units shall be equal to 0

- JVET-O0044 Signalling zero or more Sub-Profiles

- JVET-O0452: Brick signaling condition on Tile Height

- JVET-O0241 : Decoding process for generating unavailable reference pictures

- JVET-O0245: add TemporalID constraint for NALs in AU, add APS content constraints for AU, other fixes

- various bug fixes and cleanups

# CTC Performance

The following table shows VTM 6.1 performance over HM 16.20:

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-16.20 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -28.12% | -33.44% | -33.64% | 1627% | 174% |
| Class A2 | -27.86% | -20.72% | -13.06% | 2625% | 183% |
| Class B | -21.10% | -19.86% | -27.27% | 2927% | 190% |
| Class C | -21.81% | -19.77% | -23.98% | 4098% | 204% |
| Class E | -25.16% | -21.29% | -25.60% | 2393% | 176% |
| Overall | -24.23% | -22.48% | -24.96% | 2716% | 187% |
| Class D | -17.70% | -14.41% | -15.71% | 4741% | 198% |
| Class F | -38.92% | -39.43% | -41.80% | 4338% | 186% |
|  |  |  | Random Access |  |  |
|  |  |  | Over HM-16.20 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -37.32% | -38.56% | -44.50% | 843% | 162% |
| Class A2 | -41.64% | -38.65% | -32.64% | 932% | 174% |
| Class B | -33.98% | -41.65% | -43.13% | 902% | 160% |
| Class C | -28.65% | -32.77% | -34.87% | 1144% | 174% |
| Class E |  |  |  |  |  |
| Overall | -34.76% | -38.06% | -39.10% | 954% | 167% |
| Class D | -26.40% | -29.45% | -30.18% | 1231% | 178% |
| Class F | -40.66% | -45.33% | -46.77% | 640% | 142% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over HM-16.20 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -26.95% | -30.69% | -31.68% | 861% | 150% |
| Class C | -22.91% | -23.65% | -25.50% | 1005% | 153% |
| Class E | -25.56% | -31.33% | -33.57% | 435% | 121% |
| Overall | -25.25% | -28.50% | -30.09% | 764% | 143% |
| Class D | -21.34% | -19.29% | -21.28% | 1024% | 169% |
| Class F | -37.72% | -41.80% | -43.27% | 526% | 116% |
|  |  |  | Low Delay P |  |  |
|  |  |  | Over HM-16.20 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -31.87% | -34.39% | -35.33% | 809% | 149% |
| Class C | -25.45% | -25.00% | -26.93% | 964% | 160% |
| Class E | -29.82% | -36.00% | -37.95% | 425% | 123% |
| Overall | -29.22% | -31.66% | -33.18% | 730% | 145% |
| Class D | -23.47% | -21.35% | -22.26% | 987% | 171% |
| Class F | -37.62% | -41.91% | -43.00% | 575% | 122% |

The following table shows VTM 6.0 performance compared to VTM 5.2:

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.2 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.91% | 9.60% | 4.82% | 73% | 99% |
| Class A2 | -3.68% | 7.65% | 7.61% | 75% | 100% |
| Class B | -0.81% | 6.57% | 7.39% | 77% | 101% |
| Class C | -0.69% | 2.95% | 4.11% | 82% | 102% |
| Class E | -0.74% | 3.55% | 5.32% | 84% | 98% |
| Overall | -1.43% | 5.95% | 5.92% | 78% | 100% |
| Class D | -0.69% | 4.75% | 6.19% | 82% | 106% |
| Class F | -1.38% | 0.54% | 0.84% | 91% | 102% |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-5.2 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.37% | -0.93% | -5.52% | 95% | 102% |
| Class A2 | -3.85% | -4.75% | -4.40% | 97% | 103% |
| Class B | -1.77% | -8.48% | -7.39% | 92% | 102% |
| Class C | -0.99% | -8.26% | -6.28% | 92% | 101% |
| Class E |  |  |  |  |  |
| Overall | -2.30% | -6.16% | -6.12% | 93% | 102% |
| Class D | -0.73% | -7.13% | -4.31% | 85% | 99% |
| Class F | -1.25% | -7.69% | -6.67% | 88% | 101% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-5.2 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.27% | -13.27% | -12.74% | 107% | 107% |
| Class C | -0.55% | -7.38% | -6.24% | 105% | 113% |
| Class E | -0.12% | -11.51% | -9.23% | 99% | 116% |
| Overall | -0.74% | -10.87% | -9.69% | 105% | 111% |
| Class D | -0.08% | -7.48% | -6.89% | 100% | 107% |
| Class F | -2.10% | -9.06% | -8.59% | 98% | 109% |
|  |  |  | Low Delay P |  |  |
|  |  |  | Over VTM-5.2 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.01% | -14.22% | -14.41% | 109% | 107% |
| Class C | -1.12% | -8.10% | -7.08% | 108% | 112% |
| Class E | -1.20% | -12.64% | -10.95% | 107% | 116% |
| Overall | -1.51% | -11.78% | -11.10% | 108% | 111% |
| Class D | -0.65% | -8.45% | -7.17% | 100% | 106% |
| Class F | -2.09% | -9.57% | -9.29% | 102% | 110% |

The following table shows VTM 6.1 performance compared to VTM 6.0:

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | 0.01% | 99% | 93% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 93% |
| Class B | 0.01% | 0.01% | 0.01% | 103% | 96% |
| Class C | 0.03% | 0.03% | 0.03% | 100% | 97% |
| Class E | 0.03% | 0.03% | 0.03% | 101% | 96% |
| Overall | 0.02% | 0.02% | 0.02% | 101% | 95% |
| Class D | 0.05% | 0.03% | 0.03% | 100% | 91% |
| Class F | 0.03% | 0.02% | 0.02% | 100% | 96% |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | 0.01% | 98% | 87% |
| Class A2 | 0.00% | -0.04% | 0.04% | 98% | 90% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 88% |
| Class C | 0.00% | -0.01% | 0.05% | 99% | 88% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.02% | 99% | 88% |
| Class D | 0.00% | 0.03% | 0.08% | 100% | 83% |
| Class F | 0.02% | 0.01% | 0.01% | 102% | 89% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.09% | -0.13% | 99% | 85% |
| Class C | 0.00% | 0.06% | -0.06% | 99% | 81% |
| Class E | 0.02% | 0.23% | 0.06% | 100% | 83% |
| Overall | 0.01% | 0.04% | -0.06% | 99% | 83% |
| Class D | -0.02% | 0.29% | -0.09% | 100% | 83% |
| Class F | 0.03% | 0.02% | -0.03% | 99% | 81% |
|  |  |  | Low Delay P |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.04% | 0.07% | 98% | 84% |
| Class C | 0.00% | 0.14% | 0.00% | 99% | 81% |
| Class E | 0.13% | 0.01% | 0.26% | 100% | 82% |
| Overall | 0.03% | 0.03% | 0.10% | 99% | 82% |
| Class D | 0.06% | -0.57% | -0.30% | 100% | 83% |
| Class F | -0.03% | -0.11% | 0.25% | 101% | 82% |

Full results are attached to this AHG report as Excel files.

# Issues encountered during the software implementation process

Several issues were encountered during software development:

  - Vacation time caused delays because people responsible for submitting software patches were not available.

  - The implementation of “JVET-O0050: avoid small intra prediction with a local dual-tree technique” was much larger than expected (~1000 lines of code changed) and the merge request was provided close to the VTM-6.0 deadline. It is generally a problem when large patches are provided late, because this gives software coordinators little time to review and test them and may thus cause delays in the release process.

  - It was discovered that “JVET-O0119: Base palette mode for 4:4:4” causes a significant increase in memory usage, even if not enabled (as in CTC). Coding tools should only have minimal impact on memory and coding time, when disabled. This needs to be fixed.

  - It is generally concerning that for many HLS implementations work seemed to start only close to the deadline or after the deadline. HLS implementations can start in parallel to the low-level implementations, because there are few conflicts to be expected.

  - Some software patches were submitted completely untested.

  - The quality of submitted SIMD code is generally not very high. A lot of SIMD code has been rewritten.

  - Configuration files were incorrect in VTM 6.0 release candidate 1 (incorrect naming of DBPCM parameter). This resulted in BDPCM not being enabled in the set (based on rc1) that was used to retrain initial CABAC states.

  - There was some confusion about how to configure QP settings in CTC for HDR PQ sequences following the adoption of JVET-O0650 (chroma QP mapping). In particular, the confusion arose from the fact that chroma offsets signaled in the PPS are now applied after QP mapping instead of before. This has a significant effect for PQ sequences, where custom chroma offsets are computed. The luma/chroma balance for PQ content may thus be significantly different in VTM 6 than in VTM 5. This should be addressed at the 16th meeting.

  - JVET-O0282 (low QP bug fix) was integrated under the macro of JVET-O0199 (base palette for 444), which led to confusion. Such practice should be avoided in the future.

  - An issue was identified regarding text and software integration of JVET-N0278, and the expectations of proponents, what would have been integrated. Draft 5 version 1 integrated the exact text of JVET-N0278, which defines an input filter for the decoder, that removes all layers, except for the target layer (LIdTarget). Software was provided implementing this filter into VTM. Later, in Draft 5 version 4 “fixes for JVET-N0278” were integrated that refer to layers in multiple locations of the specification, including definitions, reference picture list construction, and constraints on different NAL unit types. These changes are likely language from layered HEVC, but not required in the case, where a decoder would see only one specific value of nuh_layer_id. Proponents modified these editorially added constraints later on in the Gothenburg meeting. They expected the original constraints to be implemented in software. Also, for the implementation of scalable coding, proponents expected that the software would already contain some infrastructure for multi-layer coding, which does not exist.

# Proposals not implemented in VTM

At the beginning of the 16th meeting, the following implementations are still pending:

  - JVET-O0357 Maximum TU size for chroma format 4:2:2 and 4:4:4

  - JVET-O0235: Constraints on nal_unit_type, TemporalId, etc

  - JVET-O1143. Subpictures

  - JVET-O1159: Scalability

For the following implementations, issues remain:

  - JVET-O0147/JVET-O0226: A constraint was changed in the contributions to enable the use of more efficient field coding GOPs. An example field coding config file should be provided to guide users.

  - JVET-O0145/JVET-O0215: The number of entry points is derived instead of being signalled. For this the number of bricks needs to be known in the slice header. The software needs to be restructured to allow derivation of the slice/tile/brick related variables within the slice header parsing process. An initial version of the code has been merged, but disabled.

  - JVET-O0042: The syntax was included with JVET-O0041, but no configuration options exist for frame repetitions. Also, the decoder does not repeat frames.

# Software manual

Few merge requests included the required contributions to the software manual. Updates were only provided after being requested by the software coordinators.

Many parameters still have their outdated HEVC documentation and need to be updated. Other parameters for VVC tools are completely missing.

To make the software manual a valuable document, those missing parts need to be added.

# CE software

For each CE a group was created in GitLab and CE coordinators were given owner rights to the group. This way they could clone VTM as required, create branches for different tests and assign user access to the group themselves.

The CE development workflow is described at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/Core-experiment-development-workflow

CE read access is available using shared accounts: One account exists for MPEG members, which uses the usual MPEG account data (as announced on the appropriate email lists). A second account exists for VCEG members. The account information for VCEG members is available in the TIES system:

https://www.itu.int/ifa/t/2017/sg16/exchange/wp3/q06/vceg_account.txt

# Bug tracking

The bug tracker for VTM and specification text is located at:

https://jvet.hhi.fraunhofer.de/trac/vvc

The bug tracker uses the same accounts as the HM software bug tracker. Users may need to log in again due to the different sub-domain. For spam fighting reasons account registration is only possible at the HM software bug tracker at

https://hevc.hhi.fraunhofer.de/trac/hevc

Please file all issues related to the VVC reference software into the bug tracker. Try to provide all the details, which are necessary to reproduce the issue. Patches for solving issues and improving the software are always appreciated.

# Recommendations

The AHG recommends to:

- Continue to develop the VTM reference software

- Improve documentation, especially the software manual

- Resolve any normative issues resulting from the large number of integrations in the most recent development cycle

- Encourage people to test VTM software more extensively outside of common test conditions.

- Encourage people to report all (potential) bugs that they are finding.

- Encourage people to submit bit-streams/test cases that trigger bugs in VTM.

- Encourage people to submit non-normative changes that reduce encoder run time without significantly sacrificing compression performance

- Make sure that contributions considered for adoption in the future are subject to adequate text and software review by the JVET at large

- Identify which recent additions contribute to the increase of encoder run time for low delay configurations, in particular for low QP values