| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0016-v1 |
| --- | --- |

| Title: | JVET AHG report: Implementation studies (AHG16) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | M. Zhou, J. An, E. Chai, K. Choi, S. Sethuraman, T. Hsieh, X. Xiu | Email: | minhua.zhou@broadcom.com jicheng.ajc@alibaba-inc.com eric.chai@ublnx.com kiho14.choi@samsung.com sriram.sethuraman@ittiam.com thsieh@qti.qualcomm.com xiaoyuxiu@kwai.com |
| Source: | AHG16 |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG16: implementation studies, between the 15th JVET meeting in Gothenburg, SE (3-12 July 2019) and the 16th JVET meeting in Geneva, CH (1–11 October 2019).

The AHG was established with the following mandates:

- Study draft and proposed coding tools to identify implementation issues relating to decoder pipelines, decoder throughput, and other aspects of implementation difficulty.

- Solicit hardware analysis of complex tools.

- Provide feedback on potential solutions to address identified issues.

# Activities

Active email discussions were held on the main JVET email reflector (jvet@lists.rwth-aachen.de) with an [AHG16] indication on message headers. A summary of the AHG activities is provided as follows:

## Topics discussed on the email reflector

- VVC subpicture  support (also for AHG12/AHG17)

- Confirm understanding of the adoption and design philosophy.

- Provide feedback on the implementation implications.

- Motivate development of alternative approaches to reduce the implementation impact while still satisfying the general subpicture functionality.

- Design inconsistency in the de-blocking of the affine mode and TPM mode.

- In the affine mode, two 4x4 subblock edges can be treated differently even if they have the exactly same conditions (i.e. having no non-zero coefficients and the same amount of large enough MV differences that can trigger de-blocking along the edge).

- In the TPM mode, an inner TU edge with no non-zero coefficients can still be de-blocked if the edge is located along the diagonal TPM boundary and the triangle block MV difference is large enough to trigger de-blocking.

- 16-bit overflow issue in the newly adopted PROF (Prediction Refinement with Optical Flow)

- The adopted version of PROF does not include the clipping of the refinement offset to 14-bit that was in the original PROF proposal.

- The prediction samples after the PROF refinement can exceed 16-bit, which overflows the 16-bit multiplication logic in the weighted prediction.

- Tile column boundary processing

- It was proposed to disable the longer-tap de-blocking filtering along the vertical CTU boundary as well, to reduce the column buffer size for decoder implementations that run in the tile-scan order (instead of raster scan order).

- It was commented that such a change is not needed from the decoder point of view, as there is already an existing standard to support that requires 18-sample wide column buffers for in-loop filters. The VVC so far requires 15-sample wide column buffers.

- It was commented that the is not required from the encoder point of view, as there are multiple ways to achieve quality and cost trade-offs on the encoder side.

- ALF sample padding

- There two padding methods defined in the current draft, i.e. the repetitive padding and the ALF virtual boundaries (VB) mirrored padding.

- For the ALF filtering, the repetitive padding is applied to the picture boundaries and to the 360 video VBs that are not CTU boundary aligned.

- For the ALF filtering, the ALF VB mirrored padding is applied to the sub-picture/slice/tile/brick boundaries and to the 360 video VBs that are CTU boundary aligned.

- For the 4x4 based ALF classifications, the repetitive padding is applied along all the boundaries.

- It was asserted that the current design causes implementation difficulties.

- To simply the design and to improve the design consistency, it was advocated using the ALF VB mirrored padding along the ALF VBs only and the repetitive padding everywhere else, and considering the removal of the 360 VBs that are not CTU boundary aligned.

## Feedback provided on tools being tested in CEs

- RPR (Reference Picture Resampling)

  - Recommended modifying the down-sampling filter coefficients to avoid overflowing the 16-bit weighted prediction logic.

  - Recommended using the 6-tap filters for affine motion compensation.

- GEO (Geometric partitions)

  - The partial transform is sort of “generalized SBT”, populating the residual data from the received partial transform block (which also includes the residual padding to mitigate potential visual artefacts in de-blocking) to the coding unit creates an additional processing step in the inverse transform.

  - Using the sample weights to select motion vectors for the GEO motion context storage requires a decoder either to compute the sample weights twice or to carry the sample weights through multiple pipeline stages until they are consumed by the GEO MC.

  - The need of supporting up to 140 GEO partitioning types is likely a concern too (the current TPM has only two partitioning types).

- Using DMVR-refined MVs in de-blocking

  - Recommended to use the refined MVs along the both sides of a vertical CTU boundary. Otherwise, the un-refined MVs along the vertical CTU boundary would also need to be passed from the MV reconstruction block to the DMVR block.

- Combined bilateral and SAO filter

  - Recommended to use the repetitive padding (as opposed to the ALF VB mirrored padding) to simply the design.

- Cross-component ALF (CCALF)

  - Suggested to cut the filter coefficient precision from 11-bit to 8-bit, to reduce the number of multiplications, and to align the CCALF on/off granularity with the existing ALF design (i.e. CTU based on/off).

## Additional comments posted on the email reflector

- The newly adopted CU level dual tree that removes the 2x2/4x2/2x4 chroma intra prediction blocks has re-introduced 4x4 inter PUs back into the design. This should be fixed.

- It is desirable to lower the worst case number of context coded bins per residual from 2 to 1.75 in the transform skip residual coding. This change would improve the overall CABAC throughput and make the CABAC design more consistent. Currently the worst case number in the transform coefficients coding is 1.75 context coded bins per coefficient.

- It is worth taking a second look whether the shared merge is absolutely essential for the small block size IBC merge list derivation. The shared merge requires more conditional checks and more bookkeeping to keep up with the required throughput.

- The multiplication overhead of the CCALF being tested in the CE is about 50% of the current luma and chroma ALF design, while the expected gain is about 1% (on the top of existing ~5% ALF gain in RA).  It would be desirable to study whether a better trade-off exists outside the CE.

## Memory bandwidth study for VTM6.0

Broadcom conducted a memory bandwidth study by running both the VTM6.0 and VTM5.0 for CTC with a commercial motion compensation cache model integrated. The summary results of the random access configuration are provided in the Table 1 (for informational purposes).

For class A1 and A2 4K video both the average (see ABW_diff column) and the peak memory bandwidth consumption (see MBW_diff column) are slightly increased. Changing the DMVR reference block padding to 16x16 based and the adoption of PROF do not have a noticeable impact on the memory bandwidth consumption as expected.

|  | Random Access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |
|  | Y | U | V | TCM_diff | ABW_diff | MBW_diff | EncT | DecT |
| Class A1 | -3.36% | -0.91% | -5.52% | 0.38% | 1.29% | 1.29% | 93% | 112% |
| Class A2 | -3.81% | -4.62% | -4.39% | 0.88% | 1.82% | 1.60% | 94% | 113% |
| Class B | -1.74% | -8.44% | -7.61% | 0.43% | 1.18% | -3.30% | 95% | 117% |
| Class C | -0.97% | -8.36% | -6.25% | -0.25% | 0.46% | -0.71% | 91% | 115% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -2.27% | -6.15% | -6.19% | 0.36% | 1.19% | -0.28% | 93% | 115% |
| Class D | -0.43% | -6.82% | -3.99% | -0.71% | -0.09% | 1.49% | 92% | 127% |
| Class F (optional) | -1.10% | -7.65% | -6.59% | -1.27% | -0.16% | -0.31% | 89% | 125% |

Table 1. Memory bandwidth comparison (VTM6.0 vs. VTM5.0, RA)

Where

- TCM_diff : Total cache misses (over all the frames coded), percentage difference relative to VTM5.0.

- ABW_diff: Average memory bandwidth (over all the frames coded), percentage difference relative to VTM4.0.

- MBW_diff : Worst case (Max) memory bandwidth (among all the frames coded), percentage difference relative to VTM5.0.

MBW_diff is the most important measure, which shows the worst case memory bandwidth consumption difference of VTM6.0 relative to VTM5.0.

## Bin to bit ratio study for VTM6.0

It has been reported that the bin to bit ratio has decreased from VTM5.0 to VTM6.0, and returned to a similar level as VTM4.0, thanks to the reduction of the maximum number of context coded bins from 2 to 1.75 per coefficient in the transform coefficient coding. The summary results are provided in Table 2 and Table 3 for informational purposes (data from JVET-P0050).

| Bin2bit ratio (peak, weighted) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HM16.19 | VTM4.0 | VTM5.0 | VTM6.0 | diff (%, VMT4.0 vs. HM16.19) | diff (%, VMT5.0 vs. HM16.19) | diff (%, VMT6.0 vs. HM16.19) | diff* (%, VMT5.0 vs. VTM4.0) | diff* (%, VMT6.0 vs. VTM5.0) | diff* (%, VMT6.0 vs. VTM4.0) |
|  | QP = (22, 27, 32, 37), CTC |  |  |  |  |  |  |  |  |  |
| AI | 0.95 | 1.10 | 1.13 | 1.14 | 15.72% | 19.71% | 19.89% | 3.99% | 0.18% | 4.17% |
| RA | 0.93 | 1.09 | 1.12 | 1.13 | 17.39% | 20.88% | 21.70% | 3.49% | 0.82% | 4.31% |
| LDB | 0.96 | 1.09 | 1.13 | 1.13 | 13.34% | 16.99% | 17.14% | 3.64% | 0.16% | 3.80% |
| LDP | 0.96 | 1.09 | 1.13 | 1.13 | 13.24% | 17.30% | 17.21% | 4.06% | -0.09% | 3.97% |
|  | QP = (2, 7, 12, 17), 100 frames |  |  |  |  |  |  |  |  |  |
| AI | 0.79 | 0.94 | 0.95 | 0.93 | 18.11% | 20.24% | 17.71% | 2.12% | -2.53% | -0.40% |
| RA | 0.74 | 0.87 | 0.88 | 0.86 | 17.17% | 19.00% | 15.15% | 1.83% | -3.85% | -2.02% |
| LDB | 0.78 | 0.92 | 0.93 | 0.92 | 17.67% | 19.67% | 17.73% | 2.00% | -1.94% | 0.06% |
| LDP | 0.78 | 0.92 | 0.93 | 0.92 | 17.46% | 19.46% | 17.52% | 2.00% | -1.94% | 0.06% |
| Note*: Percentage difference is measured against HM16.19 |  |  |  |  |  |  |  |  |  |  |

Table 2. Bin to bit ratio in VTM4.0, VTM5.0 and VTM6.0 (weighted)

| Bin2bit ratio (peak, unweighted) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HM16.19 | VTM4.0 | VTM5.0 | VTM6.0 | diff (%, VMT4.0 vs. HM16.19) | diff (%, VMT5.0 vs. HM16.19) | diff (%, VMT6.0 vs. HM16.19) | diff* (%, VMT5.0 vs. VTM4.0) | diff* (%, VMT6.0 vs. VTM5.0) | diff* (%, VMT6.0 vs. VTM4.0) |
|  | QP = (22, 27, 32, 37), CTC |  |  |  |  |  |  |  |  |  |
| AI | 1.23 | 1.38 | 1.40 | 1.38 | 12.38% | 14.21% | 12.91% | 1.82% | -1.30% | 0.53% |
| RA | 1.21 | 1.38 | 1.40 | 1.39 | 13.75% | 15.25% | 14.36% | 1.50% | -0.89% | 0.61% |
| LDB | 1.24 | 1.37 | 1.39 | 1.38 | 11.25% | 12.88% | 11.57% | 1.63% | -1.30% | 0.32% |
| LDP | 1.23 | 1.37 | 1.39 | 1.38 | 11.08% | 13.02% | 11.49% | 1.94% | -1.53% | 0.42% |
|  | QP = (2, 7, 12, 17), 100 frames |  |  |  |  |  |  |  |  |  |
| AI | 1.19 | 1.31 | 1.33 | 1.31 | 10.45% | 11.40% | 9.85% | 0.95% | -1.55% | -0.60% |
| RA | 1.18 | 1.28 | 1.28 | 1.27 | 7.63% | 8.39% | 6.87% | 0.76% | -1.52% | -0.76% |
| LDB | 1.18 | 1.30 | 1.31 | 1.29 | 9.54% | 10.28% | 8.98% | 0.74% | -1.30% | -0.56% |
| LDP | 1.18 | 1.30 | 1.31 | 1.29 | 9.50% | 10.24% | 8.94% | 0.74% | -1.30% | -0.56% |
| Note*: Percentage difference is measured against HM16.19 |  |  |  |  |  |  |  |  |  |  |

Table 3. Bin to bit ratio in VTM4.0, VTM5.0 and VTM6.0 (un-weighted)

In Table 2 and Table 3, a bypass bin is counted as 0.25 context coded bins in the weighted measurements; and a bypass bin and a context coded bin carry an equal weight (1:1) in the unweighted measurements.

As far as low-QP AI configuration is concerned, the weighted and un-weighted bin to bit ratio of the VTM6.0 is roughly 18% and 10% higher than that of VTM16.19, respectively.

# Related contributions

The following contributions are identified for the AHG.

## Contributions related to de-blocking of the affine and TPM mode

- JVET-P0043, “AHG16/Non-CE5: A cleanup for de-blocking in the affine and TPM mode”, B. Heng, M. Zhou (Broadcom)

- JVET-P0086, “AHG16/Non-CE5: Deblocking boundary strength fix for Affine and TPM’, A. M. Kotra, S. Esenlik, H. Gao, B. Wang, E. Alshina (Huawei)

- JVET-P0087, “Non-CE5: Deblocking boundary strength modification for triangle and affine mode”, K. Misra, F. Bossen, A. Segall (Sharp Labs of America)

- JVET-P0262, " AHG16/Non-CE5: Deblocking boundary design cleanup for affine and TPM mode”, X.W. Meng (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU)

## Contributions aiming for addressing the 16-bit PROF overflow issue

- JVET-P0094, “AHG16/Non-CE4: Fix on overflow issue in PROF”, H. Chen, H. Yang (Huawei)

- JVET-P0154, “CE4-related: PROF prediction sample range reduction”, Y.-L. Hsiao, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)

- JVET-P0172, “AHG16/Non-CE4: Prediction sample value clipping for PROF”, J. Li, C. S. Lim (Panasonic)

- JVET-P0281, “Non-CE4: Corrections on parameter calculation for PROF and BDOF”, N. Park, J. Nam, H. Jang, J. Lim, S. Kim (LGE),

- JVET-P0413, “CE4-related: Clipping for PROF”, W. Chen, Y. He (InterDigital)

- JVET-P0518, “AHG16/Non-CE4: Addressing 16-bit multiplication overflow issue of the PROF”, X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)

## Contributions related to ALF boundary (sample) padding

- JVET-P0053, “AHG16/Non-CE5: A cleanup for the ALF sample padding”, M. Zhou (Broadcom)

- JVET-P0157, “CE5-related: Align ALF padding processes at picture and subpicture boundaries”, C.-Y. Lai,O. Chubach,L. Chen,C.-Y. Chen,T.-D. Chuang,C.-W. Hsu,Y.-W. Huang,S.-M. Lei (MediaTek)

- JVET-P0551, “AHG16/Non-CE5: On ALF boundary padding”, N. Hu, V. Seregin, M. Karczewicz (Qualcomm)

## Contribution that removes 4x4 inter PUs

- JVET-P0063, “AHG16: Fix on local dual tree”, Y. Zhao, H. Yang (Huawei), Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)

## Contributions related to the entropy coding

- JVET-P0050, “AHG16/Non-CE7: A study of bin to bit ratio for VTM6.0”, M. Zhou (Broadcom)

- JVET-P0585, “CE7-related: Alignment of number of context coded bins per coefficients for TS residual coding and transform coefficient coding”, M. Coban, M. Karczewicz (Qualcomm)

## Contributions advocating removal of the shared merge list

- JVET-P0175, “AHG16/Non-CE4: A clean-up to merge list generation by removing shared merge list”, J. Li, C.S. Lim (Panasonic)

- JVET-P0400, “AhG16/Non-CE8: Removal of shared merge list”, Y. Wang, L. Zhang, H. Liu, K. Zhang, J. Xu, Y. Wang (Bytedance)

## Contributions for CC-ALF complexity reduction

- JVET-P0106, “AHG16/CE5-Related: Simplifications for Cross Component Adaptive Loop Filter”, A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei)

- JVET-P0165, “CE5-related: Simplified CCALF”, O. Chubach, C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)

- JVET-P0173, “AHG16/Non-CE5: Cross component ALF simplification”, J. Li, C.S. Lim (Panasonic)

- JVET-P0178, “CE5-related: Unification of CCALF and ALF”, C.-W. Kuo, J. Li, C.S. Lim (Panasonic)

- JVET-P0251, “CE5-related: Simplified CCALF with 6 filter coefficients”, Y. Zhao, H. Yang (Huawei)

- JVET-P0290, “CE5-related: Clipping of intermediate value in CC-ALF”, Y. Yasugi, T. Ikai (Sharp)

- JVET-P0330, “Non-CE5: CC-ALF filtering simplification”, J. Taquet, P. Onno, C. Gisquet, G. Laroche (Canon)

- JVET-P0331, “Non-CE5: CC-ALF filtering simplification”, J. Taquet, P. Onno, C. Gisquet, G. Laroche (Canon)

- JVET-P0372, “CE5-related: Joint chroma cross-component adaptive loop filtering”, H. Yang, Y. He, H. Li (InterDigital)

- JVET-P0468, “CE5-Related: Reducing multiplier count in CC-ALF”, K. Misra, F. Bossen, A. Segall (Sharp Labs of America)

- JVET-P470, “Non-CE5: Simplification of Cross Component Adaptive Loop Filter”, X. Li, X. Zhao, L. Zhao, Y. Du, S. Liu (Tencent)

- JVET-P0543, “CE5-related: Cleanup on cross-component adaptive loop filter”, J. Nam, J. Choi, J. Lim, S. Kim (LGE)

- JVET-P0555, “CE5-related: Dynamic range reduction for coefficients of cross component adaptive loop filter”, N. Hu, J. Dong, V. Seregin, M. Karczewicz (Qualcomm)

- JVET-P0556, “CE5-related: Temporal buffer removal for cross component adaptive loop filter”, N. Hu, J. Dong, V. Seregin, M. Karczewicz (Qualcomm)

- JVET-P0557, “CE5-related: Multiplication removal for cross component adaptive loop filter”, N. Hu, J. Dong, V. Seregin, M. Karczewicz (Qualcomm)

- JVET-P0558, “CE5-related: Reduced filter shape for cross component adaptive loop filter”, N. Hu, J. Dong, V. Seregin, M. Karczewicz (Qualcomm)

## Other AHG16-related contributions

- JVET-P0242, “AHG12/AHG16/AHG17: Subpicture Support”, 	W. Wan, T. Hellman, M. Zhou, B. Heng, P. Chen (Broadcom)  (and see many others in AHG12/AHG17)

- JVET-P0307,  “AHG16: Early Implementation of VVC software player and Demonstration on Mobile device”,	 Jeeva Raj Arumugam, Sagar Kotecha, Shailesh Ramamurthy, Abhisek Chelawat, Jayasanker J, Anil Kumar Bedgujar, Neha Mary Thomas, Saurabh Agrawal, Vijayakumar GR, Kaustubh Patankar (Ittiam)

- JVET-P0419, “AHG16/AHG17: Simplification of CU Splitting Controls	“,T. Hellman, W. Wan, M. Zhou, B. Heng (Broadcom)

- JVET-P0561, “AHG16: Context restriction on CTU boundary for line buffer reduction”,	 T. H.-J. Jhu, Y.-W. Chen, X. Xiu, T.-C. Ma, X. Wang (Kwai Inc.)

- JVET-P0582, “AHG16: On dynamic internal bit depth”, Y. Du, X. Zhao, X. Li, L. Zhao, S. Liu (Tencent)

# Recommendations

The AHG recommends reviewing the input contributions.