| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0018-v1 |
| --- | --- |

| Title: | JVET AHG report: Lossless and near-lossless coding tools (AHG18) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | T. Nguyen and T.-C. Ma (co-chairs), M. Ikeda, S. Iwamura, H. Jang, X. Zhao (vice-chairs) | Email: | tung.nguyen@hhi.fraunhofer.de tsung-chuanma@kwai.com masaru.ikeda@sony.com iwamura.s-gc@nhk.or.jp hm.jang@lge.com xinzzhao@tencent.com |
| Source: | AHG18 on lossless and near-lossless coding tools |  |  |

_____________________________

# Abstract

This document reports the activity of AHG 18 on lossless and near-lossless coding tools between the 15th JVET meeting in Gothenburg and the 16th Meeting in Geneva.

# Introduction

The JVET established the AHG 18 at the Gothenburg meeting with the following mandates:

- Study coding tools for lossless and near-lossless coding, including transform skip, BDPCM, and other potential technologies

- Consider the interaction between coding tools and other processing such as loop filtering and LMCS for lossless and near-lossless coding

- Develop proposals for lossless and near-lossless coding for chroma and non-YCbCr colour space content

- Consider throughput bottlenecks for lossless and near-lossless coding at high resolutions and frame rates

# Activities

Discussions related to AHG18 used the JVET email reflector (jvet@lists.rwth-aachen.de), and the AHG chairs sent a kick-off message on 27th August 2019. In total, the participants exchanged 18 emails related to the AHG.

The AHG identified the following issues:

- Design of the lossless support in VVC

- Mixed lossy/lossless

- BDPCM not activated (due to implementation) for lossless

The AHG chairs provided a reference implementation that reused the existing cu_transquant_bypass syntax in the VTM-6.0 software. Note that the syntax is not part of the current VVC Draft. The AHG chairs also provided lossless results using the sequences specified for the CTC, generated with HM-16.20 as representative for HEVC Main/Main10 and HEVC RExt.

The results for HEVC RExt relative to HEVC Main/Main10 are as follows.

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM-16.20 | HM-16.20 RExt |  | HM-16.20 | HM-16.20 RExt |  |
| Class A1 | 2.1 | 2.2 | -5.25% | 2.2 | 2.3 | -4.07% |
| Class A2 | 1.7 | 1.8 | -5.87% | 1.8 | 1.9 | -4.51% |
| Class B | 2.0 | 2.1 | -4.84% | 2.2 | 2.3 | -2.00% |
| Class C | 2.0 | 2.1 | -5.34% | 2.5 | 2.6 | -1.93% |
| Class D | 1.8 | 2.0 | -7.45% | 2.6 | 2.7 | -2.64% |
| Class E | 2.7 | 3.0 | -8.19% |  |  |  |
| Class F | 4.5 | 5.2 | -11.93% | 30.3 | 35.5 | -9.73% |
| TGM | 6.2 | 8.2 | -23.29% | 81.3 | 106.3 | -20.34% |
| Overall | 2.1 | 2.2 | -5.75% | 2.2 | 2.3 | -2.89% |
| Enc Time[%] | 94% |  |  | 106% |  |  |
| Dec Time[%] | 94% |  |  | 94% |  |  |

The results for VVC VTM-6.0 relative to HEVC Main/Main10 are as follows.

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM-16.20 | VTM6 |  | HM-16.20 | VTM6 |  |
| Class A1 | 2.1 | 2.2 | -5.97% | 2.2 | 2.3 | -4.87% |
| Class A2 | 1.7 | 1.7 | -3.63% | 1.8 | 1.9 | -3.18% |
| Class B | 2.0 | 2.1 | -4.21% | 2.2 | 2.3 | -3.20% |
| Class C | 2.0 | 2.1 | -5.47% | 2.5 | 2.6 | -3.52% |
| Class D | 1.8 | 1.9 | -5.69% | 2.6 | 2.8 | -3.87% |
| Class E | 2.7 | 2.9 | -7.62% |  |  |  |
| Class F | 4.5 | 5.5 | -16.21% | 30.3 | 39.4 | -13.89% |
| TGM | 6.2 | 12.3 | -45.77% | 81.3 | 112.6 | -29.01% |
| Overall | 2.1 | 2.2 | -5.26% | 2.2 | 2.3 | -3.62% |
| Enc Time[%] | 5667% |  |  | 1613% |  |  |
| Dec Time[%] | 193% |  |  | 157% |  |  |

The results for VVC VTM-6.0 relative to HEVC RExt are as follows.

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM-16.20 RExt | VTM6 |  | HM-16.20 RExt | VTM6 |  |
| Class A1 | 2.2 | 2.2 | -0.75% | 2.3 | 2.3 | -0.83% |
| Class A2 | 1.8 | 1.7 | 2.41% | 1.9 | 1.9 | 1.44% |
| Class B | 2.1 | 2.1 | 0.71% | 2.3 | 2.3 | -1.21% |
| Class C | 2.1 | 2.1 | -0.13% | 2.6 | 2.6 | -1.62% |
| Class D | 2.0 | 1.9 | 1.95% | 2.7 | 2.8 | -1.25% |
| Class E | 3.0 | 2.9 | 0.62% |  |  |  |
| Class F | 5.2 | 5.5 | -5.07% | 35.5 | 39.4 | -4.80% |
| TGM | 8.2 | 12.3 | -29.76% | 106.3 | 112.6 | -10.17% |
| Overall | 2.2 | 2.2 | 0.55% | 2.3 | 2.3 | -0.71% |
| Enc Time[%] | 6022% |  |  | 1529% |  |  |
| Dec Time[%] | 205% |  |  | 167% |  |  |

# Related contributions

The AHG chairs also found out that several contributions related to the interaction between deblocking filter and other coding tools were input to this meeting. It is recommended that these contributions are reviewed in combination with CE5 and AHG16 discussions which also focus on deblocking filter process.

JVET-P0082: AHG18: BDPCM for Lossless

JVET-P0109: AHG18/Non-CE5: Boundary strength fix for coding units using BCW

JVET-P0147: AHG18: BDPCM in lossless coding

JVET-P0148: AHG18: Disabling dependent quantization in lossless coding

JVET-P0149: AHG18: Disabling LMCS for lossless coding

JVET-P0176: AHG18: Lossless coding support related to cross-component ALF

JVET-P0258: AHG18: Disabling Dependent Quantization for Lossless

JVET-P0269: AHG18 / non-CE5: Deblocking for TPM and BCW

JVET-P0327: AHG18: Configurable Maximum Transform Size for Mixed Lossy and Lossless Coding

JVET-P0463: AHG18: Residual coding method for lossless mode

JVET-P0466: AHG18: Residual coding for lossless video coding

JVET-P0467: AHG18: Rice parameter extension

JVET-P0477: AHG18: On Last Position Signaling for Lossless

JVET-P0485: AHG18: Performance of CE7-1.2d and CE7-2.2 on lossless coding

JVET-P0502: AHG18: Interaction between lossless coding and max transform size

JVET-P0514: AHG18: Enable lossless coding for VVC

JVET-P0521: AHG18: Disabling DMVR and BDOF for lossless coding

JVET-P0525: AHG18: Residual coding selection signaling for lossless VVC

JVET-P0528: AHG18: On residual scanning order for lossless coding

JVET-P0559: AHG18: Rice parameter derivation for coefficient level coding in lossless

JVET-P0574: AHG18: BDPCM residual coding for lossless coding

JVET-P0576: AHG18: on low level coding for lossless

JVET-P0584: AHG18: BDPCM for luma and chroma in lossless coding

JVET-P0586: [AHG18][Non-CE5] Proposed cleanup of deblocking filter process

JVET-P0602: [AHG18][non-CE5] Boundary strength derivation for CUs with TPM

JVET-P0606: AHG18: Enabling lossless coding with minimal impact on VVC design

JVET-P0607: AHG18: Two Stage Residual Coding for Lossless

JVET-P0612: AHG18: Non-Lossless Coding Tools in VVC

# Recommendations

The AHG recommends:

- To review all related contributions

- Solving the aspect on the design for lossless support

- To continue the investigation on lossless and near-lossless performance of VVC