| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0021-v2 |
| --- | --- |

| Title: | CE1: Summary report on reference picture resampling filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jiancong Luo Vadim Seregin Wade Wan | Email: | jiancong.luo@alibaba-inc.com vseregin@qti.qualcomm.com wade.wan@broadcom.com |
| Source: | CE1 coordinators |  |  |

_____________________________

# Abstract

This contribution provides a summary report of Core Experiment 1 on reference picture resampling filters.

All techniques as planned in the context of CE1 are implemented on top of and tested against VTM-6.1 with the affine patch. Simulation results and crosschecking reports of each test specified in this document are provided.

# Test specification

| Test # | Description | Source | Tester | Cross-checker |
| --- | --- | --- | --- | --- |
| CE1-1.1 | Use integer version of 8-tap Lanczos2 filter when reference picture is larger than current picture | JVET-O0240 | Jonatan Samuelsson samuelssonj@sharplabs.com | Peisong Chen peisong.chen@broadcom.com Krishna Rapaka krapaka@apple.com |
| CE1-1.2 | Use integer version of 8-tap Lanczos1 filter when reference picture is larger than current picture | JVET-O0240 | Jonatan Samuelsson samuelssonj@sharplabs.com | Peisong Chen peisong.chen@broadcom.com Krishna Rapaka krapaka@apple.com |
| CE1-1.3 | Use integer version of 8-tap Lanczos2.67 filter when reference picture is larger than current picture | JVET-O0240 | Jonatan Samuelsson samuelssonj@sharplabs.com | Peisong Chen peisong.chen@broadcom.com Krishna Rapaka krapaka@apple.com |
| CE1-2.1 | Use 9-tap downsampling when the reference picture is larger than the current picture, and extended SHVC filters for upsampling when it is smaller. Resampling and motion compensation are done in one stage as in VTM. | JVET-O0319 | Pankaj Topiwala pankajtva@gmail.com |  |
| CE1-2.2 | Use 9-tap downsampling when the reference picture is larger than the current picture, and extended SHVC filters for upsampling when it is smaller. Resampling is done separately from motion compensation. The tester clarified that this test is not conforming to the agreed CTC of CE1. It is agreed that this test is removed from CE1. | JVET-O0319 | Pankaj Topiwala pankajtva@gmail.com |  |
| CE1-3 | Use SHM filters for downsampling | JVET-O0641 | Mohammed Golam Sarwer m.sarwer@alibaba-inc.com | Vadim Seregin vseregin@qti.qualcomm.com |

# Simulation results

## Resampling ratio 1.5 : 1

|  | Low delay Main 10 PSNR1 |  |  |  |  | Low delay B Main10 PSNR2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1-1.1 | 1.12% | 1.00% | 1.19% | 100% | 102% | 0.97% | 0.76% | 0.97% | 100% | 102% |
| 1-1.2 | - | - | - | - | - | - | - | - | - | - |
| 1-1.3 | -2,23% | -1,23% | -1,06% | 100% | 102% | -1,85% | -1,68% | -1,34% | 100% | 102% |
| 1-2.1 | - | - | - | - | - | - | - | - | - | - |
| 1-3 | -3.13% | -2.50% | -2.28% | 99% | 101% | -2.23% | -2.21% | -1.87% | 99% | 101% |

## Resampling ratio 2 : 1

|  | Low delay Main 10 PSNR1 |  |  |  |  | Low delay B Main10 PSNR2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1-1.1 Lanczos1 | -4,45% | -2,70% | -2,47% | 100% | 100% | -2,89% | -1,79% | -1,72% | 100% | 100% |
| 1-1.2 Lanczos2 | 2.97% | 1.74% | 1.63% | #DIV/0! | #DIV/0! | 2.18% | 1.43% | 1.27% | #DIV/0! | 99% |
| 1-1.3 Lanczos2.67 | -2.80% | -1.95% | -1.83% | 99% | 99% | -1.77% | -1.28% | -1.25% | 99% | 99% |
| 1-2.1 | -2.05% | 1.17% | 0.77% | N/A | N/A | -0.25% | 1.39% | 1.02% | N/A | N/A |
| 1-3 SHM | -6.47% | -4.55% | -4.62% | 99% | 102% | -3.98% | -2.67% | -3.00% | 99% | 102% |

## Resampling ratio 4 : 1 (Optional)

|  | Low delay Main 10 PSNR1 |  |  |  |  | Low delay B Main10 PSNR2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1-1.1 Lanczos1 | -3.48% | -6.81% | -6.77% | 99% | 99% | -1.63% | -3.36% | -3.30% | 99% | 99% |
| 1-1.2 Lanczos2 | -9,47% | -9,10% | -9,16% | 99% | 99% | -4,57% | -4,74% | -4,76% | 99% | 99% |
| 1-1.3 Lanczos2.67 | - | - | - | - | - | - | - | - | - | - |
| 1-2.1 | - | - | - | - | - | - | - | - | - | - |
| 1-3 SHM | - | - | - | - | - | - | - | - | - | - |

# Crosscheck report

| Test# | Crosschecker | SW match CE description (Y/N) | Text match SW (Y/N) | Simulation results (Y/N) | EncT/DecT match (Y/N) |
| --- | --- | --- | --- | --- | --- |
| 1-1.1 | Peisong Chen | Y | Text match with SW except below aspect *** | Y | not evaluated because of a heterogeneous cluster |
| 1-1.2 | Krishna Rapaka | Y | Text match with SW except below aspect *** | Y | Run on a heterogenous cluster, so enc & dec times not reliable |
| 1-1.3 | Krishna Rapaka | Y | Text match with SW except below aspect ***( | Y | Run on a heterogenous cluster, so enc & dec times not reliable |
| 1-2.1 |  |  |  |  |  |
| 1-3 | Vadim Seregin | Y | Text is not provided | Y | Y |

SW match CE description: Y SW implementation match CE description, N mismatch.

Text match SW: Y text match SW, N mismatch.

Simulation results: Y substantial results (all but class A for RA, all but class B for LDB) obtained and match results from tester, N mismatch or no substantial results.

EncT/DecT match: Y encoder and decoder run time match those from tester, N mismatch.

***	The thresholds for which scaling ratio to use are signaled in the SPS in the SW code but the specification uses fixed thresholds (no signaling).

# Detailed description of tests

### CE1-1 Use integer versions of 8-tap Lanczos filters when reference picture is larger than current picture (JVET-O0240)

The contribution JVET-O0240 proposes two alternative sets of filter coefficients to be used for motion compensation interpolation filtering when the reference picture is of higher resolution than the current picture. The contribution suggests that the scaling factor between the current picture and the reference picture is used to determine which filter to use. The Lanczos2 filter has filtering characteristics adjusted for 2x scaling factor. The Lanczos1 filter has filtering characteristics adjusted for 4x scaling factor. In the Core Experiment the two proposed filters are tested individually for 2x scaling case and 4x scaling case. The Lanczos2 filter is also tested for 1.5x scaling case. The precision of the filter coefficients have been adjusted in the CE compared to the original proposal in order to align the filters with the existing design in the VVC draft. Additionally, a Laczoss2.67 filter (adjusted for 1.5x scaling factor) is tested for the 1.5x scaling case and the 2x scaling case. The filters used for downsampling in CE1-1 are applied for conventional inter prediction only, and not for the affine mode. The affine mode uses the existing 6-tap filters in the VVC draft also for the case when reference picture resampling is applied.

### CE1-1.1 Use integer version of 8-tap Lanczos2 filter when reference picture is larger than current picture

For luma, the following filter is used when the reference picture is larger than current picture:

| Fractional sample position p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p ][ 0 ] | fL[ p ][ 1 ] | fL[ p ][ 2 ] | fL[ p ][ 3 ] | fL[ p ][ 4 ] | fL[ p ][ 5 ] | fL[ p ][ 6 ] | fL[ p ][ 7 ] |
| 0 | −2 | 2 | 18 | 28 | 18 | 2 | −2 | 0 |
| 1 | −2 | 1 | 17 | 28 | 19 | 3 | −2 | 0 |
| 2 | −2 | 1 | 16 | 28 | 20 | 4 | −2 | −1 |
| 3 | −2 | 0 | 15 | 28 | 21 | 5 | −2 | −1 |
| 4 | −2 | 0 | 14 | 28 | 22 | 5 | −2 | −1 |
| 5 | −2 | −1 | 13 | 28 | 23 | 6 | −2 | −1 |
| 6 | −2 | −1 | 12 | 27 | 24 | 7 | −2 | −1 |
| 7 | −2 | −1 | 11 | 26 | 25 | 8 | −2 | −1 |
| 8 | −1 | −2 | 9 | 26 | 26 | 9 | −2 | −1 |
| 9 | −1 | −2 | 8 | 25 | 26 | 11 | −1 | −2 |
| 10 | −1 | −2 | 7 | 24 | 27 | 12 | −1 | −2 |
| 11 | −1 | −2 | 6 | 23 | 28 | 13 | −1 | −2 |
| 12 | −1 | −2 | 5 | 22 | 28 | 14 | 0 | −2 |
| 13 | −1 | −2 | 5 | 21 | 28 | 15 | 0 | −2 |
| 14 | −1 | −2 | 4 | 20 | 28 | 16 | 1 | −2 |
| 15 | 0 | −2 | 3 | 19 | 28 | 17 | 1 | −2 |

The regular VVC 8-tap filter is used when the reference picture is smaller than current picture.

For chroma, the following filter is used when the reference picture is larger than current picture:

| Fractional sample position p | interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | fC[ p ][ 0 ] | fC[ p ][ 1 ] | fC[ p ][ 2 ] | fC[ p ][ 3 ] |
| 0 | 16 | 32 | 16 | 0 |
| 1 | 15 | 32 | 16 | 1 |
| 2 | 14 | 32 | 17 | 1 |
| 3 | 13 | 32 | 18 | 1 |
| 4 | 13 | 31 | 19 | 1 |
| 5 | 12 | 32 | 19 | 1 |
| 6 | 11 | 32 | 20 | 1 |
| 7 | 10 | 31 | 21 | 2 |
| 8 | 10 | 30 | 22 | 2 |
| 9 | 9 | 30 | 23 | 2 |
| 10 | 8 | 30 | 23 | 3 |
| 11 | 8 | 29 | 24 | 3 |
| 12 | 7 | 29 | 25 | 3 |
| 13 | 7 | 28 | 25 | 4 |
| 14 | 6 | 28 | 26 | 4 |
| 15 | 6 | 27 | 26 | 5 |
| 16 | 5 | 27 | 27 | 5 |
| 17 | 5 | 26 | 27 | 6 |
| 18 | 4 | 26 | 28 | 6 |
| 19 | 4 | 25 | 28 | 7 |
| 20 | 3 | 25 | 29 | 7 |
| 21 | 3 | 24 | 29 | 8 |
| 22 | 3 | 23 | 30 | 8 |
| 23 | 2 | 23 | 30 | 9 |
| 24 | 2 | 22 | 30 | 10 |
| 25 | 2 | 21 | 31 | 10 |
| 26 | 1 | 20 | 32 | 11 |
| 27 | 1 | 19 | 32 | 12 |
| 28 | 1 | 19 | 31 | 13 |
| 29 | 1 | 18 | 32 | 13 |
| 30 | 1 | 17 | 32 | 14 |
| 31 | 1 | 16 | 32 | 15 |

The regular VVC 4-tap filer is used when the reference picture is smaller than current picture.

### CE1-1.2 Use integer version of 8-tap Lanczos1 filter when reference picture is larger than current picture

For luma, the following filter is used when the reference picture is larger than current picture:

| Fractional sample position p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p ][ 0 ] | fL[ p ][ 1 ] | fL[ p ][ 2 ] | fL[ p ][ 3 ] | fL[ p ][ 4 ] | fL[ p ][ 5 ] | fL[ p ][ 6 ] | fL[ p ][ 7 ] |
| 0 | 4 | 8 | 13 | 14 | 13 | 8 | 4 | 0 |
| 1 | 3 | 8 | 12 | 14 | 13 | 9 | 4 | 1 |
| 2 | 3 | 8 | 12 | 14 | 13 | 9 | 4 | 1 |
| 3 | 3 | 7 | 12 | 15 | 13 | 9 | 4 | 1 |
| 4 | 3 | 7 | 12 | 14 | 13 | 9 | 5 | 1 |
| 5 | 3 | 7 | 11 | 14 | 13 | 10 | 5 | 1 |
| 6 | 2 | 6 | 11 | 14 | 14 | 10 | 5 | 2 |
| 7 | 2 | 6 | 11 | 14 | 14 | 10 | 5 | 2 |
| 8 | 2 | 6 | 10 | 14 | 14 | 10 | 6 | 2 |
| 9 | 2 | 5 | 10 | 14 | 14 | 11 | 6 | 2 |
| 10 | 2 | 5 | 10 | 14 | 14 | 11 | 6 | 2 |
| 11 | 1 | 5 | 10 | 13 | 14 | 11 | 7 | 3 |
| 12 | 1 | 5 | 9 | 13 | 14 | 12 | 7 | 3 |
| 13 | 1 | 4 | 9 | 13 | 15 | 12 | 7 | 3 |
| 14 | 1 | 4 | 9 | 13 | 14 | 12 | 8 | 3 |
| 15 | 1 | 4 | 9 | 13 | 14 | 12 | 8 | 3 |

The regular VVC 8-tap filter is used when the reference picture is smaller than current picture.

For chroma, the following filter is used when the reference picture is larger than current picture.

| Fractional sample position p | interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | fC[ p ][ 0 ] | fC[ p ][ 1 ] | fC[ p ][ 2 ] | fC[ p ][ 3 ] |
| 0 | 19 | 26 | 19 | 0 |
| 1 | 17 | 24 | 18 | 5 |
| 2 | 17 | 24 | 18 | 5 |
| 3 | 16 | 24 | 19 | 5 |
| 4 | 15 | 24 | 19 | 6 |
| 5 | 15 | 24 | 19 | 6 |
| 6 | 15 | 24 | 19 | 6 |
| 7 | 14 | 24 | 19 | 7 |
| 8 | 14 | 23 | 20 | 7 |
| 9 | 13 | 23 | 20 | 8 |
| 10 | 13 | 23 | 20 | 8 |
| 11 | 12 | 23 | 21 | 8 |
| 12 | 12 | 22 | 21 | 9 |
| 13 | 12 | 22 | 21 | 9 |
| 14 | 11 | 22 | 21 | 10 |
| 15 | 11 | 22 | 21 | 10 |
| 16 | 10 | 22 | 22 | 10 |
| 17 | 10 | 21 | 22 | 11 |
| 18 | 10 | 21 | 22 | 11 |
| 19 | 9 | 21 | 22 | 12 |
| 20 | 9 | 21 | 22 | 12 |
| 21 | 8 | 21 | 23 | 12 |
| 22 | 8 | 20 | 23 | 13 |
| 23 | 8 | 20 | 23 | 13 |
| 24 | 7 | 20 | 23 | 14 |
| 25 | 7 | 19 | 24 | 14 |
| 26 | 6 | 19 | 24 | 15 |
| 27 | 6 | 19 | 24 | 15 |
| 28 | 6 | 19 | 24 | 15 |
| 29 | 5 | 19 | 24 | 16 |
| 30 | 5 | 18 | 24 | 17 |
| 31 | 5 | 18 | 24 | 17 |

The regular VVC 4-tap filer is used when the reference picture is smaller than current picture.

### CE1-1.3 Use integer version of 8-tap Lanczos2.67 filter when reference picture is larger than current picture

For luma, the following filter is used when the reference picture is larger than current picture:

| Fractional sample position p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p ][ 0 ] | fL[ p ][ 1 ] | fL[ p ][ 2 ] | fL[ p ][ 3 ] | fL[ p ][ 4 ] | fL[ p ][ 5 ] | fL[ p ][ 6 ] | fL[ p ][ 7 ] |
| 0 | -1 | -5 | 17 | 42 | 17 | -5 | -1 | 0 |
| 1 | 0 | -5 | 15 | 41 | 19 | -5 | -1 | 0 |
| 2 | 0 | -5 | 13 | 40 | 21 | -4 | -1 | 0 |
| 3 | 0 | -5 | 11 | 39 | 24 | -4 | -2 | 1 |
| 4 | 0 | -5 | 9 | 38 | 26 | -3 | -2 | 1 |
| 5 | 0 | -5 | 7 | 38 | 28 | -2 | -3 | 1 |
| 6 | 1 | -5 | 5 | 36 | 30 | -1 | -3 | 1 |
| 7 | 1 | -4 | 3 | 35 | 32 | 0 | -4 | 1 |
| 8 | 1 | -4 | 2 | 33 | 33 | 2 | -4 | 1 |
| 9 | 1 | -4 | 0 | 32 | 35 | 3 | -4 | 1 |
| 10 | 1 | -3 | -1 | 30 | 36 | 5 | -5 | 1 |
| 11 | 1 | -3 | -2 | 28 | 38 | 7 | -5 | 0 |
| 12 | 1 | -2 | -3 | 26 | 38 | 9 | -5 | 0 |
| 13 | 1 | -2 | -4 | 24 | 39 | 11 | -5 | 0 |
| 14 | 0 | -1 | -4 | 21 | 40 | 13 | -5 | 0 |
| 15 | 0 | -1 | -5 | 19 | 41 | 15 | -5 | 0 |

The regular VVC 8-tap filter is used when the reference picture is smaller than current picture.

For chroma, the following filter is used when the reference picture is larger than current picture.

| Fractional sample position p | interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | fC[ p ][ 0 ] | fC[ p ][ 1 ] | fC[ p ][ 2 ] | fC[ p ][ 3 ] |
| 0 | 12 | 40 | 12 | 0 |
| 1 | 11 | 40 | 13 | 0 |
| 2 | 10 | 40 | 15 | -1 |
| 3 | 9 | 40 | 16 | -1 |
| 4 | 8 | 40 | 17 | -1 |
| 5 | 8 | 39 | 18 | -1 |
| 6 | 7 | 39 | 19 | -1 |
| 7 | 6 | 38 | 21 | -1 |
| 8 | 5 | 38 | 22 | -1 |
| 9 | 4 | 38 | 23 | -1 |
| 10 | 4 | 37 | 24 | -1 |
| 11 | 3 | 36 | 25 | 0 |
| 12 | 3 | 35 | 26 | 0 |
| 13 | 2 | 34 | 28 | 0 |
| 14 | 2 | 33 | 29 | 0 |
| 15 | 1 | 33 | 30 | 0 |
| 16 | 1 | 31 | 31 | 1 |
| 17 | 0 | 30 | 33 | 1 |
| 18 | 0 | 29 | 33 | 2 |
| 19 | 0 | 28 | 34 | 2 |
| 20 | 0 | 26 | 35 | 3 |
| 21 | 0 | 25 | 36 | 3 |
| 22 | -1 | 24 | 37 | 4 |
| 23 | -1 | 23 | 38 | 4 |
| 24 | -1 | 22 | 38 | 5 |
| 25 | -1 | 21 | 38 | 6 |
| 26 | -1 | 19 | 39 | 7 |
| 27 | -1 | 18 | 39 | 8 |
| 28 | -1 | 17 | 40 | 8 |
| 29 | -1 | 16 | 40 | 9 |
| 30 | -1 | 15 | 40 | 10 |
| 31 | 0 | 13 | 40 | 11 |

The regular VVC 4-tap filer is used when the reference picture is smaller than current picture.

### CE1-2 Use 9-tap downsampling and extended SHVC filters for upsampling (JVET-O0319)

Our vision for ARC (or reference picture resampling) functionality is to define resampling of both reference pictures and source pictures using a single set of co-designed resampling filters, to allow for high quality prediction. This is initially intended to be outside the MEC process. The ARC functionality can be embedded within the interpolation process. For the purpose of testing both approaches, we offer phased versions of both our downsampler and an existing upsampler (the upsampler is from table 8-9 and 8-13 in the Draft VVC specification, JVET-2001).

- Downsampling: [1 0 -8 16 46 16 -8 0 1] / 64 (complexity of 7-tap) is .used for downsampling and motion compensation interpolation for both luma and chroma components when the reference picture is larger than the current picture. Phased version below..

- Upsampling: (from table 8-9 from JVET-O2001). This is used when the reference picture is smaller than the current picture. The detailed filter coefficients are given in tables below.

We present our filters in the traditional [-pi, pi] phase format, but we have [0, 2pi] versions if needed.

| Fractional sample position p | downsampling filter coefficients |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | p[ 0 ] | p[ 1 ] | p[ 2 ] | p[ 3 ] | p[ 4 ] | p[ 5 ] | p[ 6 ] | p[ 7 ] | p[ 8 ] |
| 0 | 4 | -6 | 0 | 32 | 32 | 0 | -6 | 4 | 4 |
| 1 | 4 | -6 | 1 | 34 | 30 | -2 | -5 | 4 | 4 |
| 2 | 3 | -7 | 3 | 36 | 28 | -3 | -4 | 4 | 4 |
| 3 | 3 | -7 | 5 | 37 | 26 | -4 | -4 | 4 | 4 |
| 4 | 3 | -7 | 6 | 39 | 24 | -5 | -3 | 4 | 3 |
| 5 | 2 | -8 | 9 | 41 | 22 | -6 | -2 | 3 | 3 |
| 6 | 2 | -8 | 11 | 43 | 20 | -7 | -2 | 3 | 2 |
| 7 | 1 | -8 | 13 | 44 | 18 | -7 | -1 | 2 | 2 |
| 8 | 1 | 0 | -8 | 16 | 46 | 16 | -8 | 0 | 1 |
| 9 | 2 | -1 | -7 | 18 | 45 | 13 | -8 | 1 | 1 |
| 10 | 3 | -2 | -7 | 20 | 44 | 11 | -8 | 1 | 2 |
| 11 | 3 | -2 | -6 | 22 | 42 | 8 | -8 | 2 | 3 |
| 12 | 4 | -3 | -5 | 24 | 39 | 6 | -7 | 3 | 3 |
| 13 | 4 | -4 | -4 | 26 | 37 | 5 | -7 | 3 | 4 |
| 14 | 4 | -4 | -3 | 28 | 36 | 3 | -7 | 3 | 4 |
| 15 | 4 | -5 | -1 | 30 | 33 | 1 | -6 | 4 | 4 |

CE1-2.1.  Test within the interpolation process, similar to CE1-1, but using our filters in place of the existing interpolation filters. (This process does not rescale the reference pictures, nor stores them in the DPB, but rather works with block-based methods to scale and perform MEC as needed.)  This functionality is provided for comparison with the method of CE1-1.

CE1-2.2.  Test outside of the MEC process, where reference pictures are resampled to match the resolution of input source frames, and then MEC is performed using existing MEC process. We will illustrate the application of these filters, for either reference or source picture downsampling, for just the luma samples in two important subcases, 2:1 and 1.5:1.

For 2:1 downsampling, one simply applies the 0-phase downsampling filter (p=8 filter above), and then selects every other sample.

For 3:2 (1.5:1) downsampling, it is more complicated. One needs to replace every 3 points with 2 points. That is, for points at positions [0, 1, 2, 3, 4, 5] etc, one replaces them with points at [0.25, 1.75, 3.25, 4.75] etc.  Since the 0-phase here is the p=8 filter, in a 16-phase family, for the 0.25 position, one uses the p=12 filtter, and for the 1.75 position, one uses the position p=4 filter.

We note again that our upsampler, in the table below, is Table 8-9 from the VVC Draft Specification, JVET-2001.

| Fractional sample position p | Upsampling filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | f[ p ][ 0 ] | f[ p ][ 1 ] | f[ p ][ 2 ] | f[ p ][ 3 ] |
| 0 | 0 | 64 | 0 | 0 |
| 1 | −1 | 63 | 2 | 0 |
| 2 | −2 | 62 | 4 | 0 |
| 3 | −2 | 60 | 7 | −1 |
| 4 | −2 | 58 | 10 | −2 |
| 5 | −3 | 57 | 12 | −2 |
| 6 | −4 | 56 | 14 | −2 |
| 7 | −4 | 55 | 15 | −2 |
| 8 | −4 | 54 | 16 | −2 |
| 9 | −5 | 53 | 18 | −2 |
| 10 | −6 | 52 | 20 | −2 |
| 11 | −6 | 49 | 24 | −3 |
| 12 | −6 | 46 | 28 | −4 |
| 13 | −5 | 44 | 29 | −4 |
| 14 | −4 | 42 | 30 | −4 |
| 15 | −4 | 39 | 33 | −4 |
| 16 | −4 | 36 | 36 | −4 |
| 17 | −4 | 33 | 39 | −4 |
| 18 | −4 | 30 | 42 | −4 |
| 19 | −4 | 29 | 44 | −5 |
| 20 | −4 | 28 | 46 | −6 |
| 21 | −3 | 24 | 49 | −6 |
| 22 | −2 | 20 | 52 | −6 |
| 23 | −2 | 18 | 53 | −5 |
| 24 | −2 | 16 | 54 | −4 |
| 25 | −2 | 15 | 55 | −4 |
| 26 | −2 | 14 | 56 | −4 |
| 27 | −2 | 12 | 57 | −3 |
| 28 | −2 | 10 | 58 | −2 |
| 29 | −1 | 7 | 60 | −2 |
| 30 | 0 | 4 | 62 | −2 |
| 31 | 0 | 2 | 63 | −1 |

| Fractional sample position p | Upsampling filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | p [ 0 ] | p [ 1 ] | p [ 2 ] | p [ 3 ] |
| 1 | −1 | 63 | 2 | 0 |
| 2 | −2 | 62 | 4 | 0 |
| 3 | -2 | 60 | 7 | −1 |
| 4 | −2 | 58 | 10 | −2 |
| 5 | −3 | 57 | 12 | −2 |
| 6 | −4 | 56 | 14 | −2 |
| 7 | −4 | 55 | 15 | −2 |
| 8 | −4 | 54 | 16 | −2 |
| 9 | −5 | 53 | 18 | −2 |
| 10 | −6 | 52 | 20 | −2 |
| 11 | −6 | 49 | 24 | −3 |
| 12 | −6 | 46 | 28 | −4 |
| 13 | −5 | 44 | 29 | −4 |
| 14 | −4 | 42 | 30 | −4 |
| 15 | −4 | 39 | 33 | −4 |
| 16 | −4 | 36 | 36 | −4 |
| 17 | −4 | 33 | 39 | −4 |
| 18 | −4 | 30 | 42 | −4 |
| 19 | −4 | 29 | 44 | −5 |
| 20 | −4 | 28 | 46 | −6 |
| 21 | −3 | 24 | 49 | −6 |
| 22 | −2 | 20 | 52 | −6 |
| 23 | −2 | 18 | 53 | −5 |
| 24 | −2 | 16 | 54 | −4 |
| 25 | −2 | 15 | 55 | −4 |
| 26 | −2 | 14 | 56 | −4 |
| 27 | −2 | 12 | 57 | −3 |
| 28 | −2 | 10 | 58 | −2 |
| 29 | −1 | 7 | 60 | −2 |
| 30 | 0 | 4 | 62 | −2 |
| 31 | 0 | 2 | 63 | −1 |

### CE1-3 Use SHM filters for downsampling (JVET-O0641)

In the motion compensation for reference picture downsampling, the 12-tap SHM downsampling filters are used. The selection of filters is based on the downsampling ratio as in SHM-12.4. The same filter is used for both luma and chroma components. In the motion compensation for reference picture upsampling, the existed VTM interpolation filters (i.e., the 8-tap filter for luma, the 6-tap for luma 4x4 sub-block, the 6-tap filter for halp-pel AMVR, and the 4–tap for chroma) are used. The detail downsampling filter coefficients are given below.

| Fractional sample position p | interpolation filter coefficients for 2:1 downsampling |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | p0 | p1 | p2 | p3 | p4 | p5 | p6 | p7 | p8 | p9 | p10 | p11 |
| 0 | 2 | -3 | -9 | 6 | 39 | 58 | 39 | 6 | -9 | -3 | 2 | 0 |
| 1 | 2 | -3 | -9 | 4 | 38 | 58 | 43 | 7 | -9 | -4 | 1 | 0 |
| 2 | 2 | -2 | -9 | 2 | 35 | 58 | 44 | 9 | -8 | -4 | 1 | 0 |
| 3 | 1 | -2 | -9 | 1 | 34 | 58 | 46 | 11 | -8 | -5 | 1 | 0 |
| 4 | 1 | -1 | -8 | -1 | 31 | 57 | 47 | 13 | -7 | -5 | 1 | 0 |
| 5 | 1 | -1 | -8 | -2 | 29 | 56 | 49 | 15 | -7 | -6 | 1 | 1 |
| 6 | 1 | 0 | -8 | -3 | 26 | 55 | 51 | 17 | -7 | -6 | 1 | 1 |
| 7 | 1 | 0 | -7 | -4 | 24 | 54 | 52 | 19 | -6 | -7 | 1 | 1 |
| 8 | 1 | 0 | -7 | -5 | 22 | 53 | 53 | 22 | -5 | -7 | 0 | 1 |
| 9 | 1 | 1 | -7 | -6 | 19 | 52 | 54 | 24 | -4 | -7 | 0 | 1 |
| 10 | 1 | 1 | -6 | -7 | 17 | 51 | 55 | 26 | -3 | -8 | 0 | 1 |
| 11 | 1 | 1 | -6 | -7 | 15 | 49 | 56 | 29 | -2 | -8 | -1 | 1 |
| 12 | 0 | 1 | -5 | -7 | 13 | 47 | 57 | 31 | -1 | -8 | -1 | 1 |
| 13 | 0 | 1 | -5 | -8 | 11 | 46 | 58 | 34 | 1 | -9 | -2 | 1 |
| 14 | 0 | 1 | -4 | -8 | 9 | 44 | 58 | 35 | 2 | -9 | -2 | 2 |
| 15 | 0 | 1 | -4 | -9 | 7 | 43 | 58 | 38 | 4 | -9 | -3 | 2 |

| Fractional sample position p | interpolation filter coefficients for 1.5 : 1 downsampling |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | p0 | p1 | p2 | p3 | p4 | p5 | p6 | p7 | p8 | p9 | p10 | p11 |
| 0 | 0 | 5 | -6 | -10 | 37 | 76 | 37 | -10 | -6 | 5 | 0 | 0 |
| 1 | 0 | 5 | -4 | -11 | 33 | 76 | 40 | -9 | -7 | 5 | 0 | 0 |
| 2 | -1 | 5 | -3 | -12 | 29 | 75 | 45 | -7 | -8 | 5 | 0 | 0 |
| 3 | -1 | 4 | -2 | -13 | 25 | 75 | 48 | -5 | -9 | 5 | 1 | 0 |
| 4 | -1 | 4 | -1 | -13 | 22 | 73 | 52 | -3 | -10 | 4 | 1 | 0 |
| 5 | -1 | 4 | 0 | -13 | 18 | 72 | 55 | -1 | -11 | 4 | 2 | -1 |
| 6 | -1 | 4 | 1 | -13 | 14 | 70 | 59 | 2 | -12 | 3 | 2 | -1 |
| 7 | -1 | 3 | 1 | -13 | 11 | 68 | 62 | 5 | -12 | 3 | 2 | -1 |
| 8 | -1 | 3 | 2 | -13 | 8 | 65 | 65 | 8 | -13 | 2 | 3 | -1 |
| 9 | -1 | 2 | 3 | -12 | 5 | 62 | 68 | 11 | -13 | 1 | 3 | -1 |
| 10 | -1 | 2 | 3 | -12 | 2 | 59 | 70 | 14 | -13 | 1 | 4 | -1 |
| 11 | -1 | 2 | 4 | -11 | -1 | 55 | 72 | 18 | -13 | 0 | 4 | -1 |
| 12 | 0 | 1 | 4 | -10 | -3 | 52 | 73 | 22 | -13 | -1 | 4 | -1 |
| 13 | 0 | 1 | 5 | -9 | -5 | 48 | 75 | 25 | -13 | -2 | 4 | -1 |
| 14 | 0 | 0 | 5 | -8 | -7 | 45 | 75 | 29 | -12 | -3 | 5 | -1 |
| 15 | 0 | 0 | 5 | -7 | -9 | 40 | 76 | 33 | -11 | -4 | 5 | 0 |

# Test conditions and guidelines

VTM-6.1 software with the affine patch shall be used as the anchor and the base software for CE development.

Low delay B (LB) test configuration is used to evaluate tests in the CE:

- only 5 second length (half of the CTC frame count)

- 0.5 second switch points

- 2:1 and 1.5:1 as primary; 4:1 optional, with both dimensions changing together

Configuration files for 1.5x, 2.0x, and 4.0x scaling ratios can be found in the cfg/rpr directory of the VTM software package.

SHM filters are used for both luma and chroma components to downsample the source and the downsampled picture is encoded when the resolution is reduced. In the source downsampling, zero position alignment (type 2, see Fig. 1) between luma and choma components is assumed.

Figure 1. Chroma sampling types.

For test comparison purposes, the fixed input source downsampling (SHM filters) and fixed upsampling of decoded pictures (VTM interpolation filters) are used in all tests as depicted on Fig. 2.

Figure 2. Fixed source downsampling and fixed decoded picture upsampling

Two PSNRs are requested to be provided:

- PSNR1: it is measured on the decoded picture size by calculating MSE for each picture, accumulating MSEs for all pictures and calculating average PSNR from the accumulated MSE.

- PSNR2: it is measured after upsampling of the decoded picture if the decoded picture size is in smaller resolution, the existed VTM interpolation filters are used for upsampling.

Figure 3. Source downsampling and PSNR calculation

The existing interpolation filters (luma 8-tap and chroma 4-tap interpolation filters) for upsampling/downsampling in the reference picture resampling are used as the anchor for test results comparison.

# Non-CE1/CE1-related contributions

| JVET number | Title |
| --- | --- |
| JVET-P0119 | AHG8: On reference picture resampling (RPR) |
| JVET-P0241 | AHG17/CE1-related: Specifying Scaling Regions for Reference Picture Resampling |
| JVET-P0274 | Non-CE1: Enabling TMVP in RPR |
| JVET-P0353 | CE1-related: Reference picture resampling filters |
| JVET-P0381 | Non-CE1: A fix on reference sample offset for RPR |
| JVET-P0382 | Non-CE1: Simplified motion compensation in RPR |
| JVET-P0390 | CE1-related: Anti-aliasing motion compensation interpolation downsampling filters for reference picture resampling |
| JVET-P0409 | CE1-related: Enable PROF for RPR |
| JVET-P0592 | AHG8/Non-CE1: Phase shifts for resampling |
| JVET-P0593 | AHG8/CE1-related: Signalling filter coefficients for resampling |