| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0024-v1 |
| --- | --- |

| Title: | CE4: Summary report on inter prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen Haitao Yang Xiaoyu Xiu | Email: | chunchic@qti.qualcomm.com haitao.yang@huawei.com xiaoyuxiu@kwai.com |
| Source: | CE4 coordinators |  |  |

_____________________________

- Abstract

This contribution provides a summary report of Core Experiment 4 on inter prediction. This CE comprises two categories,

- Geometric partition related merge modes,

- Simplification on PROF and BDOF.

All techniques as planned in the context of CE4 are implemented on top of and tested against VTM-6.0. Simulation results and crosschecking reports of each test specified in this document are provided.

Subjective evaluation of representative CE4-1 tests is planned to identify whether there is an observable subjective improvement. Test materials are provided for consideration.

CE4-1: Geometric partition related merge modes

Test specification

| Test # | Description | Source |
| --- | --- | --- |
| 4-1.1* | Geometric (GEO) merge mode as proposed in JVET-O0489 with 32 angles and 5 distance values | JVET-P0068 |
| 4-1.2a | Geometric (GEO) merge mode with reduced number of partitions with N angles and 4 distance values, i.e., N=32 | JVET-P0068 |
| 4-1.2b | Geometric (GEO) merge mode with reduced number of partitions with N angles and 4 distance values, i.e., N=24 | JVET-P0068 |
| 4-1.5 | GEO + ¼ and ½ adaptive-size partial transform | JVET-P0074 |
| 4-1.6 | GEO + ½ fixed-size partial transform | JVET-P0074 |
| 4-1.7 | Adaptive blending filtering for TPM | JVET-P0069 |
| 4-1.8 | Test 4-1.7 + GEO | JVET-P0069 |
| 4-1.9 | CIIP with triangular partitions | JVET-P0071 |
| 4-1.10 | Test 4-1.9 + GEO | JVET-P0070 |
| 4-1.12 | Test 4-1.9 + Partial transform as in Test 4.1.5 | JVET-P0075 |
| 4-1.13 | Test 4-1.12 + Test 4.1.5 (GEO with partial transform) | JVET-P0075 |
| 4-1.14 | GEO with TPM-like motion storage method vs GEO with the proposal motion storage method in JVET-N0489 | JVET-P0068 |

* It is noted that all the GEO-related tests in CE4-1 signal GEO flag at the bottom of the merge tree to switch between TPM and GEO.

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4-1.1* | -0.36% | -0.45% | -0.54% | 104% | 100% | -0.72% | -0.66% | -0.76% | 105% | 100% |
| 4-1.2a | -0.34% | -0.42% | -0.54% | 104% | 100% | -0.73% | -0.69% | -0.76% | 104% | 99% |
| 4-1.2b | -0.34% | -0.39% | -0.52% | 103% | 100% | -0.68% | -0.46% | -0.60% | 103% | 99% |
| 4-1.5 | -0.35% | -0.41% | -0.49% | 104% | 100% | -0.83% | -0.67% | -0.67% | 105% | 100% |
| 4-1.6 | -0.34% | -0.43% | -0.49% | 104% | 100% | -0.87% | -0.56% | -0.60% | 105% | 100% |
| 4-1.7 | / | / | / | / | / | / | / | / | / | / |
| 4-1.8 | / | / | / | / | / | / | / | / | / | / |
| 4-1.9 | -0.07% | -0.05% | -0.05% | 101% | 100% | -0.18% | -0.14% | -0.15% | 103% | 99% |
| 4-1.10 | -0.39% | -0.48% | -0.49% | 104% | 100% | -0.77% | -0.84% | -0.62% | 106% | 98% |
| 4-1.12 | -0.09% | -0.05% | -0.01% | 103% | 100% | -0.31% | 0.02% | -0.08% | 103% | 100% |
| 4-1.13 | -0.41% | -0.47% | -0.47% | 106% | 100% |  |  |  |  |  |
| 4-1.14 | -0.37% | -0.40% | -0.53% | 104% | 100% | -0.71% | -0.71% | -0.59% | 105% | 100% |

Results for Class F and Class SCC is shown for CE4-1.7 and CE4-1.8 since the two tests applies to screen content only.

|  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Sequence | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE4-1.7 | Class F | -0.02% | -0.02% | -0.03% | 100% | 100% | 0.02% | 0.00% | 0.48% | 99% | 100% |
|  | Class SCC | -0.73% | -0.58% | -0.55% | 100% | 100% | -0.94% | -0.69% | -0.68% | 98% | 100% |
| CE4-1.8 | Class F | -0.47% | -0.55% | -0.48% | 101% | 100% | -0.70% | -0.83% | -0.49% | 102% | 99% |
|  | Class SCC | -2.35% | -2.01% | -1.98% | 100% | 100% | -3.11% | -2.77% | -2.72% | 99% | 100% |

Crosscheck report

| Test# | Crosschecker | SW match CE description (Y/N) | Text match SW (Y/N) | Simulation results (Y/N) | EncT/DecT match (Y/N) |
| --- | --- | --- | --- | --- | --- |
| 4-1.1 |  |  |  |  |  |
| 4-1.2a |  |  |  |  |  |
| 4-1.2b |  |  |  |  |  |
| 4-1.5 |  |  |  |  |  |
| 4-1.6 |  |  |  |  |  |
| 4-1.7 |  |  |  |  |  |
| 4-1.8 |  |  |  |  |  |
| 4-1.9 |  |  |  |  |  |
| 4-1.10 |  |  |  |  |  |
| 4-1.12 |  |  |  |  |  |
| 4-1.13 |  |  |  |  |  |
| 4-1.14 |  |  |  |  |  |

SW match CE description: Y SW implementation match CE description, N mismatch.

Text match SW: Y text match SW, N mismatch.

Simulation results: Y substantial results (all but class A for RA, all but class B for LDB) obtained and match results from tester, N mismatch or no substantial results.

EncT/DecT match: Y encoder and decoder run time match those from tester, N mismatch.

Detailed description of tests

- Test 4-1.1, 4-1.2 and 4-1.14 in JVET-P0068 (Huawei, RWTH Aachen)

Basic idea

Geometric partition mode extends the current triangle partition mode to support various binary partitions other thant he current diagonal and anti-diagonal partition mode. As shown in the figure below.

VVC-6.0 TPM            Extend to GEO

The split boundary of geometric merge mode is descripted by angle φi and distance offset ρi as shown in Figure 2. Angle  φi represents a quantized angle between 0 and 360 degrees and distance offset ρi represents a quantized offset of the largest distance ρmax .In addition, the split directions overlapped with binary tree splits and TPM splits are excluded.

Syntax change

| if( !ciip_flag[ x0 ][ y0 ] && MaxNumTriangleMergeCand > 1 ) { |  |
| --- | --- |
| if (cbWidth > = 8 && cbHeight >= 8 ) |  |
| merge_geo_flag[ x0 ][ y0 ] | ae(v) |
| if ( merge_geo_flag[ x0 ][ y0 ] ) |  |
| merge_geo_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |

The merge_geo_flag[][] is sinalged with 4 CABAC context models. The geo_partition_idx[][] is used as an index to the lookup table that stores the angle φi and distance ρi pairs. The geo_partition_idx coded truncated binary and binarized using bypass.

Angles and distances

|  | CE4-1.1 | CE4-1.2a | CE4-1.2b |
| --- | --- | --- | --- |
| Number of angles | 32 | 32 | 24 |
| Number of distance | 5 | 4 | 4 |
| Number of partitions | 140 | 108 | 80 |

Max distance ρmax is calculated and then quantified to derive the 4 or 5 distances in the table.

ρmaxφ,w,h=cos(φ)h2tanπ2-φ+w2-1.0, 0<φ<π2 				(1)

Directions close to the vertical direction is removed to reduce the number angles from 32 to 24 as illustrated below.

Blending of two predictions

The blending operation itself is the same as TPM, applying the 3-bit mask for weighted prediction of pixels around the partition boundary.

The derivation of blending mask is illustrated by the three equations below,

distFromLine = ((x<<1) + 1)*Dis[displacementX] + ((y<<1) + 1))*Dis[displacementY] –rho	(3)

distScaled = Clip3(0, 26, (abs(distFromLine) + 4) >> 3)						(4)

sampleWeightL[ x ][ y ] = distFromLine <= 0 ? GeoFilter[distScaled] : 8 − GeoFilter[distScaled]	(5)

Where 3 look-up tables, Dis[.] with 32 entries, StepDis[.] with 36 entries and GeoFilter[.] with 26 entries are involved.

Motion vector storage

In CE4-1.1 and CE4-1.2, luma sample weights at the four corners of a 4x4 motion storage unit is summed up. Then the sum is compared with 2 thresholds to decide whether one of two uni-prediction motion information or bi-prediction motion information is stored. The bi-prediction motion information is derived using the same process as TPM.

In CE4-1.14, the distance between the central position of a 4x4 motion storage unit and the split boundary is calculated and compared with a fixed threshold to decide whether uni- or bi prediction motion information is stored.

- Test 4-1.5 and 4-1.6 in JVET-P0074 (Huawei)

Basic idea

Align residual signal sampling with residual transform so that the residual signal is localized within a PU by the means of a PT area. The method is close to the one known as sub-block transform (SBT). The shape of the area to be transformed could be a parallelogram. The parallelogram of residual, the PT area, is further warped to a rectangular shape and then then existing transform mechanism applied.

The proposed partial transform is applied to GEO and TPM blocks. Either the width or the height of PT area must be the same as the width or the height of the CB. The other side of PT, either height or width, could be ½ or ¼ size of the height or width of the CB.

Syntax

| if( tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| --- | --- |
| if (partial_transform enabled && (MergeTriangleFlag ¦¦ geo_flag[ x0 ][ y0 ] ) && !cu_skip_flag[ x0 ][ y0 ] && !cu_skip_flag[ x0 ][ y0 ] && !cu_transquant_bypass_flag) { |  |
| part_trans_flag[ x0 ][ y0 ] | ae(v) |
| if (part_trans_flag) |  |
| part_trans_idx[ x0 ][ y0 ] | ae(v) |
| } |  |

In test CE4-1.5, partial transform size could be ½ or ¼ that is indicated by the syntax element part_trans_idx. If it is assign to 0, partial transform size is ½. Otherwise, partial transform size is ¼. PT size denotes the size of the shorter side of the PT area. part_trans_idx is not signaled for CE4-1.6, where ½ fixed-size partial transform is applied. So, the syntax is as follows:

| if( tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| --- | --- |
| if (partial_transform enabled && (MergeTriangleFlag ¦¦ geo_flag[ x0 ][ y0 ] ) && !cu_skip_flag[ x0 ][ y0 ] && !cu_skip_flag[ x0 ][ y0 ] && !cu_transquant_bypass_flag) |  |
| part_trans_flag[ x0 ][ y0 ] | ae(v) |
| … |  |

Derivation of PT area

Starting and ending points of the PT line that defines a partial transform area (PT area) is derived from the parameters of the GEO splitting line and the shape of the PU. The PT area is specified by this PT line. In particular, for each position along this line, the number of samples, which is equal to ½ or ¼ of the shorter side, are selected column-wise in the case when the shorter side is the left or right side of the PU, or row-wise in case the shorter side is the top side or bottom side of the PU. The center position of the selected samples is the position on the PT line.

- Test 4-1.7 and 4-1.8 in JVET-P0069 (Huawei, RWTH Aachen)

TPM blending operation is over-smoothing when the content is known to include sharp edges, hence may harm screen content coding. Proposed solution introduces a sequence level and a slice level control flag to disable the blending filtering operation. When the blending filtering is disabled, the sample weight for weighted combination can either be 0 or 8, the weighting matrix example is shown as in figure below.

In Test 4-1.7, the blending off method is applied to TPM.

In Test 4-1.8, the blending off method is applied to both TPM and GEO.

- Test 4-1.9 in JVET-P0071 (BBC)

CIIP with triangular partitions are proposed. An intra predictor and an inter predictor are computed for a block. The inter predictor and the intra predictor are combined using the partitioning scheme as in TPM mode. The intra predictor is used in one of the two triangular partitions, and the inter predictor is used in the other triangular partition. The blending scheme as used in TPM is used to blend the two triangular partitions.

Three possible Triangle CIIP modes are considered with different triangle split and/or usage of inter or intra prediction in the two partitions, as illustrated below.

- Test 4-1.10 in JVET-P0070 (BBC, Huawei, RWTH Aachen)

This test is a combination of test 4-1.9 and the geometric partition.

- Test 4-1.12 and 4-1.13 in JVET-P0075 (Huawei, BBC)

Test 4-1.12 is a combination of test 4-1.9 and the partial transform in test 4-1.5.

Test 4-1.13 is a combination of test 4-1.9 and the partial transform in test 4-1.5 applied to GEO, TPM and CIIP with triangular partitions.

CE4-2: Simplification on PROF and BDOF

Test specification

| Test # | Description | Source |
| --- | --- | --- |
| 4-2.1 | 1/32-pel precision of PROF motion refinement + [-32, 31] for PROF clipping range | JVET-P0057 |
| 4-2.2 | [-32, 31] for PROF clipping range | JVET-P0057 |

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4-2.1 | 0.01% | 0.02% | 0.01% | 100% | 99% | 0.02% | 0.01% | 0.04% | 100% | 100% |
| 4-2.2 | 0.01% | 0.05% | -0.05% | 101% | 101% | 0.02% | 0.00% | 0.04% | 100% | 101% |

Crosscheck report

| Test# | Crosschecker | SW match CE description (Y/N) | Text match SW (Y/N) | Simulation results (Y/N) | EncT/DecT match (Y/N) |
| --- | --- | --- | --- | --- | --- |
| 4-2.1 | G. Li | Y | Y | Y | Y |
| 4-2.2 | G. Li | Y | Y | Y | Y |

SW match CE description: Y SW implementation match CE description, N mismatch.

Text match SW: Y text match SW, N mismatch.

Simulation results: Y substantial results (all but class A for RA, all but class B for LDB) obtained and match results from tester, N mismatch or no substantial results.

EncT/DecT match: Y encoder and decoder run time match those from tester, N mismatch.

Detailed description of tests

- Test 4-2.1 and 4-1.2 in JVET-P0057 (Kwai, Qualcomm, Mediatek, InterDigital)

Common modifications in both tests,

- The range of BDOF motion refinements is changed from [-32, 32] to [-32, 31],

Modifications specific to test 4-2.1,

- PROF motion refinement is clipped into the range of [-32, 31], and

- The precision of the PROF motion refinement is changed from 1/64-pel to 1/32-pel.

Modifications specific to test 4-2.2,

- PROF motion refinement is clipped into the range of [-32, 31].

Non-CE4/CE4-related contributions

| JVET number | Title |
| --- | --- |

Recommendations

Conduct subjective evaluation to representative tests in CE4-1.

Discuss CE4 tests and review all Non-CE4/CE4-related contributions.