| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0024-v4 |
| --- | --- |

| Title: | CE4: Summary report on inter prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen Haitao Yang Xiaoyu Xiu | Email: | chunchic@qti.qualcomm.com haitao.yang@huawei.com xiaoyuxiu@kwai.com |
| Source: | CE4 coordinators |  |  |

_____________________________

- Abstract

This contribution provides a summary report of Core Experiment 4 on inter prediction. This CE comprises two categories,

- Geometric partition related merge modes,

- Simplification on PROF and BDOF.

All techniques as planned in the context of CE4 are implemented on top of and tested against VTM-6.0. Simulation results and crosschecking reports of each test specified in this document are provided.

Subjective evaluation of representative CE4-1 tests is planned to identify whether there is an observable subjective improvement. Test materials are provided for consideration.

Summary of late changes to JVET-O2024

The v3 version of CE4 document was provided on Sep. 19th. The late changes are summarized below.

- Complete cross-checker list,

- Remove tests that are withdrawn,

- Assign new test number for each subtest in CE4-1.2,

- Include proponent-provided clarification on vague test descriptions that were asked by experts on CE4 reflector (CE4-1.1, 4-2.1, 4-2.2),

- Revise the description of subjective evaluation.

CE4-1: Geometric partition related merge modes

There are four tools for CE4-1, geometric partition, partial transform for geometric partition, adaptive blending for screen content and CIIP with triangle partition.

Variants of the four tools and the combination are tested.

Test specification

| Test # | Description | Source |
| --- | --- | --- |
| 4-1.1* | Geometric (GEO) merge mode as proposed in JVET-O0489 with 32 angles and 5 distance values | JVET-P0068 |
| 4-1.2a | Geometric (GEO) merge mode with reduced number of partitions with N angles and 4 distance values, i.e., N=32 | JVET-P0068 |
| 4-1.2b | Geometric (GEO) merge mode with reduced number of partitions with N angles and 4 distance values, i.e., N=24 | JVET-P0068 |
| 4-1.5 | GEO + ¼ and ½ adaptive-size partial transform | JVET-P0074 |
| 4-1.6 | GEO + ½ fixed-size partial transform | JVET-P0074 |
| 4-1.7 | Adaptive blending filtering for TPM | JVET-P0069 |
| 4-1.8 | Test 4-1.7 + GEO | JVET-P0069 |
| 4-1.9 | CIIP with triangular partitions | JVET-P0071 |
| 4-1.10 | Test 4-1.9 + GEO | JVET-P0070 |
| 4-1.12 | Test 4-1.9 + Partial transform as in Test 4.1.5 | JVET-P0075 |
| 4-1.13 | Test 4-1.12 + Test 4.1.5 (GEO with partial transform) | JVET-P0075 |
| 4-1.14 | GEO with TPM-like motion storage method vs GEO with the proposal motion storage method in JVET-O0489 | JVET-P0068 |

* It is noted that all the GEO-related tests in CE4-1 signal GEO flag at the bottom of the merge tree to switch between TPM and GEO.

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4-1.1* | -0.36% | -0.45% | -0.54% | 104% | 100% | -0.72% | -0.66% | -0.76% | 105% | 100% |
| 4-1.2a | -0.34% | -0.42% | -0.54% | 104% | 100% | -0.73% | -0.69% | -0.76% | 104% | 99% |
| 4-1.2b | -0.34% | -0.39% | -0.52% | 103% | 100% | -0.68% | -0.46% | -0.60% | 103% | 99% |
| 4-1.5 | -0.35% | -0.41% | -0.49% | 104% | 100% | -0.83% | -0.67% | -0.67% | 105% | 100% |
| 4-1.6 | -0.34% | -0.43% | -0.49% | 104% | 100% | -0.87% | -0.56% | -0.60% | 105% | 100% |
| 4-1.7 | / | / | / | / | / | / | / | / | / | / |
| 4-1.8 | / | / | / | / | / | / | / | / | / | / |
| 4-1.9 | -0.07% | -0.05% | -0.05% | 101% | 100% | -0.18% | -0.14% | -0.15% | 103% | 99% |
| 4-1.10 | -0.39% | -0.48% | -0.49% | 104% | 100% | -0.77% | -0.84% | -0.62% | 106% | 98% |
| 4-1.12 | -0.09% | -0.05% | -0.01% | 103% | 100% | -0.31% | 0.02% | -0.08% | 103% | 100% |
| 4-1.13 | -0.41% | -0.47% | -0.47% | 106% | 100% | -0.97% | -0.79% | -0.63% | 108% | 100% |
| 4-1.14 | -0.37% | -0.40% | -0.53% | 104% | 100% | -0.71% | -0.71% | -0.59% | 105% | 100% |

Results for Class F and Class SCC is shown for CE4-1.7 and CE4-1.8 since the two tests applies to screen content only.

|  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Sequence | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE4-1.7 | Class F | -0.02% | -0.02% | -0.03% | 100% | 100% | 0.02% | 0.00% | 0.48% | 99% | 100% |
|  | Class SCC | -0.73% | -0.58% | -0.55% | 100% | 100% | -0.94% | -0.69% | -0.68% | 98% | 100% |
| CE4-1.8 | Class F | -0.47% | -0.55% | -0.48% | 101% | 100% | -0.70% | -0.83% | -0.49% | 102% | 99% |
|  | Class SCC | -2.35% | -2.01% | -1.98% | 100% | 100% | -3.11% | -2.77% | -2.72% | 99% | 100% |

Crosscheck report

| Test# | Crosschecker | SW match CE description (Y/N) | Text match SW (Y/N) | Simulation results (Y/N) | EncT/DecT match (Y/N) |
| --- | --- | --- | --- | --- | --- |
| 4-1.1* | A. Wieckowski, K. Reuzé | Y | Y | Y | Y |
| 4-1.2a | A. Wieckowski | Y | Y | Y | Y |
| 4-1.2b | A. Wieckowski | Y | Y | Y | Y |
| 4-1.5 | J. Pfaff, K. Reuzé | Y | Y | Y | Y |
| 4-1.6 | J. Pfaff | Y | Y | Y | Y |
| 4-1.7 | Y.-L. Hsiao | Y | Y | Y | Y |
| 4-1.8 | Y.-L. Hsiao | Y | Y | Y | Y |
| 4-1.9 | Man-Shu Chiang | Y | Y | Y | Y |
| 4-1.10 | Man-Shu Chiang | Y | Y | Y | Y |
| 4-1.12 | J. Pfaff | Y | Y | Y | Y |
| 4-1.13 | J. Pfaff | Y | Y | Y | Y |
| 4-1.14 | X. Xiu, K. Reuzé | Y | Y | Y | Y |

SW match CE description: Y SW implementation match CE description, N mismatch.

Text match SW: Y text match SW, N mismatch.

Simulation results: Y substantial results (all but class A for RA, all but class B for LDB) obtained and match results from tester, N mismatch or no substantial results.

EncT/DecT match: Y encoder and decoder run time match those from tester, N mismatch.

Subjective evaluation

Suggestion below is from the discussion of subjective test planning in Track A.

In total 3 viewing sessions (3 experts each), 5 groups, 20x10min, approx. 3 hrs.

Anchor is always VTM6 CTC.

Session 1 on geometric partition

Tools under test: JVET-P0884 & JVET-P0885 (exactly the same as P0884)

Test configuration

| # | Sequence name | Resolution | Fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| S01 | Cactus | 1920x1080 | 50fps | 500 | RA | 32, 37 |
| S02 | ParkScene | 1920x1080 | 24fps | 240 | RA | 32, 37 |
| S03 | BQMall | 832x480 | 60fps | 600 | LDB | 32, 37 |
| S04 | RaceHorsesC | 832x480 | 30fps | 300 | RA | 32, 37 |
| S05 | Johnny | 1280x720 | 60fps | 600 | LDB | 32, 37 |

Session 2 on CIIP with TPM

Tools under test: CE4-1.9

Test configuration: 2 QP, 5 sequences

| # | Sequence name | Resolution | Fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| S01 | Cactus | 1920x1080 | 50fps | 500 | RA | 32, 37 |
| S02 | ParkScene | 1920x1080 | 24fps | 240 | RA | 32, 37 |
| S03 | BQMall | 832x480 | 60fps | 600 | LDB | 32, 37 |
| S04 | RaceHorsesC | 832x480 | 30fps | 300 | RA | 32, 37 |
| S05 | Johnny | 1280x720 | 60fps | 600 | LDB | 32, 37 |

Session 3 on adaptive blending for screen content

Tools under test: a) CE4-1.7, and b) CE4-1.8.

Test configuration: 2 QP, 3 sequences

| # | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| S01 | Desktop_420 | 1920x1080 | 60fps | 600 | RA | 32, 37 |
| S02 | Desktop_444 | 1920x1080 | 60fps | 600 | LDB | 32, 37 |
| S03 | Console_444 | 1920x1080 | 60fps | 600 | LDB | 32, 37 |

Detailed description of tests

- Test 4-1.1, 4-1.2 and 4-1.14 in JVET-P0068 (Huawei, RWTH Aachen)

Basic idea

Geometric partition mode extends the current triangle partition mode to support various binary partitions other than the current diagonal and anti-diagonal partition mode. As shown in the figure below.

VVC-6.0 TPM            Extend to GEO

The split boundary of geometric merge mode is descripted by angle φi and distance offset ρi as shown in the figure below. Angle φi represents a quantized angle between 0 and 360 degrees and distance offset ρi represents a quantized offset of the largest distance ρmax. In addition, the split directions overlapped with binary tree splits and TPM splits are excluded.

Syntax change

| if( !ciip_flag[ x0 ][ y0 ] && MaxNumTriangleMergeCand > 1 ) { |  |
| --- | --- |
| if (cbWidth > = 8 && cbHeight >= 8 ) |  |
| merge_geo_flag[ x0 ][ y0 ] | ae(v) |
| if ( merge_geo_flag[ x0 ][ y0 ] ) |  |
| merge_geo_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |

The merge_geo_flag[][] is sinalged with 4 CABAC context models. The geo_partition_idx[][] is used as an index to the lookup table that stores the angle φi and distance ρi pairs. The geo_partition_idx coded truncated binary and binarized using bypass.

Angles and distances

|  | CE4-1.1 | CE4-1.2a | CE4-1.2b |
| --- | --- | --- | --- |
| Number of angles | 32 | 32 | 24 |
| Number of distance | 5 | 4 | 4 |
| Number of partitions | 140 | 108 | 80 |

Max distance ρmax is calculated and then quantified to derive the 4 or 5 distances in the table.

ρmaxφ,w,h=cos(φ)h2tanπ2-φ+w2-1.0, 0<φ<π2 				(1)

Directions close to the vertical direction is removed to reduce the number angles from 32 to 24 as illustrated below.

Blending of two predictions

The blending operation itself is the same as TPM, applying the 3-bit mask for weighted prediction of pixels around the partition boundary.

The derivation of blending mask is illustrated by the three equations below,

distFromLine = ((x<<1) + 1)*Dis[displacementX] + ((y<<1) + 1))*Dis[displacementY] –rho	(3)

distScaled = Clip3(0, 26, (abs(distFromLine) + 4) >> 3)						(4)

sampleWeightL[ x ][ y ] = distFromLine <= 0 ? GeoFilter[distScaled] : 8 – GeoFilter[distScaled]	(5)

Where 3 look-up tables, Dis[.] with 32 entries, StepDis[.] with 36 entries and GeoFilter[.] with 26 entries are involved.

Motion vector storage

In CE4-1.1 and CE4-1.2, luma sample weights at the four corners of a 4x4 motion storage unit is summed up. Then the sum is compared with 2 thresholds to decide whether one of two uni-prediction motion information or bi-prediction motion information is stored. The bi-prediction motion information is derived using the same process as TPM.

In CE4-1.14, the distance between the central position of a 4x4 motion storage unit and the split boundary is calculated and compared with a fixed threshold to decide whether uni- or bi prediction motion information is stored.

- Test 4-1.5 and 4-1.6 in JVET-P0074 (Huawei)

Basic idea

Align residual signal sampling with residual transform so that the residual signal is localized within a PU by the means of a PT area. The method is close to the one known as sub-block transform (SBT). The shape of the area to be transformed could be a parallelogram. The parallelogram of residual, the PT area, is further warped to a rectangular shape and then then existing transform mechanism applied.

The proposed partial transform is applied to GEO and TPM blocks. Either the width or the height of PT area must be the same as the width or the height of the CB. The other side of PT, either height or width, could be ½ or ¼ size of the height or width of the CB.

Syntax

| if( tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| --- | --- |
| if (partial_transform enabled && (MergeTriangleFlag ¦¦ geo_flag[ x0 ][ y0 ] ) && !cu_skip_flag[ x0 ][ y0 ] && !cu_skip_flag[ x0 ][ y0 ] && !cu_transquant_bypass_flag) { |  |
| part_trans_flag[ x0 ][ y0 ] | ae(v) |
| if (part_trans_flag) |  |
| part_trans_idx[ x0 ][ y0 ] | ae(v) |
| } |  |

In test CE4-1.5, partial transform size could be ½ or ¼ that is indicated by the syntax element part_trans_idx. If it is assign to 0, partial transform size is ½. Otherwise, partial transform size is ¼. PT size denotes the size of the shorter side of the PT area. Part_trans_idx is not signaled for CE4-1.6, where ½ fixed-size partial transform is applied. So, the syntax is as follows:

| if( tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| --- | --- |
| if (partial_transform enabled && (MergeTriangleFlag ¦¦ geo_flag[ x0 ][ y0 ] ) && !cu_skip_flag[ x0 ][ y0 ] && !cu_skip_flag[ x0 ][ y0 ] && !cu_transquant_bypass_flag) |  |
| part_trans_flag[ x0 ][ y0 ] | ae(v) |
| … |  |

Derivation of PT area

Starting and ending points of the PT line that defines a partial transform area (PT area) is derived from the parameters of the GEO splitting line and the shape of the PU. The PT area is specified by this PT line. In particular, for each position along this line, the number of samples, which is equal to ½ or ¼ of the shorter side, are selected column-wise in the case when the shorter side is the left or right side of the PU, or row-wise in case the shorter side is the top side or bottom side of the PU. The center position of the selected samples is the position on the PT line.

- Test 4-1.7 and 4-1.8 in JVET-P0069 (Huawei, RWTH Aachen)

TPM blending operation is over-smoothing when the content is known to include sharp edges, hence may harm screen content coding. Proposed solution introduces a sequence level and a slice level control flag to disable the blending filtering operation. When the blending filtering is disabled, the sample weight for weighted combination can either be 0 or 8, the weighting matrix example is shown as in figure below.

In Test 4-1.7, the blending off method is applied to TPM.

In Test 4-1.8, the blending off method is applied to both TPM and GEO.

- Test 4-1.9 in JVET-P0071 (BBC)

CIIP with triangular partitions are proposed. An intra predictor and an inter predictor are computed for a block. The inter predictor and the intra predictor are combined using the partitioning scheme as in TPM mode. The intra predictor is used in one of the two triangular partitions, and the inter predictor is used in the other triangular partition. The blending scheme as used in TPM is used to blend the two triangular partitions.

Three possible Triangle CIIP modes are considered with different triangle split and/or usage of inter or intra prediction in the two partitions, as illustrated below.

- Test 4-1.10 in JVET-P0070 (BBC, Huawei, RWTH Aachen)

This test is a combination of test 4-1.9 and the geometric partition.

- Test 4-1.12 and 4-1.13 in JVET-P0075 (Huawei, BBC)

Test 4-1.12 is a combination of test 4-1.9 and the partial transform in test 4-1.5.

Test 4-1.13 is a combination of test 4-1.9 and the partial transform in test 4-1.5 applied to GEO, TPM and CIIP with triangular partitions.

CE4-2: Simplification on PROF and BDOF

Test specification

| Test # | Description | Source |
| --- | --- | --- |
| 4-2.1 | 1/32-pel precision of PROF motion refinement + [-32, 31] for PROF clipping range | JVET-P0057 |
| 4-2.2 | [-32, 31] for PROF clipping range | JVET-P0057 |

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4-2.1 | 0.01% | 0.02% | 0.01% | 100% | 99% | 0.02% | 0.01% | 0.04% | 100% | 100% |
| 4-2.2 | 0.01% | 0.05% | -0.05% | 101% | 101% | 0.02% | 0.00% | 0.04% | 100% | 101% |

Crosscheck report

| Test# | Crosschecker | SW match CE description (Y/N) | Text match SW (Y/N) | Simulation results (Y/N) | EncT/DecT match (Y/N) |
| --- | --- | --- | --- | --- | --- |
| 4-2.1 | G. Li | Y | Y | Y | Y |
| 4-2.2 | G. Li | Y | Y | Y | Y |

SW match CE description: Y SW implementation match CE description, N mismatch.

Text match SW: Y text match SW, N mismatch.

Simulation results: Y substantial results (all but class A for RA, all but class B for LDB) obtained and match results from tester, N mismatch or no substantial results.

EncT/DecT match: Y encoder and decoder run time match those from tester, N mismatch.

Detailed description of tests

- Test 4-2.1 and 4-1.2 in JVET-P0057 (Kwai, Qualcomm, Mediatek, InterDigital)

Common modifications in both tests,

- The range of BDOF motion refinements is changed from [-32, 32] to [-32, 31],

Modifications specific to test 4-2.1,

- PROF motion refinement is clipped into the range of [-32, 31], and

- The precision of the PROF motion refinement is changed from 1/64-pel to 1/32-pel.

Modifications specific to test 4-2.2,

- PROF motion refinement is clipped into the range of [-32, 31].

Non-CE4/CE4-related contributions

Geometric motion partition (10)

| JVET number | Title | Tag |
| --- | --- | --- |
| JVET-P0107 | CE4-Related: Geometric Merge Mode (GEO) Simplifications | SIMPLIFICATION |
| JVET-P0174 | Non-CE4: Modifications of TPM and GEO | TPM UNIFICATION |
| JVET-P0248 | CE4-related: Modification of GEO mode | TPM UNIFICATION |
| JVET-P0250 | CE4-related: On simplification for GEO weight derivation | SIMPLIFICATION |
| JVET-P0264 | CE4-related: Simplification of GEO using angles with power-of-two tangents | SIMPLIFICATION |
| JVET-P0304 | CE4-related: Simplification of blending weights and motion field storage in geometric merge mode | SIMPLIFICATION |
| JVET-P0305 | CE4-related: Unification of triangle partition mode and geometric merge mode | TPM UNIFICATION |
| JVET-P0583 | CE4-related: On the SBT in CE4-1.1 | TPM UNIFICATION |
| JVET-P0884 | Simplified GEO without multiplication and minimum blending mask storage (harmonization of JVET-P0107, JVET-P0264 and JVET-P0304) | SIMPLIFICATION |
| JVET-P0885 | Simplified GEO without multiplication and minimum blending mask storage (harmonization of JVET-P0107, JVET-P0264 and JVET-P0304) | SIMPLIFICATION |

Affine motion compensation (3)

| JVET number | Title | Tag |
| --- | --- | --- |
| JVET-P0209 | Non-CE4: On the affine AMVR |  |
| JVET-P0261 | Non-CE4: On Affine Motion Vector Restriction |  |
| JVET-P0600 | CE4-related: Support large rotation and flipping in affine and PROF |  |

Merge mode modifications (23)

| JVET number | Title | Tag |
| --- | --- | --- |
| JVET-P0137 | Non-CE4: On coding of merge triangle index | TPM |
| JVET-P0155 | CE4-related: An encoder switch for forcing temporal MV to zero in SbTMVP | OTHERS |
| JVET-P0214 | CE4-related: Simplification of the CIIP chroma Intra Prediction Mode | CIIP |
| JVET-P0249 | CE4-related: On simplification for CIIP with triangular partitions | CIIP |
| JVET-P0275 | Non-CE4/AHG17: On slice-level syntax for TPM and GEO | TPM |
| JVET-P0278 | Non-CE4: HMVP buffer update for TPM block | TPM |
| JVET-P0285 | Non-CE4:Intra prediction mode of CIIP selection using left adjacent coded block | CIIP |
| JVET-P0286 | Non-CE4:Intra prediction mode of CIIP selection using left and above adjacent coded block | CIIP |
| JVET-P0287 | Non-CE4:Modefied MVD derivation method for bidirectional MMVD using the magnitude of MV | MMVD |
| JVET-P0288 | Non-CE4:Modefied MVD derivation method for bidirectional MMVD using the magnitude of MV and POC distance | MMVD |
| JVET-P0291 | AHG17: MMVD and CIIP parameters in PPS or slice header | HLS |
| JVET-P0309 | Non-CE4: Improved signaling method for merge modes | OTHERS |
| JVET-P0322 | Non-CE4: CIIP size restriction | CIIP |
| JVET-P0325 | Construction of spatial merge candidates | OTHERS |
| JVET-P0384 | non-CE4 flexible MMVD candidates | MMVD |
| JVET-P0387 | non-CE4 flexible CIIP mode | CIIP |
| JVET-P0416 | Non-CE4: Using integer MV in CIIP | CIIP |
| JVET-P0449 | CE-4 related: Thin Partition Mode | TPM |
| JVET-P0498 | Non-CE4: On weight derivation process in CIIP | CIIP |
| JVET-P0499 | Non-CE4: Removal of MVD scaling process in MMVD | MMVD |
| JVET-P0530 | Non-CE4: Alignment of luma and chroma weight calculation for TPM blending | TPM |
| JVET-P0595 | Non-CE4: Neighboring locations for CIIP | CIIP |
| JVET-P0629 | CE4-related: CIIP Simplification | CIIP |

Document JVET-P0152 and JVET-P0605 is about fixing bugs introduced by the constant slice header parameter set adopted in the previous meeting. The two contributions fit into the summary report of HLS proposals in JVET-P0687. And therefore they are not included in the table above though the document title indicates them as CE4 related.

Document JVET-P0175 talks about changes on shared merge list, which only applies to IBC mode. And therefore it is not included in the table above though the document title indicates them as CE4 related.

Document JVET-P0325 propose modifications of merge mode, and therefore is put in the table above though it is not indicated as CE4 related in the title.

Document JVET-P0291 propose additional control parameters for MMVD and CIIP in PPS and slice header, and is suggested to be discussed in the context of inter prediction.

Decoder side motion vector derivation (36)

| JVET number | Title | Tag |
| --- | --- | --- |
| JVET-P0089 | Non-CE4: DMVR control by reference picture type | ONOFF_CONTROL |
| JVET-P0091 | CE4-related:Simplification of PROF and BDOF | VAR_SHIFT |
| JVET-P0094 | AHG16/Non-CE4: Fix on overflow issue in PROF | VAR_CLIP |
| JVET-P0153 | CE4-related: Overlapped block optical flow | OTHERS |
| JVET-P0154 | CE4-related: PROF prediction sample range reduction | VAR_CLIP |
| JVET-P0172 | AHG16/Non-CE4: Prediction sample value clipping for PROF | VAR_CLIP |
| JVET-P0191 | Non-CE4: Enabling BDOF and DMVR according to reference picture types | ONOFF_CONTROL |
| JVET-P0200 | Non-CE4: Modifications on BDOF intermediate parameter derivation | VAR_SHIFT |
| JVET-P0201 | Non-CE4: Simplification on BDOF offset calculation | VAR_SHIFT |
| JVET-P0237 | Non-CE4: On conditions for PROF | ONOFF_CONTROL |
| JVET-P0238 | CE4-related: On definition of motion vector range for PROF and BDOF | VAR_CLIP |
| JVET-P0253 | Non-CE4: Simplify the division process of DMVR | OTHERS |
| JVET-P0263 | Non-CE4: On PROF conditions | ONOFF_CONTROL |
| JVET-P0279 | Non-CE4: Interaction between PROF and other tools | ONOFF_CONTROL |
| JVET-P0281 | Non-CE4: Corrections on parameter calculation for PROF and BDOF | VAR_CLIP |
| JVET-P0282 | Non-CE4: Block size alignment between PROF and BDOF | ONOFF_CONTROL |
| JVET-P0310 | Non-CE4: On simplification of PROF and BDOF | OTHERS |
| JVET-P0311 | Non-CE4: On enabling condition of BDOF and DMVR | ONOFF_CONTROL |
| JVET-P0314 | Non-CE4: Harmonization of PROF, BDOF and DMVR syntax | ONOFF_CONTROL |
| JVET-P0320 | Non-CE4/AHG17: On slice-level syntax for BDOF and DMVR | OTHERS |
| JVET-P0408 | CE4-related: On PROF on/off control | ONOFF_CONTROL |
| JVET-P0413 | CE4-related: Clipping for PROF | VAR_CLIP |
| JVET-P0415 | Non-CE4: Unification of DMVR and BDOF enabling conditions | ONOFF_CONTROL |
| JVET-P0434 | Non-CE4: Enabling conditions of PROF | ONOFF_CONTROL |
| JVET-P0443 | Non-CE4: On DMVR early termination | OTHERS |
| JVET-P0444 | Non-CE4: On DMVR without padding | OTHERS |
| JVET-P0491 | CE4-related: On MVD derivation in PROF | VAR_CLIP |
| JVET-P0518 | AHG16/Non-CE4: Addressing 16-bit multiplication overflow issue of the PROF | VAR_CLIP |
| JVET-P0519 | Non-CE4: On SAD threshold for BDOF early termination | OTHERS |
| JVET-P0524 | On high-level control flags of BDOF and PROF | ONOFF_CONTROL |
| JVET-P0544 | Non-CE4: A slice header flag disabling PROF | ONOFF_CONTROL |
| JVET-P0546 | Non-CE4: An applying condition of BDOF | ONOFF_CONTROL |
| JVET-P0598 | CE4-related: Harmonized CU-level Condition Check in BDOF and DMVR | ONOFF_CONTROL |
| JVET-P0601 | CE4-related: Support slice level disabling for PROF | ONOFF_CONTROL |
| JVET-P0653 | Non-CE4: On BDOF and PROF parameter derivation | VAR SHIFT |
| JVET-P0654 | Non-CE4: Unified BDOF and DMVR early termination threshold | OTHERS |

Document JVET-P0524 is about slice level on/off control of PROF on which there are several other contributions in the table above. And therefore this contribution is put here though it is not indicated as CE4 related in the title.

Weighted prediction (5)

| JVET number | Title | Tag |
| --- | --- | --- |
| JVET-P0234 | AHG-2/Non-CE4: On clarification of weightedPredFlag |  |
| JVET-P0236 | Non CE4: On a simplification for triangle merge mode |  |
| JVET-P0280 | Non-CE4: Fix the behavior between BCW and WP |  |
| JVET-P0317 | Non-CE4: BCW clean-up for weight signaling |  |
| JVET-P0617 | Non-CE4: On TPM merge mode in the presence of weighted prediction |  |

Switchable interpolation filter (15)

| JVET number | Title | Tag |
| --- | --- | --- |
| JVET-P0151 | CE4-related: Simplification of half-pel switchable interpolation filter | IDX_DERIVATION |
| JVET-P0260 | CE4-related: On inheritance of half-pel interpolation filter in merge mode | IDX_DERIVATION |
| JVET-P0340 | Non-CE4: Unification of merge interpolation filter (triangle and pairwise-average) | IDX_DERIVATION |
| JVET-P0341 | Non-CE4: Coding of interpolation filter index in non-merge inter mode | IDX_DERIVATION |
| JVET-P0414 | Non-CE4: Simplification of SIF | OTHERS |
| JVET-P0439 | Non-CE4: Motion pruning with alternative half-pel interpolation filter flag | MV_PRUNING |
| JVET-P0440 | Non-CE4: On inheritance of alternative half-pel interpolation filter flag in MMVD | IDX_DERIVATION |
| JVET-P0461 | Non-CE4: Usage of half-pel switchable interpolation filter for pairwise candidate | IDX_DERIVATION |
| JVET-P0490 | Non-CE4: Cleanup of half-pel switchable interpolation filter | IDX_DERIVATION |
| JVET-P0496 | Non-CE4: Switchable interpolation filter for TPM | IDX_DERIVATION |
| JVET-P0513 | Non-CE4: Extending switchable interpolation filter to affine mode | OTHERS |
| JVET-P0532 | CE4-related: Cleanup of applicable condition of alternative interpolation filter for half-sample position in merge mode. | IDX_DERIVATION |
| JVET-P0533 | CE4-related: Cleanup of applicable condition of alternative interpolation filter for half-sample position in merge mode. | IDX_DERIVATION |
| JVET-P0542 | Non-CE4: Cleanup on alternative half-pel interpolation filter | IDX_DERIVATION |
| JVET-P0621 | Non-CE4: On motion information comparison | MV_PRUNING |

Others (10)

| JVET number | Title | Tag |
| --- | --- | --- |
| JVET-P0090 | Non-CE4: 32 bits coding for abs_mvd_minus2 |  |
| JVET-P0094 | AHG16/Non-CE4: Fix on overflow issue in PROF |  |
| JVET-P0312 | Non-CE4: On condition of SMVD |  |
| JVET-P0385 | Non-CE4: MV rounding in sbTMVP |  |
| JVET-P0442 | Non-CE4: Unified AMVR signaling |  |
| JVET-P0445 | Non-CE4: Encoder optimization for subblock-based merge candidate search |  |
| JVET-P0512 | Non-CE4: SIMD support for motion compensated prediction at high internal bit-depth |  |
| JVET-P0541 | Non-CE4: Context modeling for inter prediction mode |  |
| JVET-P0604 | Non-CE4: Motion field storage optimization for the line buffer |  |
| JVET-P0620 | non-CE4: flexible sbt mode |  |

Recommendations

Define the subjective evaluation of CE4-1, including the selection of representative CE 4-1 tests, the selection of test sequences and the encoder configuration for the generation of all subtests. And then conduct the subjective evaluation.

Discuss CE4 tests and review all Non-CE4/CE4-related contributions.