| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0026-v1 |
| --- | --- |

| Title: | CE6: Summary Report on Transforms and Transform Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xin Zhao Hilmi E. Egilmez Mehdi Salehifar | Tel: Email: | xinzzhao@tencent.com hegilmez@qti.qualcomm.com mehdi.salehifar@lge.com |
| Source: | CE6 coordinators |  |  |

_____________________________

# Abstract

This contribution summarizes the activities of Core Experiment (CE) on Transforms and Transform Signaling. The goal of this CE is to study transform design and signaling for the VVC standard. The CE studies were divided into two categories, including:

(1) CE6-1: LFNST with one mode (6 tests), targeting for encoder run-time saving.

(2) CE6-2: LFNST with reduced kernels (5 tests), targeting for memory cost reduction (all tests), unification (CE6-2.2 a/c/d) and multiplication reduction (CE6-2.2d, CE6-2.3).

In this CE all experiments were done using based on the VTM-6.0 SW. This document summarizes the test results, brief experiment definition, cross-check reports and complexity measurements, and also related contributions.

# Introduction

The purpose of this Core Experiment (CE) is to investigate the coding tools related to transforms and transform signaling on top of the VTM-6.0. In this CE, the tradeoff between coding efficiency and complexity of different transformation tools will be investigated.

# Experiment List

Core experiments (CEs) are organized according to the following categories, and the following table lists all the experiments in each category, and the corresponding input document to the Geneva meeting. In total 11 tests were studied in this CE.

The 11 tests reported in this CE6 summary are listed below.

| Test # | Doc # | Description | Tester | Cross-checker |
| --- | --- | --- | --- | --- |
| 6-1.1a | JVET-P0041 | LFNST kernel selection according to the equation in JVET-O0292 with normative change | M.-S. Chiang (MediaTek) J. Lainema (Nokia) | M. Koo (LGE) |
| 6-1.1b | JVET-P0041 | LFNST kernel selection according to the equation in JVET-O0292 with encoder-only change | M.-S. Chiang (MediaTek) J. Lainema (Nokia) | M. Koo (LGE) |
| 6-1.1c | JVET-P0041 | LFNST kernel selection according to the equation in JVET-O0292 with encoder-only change and the last bit used to signal LFNST index is context coded. | M.-S. Chiang (MediaTek) J. Lainema (Nokia) | M. Koo (LGE) |
| 6-1.1d | JVET-P0041 | LFNST kernel selection according to the equation in JVET-O0350 with normative change | J. Lainema (Nokia) M.-S. Chiang (MediaTek) | M. Koo (LGE) |
| 6-1.1e | JVET-P0041 | LFNST kernel selection according to the equation in JVET-O0350 with encoder-only change | J. Lainema (Nokia) M.-S. Chiang (MediaTek) | M. Koo (LGE) |
| 6-1.1f | JVET-P0041 | LFNST kernel selection according to the equation in JVET-O0350 with encoder-only change and the last bit used to signal LFNST index is context coded. | J. Lainema (Nokia) M.-S. Chiang (MediaTek) | M. Koo (LGE) |
| CE6-2.1 | JVET- P0044 | Reduce 4 transform sets to 3 transform sets - set 0 for DC / Planar - set 1 for angular modes between 19 and 49 - set 3 for the other angular modes | Y. Zhao (Huawei) | M. Salehifar (LGE) |
| CE6-2.2a | JVET-P0051 | LFNST with the original 16x48 kernels defined in VTM-6.0 and apply them to all block sizes | X. Zhao (Tencent) | M.Siekmann (HHI) M. Salehifar (LGE) H. E. Egilmez (Qualcomm) |
| CE6-2.2c | JVET-P0051 | Combined test of CE6-2.2a and CE6-2.1 | X. Zhao (Tencent) | M.Siekmann (HHI) M. Salehifar (LGE) |
| CE6-2.2d | JVET-P0052 | Combination of CE6-2.2a and CE6-2.3a - For larger block sizes, the new kernels in CE6-2.3a are applied - For 4xN and Nx4 blocks, the new kernels in CE6-2.3a are applied, but only pick the overlapping part with current block as output, as proposed in CE6-2.2a | X. Zhao (Tencent) T. Zhou (Sharp) | M.Siekmann (HHI) M. Salehifar (LGE) H. E. Egilmez (Qualcomm) |
| CE6-2.3 | JVET-P0065 | Simplification of the low frequency separable transform using 36x16 RST matrices | T. Zhou (Sharp) | M.Siekmann (HHI) M. Salehifar (LGE) X. Zhao (Tencent) C. Rosewarne (Canon) |

# Test conditions

## Software

The Test Model VTM-6.0 shall be used for these experiments, as specified in JVET-N1010 [1].

## Test configurations

For each test in this CE, it is mandatory to provide coding results under common test conditions (CTC) over VTM-6.0, and the results shall be compared against the anchor obtained using VTM-6.0 under CTC.

The following requirements apply to all tests defined in this CE:

- For experiments having an impact on the transform precision, i.e., modifications on transform cores, results using low-QP configurations, i.e., QP 2, 7, 12 and 17 shall be provided. For low-QP configuration, simulation results using All Intra configuration are mandatory and the total number of frames to be coded is modified using the following encoder configuration: --FramesToBeEncoded=100.

- Any encoder or decoder optimizations (e.g., SIMD or encoder optimizations) that are not an essential part of the proposed method shall be described, and individual experiment results on the related optimizations shall be reported.

Proponents and non-proponents are encouraged to report results under non-CTC conditions.

# Timeline

The following table lists the timeline for the CE6 study planned in the meeting.

| T1 | Aug. 02, 2019 | - CE description is finalized and uploaded |
| --- | --- | --- |
| T2 | Sep. 02, 2019 Two weeks after VTM-6.0 software release | - Proponents provide initial software of CE tools - Cross-checkers may start initial study of tools/software |
| T3 | Sep. 10, 2019 Three weeks before the next JVET meeting or one week after T2, whichever date is later | - Software of each CE test is finalized - Cross-checkers start final cross checks |
| T4 | Sep. 17, 2019 One week before the regular document submission deadline | - CE contribution documents including specification text and complete results shall be uploaded to the JVET repository (particularly for proposals targeting to be promoted to the draft standard at the next meeting) - At the same time, crosscheckers shall send their findings out |
| T5 | Sep. 25, 2019 Regular document submission deadline | - The CE summary report shall be available. This shall include documentation about crosscheck of software, matching of CE description and confirmation of the appropriateness of the text changes as well as sufficient crosscheck results to create evidence about correctness (crosscheckers must send this information to the CE coordinator at least 3 days ahead of the document deadline) |

It is reported that the initial SW of all CE tools has been committed to associated branches by T2 (Sept. 2nd) and finalized by T3 (Sept. 10), and contributions of all CE tests have been registered and uploaded by T4 (Sept. 17).

# Summary of results

The following table summarizes the results of CE6 tests using CTC configuration and VTM-6.0 as anchor.

|  | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-1.1a | 0.11% | 0.66% | 0.82% | 90% | 101% | 0.09% | 0.49% | 0.50% | 97% | 102% |
| CE6-1.1b | 0.53% | 0.99% | 1.06% | 92% | 101% | 0.29% | 0.53% | 0.63% | 98% | 103% |
| CE6-1.1c | 0.48% | 0.99% | 1.11% | 91% | 101% | 0.27% | 0.50% | 0.56% | 98% | 103% |
| CE6-1.1d | 0.11 % | 0.64 % | 0.80 % | 90% | 99% | 0.07 % | 0.48 % | 0.49 % | 98% | 100% |
| CE6-1.1e | 0.53 % | 0.97 % | 1.10 % | 91% | 99% | 0.29 % | 0.53 % | 0.60 % | 98% | 99% |
| CE6-1.1f | 0.46 % | 0.97 % | 1.08 % | 91% | 99% | 0.27 % | 0.53 % | 0.56 % | 98% | 100% |
| CE6-2.1 | 0.03% | 0.06% | 0.11% | 100% | 101% | -0.01% | 0.06% | 0.03% | 100% | 100% |
| CE6-2.2a | -0.02% | 0.03% | 0.07% | 100% | 98% | -0.02% | 0.06% | 0.04% | 100% | 99% |
| CE6-2.2c | 0.01% | 0.12% | 0.14% | 100% | 98% | -0.01% | -0.01% | -0.01% | 100% | 99% |
| CE6-2.2d | -0.01% | 0.14% | 0.16% | 99% | 96% | -0.02% | 0.12% | 0.03% | 99% | 97% |
| CE6-2.3 | 0.02% | 0.09% | 0.10% | 100% | 100% | 0.03% | 0.11% | 0.05% | 100% | 101% |

Comparison on memory saving for CE6-2 tests:

| Test | Memory cost |
| --- | --- |
| VTM-6.0 | 8KB |
| CE6-2.1 | 6KB (25% reduction) |
| CE6-2.2a | 6KB (25% reduction) |
| CE6-2.2c (CE6-2.2a + CE6-2.1) | 4.6KB (44% reduction) |
| CE6-2.2d (CE6-2.2a + CE6-2.3) | 4.6KB (44% reduction) |
| CE6-2.3 | 6.7KB (19% reduction) |

The following table summarizes the results of CE6 tests using Low QP configuration and VTM-6.0, as specified in JVET-O2026.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT |
| CE6-1.1a |  |  |  |  |  |
| CE6-1.1b |  |  |  |  |  |
| CE6-1.1c |  |  |  |  |  |
| CE6-1.1d |  |  |  |  |  |
| CE6-1.1e |  |  |  |  |  |
| CE6-1.1f |  |  |  |  |  |
| CE6-2.1 |  |  |  |  |  |
| CE6-2.2a | 0.02% | 0.04% | 0.04% | 98% | 98% |
| CE6-2.2c | 0.04% | 0.01% | 0.03% | 98% | 97% |
| CE6-2.2d | 0.03% | 0.03% | 0.04% | 98% | 97% |
| CE6-2.3 | 0.00% | -0.01% | 0.00% | 100% | 100% |

# Complexity Analysis

For the understanding of complexity, the following information shall be provided for each test planned for this CE:

- Operation counts (number of multiplications, additions, and bit-shift operations) for individual transforms. If encoder and decoder use different number of transform operations as compared to the VTM, operation counts shall be reported for both encoder and decoder separately.

- Memory requirements for storing transform cores (in bytes).

- List of all combinations of transforms block size and transform type used for primary/secondary transformation that are different as compared to VTM.

For the understanding of training transform kernels applied in CE test, if a re-training is performed to obtain a new transform kernel(s) in a CE test, the following questionnaire shall be filled by the Proponent (or Tester) by T3, and the Crosschecker by T4.

|  | Tester | Crosschecker |
| --- | --- | --- |
| Training sequence set (sequences outside CTC test set should be used) | Which sequence set has been used for training the LFNST kernels? | Whether it has been confirmed that the training sequences provided by Tester are used for deriving the modified LFNST kernels applied in CE test. |
| Training software | Whether the software for deriving the LFNST kernels given the training data has been provided. | Whether the training software has been received and confirmed for deriving the modified LFNST kernels applied in CE test. |
| VTM software version used for retrieving training data (test configuration, e.g. CTC) | Provide the VTM software version which is used for retrieving the training data used for generating LFNST kernels. | Whether it has been confirmed that the VTM software version provided by Tester is used for retrieving training data which is used for generating the LFNST kernels applied in CE test. |

Proponents and non-proponents are encouraged to provide analysis on the orthogonality of LFNST kernels.

## CE6-1.1

- Operation counts.

  - Same with VTM-6.0.

- Memory requirements for storing transform cores (in bytes).

  - Same with VTM-6.0.

- List of all combinations of transforms block size and transform type used for primary/secondary transformation that are different as compared to VTM.

  - TBA.

## CE6-2.1

- Operation counts.

  - Same with VTM-6.0.

- Memory requirements for storing transform cores (in bytes).

  - CE6-2.1: 6.1 KB for storing LFNST matrices, i.e., 2.0 KB (25%) reduction.

- List of all combinations of transforms block size and transform type used for primary/secondary transformation that are different as compared to VTM.

  - TBA.

## CE6-2.2

- Operation counts.

  - Worst case same with VTM-6.0, i.e., 8-mults per sample.

- Memory requirements for storing transform cores (in bytes).

  - CE6-2.2a: 6.1 KB for storing LFNST matrices, i.e., 2.0 KB (25%) reduction.

  - CE6-2.2c: 4.6 KB for storing LFNST matrices, i.e., 3.6 KB (44%) reduction.

  - CE6-2.2d: 4.6 KB for storing LFNST matrices, i.e., 3.6 KB (44%) reduction.

- List of all combinations of transforms block size and transform type used for primary/secondary transformation that are different as compared to VTM.

  - Same with VTM-6.0.

## CE6-2.3

- Operation counts.

  - TBA

- Memory requirements for storing transform cores (in bytes).

  - CE6-2.3: 6.7 KB for storing LFNST matrices, i.e., 1.5 KB (19%) reduction.

- List of all combinations of transforms block size and transform type used for primary/secondary transformation that are different as compared to VTM.

  - Same with VTM-6.0.

# Test descriptions

CE6-1.1: JVET-O0292 CE6-related: Simplifications for LFNST [M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)] and JVET-O0350 CE6-related: LFNST with one mode [J. Lainema (Nokia)]

It is proposed to remove the option of lfnst_idx equal to 2 for encoding time reduction. lfnst_idx equal to 0 means LFNST is not used and lfnst_idx equal to 1 means LFNST is used. The LFNST kernel in a transform set is specified according to a pre-defined method in terms of the intra prediction mode of the current CU. Different methods with normative and encoder-only change are tested. The method described in JVET-O0292 is used in test CE6-1.1.a and CE6-1.1.b and the method described in JVET-O0350 is used in test CE6-1.1.c and CE6-1.1.d.

Method described in JVET-O0292:

lfnst_idx 0: LFNST is not used

lfnst_idx 1: The LFNST kernel A, where A is 1 or 2, is selected by intra prediction mode (INTRA_LT_CCLM, INTRA_L_CCLM, INTRA_T_CCLM, INTRA_PLANAR, INTRA_DC, INTRA_ANGULAR2~INTRA_ANGULAR66).

If intra prediction mode is 0~34, A is set to be “Intra prediction mode % 2 == 0 ? 1 : 2.”

Otherwise if intra prediction mode is 35~83, A is set to be “Intra prediction mode % 2 == 0 ? 2 : 1.”

Method described in JVET-O0350:

lfnst_idx 0: LFNST is not used

lfnst_idx 1: The LFNST kernel index lfnstIdx, where lfnstIdx is 1 or 2, is calculated using the variable predModeIntra specifying the intra prediction mode for LFNST set selection as follows:

lfnstIdx = ( predModeIntra % 2 ) + 1

CE6-2.1: JVET-O0195 Non-CE6: Simplification on LFNST [Y. Zhao, H. Yang (Huawei)]

It is proposed to use 3 transform sets. Transform set 2 (thus the matrices related to this set) is removed. Accordingly, the mapping table from predModeIntra to lfnstTrSetIdx is changed to Table 1. LFNST transform matrices are not modified. Because of the reduction of transform matrices from 4 sets to 3 sets, 25% LFNST matrix memory (i.e., 2KB ) is saved.

Table 1 Mapping table with three transform sets

| predModeIntra | lfnstTrSetIdx |
| --- | --- |
| predModeIntra < 0 | 1 |
| 0 <= predModeIntra <= 1 | 0 |
| 2 <= predModeIntra <= 18 | 1 |
| 19 <= predModeIntra <= 49 | 3 |
| 50 <= predModeIntra <= 80 | 1 |
| 81 <= predModeIntra <= 83 | 0 |

According to the new mapping table, directional prediction modes with reference samples from both left and top sides uses transform set 3, and those with reference samples from only one side uses transform set 1, as illustrated in Fig. 1.

Fig. 1 Transform set mapping with 4 sets in VTM5 (left) and the proposed method with 3 sets (right)

CE6-2.2: JVET-O0539 CE6-related: Unified LFNST using block size independent kernel [X. Zhao, X. Li, S. Liu (Tencent)]

The current LFNST design applies two different sizes of non-separable secondary transform kernel, the one with size 16x16 is applied for 4xN and Nx4 blocks, and the other one with size 16x48 is applied for larger blocks. To unify the LFNST design, it is proposed to use only the LFNST kernels for all block sizes. For 4xN and Nx4 blocks, only the overlapping part between the current block and the output of the LFNST is calculated.

Combination tests with CE6-2.1 and CE6-2.3 are also performed.

CE6-2.3: JVET-O0620 Non-CE6: Simplification with new LFNST transform basis [T. Zhou, T. Hashimoto, T. Ikai (Sharp)] [miss] [late]

This test studies the reduction of LFNST matrices by replacing 48x16 matrices with 36x16 matrices. The corresponding position of 36 coefficients are decided depending on stIdx.

The training sequences and software released at the software finalization deadline, i.e. T3.

CE6-2.3: Use 36x16 matrices

8 6 6 6 4 4 1 1 6 6 6 6 4 4 4 0 ( lfnstIdx == 0) ( lfnstIdx == 1). Figure 2: 36 coefficients pattern.

# Crosscheck Reports

The following table reports the crosschecker’s output on each CE6 test.

| Test # | Crosscheckers | Implementation is aligned with the algorithm description? | Does crosschecking confirm whether ALL data points match with the results provided by proponent? | Additional comments? |
| --- | --- | --- | --- | --- |
| CE6-1.1a | M. Koo (LGE) | Yes | Yes (CTC) |  |
| CE6-1.1b | M. Koo (LGE) | Yes | Yes (CTC) |  |
| CE6-1.1c | M. Koo (LGE) | Yes | Yes (CTC) |  |
| CE6-1.1d | M. Koo (LGE) | Yes | Yes (CTC) |  |
| CE6-1.1e | M. Koo (LGE) | Yes | Yes (CTC) |  |
| CE6-1.1f | M. Koo (LGE) | Yes | Yes (CTC) |  |
| CE6-2.1 | M.Salehifar (LGE) | Yes | Yes (CTC) |  |
| CE6-2.2a | M.Siekmann (HHI) | Yes | Yes |  |
|  | M. Salehifar (LGE) | Yes | Yes (CTC) |  |
|  | H. E. Egilmez (Qualcomm) | Yes | Yes (CTC) | Additional aspects are crosschecked in JVET-P0594. |
| CE6-2.2c | M.Siekmann (HHI) | Yes | Yes |  |
|  | M. Salehifar (LGE) | Yes | Yes (CTC) |  |
| CE6-2.2d | M.Siekmann (HHI) | Yes | Yes |  |
|  | M. Salehifar (LGE) | Yes | Yes (CTC) |  |
|  | H. E. Egilmez (Qualcomm) | Yes | Yes (CTC) | Additional aspects are crosschecked in JVET-P0594. |
| CE6-2.3 | M.Siekmann (HHI) | Yes | Yes |  |
|  | M. Salehifar (LGE) | Yes | Yes (CTC) |  |
|  | X. Zhao (Tencent) | Yes | Yes |  |
|  | J. Gan (Canon) | Yes | Yes |  |

The crosscheck report on the training process used in CE6-2.3 are summarized as follows:

|  | Training sequence set (sequences outside CTC test set should be used) | Training software | VTM software version used for retrieving training data (test configuration, e.g. CTC) |
| --- | --- | --- | --- |
| Tester T. Zhou (Sharp) | Training sequence set can be available in: ftp://ftp.ient.rwth-aachen.de/ce/ JVET-O2026_Transforms/ ftp://ftp.hhi.fraunhofer.de/ce/ JVET-O2026_Transforms/ | Yes | VTM-6.0 |
| M.Siekmann (HHI) | Confirmed | Confirmed | Confirmed |
| M. Koo (LGE) | Confirmed | Confirmed | Confirmed |
| X. Zhao (Tencent) | Confirmed | Confirmed | Confirmed |
| J. Gan (Canon) | Confirmed | Confirmed | Confirmed |

# CE related contributions

# JVET-P0166 CE6-related: Transform selection with intra prediction mode for implicit MTS [M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

# JVET-P0196 CE6-related: Latency reduction for LFNST signaling [J. Lainema (Nokia)]

# JVET-P0197 CE6-related: LFNST transform mapping [J. Lainema (Nokia)]

# JVET-P0215 CE6 related: Harmonization of CE6-1.1 and two-mode LFNST [X. Zhao, X. Li, S. Liu (Tencent)]

# JVET-P0216 Non-CE6 : Simplified SBT mode coding [J. Park, B. Jeon (SKKU)]

# JVET-P0259 Non-CE6: On LFNST reduced kernels [J. Gan, C. Rosewarne (Canon)]

# JVET-P0266 Non-CE6: LFNST signaling [C. Rosewarne, J. Gan (Canon)]

# JVET-P0271 Non-CE6: Context modeling for LFNST index coding [T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)]

# JVET-P0273 Non-CE6: On MTSIntraMaxCand [C. Hollmann, D. Saffar, J. Ström (Ericsson)]

# JVET-P0276 Non-CE6:On Supporting 64x64 Chroma Transform Unit with Composite VPDU [Wenting Cai, Jianqing Zhu, Jie Yao, K. Kazui]

# JVET-P0301 Non-CE6: Decoding process of implicit TU partitioning [G. Ko, D. Kim, J. Jung, J. Son, J. Kwak (WILUS)]

# JVET-P0313 Non-CE6: Simplification of LFNST LUT [S. Shrestha, A. Kumar, B. Lee (Chosun Univ.), Y. Lee, J. Park (Humax)]

# JVET-P0342 CE6-related: LFNST applied to ISP mode [F. Le Leannec, K. Naser, F. Galpin]

# JVET-P0346 Non-CE6: SPS Clean-up of SBT [Karam Naser, Tangi Poirier, Fabrice Le Léannec, Frank Galpin (InterDigital)]

# JVET-P0349 CE6 Related: MIP Adaptation On Top of CE6-1.1d [Karam Naser, Frank Galpin, Tangi Poirier, Ya Chen (InterDigital)]

# JVET-P0350 CE6 Related: Modified LFNST Index Coding For Fast Encoder Implementation [Karam Naser, Frank Galpin, Tangi Poirier, Ya Chen (InterDigital)]

# JVET-P0352 Non-CE6 / Non-CE3: MIP UP TO 64x64 CU’s [Karam Naser, Tangi Poirier, Fabrice Le Léannec, Frank Galpin (InterDigital)]

# JVET-P0354 Non-CE6: LFNST Up to 64x64 CU’s [Karam Naser, Tangi Poirier, Fabrice Le Léannec, Frank Galpin (InterDigital)]

# JVET-P0376 Non-CE6: Refined LFNST restriction with MIP [H. Yang, Y. He (InterDigital)]

# JVET-P0379 Non-CE6: Unified zero-out range for 4x4 LFNST [K. Fan, L. Zhang, K. Zhang, Y. Wang (Bytedance)]

# JVET-P0391 Non-CE6: A clean-up of SPS maximum SBT size signaling [M. G. Sarwer, R. L. Liao, J. Luo, Y. Ye (Alibaba)]

# JVET-P0392 Non-CE6: Combination of ISP and LFNST [S. De-Luxán-Hernández, V. George, G. Venugopal, J. Brandenburg, B. Bross, H. Schwarz, D. Marpe, T. Wiegand (HHI), M. Koo, M. Salehifar, J. Lim, S. Kim (LGE)]

# JVET-P0405 Non-CE6: Cleanups of maximum transform size related syntax elements [Z. Deng, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)]

# JVET-P0493 CE6-related: Further simplification with new LFNST transform basis []

# JVET-P0495 Non-CE6: On implicit MTS [Y. Wang, H. Liu, L. Zhang, K. Zhang, Z. Deng, Y. Wang (Bytedance)]

# JVET-P0501 Non-CE6: Cleanup on MTS related HLS [X. Zhao, X. Li, S. Liu (Tencent)]

# JVET-P0527 Non-CE6: On context model for coding last NZ position []

# JVET-P0538 Non-CE6: Transform skip with fixed transform shift [J. Xu, L. Zhang, W. Zhu, K. Zhang, H. Liu, Z. Deng (Bytedance)]

# JVET-P0540 Non-CE6: Disabling MTS for 64xN and Nx64 CUs []

# JVET-P0545 CE6-2.3-related: further reduced 8x8 LFNST matrices [K. Fan, L. Zhang, K. Zhang, Y. Wang (Bytedance)]

# JVET-P0566 CE6-related: On LFNST Support Patterns [H. E. Egilmez, A. Said, V. Seregin, M Karczewicz (Qualcomm)]

# JVET-P0567 CE6-related: Optimized LFNST matrices for CE6-2.1 [H. E. Egilmez, A. Said, V. Seregin, M. Karczewicz (Qualcomm)]

# JVET-P0568 CE6-related: An LFNST Index Signaling with Bin Prediction [H. E. Egilmez, V. Seregin, A. Nalci, A. Said, M. Karczewicz (Qualcomm)]

# JVET-P0569 AHG17/Non-CE6: High-level syntax for MTS and Implicit Transform Derivations [H. E. Egilmez, V. Seregin, M. Karczewicz (Qualcomm)]

# References

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-N1010, 14th JVET meeting, Geneva, 19–27 Mar. 2019.

- H. E. Egilmez, M. Salehifar, X. Zhao, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-O2026, 15th Meeting: Gothenburg, SE, 3–12 July 2019.