| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0046-v1 |
| --- | --- |

| Title: | CE7-1.1 and CE7-2.1: Improving transform skip residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Tzu-Der Chuang, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, peter.chuang, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of CE7-1.1 and CE7-2.1 for improving transform skip residual coding. In CE7-1.1, the syntax element par_level_flag is signalled in a separate pass after signalling abs_level_gtx[n]. In CE7-2.1, the syntax element par_level_flag is further entropy coded conditioned on the partially reconstructed absolute level value of the current coefficient. The luma BD bitrate results are summarized as follows:

Over standard QPs 22, 27, 32, 37

- CE7-1.1: -0.06% (AI), -0.02% (RA), and -0.02% (LB) for Class F, and -0.04% (AI), -0.04% (RA), and -0.04% (LB) for Class SCC

- CE7-2.1: -0.10% (AI), -0.08% (RA), and -0.07% (LB) for Class F, and -0.19% (AI), -0.14% (RA), and -0.17% (LB) for Class SCC

Over low QPs 2, 7, 12, 17

- CE7-1.1: -0.09% (AI), -0.05% (RA), and -0.13% (LB) for Class F, and -0.27% (AI), -0.30% (RA), and -0.20% (LB) for Class SCC

- CE7-2.1: -0.12% (AI), -0.26% (RA), and -0.06% (LB) for Class F, and -0.32% (AI), -0.31% (RA), and -0.28% (LB) for Class SCC

# Introduction

This contribution reports the results of CE7-1.1 and CE7-2.1 based on the proposed methods in [1] for improving transform skip residual coding.

# CE description

In CE7-1.1, the syntax elements abs_level_gtx_flag[n][j], j = 1..4, is signalled in a single pass. The syntax par_level_flag is signalled in a separate pass after signalling abs_level_gtx[n].

In CE7-2.1, the syntax element par_level_flag is entropy coded conditioned on the partially reconstructed absolute level value of the current coefficient. This test is conducted on top of CE7-1.1.

The context increment, ctxInc, for entropy coding par_level_flag is derived by

ctxInc = (AbsLevelPassX < 10) ? ((AbsLevelPassX – 2 ) >> 1) : 4,

where AbsLevelPassX is the partially reconstructed level of the current coefficient and is given by

AbsLevelPassX =	sig_coeff_flag[n] + abs_level_gtX[n][0] +

2 * (abs_level_gtX[n][1] + abs_level_gtX[n][2]+ abs_level_gtX[n][3]+ abs_level_gtX[n][4]).

# Experimental results

The proposed method was integrated into the VTM6.0 reference software [2]. The coding results generated by the proposed method were evaluated against the anchor results generated by the unmodified VTM6.0 reference software under the SCC common test conditions (CTCs) defined for CE7 in [3]. Tables 1 and 2 summarize the experimental results for CE7-1.1 and CE7-2.1, respectively, conducted under the CTCs over standard QPs 22, 27, 32, and 37. Tables 3 and 4 summarize the experimental results for CE7-1.1 and CE7-2.1, respectively, conducted under the CTC over low QPs 2, 7, 12, and 17. Please note the run time results may not be accurate in the tables. The detailed simulation results can be found in the attached spreadsheets.

Table 1: Summary of the average BD bitrate results of CE7-1.1 over standard QPs 22, 27, 32, 37 by coding par_level_flag[n] in a separate subblock coding pass after coding abs_level_gtx_flag[n][j]

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | -0.01% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | -0.01% | 100% | 99% |
| Class B | 0.00% | -0.02% | -0.01% | 101% | 99% |
| Class C | 0.00% | -0.08% | 0.09% | 99% | 98% |
| Class E | 0.00% | 0.04% | -0.02% | 100% | 101% |
| Overall | 0.00% | -0.01% | 0.01% | 100% | 99% |
| Class D | -0.01% | -0.02% | -0.06% | 98% | 99% |
| Class F | -0.06% | -0.09% | -0.07% | 102% | 100% |
| TGM | -0.04% | -0.03% | -0.05% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | 0.00% | 100% | 102% |
| Class A2 | 0.04% | 0.12% | 0.01% | 100% | 102% |
| Class B | 0.00% | -0.02% | -0.13% | 100% | 100% |
| Class C | 0.00% | 0.15% | 0.01% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.06% | -0.04% | 100% | 100% |
| Class D | -0.04% | -0.51% | 0.03% | 100% | 99% |
| Class F | -0.02% | -0.08% | -0.03% | 100% | 95% |
| TGM | -0.04% | -0.10% | -0.07% | 100% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.01% | -0.50% | 100% | 108% |
| Class C | -0.04% | -0.14% | -0.01% | 102% | 104% |
| Class E | -0.02% | -0.39% | -0.09% | 100% | 104% |
| Overall | -0.02% | -0.14% | -0.23% | 101% | 106% |
| Class D | 0.01% | -0.07% | -0.23% | 104% | 105% |
| Class F | -0.02% | -0.07% | 0.30% | 99% | 98% |
| TGM | -0.04% | 0.00% | 0.07% | 101% | 100% |

Table 2: Summary of the average BD bitrate results of CE7-2.1 over standard QPs 22, 27, 32, 37 by coding par_level_flag[n] in a separate subblock coding pass after coding abs_level_gtx_flag[n][j] with additional contexts

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.02% | 104% | 102% |
| Class A2 | 0.00% | 0.01% | 0.02% | 103% | 105% |
| Class B | 0.01% | 0.01% | -0.05% | 103% | 102% |
| Class C | 0.01% | -0.12% | 0.04% | 101% | 100% |
| Class E | 0.01% | 0.04% | -0.09% | 99% | 98% |
| Overall | 0.00% | -0.02% | -0.01% | 102% | 101% |
| Class D | -0.01% | -0.06% | -0.05% | 100% | 97% |
| Class F | -0.10% | -0.05% | -0.14% | 100% | 97% |
| TGM | -0.19% | -0.16% | -0.14% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | 0.02% | 106% | 103% |
| Class A2 | 0.01% | 0.05% | 0.10% | 105% | 96% |
| Class B | -0.01% | -0.03% | 0.05% | 102% | 98% |
| Class C | -0.02% | 0.08% | 0.11% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.07% | 103% | 99% |
| Class D | -0.07% | -0.11% | -0.16% | 100% | 106% |
| Class F | -0.08% | -0.11% | -0.06% | 101% | 106% |
| TGM | -0.14% | -0.16% | -0.16% | 102% | 93% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.14% | -0.30% | 101% | 98% |
| Class C | -0.07% | -0.06% | -0.06% | 103% | 101% |
| Class E | -0.15% | 0.58% | 0.03% | 101% | 102% |
| Overall | -0.06% | 0.19% | -0.13% | 101% | 100% |
| Class D | -0.04% | -0.38% | 0.10% | 101% | 105% |
| Class F | -0.07% | -0.06% | 0.45% | 100% | 102% |
| TGM | -0.17% | -0.07% | -0.08% | 100% | 100% |

Table 3: Summary of the average BD bitrate results of CE7-1.1 over low QPs 2, 7, 12, 17 by coding par_level_flag[n] in a separate subblock coding pass after coding abs_level_gtx_flag[n][j] (will update results of camera captured contents for RA)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | 0.00% | 99% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 96% | 96% |
| Class B | 0.00% | 0.00% | 0.01% | 95% | 98% |
| Class C | 0.00% | 0.00% | 0.02% | 98% | 99% |
| Class E | 0.01% | -0.02% | 0.00% | 96% | 96% |
| Overall | 0.00% | 0.00% | 0.00% | 97% | 98% |
| Class D | 0.00% | -0.02% | 0.00% | 95% | 96% |
| Class F | -0.09% | -0.06% | -0.09% | 95% | 96% |
| TGM | -0.27% | -0.25% | -0.25% | 99% | 93% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F | -0.05% | -0.09% | -0.09% | 101% | 104% |
| TGM | -0.30% | -0.25% | -0.26% | 102% | 105% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.00% | -0.01% | 97% | 97% |
| Class C | -0.01% | 0.01% | 0.00% | 98% | 95% |
| Class E | 0.00% | -0.02% | -0.02% | 96% | 100% |
| Overall | -0.01% | 0.00% | -0.01% | 97% | 97% |
| Class D | -0.02% | 0.00% | 0.05% | 98% | 93% |
| Class F | -0.13% | -0.09% | -0.15% | 97% | 96% |
| TGM | -0.20% | -0.14% | -0.09% | 99% | 95% |

Table 4: Summary of the average BD bitrate results of CE7-2.1 over low QPs 2, 7, 12, 17 by coding par_level_flag[n] in a separate subblock coding pass after coding abs_level_gtx_flag[n][j] with additional contexts (will update results of camera captured contents for RA)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | 0.00% | 98% | 96% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class B | 0.00% | -0.01% | 0.00% | 101% | 102% |
| Class C | 0.00% | 0.01% | 0.02% | 101% | 101% |
| Class E | 0.01% | -0.03% | -0.02% | 100% | 101% |
| Overall | 0.00% | -0.01% | 0.00% | 100% | 100% |
| Class D | 0.00% | -0.03% | 0.00% | 101% | 100% |
| Class F | -0.12% | -0.09% | -0.10% | 101% | 102% |
| TGM | -0.32% | -0.26% | -0.25% | 102% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F | -0.26% | -0.29% | -0.30% | 101% | 102% |
| TGM | -0.31% | -0.25% | -0.23% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.00% | 0.00% | 98% | 95% |
| Class C | 0.00% | 0.03% | -0.03% | 97% | 96% |
| Class E | 0.01% | -0.05% | -0.02% | 96% | 98% |
| Overall | 0.00% | 0.00% | -0.01% | 97% | 96% |
| Class D | -0.03% | 0.07% | 0.03% | 98% | 97% |
| Class F | -0.06% | -0.06% | -0.03% | 103% | 99% |
| TGM | -0.28% | -0.15% | -0.09% | 103% | 97% |

# Conclusions

This contribution reports the results of CE7-1.1 and CE7-2.1 with the proposed modifications for coding the syntax element par_level_flag. The proposed method reportedly achieves -0.19%, -0.14%, and -0.17% luma BD-rates for the AI, RA, and LB settings, respectively, for Class SCC under the SCC CTCs for CE7 over standard QPs, and achieves -0.32%, -0.31%, and -0.28% luma BD-rates for the AI, RA, and LB settings, respectively, for Class SCC under the SCC CTCs for CE7 over low QPs.

# References

- S.-T. Hsiang and S.-M. Lei, “CE7-related: Modified subblock coding passes for coding TS residual,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-O00295, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- VTM6.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM6.0

- H. Schwarz and M. Coban, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-O2027, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).