| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0049-r2 |
| --- | --- |

| Title: | CE7-4.1: Coding of chroma cbf flags |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Biao Wang, Han Gao, Anand Meher Kotra, Elena Alshina | Tel: Email: | semih.esenlik@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

The current contribution proposes an alternative way of coding the coded block flags for Cb and Cr blocks. Based on the statistical observation, a first flag (tu_cbf_cb_or_cr) indicating whether Cb or Cr  blocks have residual, and a second flag (tu_cbf_cb_and_cr) indicating that both Cb and Cr blocks have residual are included in the syntax. The joint CbCr residual flag will be signalled when tu_cbf_cb_or_cr flag is true.

The objective results according to CTC with the above cbf coding scheme are indicated in table 1 below:

| Config. | Y | U | V | EncT | DecT | (8Y+U+V)/10 |
| --- | --- | --- | --- | --- | --- | --- |
| AI | 0.08% | -0.67% | -0.48% | 103% | 103% | -0.05% |
| RA | 0.14% | -1.15% | -1.39% | 103% | 102% | -0.14% |
| LDB | 0.03% | -0.41% | -2.08% | 103% | 102% | -0.23% |

Table-1: Average BD rate results for CTC

# Problem Statement

The coded block flag signalling in VVC draft 6 is depicted in Table1 below. tu_joint_cbcr_residual_flag, tu_cbf_cb and tu_cbf_cr flags control the presence and absence of the CB and CR block residuals in the table.

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) && ChromaArrayType != 0 ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex = = NumIntraSubPartitions − 1 ) ) ) { |  |
| tu_cbf_cb[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) && ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ¦ ¦ CbWidth[ chType ][ x0 ][ y0 ] > MaxTbSizeY ¦ ¦ CbHeight[ chType ][ x0 ][ y0 ] > MaxTbSizeY ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex < NumIntraSubPartitions − 1 ¦ ¦ !InferTuCbfLuma ) ) ) |  |
| tu_cbf_luma[ x0 ][ y0 ] | ae(v) |
| if (IntraSubPartitionsSplitType != ISP_NO_SPLIT ) |  |
| InferTuCbfLuma = InferTuCbfLuma && !tu_cbf_luma[ x0 ][ y0 ] |  |
| } |  |
| if( IntraSubPartitionsSplitType != ISP_NO_SPLIT && treeType = = SINGLE_TREE && subTuIndex = = NumIntraSubPartitions − 1 ) ) |  |
| xC = CbPosX[ chType ][ x0 ][ y0 ] |  |
| yC = CbPosY[ chType ][ x0 ][ y0 ] |  |
| wC = CbWidth[ chType ][ x0 ][ y0 ] / SubWidthC |  |
| hC = CbHeight[ chType ][ x0 ][ y0 ] / SubHeightC |  |
| } else |  |
| xC = x0 |  |
| yC = y0 |  |
| wC = tbWidth / SubWidthC |  |
| hC = tbHeight / SubHeightC |  |
| } |  |
| if( ( CbWidth[ chType ][ x0 ][ y0 ] > 64 ¦ ¦ CbHeight[ chType ][ x0 ][ y0 ] > 64 ¦ ¦ tu_cbf_luma[ x0 ][ y0 ] ¦ ¦ tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) && treeType != DUAL_TREE_CHROMA ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta_abs | ae(v) |
| if( cu_qp_delta_abs ) |  |
| cu_qp_delta_sign_flag | ae(v) |
| } |  |
| } |  |
| if( ( tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) { |  |
| if( cu_chroma_qp_offset_enabled_flag && !IsCuChromaQpOffsetCoded) { |  |
| cu_chroma_qp_offset_flag | ae(v) |
| if( cu_chroma_qp_offset_flag && chroma_qp_offset_list_len_minus1 > 0 ) |  |
| cu_chroma_qp_offset_idx | ae(v) |
| } |  |
| } |  |
| if( sps_joint_cbcr_enabled_flag && ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && ( tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) ) ¦ ¦ ( tu_cbf_cb[ x0 ][ y0 ] && tu_cbf_cr[ x0 ][ y0 ] ) ) ) |  |
| tu_joint_cbcr_residual_flag[ x0 ][ y0 ] | ae(v) |
| … |  |
| … |  |

Table-2: Syntax of chroma coded block flags according to VVC draft 6.

# Proposed Solution

Based on the statistical evidence, the proposal suggests to signal the coded block flags as shown in table 3 below (highlighted).

According to the proposal, tu_cbf_cb and tu_cbf_cr flags are replaced with tu_cbf_cb_or_cr and tu_cbf_cb_and_cr flags. The tu_cbf_cb_or_cr flags indicate whether at least one of Cb or Cr blocks have residual. If this flag is equal to one tu_joint_cbcr_residual is parsed, whose semantics are not changed. Afterwards;

- in the case of inter blocks, a second flag (tu_cbf_cb_and_cr) is parsed when tu_joint_cbcr_residual is equal to zero, and it indicates that both Cb and Cr blocks have residual and there is no special relationship between the residuals (i.e. Cb and Cr residuals are not jointly coded).

- in the case of intra blocks tu_cbf_cb_and_cr flag is parsed even if tu_joint_cbcr_residual flag is equal to 1.

Finally if tu_cbf_cb_and_cr flag is false, tu_cbf_cb is parsed to indicate which of the components (Cb or Cr) has residual.

tu_joint_cbcr_residual and tu_cbf_cb_and_cr flags are inferred to be equal to zero and tu_cbf_cb_or_cr, respectively, when not present in the bitstream.

## Proposed Syntax Changes

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) && ChromaArrayType != 0 ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex = = NumIntraSubPartitions − 1 ) ) ) { |  |
| tu_cbf_cb[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr [ x0 ][ y0 ] = tu_cbf_cb [ x0 ][ y0 ] = 0 |  |
| tu_cbf_cb_or_cr[ x0 ][ y0 ] | ae(v) |
| if( tu_cbf_cb_or_cr[ x0 ][ y0 ] ) { |  |
| if( sps_joint_cbcr_enabled_flag ){ |  |
| tu_joint_cbcr_residual[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr [ x0 ][ y0 ] = tu_cbf_cb [ x0 ][ y0 ] = tu_joint_cbcr_residual[ x0 ][ y0 ] |  |
| } |  |
| if( !tu_joint_cbcr_residual[ x0 ][ y0 ] ¦ ¦ CuPredMode[ chType ][ x0 ][ y0 ] == MODE_INTRA){ |  |
| tu_cbf_cb_and_cr[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr [ x0 ][ y0 ] = tu_cbf_cb [ x0 ][ y0 ] = tu_cbf_cb_and_cr[ x0 ][ y0 ] |  |
| } |  |
| if( !tu_cbf_cb_and_cr[ x0 ][ y0 ] ){ |  |
| tu_cbf_cb [ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr [ x0 ][ y0 ] = 1 − tu_cbf_cb [ x0 ][ y0 ] |  |
| } |  |
| } |  |
| } |  |

Table-3: Proposed syntax modifications

The semantics for the newly introduces syntax elements are as follows:

tu_cbf_cb_or_cr [ x0 ][ y0 ] equal to 1 specifies that at least on of the Cb transform block or Cr transform block contains one or more transform coefficient levels not equal to 0. tu_cbf_cb_or_cr [ x0 ][ y0 ] equal to 0 specifies that the Cb and Cr transform blocks contrain transform coefficient levels that are all equal to 0. The array indices x0, y0 specify the top-left location ( x0, y0 ) of the considered transform block.

When tu_cbf_cb_or_cr[ x0 ][ y0 ] is not present in the current CU, its value is inferred to be equal to 0.

tu_cbf_cb_and_cr [ x0 ][ y0 ] equal to 1 specifies that both of the Cb and Cr transform blocks contain one or more transform coefficient levels not equal to 0. The array indices x0, y0 specify the top-left location ( x0, y0 ) of the considered transform block.

When tu_cbf_cb_and_cr[ x0 ][ y0 ] is not present in the current CU, its value is inferred to be equal to tu_cbf_cb_or_cr [ x0 ][ y0 ].

# Simulation results

## Simulations results under CTC

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -1.38% | 0.37% | 102% | 102% |
| Class A2 | 0.13% | -1.61% | 0.46% | 103% | 102% |
| Class B | 0.07% | -0.33% | -1.22% | 103% | 103% |
| Class C | 0.08% | -0.34% | -0.59% | 103% | 104% |
| Class E | 0.04% | -0.01% | -0.87% | 103% | 103% |
| Overall | 0.08% | -0.67% | -0.48% | 103% | 103% |
| Class D | 0.07% | -0.32% | -0.30% | 104% | 107% |
| Class F | 0.09% | -0.33% | -0.63% | 104% | 104% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.23% | -2.13% | -0.32% | 103% | 101% |
| Class A2 | 0.18% | -1.85% | -0.11% | 103% | 101% |
| Class B | 0.09% | -0.76% | -2.41% | 103% | 102% |
| Class C | 0.09% | -0.37% | -1.89% | 103% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.14% | -1.15% | -1.39% | 103% | 102% |
| Class D | 0.02% | -1.09% | -0.84% | 103% | 105% |
| Class F | 0.09% | -0.25% | -0.59% | 103% | 104% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -0.49% | -2.97% | 103% | 102% |
| Class C | 0.07% | -0.58% | -2.07% | 103% | 102% |
| Class E | -0.08% | -0.06% | -0.62% | 102% | 102% |
| Overall | 0.03% | -0.41% | -2.08% | 103% | 102% |
| Class D | 0.07% | -0.62% | -0.85% | 102% | 102% |
| Class F | 0.11% | -0.04% | -0.09% | 102% | 102% |

## Simulations results with low QP setting

The simulations results for the low QP setting are provided according to the CE description for QP points 2, 7, 12, 17.. Only first 100 frames are encoded.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 lowQP(2, 7, 12, 17) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.08% | 0.09% | 101% | 99% |
| Class A2 | 0.02% | -0.02% | 0.01% | 100% | 99% |
| Class B | 0.03% | 0.05% | -0.05% | 101% | 99% |
| Class C | 0.06% | 0.12% | -0.02% | 100% | 99% |
| Class E | 0.05% | 0.06% | -0.07% | 101% | 99% |
| Overall | 0.04% | 0.03% | -0.01% | 101% | 99% |
| Class D | 0.04% | 0.11% | 0.06% | 100% | 99% |
| Class F (optional) | 0.08% | 0.09% | -0.11% | 100% | 99% |
|  | Random Access |  |  |  |  |
|  | Over VTM6.0 lowQP(2, 7, 12, 17) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.14% | -0.74% | 0.03% | 101% | 99% |
| Class A2 | 0.02% | -0.33% | 0.22% | 100% | 99% |
| Class B | 0.10% | 0.22% | -0.31% | 101% | 99% |
| Class C | 0.05% | 0.29% | -0.41% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | -0.07% | -0.16% | 101% | 99% |
| Class D | 0.06% | 0.13% | -0.24% | 101% | 99% |
| Class F (optional) | -0.02% | -0.02% | -0.52% | 100% | 99% |
|  | Low delay B |  |  |  |  |
|  | Over VTM6.0 lowQP(2, 7, 12, 17) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | 0.19% | -0.20% | 101% | 100% |
| Class C | 0.02% | 0.29% | -0.39% | 100% | 100% |
| Class E | 0.07% | 0.16% | -0.77% | 100% | 98% |
| Overall | 0.05% | 0.22% | -0.40% | 101% | 99% |
| Class D | 0.03% | 0.01% | -0.33% | 101% | 100% |
| Class F (optional) | -0.05% | 0.05% | -0.52% | 101% | 100% |

## Simulations results under HDR CTC

|  |  |  |  | All Intra |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over VTM-6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | wPSNR |  |  |  |  |  |  | PSNR |  |  |  |  |  |  |  |  |  |  |  |
|  |  | DE100 |  |  |  | PSNR-L100 |  |  | Y |  | U |  | V |  |  | Y |  | U |  | V |  |  | EncT |  |  | DecT |  |
| Class H1 |  | #VALUE! |  |  |  | #VALUE! |  |  | 0.13% |  | -1.01% |  | -4.27% |  |  | 0.13% |  | -1.09% |  | -5.04% |  |  | 101% |  |  | 100% |  |
| Class H2 |  |  |  |  |  | 0.08% | 0.45% | -2.42% | 100% | 100% |
| Overall |  | #VALUE! |  |  |  | #VALUE! |  |  | 0.13% |  | -1.01% |  | -4.27% |  |  | 0.11% |  | -0.53% |  | -4.08% |  |  | 100% |  |  | 100% |  |
|  |  |  |  | Random Access |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Over VTM-6.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | wPSNR |  |  |  |  |  |  | PSNR |  |  |  |  |  |  |  |  |  |  |  |
|  |  | DE100 |  |  |  | PSNR-L100 |  |  | Y |  | U |  | V |  |  | Y |  | U |  | V |  |  | EncT |  |  | DecT |  |
| Class H1 |  | #VALUE! |  |  |  | #VALUE! |  |  | 0.09% |  | -1.33% |  | -5.14% |  |  | 0.09% |  | -1.28% |  | -5.45% |  |  | 100% |  |  | 100% |  |
| Class H2 |  |  |  |  |  | 0.06% | 0.79% | -4.61% | 100% | 100% |
| Overall |  | #VALUE! |  |  |  | #VALUE! |  |  | 0.09% |  | -1.33% |  | -5.14% |  |  | 0.08% |  | -0.53% |  | -5.14% |  |  | 100% |  |  | 100% |  |

## Simulations results with modified JCCR lambda

During the CE testing the crosschecker proposed to modify the JCCR lambda to eliminate the loss in the luma channel. After the comments of the crosschecker the proponents realized that the JCCR uses modified lambda settings for intra and inter mode estimation stages. In VTM 6.0 the following lambda settings are used for the JCCR mode:

Inter prediction: JCCR lambda is 1.05 times regular chroma lambda.

Intra prediction: JCCR lambda is 1.3 times regular chroma lambda.

Therefore the proponents re-optimized the JCCR lambda, the following new settings are used:

Inter prediction: JCCR lambda is 1.25 times regular chroma lambda.

Intra prediction: JCCR lambda is 1.3 times regular chroma lambda.

The following are the results obtained according to the new lambda settings:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -1.38% | 0.37% | 100% | 100% |
| Class A2 | 0.13% | -1.61% | 0.46% | 100% | 100% |
| Class B | 0.07% | -0.33% | -1.22% | 100% | 100% |
| Class C | 0.08% | -0.34% | -0.59% | 100% | 100% |
| Class E | 0.04% | -0.01% | -0.87% | 100% | 100% |
| Overall | 0.08% | -0.67% | -0.48% | 100% | 100% |
| Class D | 0.07% | -0.32% | -0.30% | 100% | 100% |
| Class F (optional) | 0.09% | -0.33% | -0.63% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | -1.60% | 0.21% | 100% | 100% |
| Class A2 | -0.19% | -0.96% | 0.84% | 100% | 100% |
| Class B | 0.03% | -0.03% | -1.99% | 100% | 100% |
| Class C | 0.03% | 0.01% | -1.64% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.52% | -0.89% | 100% | 100% |
| Class D | -0.02% | -0.57% | -0.49% | 100% | 100% |
| Class F (optional) | 0.01% | -0.01% | -0.35% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | 2.31% | -0.83% | 100% | 100% |
| Class C | -0.06% | 0.56% | -1.30% | 100% | 99% |
| Class E | -0.08% | 0.87% | 0.56% | 100% | 100% |
| Overall | -0.06% | 1.37% | -0.64% | 100% | 100% |
| Class D | -0.03% | 1.16% | 1.37% | 100% | 100% |
| Class F (optional) | -0.09% | 0.96% | 0.39% | 100% | 99% |

# Conclusion

Based on statistical observations, a more efficient way of signalling the Coded block flag (cbf) information for Cb and Cr blocks is proposed. Considering the weighting factor of [6 1 1]/8 for [Y Cb Cr], the average BD-Rate gain of 0.08%, 0.22%, and 0.29% for the AI, RA and LDB configurations respectively according to CTC. It is recommended to adopt the proposed coded block flag signalling scheme in the next version of VVC.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).