| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0055-v1 |
| --- | --- |

| Title: | CE8-1.2: Compound Palette Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijia Zhu, Jizheng Xu, Li Zhang, Kai Zhang, Hongbin Liu, Yue Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-360-525-0535 weijia.zhu@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This document presents the results of CE 8-1.2 on the compound palette mode which combines palette mode and IBC prediction. The proposed method is implemented and evaluated with VTM6 by following CE8 test conditions. Results are reported as below:

Dual tree on:

TGM 1080p: AI: -0.33%, 127%, 102%; RA: -0.42%, 114%, 102%; LDB: -0.29%, 112%, 102%

Dual tree off:

TGM 1080p: AI: -1.31%, 124%, 103%; RA: -0.94%, 113%, 101%; LDB: -0.27%, 112%, 103%

# Introduction

VTM6 includes a palette mode coding which uses a base color representation to code screen contents. Although the palette coding can reduce redundancy in a block well, it cannot exploit non-local duplicated contents in screen pictures. IBC is another efficient tool on screen content coding which can exploit the repeating patterns in screen content coding. To take advantage of both IBC mode and palette mode, a new palette mode called compound palette mode is proposed in this contribution.

Compared with the palette mode, the samples in the compound palette mode can be reconstructed by copying the collocated sample in the IBC prediction when the current index is 0. Otherwise, the current sample will be reconstructed by the palette colors. Figure 1 illustrates the proposed compound palette mode.

Figure 1 An illustration of the proposed compound palette mode

# Proposed method

In the compound palette mode, the IBC prediction is generated by using the first available merge candidate, so there is no need to signal the motion information for the IBC prediction. In addition, the proposed method is only applied to luma blocks with both width and height are smaller than 64.

# Simulation Results

The proposed method is evaluated with VTM6. The simulation follows the CE 8 test condition [1]. The results are given in Table 1-6. More detailed results could be found in the attached spreadsheets.

Table 1. The performance of the proposed scheme versus VTM 6 under AI with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.33% | 0.00% | -0.04% | 127% | 102% |
| TGM 720p | 0.02% | -0.01% | -0.12% | 127% | 100% |
| Animation | 0.00% | 0.00% | 0.04% | 132% | 102% |
| Mixed content | 0.00% | -0.01% | 0.05% | 128% | 101% |
| Camera-Captured content | 0.00% | 0.06% | -0.01% | 131% | 101% |
| Overall | -0.08% | 0.00% | -0.02% | 129% | 101% |

Table 2. The performance of the proposed scheme versus VTM 6 under RA with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.42% | -0.25% | -0.19% | 114% | 102% |
| TGM 720p | -0.05% | -0.15% | -0.09% | 113% | 105% |
| Animation | 0.04% | 0.01% | -0.01% | 114% | 103% |
| Mixed content | 0.00% | 0.01% | 0.04% | 112% | 100% |
| Camera-Captured content | 0.00% | 0.09% | 0.11% | 113% | 102% |
| Overall | -0.11% | -0.09% | -0.05% | 113% | 102% |

Table 3. The performance of the proposed scheme versus VTM 6 under LDB with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.29% | -0.56% | -0.31% | 112% | 102% |
| TGM 720p | 0.06% | -0.12% | 0.27% | 109% | 100% |
| Animation | 0.00% | 0.06% | -0.10% | 112% | 101% |
| Mixed content | -0.20% | 0.14% | 0.08% | 112% | 103% |
| Camera-Captured content | -0.02% | -0.03% | -0.09% | 113% | 102% |
| Overall | -0.10% | -0.14% | -0.03% | 111% | 102% |

Table 4. The performance of the proposed scheme versus VTM 6 under AI with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -1.31% | -1.55% | -1.69% | 124% | 103% |
| TGM 720p | -0.02% | -0.45% | -0.61% | 123% | 101% |
| Animation | 0.01% | 0.04% | 0.06% | 125% | 99% |
| Mixed content | -0.21% | -1.02% | -0.93% | 125% | 102% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 127% | 101% |
| Overall | -0.37% | -0.68% | -0.74% | 124% | 101% |

Table 5. The performance of the proposed scheme versus VTM 6 under RA with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.94% | -1.01% | -0.78% | 113% | 101% |
| TGM 720p | -0.40% | -0.80% | -0.68% | 114% | 103% |
| Animation | -0.04% | -0.08% | -0.04% | 114% | 101% |
| Mixed content | -0.31% | -0.38% | -0.62% | 112% | 101% |
| Camera-Captured content | 0.00% | 0.00% | 0.01% | 113% | 103% |
| Overall | -0.40% | -0.54% | -0.49% | 113% | 102% |

Table 6. The performance of the proposed scheme versus VTM 6 under LDB with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.27% | -0.34% | -0.25% | 112% | 103% |
| TGM 720p | 0.03% | -0.41% | -0.27% | 111% | 100% |
| Animation | -0.03% | 0.02% | 0.03% | 111% | 100% |
| Mixed content | -0.08% | 0.53% | -0.22% | 110% | 102% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 113% | 102% |
| Overall | -0.08% | -0.08% | -0.16% | 111% | 102% |

# Text changes

The text changes below are on top of JVET-O2001-vE [2]. Changes are highlight by grey. Deleted texts are marked with strikethrough.

#### 7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA? 1 : 0 |  |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ¦ ¦ sps_palette_enabled_flag) { |  |
| … |  |
| if( ( ( ( slice_type = = I ¦ ¦ ( cbWidth = = 4 && cbHeight = = 4 ) ¦ ¦ sps_ibc_enabled_flag ) && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) ¦ ¦ ( slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) && !sps_ibc_enabled_flag && CuPredMode[ x0 ][ y0 ] != MODE_INTRA ) ) && sps_palette_enabled_flag && cbWidth <= 64 && cbHeight <= 64 && && cu_skip_flag[ x0 ][ y0 ] = = 0 && modeType != MODE_INTER ) |  |
| pred_mode_plt_flag | ae(v) |
| } |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_PLT ) { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( pred_mode_plt_flag ) { |  |
| if(cbWidth < 64 && cbHeight < 64 ) |  |
| plt_ibc_flag | ae(v) |
| if( treeType = = DUAL_TREE_LUMA ) |  |
| palette_coding( x0, y0, cbWidth, cbHeight, 0, 1 ) |  |
| else /* SINGLE_TREE */ |  |
| palette_coding( x0, y0, cbWidth, cbHeight, 0, 3 ) |  |
| } else { |  |
| … |  |
| } |  |
| … |  |
| } |  |

plt_ibc_flag specifies the use of palette mode in the current coding unit. plt_ibc_flag equal to 1 indicates that compound palette mode is applied in the current coding unit. plt_ibc_flag equal to 0 indicates that compound palette mode is not applied in the current coding unit. When plt_ibc_flag is not present, it is inferred to be equal to 0.

palette_escape_val_present_flag equal to 1 specifies that the current coding unit contains at least one escape coded sample. escape_val_present_flag equal to 0 specifies that there are no escape coded samples in the current coding unit. When not present, the value of palette_escape_val_present_flag is inferred to be equal to 1.

The variable MaxPaletteIndex specifies the maximum possible value for a palette index for the current coding unit. The value of MaxPaletteIndex is set equal to CurrentPaletteSize[ startComp ] − 1 + palette_escape_val_present_flag + (plt_ibc_flag ? 1 : 0) .

#### 8.4.5.3 Decoding process for palette mode

Inputs to this process are:

- a location ( xCb, yCb ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

- a variable startComp specifies the first colour component in the palette table,

- a variable cIdx specifying the colour component of the current block,

- two variables nCbW and nCbH specifying the width and height of the current block, respectively.

Output of this process is an array recSamples[ x ][ y ], with x = 0.. nCbW − 1, y = 0.. nCbH − 1 specifying reconstructed sample values for the block.

Depending on the value of cIdx, the variables nSubWidth and nSubHeight are derived as follows:

- If cIdx is equal to 0, nSubWidth is set to 1 and nSubHeight is set to 1.

- Otherwise, nSubWidth is set to SubWidthC and nSubHeight is set to SubHeightC.

If plt_ibc_flag is equal to 1 the following applies:

  - The derivation process for IBC luma motion vector prediction as specified in clause 8.6.2.2 is invoked with the luma location ( xCb, yCb ), the variables cbWidth and cbHeight inputs, merge_idx[ xCb ][ yCb ] is set as the first valid BV merge index, and the output being the luma motion vector mvL.

- The decoding process for ibc blocks as specified in clause 8.6.3.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight, the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY, the luma motion vectors mvL[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1, the variable cIdx set equal to 0 as inputs, and the ibc prediction samples (predSamples) that are an (cbWidth)x(cbHeight) array predSamplesL of prediction luma samples as outputs.

It is a requirement of bitstream conformance that at least one merge index indicates a valid luma block vector bvL and the bvL shall obey the following constraints:

- CtbSizeY is greater than or equal to ( ( yCb + ( bvL[ 1 ] >> 4 ) ) & ( CtbSizeY − 1 ) ) + cbHeight.

- IbcVirBuf[ 0 ][ ( x + (bvL[ 0 ] >> 4 ) ) & ( IbcVirBufWidth − 1 ) ][ ( y + (bvL[1] >> 4 ) ) & ( CtbSizeY − 1) ] shall not be equal to −1 for x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1

The ( nCbW x nCbH ) block of the reconstructed sample array recSamples at location ( xCb, yCb ) is represented by recSamples[ x ][ y ] with x = 0..nCTbW − 1 and y = 0..nCbH − 1, and the value of recSamples[ x ][ y ] for each x in the range of 0 to nCbW − 1, inclusive, and each y in the range of 0 to nCbH − 1, inclusive, is derived as follows:

- The variables xL and yL are derived as follows:

xL = palette_transpose_flag  ?  x * nSubHeight  :  x * nSubWidth	(8-268)

yL = palette_transpose_flag  ?  y * nSubWidth  :  y * nSubHeight	(8-269)

- The variable bIsEscapeSample is derived as follows:

- If PaletteIndexMap[ xCb + xL ][ yCb + yL ] is equal to MaxPaletteIndex and palette_escape_val_present_flag is equal to 1, bIsEscapeSample is set equal to 1.

- Otherwise, bIsEscapeSample is set equal to 0.

- If bIsEscapeSample is equal to 0, the following applies:

  - If plt_ibc_flag is 1, the following applies:

recSamples[ x ][ y ] = PaletteIndexMap[ xCb + xL ][ yCb + yL ] == 0 ? predSamples[ x ][ y ]:

CurrentPaletteEntries[ cIdx ][ PaletteIndexMap[ xCb + xL ][ yCb + yL ]  - 1]

  - Otherwise, the following applies:

recSamples[ x ][ y ] =   CurrentPaletteEntries[ cIdx ][ PaletteIndexMap[ xCb + xL ][ yCb + yL ] ]	(8-270)

…

It is a requirement of bitstream conformance that the value of PredictorPaletteSize[ startComp ] shall be in the range of 0 to PaletteMaxPredictorSize, inclusive.

#### 8.6.2.2 Derivation process for IBC luma block vector prediction

This process is only invoked when CuPredMode[ 0 ][ xCb ][ yCb ] is equal to MODE_IBC or plt_ibc_flag is 1, where ( xCb, yCb ) specify the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

…

#### 8.8.3.5 Derivation process of boundary filtering strength

Inputs to this process are:

- a picture sample array recPicture,

- a location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left sample of the current picture,

- a variable nCbW specifying the width of the current coding block,

- a variable nCbH specifying the height of the current coding block,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

- a variable cIdx specifying the colour component of the current coding block,

- a two-dimensional (nCbW)x(nCbH) array edgeFlags.

Output of this process is a two-dimensional (nCbW)x(nCbH) array bS specifying the boundary filtering strength.

…

For xDi with i = 0..xN and yDj with j = 0..yN, the following applies:

- If edgeFlags[ xDi ][ yDj ] is equal to 0, the variable bS[ xDi ][ yDj ] is set equal to 0.

- Otherwise, the following applies:

…

- The variable bS[ xDi ][ yDj ] is derived as follows:

…

    - Otherwise, if cIdx is equal to 0 and one or more of the following conditions are true, bS[ xDi ][ yDj ] is set equal to 1:

      - The coding subblock containing the sample p0 and the coding subblock containing the sample q0 are both coded in IBC prediction mode or their plt_ibc_flag are 1, and the absolute difference between the horizontal or vertical component of the block vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

      - For the prediction of the coding subblock containing the sample p0 different reference pictures or a different number of motion vectors are used than for the prediction of the coding subblock containing the sample q0.

NOTE 1 – The determination of whether the reference pictures used for the two coding sublocks are the same or different is based only on which pictures are referenced, without regard to whether a prediction is formed using an index into reference picture list 0 or an index into reference picture list 1, and also without regard to whether the index position within a reference picture list is different.

NOTE 2 – The number of motion vectors that are used for the prediction of a coding subblock with top-left sample covering ( xSb, ySb ), is equal to PredFlagL0[ xSb ][ ySb ] + PredFlagL1[ xSb ][ ySb ].

…

    - Otherwise, the variable bS[ xDi ][ yDj ] is set equal to 0.

### 6.4.4 Derivation process for neighbouring block availability

Inputs to this process are:

- the luma location ( xCurr, yCurr ) of the top-left sample of the current block relative to the top-left luma sample of the current picture,

- the luma location ( xNbY, yNbY ) covered by a neighbouring block relative to the top-left luma sample of the current picture,

- the variable checkPredModeY specifying whether availability depends on the prediction mode.

- the variable cIdx specifying the colour component of the current block.

Output of this process is the availability of the neighbouring block covering the location ( xNbY, yNbY ), denoted as availableN.

…

When all of the following conditions are true, availableN is set equal to FALSE.

–	checkPredModeY is equal to TRUE.

–	availableN is set equal to TRUE.

–	CuPredMode[ 0][ xNbY ][ yNbY ] is not equal to CuPredMode[ 0][ xCurr ][ yCurr ]

–  All of the following conditions are false

–   CuPredMode[ 0][ xNbY ][ yNbY ] is IBC, CuPredMode[0][ xCurr ][ yCurr ] is MODE_PLT, and plt_ibc_flag[ xCurr ][ yCurr ] is 1

–	CuPredMode[0][ xNbY ][ yNbY ] is MODE_PLT, plt_ibc_flag[ xNbY ][ yNbY ] is 1 and CuPredMode[0][ xCurr ][ yCurr ] is PLT

–	CuPredMode[ 0][ xNbY ][ yNbY ] is equal to CuPredMode[0][ xCurr ][ yCurr ]

# Conclusion

This document proposed a coding mode, called compound palette mode, to combine the palette mode and IBC prediction. Simulation results show that the average coding gain could be more than 1% with minor decoding complexity increase.

# References

- X. Xu, Y.-H Chao, Y.-C Sun and J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” JVET-O2028, Jul. 2019

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, Jul. 2019

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).