| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-Pxxxx |
| --- | --- |

| Title: | CE4: Harmonization of BDOF and PROF (Test 4-2.1 and 4-2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang Han Huang, Wei-Jung Chien, Vadim Seregin, and Marta Karczewicz Chen-Yen Lai, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei Yuwen He, Wei Chen | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, xianglinwang}@kwai.com {hanhuang, wchien, vseregin, martak}@qti.qualcomm.com {jenny.lai, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com {yuwen.he, wei.chen} @interdigital.com |
| Source: | Kwai Inc., Qualcomm Incorporated, MediaTek Inc., InterDigital Communications, Inc. |  |  |

_____________________________

# Proposed specification text change for CE4-2.2

#### 8.5.5.9 Derivation process for motion vector arrays from affine control point motion vectors

- The variables sbWidth and sbHeight, dmvLimit, posOffsetX and posOffsetY are derived as follows:

sbWidth  =  cbWidth / numSbX		(8-729)

sbHeight  =  cbHeight / numSbY		(8-730)

dmvLimit  =  1 << Max( 6, BitDepthY − 6 )5		(8-731)

posOffsetX  =  6 * dHorX + 6 * dVerX		(8-732)

posOffsetY  =  6 * dHorY + 6 * dVerY		(8-733)

- For x = 0..sbWidth − 1 and y = 0..sbHeight − 1, the following applies:

diffMv[ x ][ y ][ 0 ] = x * ( dHorX << 2 ) + y * ( dVerX << 2 ) – posOffsetX	(8-734)

diffMv[ x ][ y ][ 1 ] = x * ( dHorY << 2 ) + y * ( dVerY << 2 ) – posOffsetY	(8-735)

  - For i = 0..1, the following applieis:

    - The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to diffMv[ x ][ y ][ i ], rightShift set equal to 7, and leftShift set equal to 0 as inputs and the rounded diffMv[ x ][ y ][ i ] as output.

    - The value of diffMv[ x ][ y ][ i ] is clipped as follows:

diffMv[ x ][ y ][ i ] = Clip3( −dmvLimit, dmvLimit − 1, diffMv[ x ][ y ][ i ]) 	(8-736)

#### 8.5.6.5 Bi-directional optical flow prediction process

  - The horizontal and vertical motion offset of the current subblock are derived as:

vx = sGx2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres-1,	(8-817)
	−( sGxdI << 3 ) >> Floor( Log2( sGx2 ) ) )  :  0

vy = sGy2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres-1, ( ( sGydI << 3 ) − 	(8-818)
	( ( vx * sGxGym ) << 12 + vx * sGxGys ) >> 1 ) >> Floor( Log2( sGy2 ) ) )  :  0