| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0057 |
| --- | --- |

| Title: | CE4: Harmonization of BDOF and PROF (Test 4-2.1 and 4-2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang Han Huang, Wei-Jung Chien, Vadim Seregin, and Marta Karczewicz Chen-Yen Lai, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei Yuwen He, Wei Chen | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, xianglinwang}@kwai.com {hanhuang, wchien, vseregin, martak}@qti.qualcomm.com {jenny.lai, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com {yuwen.he, wei.chen} @interdigital.com |
| Source: | Kwai Inc., Qualcomm Incorporated, MediaTek Inc., InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of Core Experiment (CE) 4-2.1 and 4-2.2 on harmonization bidirectional optical flow (BDOF) and prediction refinement with optical flow (PROF). In CE4-2.1, the motion refinement precision of the PROF is modified to be the same as that of the BDOF, i.e., changing from 1/64-pel to 1/32-pel. Additionally, the range of the PROF motion refinement is clipped to [-32, 31]. In CE4-2.2, the range of the PROF motion refinement is directly clipped to [-32, 31] without modifying the PROF motion refinement precision. In both CE4-2.1 and CE4-2.2, the range of the BDOF motion refinement is modified from [-32, 32] to [-32, 31]. Compared to VTM-6.0 anchors, it is reported that CE4-2.1 leads to average BD- rate impacts of {0.01%, 0.02%, 0.01%} and {0.02%, 0.01%, 0.04%} for RA and LD configurations, respectively. The corresponding BD-rate impacts of CE-4.2.2 are {0.01%, 0.05%, -0.05%} and {0.02%, 0.00%, 0.04%}. Both the encoding and decoding impacts of the two CE tests are negligible.

# Introduction

In the VVC working draft (WD) 6 [1], two optical flow based sample refinement methods, namely bidirectional optical flow (BDOF) and prediction refinement with optical flow (PROF), are applied to enhance the efficiency of conventional motion compensation.

The BDOF is applied to enable the motion refinement based on optical flow for bi-predicted coding units (CUs) that has both forward and backward predictions. In the BDOF, gradients and motion refinements are derived from the initial L0 and L1 prediction samples. The gradients are calculated based on one three-tap filter [-1, 0, 1] and the motion refinements are derived in the precision of 1/32-pel and clipped in the range of [-32, 32].

The PROF is applied to refine the prediction samples of affine mode according to the optical flow model. The PROF refinement process is kept the same as the BDOF, except that the motion refinements used by the PROF are directly derived from the affine control-point motion vectors (CPMVs) instead of the initial prediction samples. In the current design, the PROF motion refinement is derived in the precision of 1/64-pel and clipped to the range [-64, 63].

# Technical description

Given that both the BDOF and the PROF are built upon the optical-flow concept, it is highly desirable to maximally harmonize the BDOF and PROF designs to facilitate hardware implementations. In [2], methods are proposed to unify the precision and the representation bit-depth of the motion refinements used by the BDOF and the PROF. The methods were included in Core Experiment (CE) 4 as test 2.1 and 2.2 for further evaluation [3].

## CE4-2.1

In CE4-2.1, it is proposed to align the precision of the motion refinement derived by the BDOF and the PROF by reducing the precision of the PROF motion refinement from 1/64-pel to 1/32-pel. Correspondingly, the range of the derived PROF motion refinement is changed from [-64, 63] to [-32, 31].

#### 8.5.5.9 Derivation process for motion vector arrays from affine control point motion vectors

- The variables sbWidth and sbHeight, dmvLimit, posOffsetX and posOffsetY are derived as follows:

sbWidth  =  cbWidth / numSbX		(8-729)

sbHeight  =  cbHeight / numSbY		(8-730)

dmvLimit  =  1 << Max( 65, BitDepthY − 67 )		(8-731)

posOffsetX  =  6 * dHorX + 6 * dVerX		(8-732)

posOffsetY  =  6 * dHorY + 6 * dVerY		(8-733)

- For x = 0..sbWidth − 1 and y = 0..sbHeight − 1, the following applies:

diffMv[ x ][ y ][ 0 ] = x * ( dHorX << 2 ) + y * ( dVerX << 2 ) – posOffsetX	(8-734)

diffMv[ x ][ y ][ 1 ] = x * ( dHorY << 2 ) + y * ( dVerY << 2 ) – posOffsetY	(8-735)

  - For i = 0..1, the following applieis:

    - The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to diffMv[ x ][ y ][ i ], rightShift set equal to 713 – Max(5, bitDepth – 7), and leftShift set equal to 0 as inputs and the rounded diffMv[ x ][ y ][ i ] as output.

    - The value of diffMv[ x ][ y ][ i ] is clipped as follows:

diffMv[ x ][ y ][ i ] = Clip3( −dmvLimit, dmvLimit − 1, diffMv[ x ][ y ][ i ]) 	(8-736)

Further, it is proposed to change the range of the BDOF motion refinement from [-32, 32] to [-32, 31].

#### 8.5.6.5 Bi-directional optical flow prediction process

  - The horizontal and vertical motion offset of the current subblock are derived as:

vx = sGx2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres-1,	(8-817)
	−( sGxdI << 3 ) >> Floor( Log2( sGx2 ) ) )  :  0

vy = sGy2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres-1, ( ( sGydI << 3 ) − 	(8-818)
	( ( vx * sGxGym ) << 12 + vx * sGxGys ) >> 1 ) >> Floor( Log2( sGy2 ) ) )  :  0

As can be seen from the above, in CE4-2.1, both the precision (i.e., 1/32-pel) and the representation bit-depth (6-bit sign value) of the BDOF and the PROF are fully unified.

## CE4-2.2

In CE4-2.2, it is proposed to directly clip the range of the PROF motion refinement to [-32, 31] while the precision of the PROF motion refinement is still kept at 1/64-pel.

#### 8.5.5.9 Derivation process for motion vector arrays from affine control point motion vectors

- The variables sbWidth and sbHeight, dmvLimit, posOffsetX and posOffsetY are derived as follows:

sbWidth  =  cbWidth / numSbX		(8-729)

sbHeight  =  cbHeight / numSbY		(8-730)

dmvLimit  =  1 << Max( 65, BitDepthY − 67 )		(8-731)

posOffsetX  =  6 * dHorX + 6 * dVerX		(8-732)

posOffsetY  =  6 * dHorY + 6 * dVerY		(8-733)

- For x = 0..sbWidth − 1 and y = 0..sbHeight − 1, the following applies:

diffMv[ x ][ y ][ 0 ] = x * ( dHorX << 2 ) + y * ( dVerX << 2 ) – posOffsetX	(8-734)

diffMv[ x ][ y ][ 1 ] = x * ( dHorY << 2 ) + y * ( dVerY << 2 ) – posOffsetY	(8-735)

  - For i = 0..1, the following applieis:

    - The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to diffMv[ x ][ y ][ i ], rightShift set equal to 713 – Max(6, bitDepth – 6), and leftShift set equal to 0 as inputs and the rounded diffMv[ x ][ y ][ i ] as output.

    - The value of diffMv[ x ][ y ][ i ] is clipped as follows:

diffMv[ x ][ y ][ i ] = Clip3( −dmvLimit, dmvLimit − 1, diffMv[ x ][ y ][ i ]) 	(8-736)

Further, it is proposed to change the range of the BDOF motion refinement from [-32, 32] to [-32, 31].

#### 8.5.6.5 Bi-directional optical flow prediction process

  - The horizontal and vertical motion offset of the current subblock are derived as:

vx = sGx2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres-1,	(8-817)
	−( sGxdI << 3 ) >> Floor( Log2( sGx2 ) ) )  :  0

vy = sGy2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres-1, ( ( sGydI << 3 ) − 	(8-818)
	( ( vx * sGxGym ) << 12 + vx * sGxGys ) >> 1 ) >> Floor( Log2( sGy2 ) ) )  :  0

As can be seen from the above, in CE4-2.2, the represent bit-depth (6-bit sign value) of the BDOF and the PROF is unified while their motion refinement precisions are different (i.e., 1/64-pel for the PROF and 1/32-pel for the BDOF).

# Simulation results

The proposed methods were implemented based on the VTM-6.0 reference software [4] and tested using the test condition and evaluation criteria that are used in CE4 [3] are applied.

Table 1 Simulation results of CE4-2.1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.03% | -0.11% | 101% | 99% |
| Class A2 | 0.03% | -0.08% | 0.06% | 100% | 98% |
| Class B | 0.01% | 0.07% | 0.03% | 101% | 100% |
| Class C | 0.00% | 0.08% | 0.03% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.02% | 0.01% | 100% | 99% |
| Class D | -0.02% | 0.04% | 0.09% | 98% | 101% |
| Class F | 0.00% | -0.01% | -0.01% | 100% | 97% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.05% | -0.37% | 102% | 99% |
| Class C | 0.01% | -0.09% | 0.24% | 98% | 99% |
| Class E | 0.04% | 0.26% | 0.46% | 98% | 103% |
| Overall | 0.02% | 0.01% | 0.04% | 100% | 100% |
| Class D | -0.05% | 0.22% | -0.13% | 99% | 100% |
| Class F | 0.10% | -0.20% | 0.41% | 99% | 99% |

Table 2 Simulation results of CE4-2.2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | -0.08% | 102% | 100% |
| Class A2 | 0.07% | 0.05% | 0.02% | 102% | 98% |
| Class B | -0.02% | 0.11% | -0.07% | 101% | 103% |
| Class C | 0.00% | 0.03% | -0.07% | 98% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.05% | -0.05% | 101% | 101% |
| Class D | -0.04% | -0.14% | -0.20% | 103% | 101% |
| Class F | 0.01% | -0.02% | 0.00% | 101% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.07% | -0.12% | 100% | 101% |
| Class C | -0.02% | -0.01% | -0.11% | 100% | 101% |
| Class E | 0.12% | 0.12% | 0.52% | 101% | 100% |
| Overall | 0.02% | 0.00% | 0.04% | 100% | 101% |
| Class D | 0.01% | -0.11% | 0.05% | 98% | 96% |
| Class F | 0.14% | 0.01% | 0.32% | 100% | 104% |

# Reference

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, July 2019, Gothenburg, Sweden.

X. Xiu, et al., “CE4/9-related: Combined Test of JVET-O0593/JVET-O0252/JVET-O0281/JVET-O0615 on harmonization on BDOF and PROF”, JVET-O1142, July 2019, Gothenburg, Sweden.

C.-C. Chen, H. Yang, X. Xiu, “Description of Core Experiment 4 (CE4): Inter prediction”, JVET-O2024, July 2019, Gothenburg, Sweden.

VTM-6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).