| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0059-v1 |
| --- | --- |

| Title: | CE8-4.1: BDPCM and Transform skip for Chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gordon Clare b<>com 1219 Avenue des Champs Blancs, 35510 Cesson-Sévigné, France Felix Henry Orange Labs 4 rue Clos Courtel 35510 Cesson-Sévigné, France Orange, b<>com Takeshi Tsukuba Masaru Ikeda Yoichi Yagasaki Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | gordon.clare@b-com.com felix.henry@orange.com Takeshi.Tsukuba@sony.com Masaru.Ikeda@sony.com Yoichi.Yagasaki@sony.com Teruhiko.S@sony.com |
| Source: | Orange, b<>com, Sony Corporation |  |  |

_____________________________

- Abstract

This document reports results of CE8-4.1, which introduces BDPCM and Transform Skip for Chroma, and applies TS residual coding to chroma-transform-skipped block with limiting the maximum number of context-coded bins to 1.75 per coefficient.

Simulation results show that:

For common QP condition,

- When DualITree turned off, average BD-rate differences (Y, U, V) are (0.80%, -1.67%, -1.57%) for AI, (0.08%, -2.86 %, -2.52%) for RA, and (0.07 %, -2.74%, -2.96 %) for LB, respectively.

- When DualITree turned on, average BD-rate differences (Y, U, V) are (-0.09%, -0.59%, -0.70%) for AI, (0.03%, -1.64%, -1.46%) for RA, and (0.19%, -1.60%, -1.85%) for LB, respectively.

For low QP condition (QP=2, 7, 12, 17),

- When DualITree turned off, average BD-rate differences (Y, U, V) are (-2.31%, -4.44%, -4.43%) for AI, (-2.59%, -9.54%, -9.89%) for RA, and (-2.69%, -6.13%, -5.96%) for LB, respectively.

- When DualITree turned on, average BD-rate differences (Y, U, V) are (-0.37%, -3.72%, -3.56%) for AI, (-1.10%, -9.04%, -9.19%) for RA, and (-2.10%, -6.29%, -6.17%) for LB, respectively.

Introduction

CE8-4.1 tests BDPCM and Transform Skip for chroma, which is a combination of CE8-3.1 (JVET-O0166 [1]) and CE8-2.1 (JVET-O0081[2]). The current design of VVC supports BDPCM and Transform Skip, which are only applied to luma component. To make the design of VVC consistency for luma and chroma, CE8-4.1 proposes to allow BDCPM and Transform skip to be used on the chroma components. Summary of proposed changes are as below:

- Signal sps_bdpcm_chroma_enabled_flag in the SPS to enable BDPCM for chroma

- Signal intra_bdpcm_chroma_flag and intra_bdcpm_dir_chroma if BDPCM for chroma is enabled (sps_bdcpm_chroma_enabled_flag is equal to 1)

- When BDPCM is available for luma only, the current behaviour is unchanged.

- When BDPCM is also available for chroma, an intra_bdpcm_chroma_flag is sent for each chroma block. This indicates whether BDPCM is used on the chroma blocks. When it is on, BDPCM is used for both chroma components, and an additional intra_bdpcm_dir_chroma flag is coded, indicating the prediction direction used for both chroma components.

- Signal transform type (transform_skip_flag and mts_idx) per component with minimum changes:

  - Additional single context model for chroma TS

  - No MTS for chroma

  - No changes of both context model and binarization for MTS signaling

  - When BDPCM for chroma is used, transform_skip_flag for chroma is not signaled and inferred to be equal to “transform skip is used”.

- Apply TS residual coding to chroma-transform-skipped block

  - TS residual coding is straightforwardly used for chroma TS as well as luma TS case.

  - Limiting the maximum number of context-coded bin in TS residual coding to 1.75 per coefficient, which was originally proposed in JVET-O1169 [3].

Text changes relative to JVET-O2001v14 are found in JVET-P0059_ DraftText.docx.

Simulation Results

The proposed BDPCM/TS for Choma has been implemented on top of VTM-6.0 and following simulations were conducted under CE8 test condition [4]:

- Test1:	VTM-6.0 DualITreeOff vs CE8-4.1 DualITreeOff under common QP condition

- Test2: VTM-6.0 DualITreeOff vs CE8-4.1 DualITreeOff under low QP condition (QP=2,7,12,17)

- Test3: VTM-6.0 DualITreeOn vs CE8-4.1 DualITreeOn under common QP condition

- Test4: VTM-6.0 DualITreeOn vs CE8-4.1 DualITreeOn under low QP condition (QP=2,7,12,17)

Table 3-1 to 3-4 summarizes results of each Test1, Test2, Test3 and Test4, respectively.

Note that palette bugfixes for ticket#512 and ticket#521 were applied to both anchor and test for simulations at QP=2/RA&LB.

A bdpcm loop filter bugfix (ticket #295) was also applied for the test as part of the bdpcm-chroma loop filter changes.

Table 3-1: Results of CE8-4.1 DualITreeOff under common QP condition (Test1)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.71% | -1.87% | -1.95% | 121% | 98% |
| TGM 720p | 1.51% | -1.52% | -1.06% | 117% | 100% |
| Animation | 0.46% | -0.57% | -0.24% | 114% | 100% |
| Mixed content | 0.67% | -4.04% | -4.22% | 117% | 98% |
| Camera-Captured content | 0.24% | 0.31% | 0.15% | 114% | 101% |
| Overall | 0.80% | -1.67% | -1.57% | 117% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.21% | -3.20% | -3.21% | 113% | 100% |
| TGM 720p | 0.66% | -2.81% | -1.73% | 109% | 100% |
| Animation | 0.31% | -1.36% | -0.68% | 109% | 100% |
| Mixed content | -0.52% | -6.06% | -6.20% | 107% | 100% |
| Camera-Captured content | 0.08% | 0.26% | 0.02% | 106% | 101% |
| Overall | 0.08% | -2.86% | -2.52% | 109% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.04% | -3.21% | -3.56% | 110% | 98% |
| TGM 720p | 0.22% | -2.76% | -2.65% | 106% | 100% |
| Animation | 0.51% | -1.30% | -1.04% | 106% | 99% |
| Mixed content | -0.41% | -5.35% | -6.49% | 104% | 103% |
| Camera-Captured content | 0.05% | -0.03% | 0.00% | 104% | 98% |
| Overall | 0.07% | -2.74% | -2.96% | 106% | 99% |

Table 3-2: Results of CE8-4.1 DualITreeOff under low QP condition (Test2)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -4.77% | -5.67% | -6.09% | 124% | 99% |
| TGM 720p | -1.61% | -3.66% | -3.49% | 119% | 99% |
| Animation | -0.25% | -3.82% | -2.46% | 116% | 100% |
| Mixed content | -3.56% | -7.42% | -8.39% | 122% | 98% |
| Camera-Captured content | 0.03% | -0.01% | -0.02% | 112% | 100% |
| Overall | -2.31% | -4.44% | -4.43% | 119% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -6.40% | -9.36% | -9.84% | 116% | 99% |
| TGM 720p | -1.15% | -9.33% | -9.98% | 111% | 99% |
| Animation | -2.39% | -8.31% | -7.66% | 112% | 98% |
| Mixed content | -1.38% | -17.59% | -18.37% | 107% | 90% |
| Camera-Captured content | 0.00% | -0.11% | -0.43% | 105% | 98% |
| Overall | -2.59% | -9.54% | -9.89% | 111% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -5.43% | -7.72% | -8.02% | 110% | 100% |
| TGM 720p | -1.25% | -4.90% | -3.96% | 108% | 100% |
| Animation | -3.48% | -6.85% | -6.27% | 108% | 99% |
| Mixed content | -1.95% | -8.97% | -9.36% | 107% | 101% |
| Camera-Captured content | -0.03% | -0.05% | -0.25% | 107% | 98% |
| Overall | -2.69% | -6.13% | -5.96% | 108% | 100% |

Table 3-3: Results of CE8-4.1 DualITreeOn under common QP condition (Test3)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.61% | -1.04% | -1.54% | 117% | 98% |
| TGM 720p | 0.31% | -0.57% | -0.44% | 110% | 96% |
| Animation | 0.38% | -0.33% | 0.27% | 110% | 97% |
| Mixed content | -0.55% | -1.11% | -1.80% | 113% | 98% |
| Camera-Captured content | 0.13% | 0.66% | 0.68% | 111% | 100% |
| Overall | -0.09% | -0.59% | -0.70% | 112% | 97% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.44% | -2.62% | -2.66% | 112% | 102% |
| TGM 720p | 0.31% | -1.28% | -1.17% | 107% | 100% |
| Animation | 0.40% | -1.21% | -0.36% | 107% | 98% |
| Mixed content | -0.10% | -2.54% | -2.65% | 105% | 99% |
| Camera-Captured content | 0.06% | 0.34% | 0.48% | 105% | 102% |
| Overall | 0.03% | -1.64% | -1.46% | 107% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.13% | -3.00% | -3.22% | 108% | 100% |
| TGM 720p | 0.40% | -1.34% | -1.66% | 104% | 98% |
| Animation | 0.72% | -0.97% | -0.82% | 104% | 94% |
| Mixed content | -0.08% | -1.83% | -2.55% | 101% | 89% |
| Camera-Captured content | 0.07% | 0.05% | 0.03% | 105% | 100% |
| Overall | 0.19% | -1.60% | -1.85% | 104% | 96% |

Table 3-4: Results of CE8-4.1 DualITreeOn under low QP condition (Test4)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.71% | -4.08% | -3.96% | 117% | 98% |
| TGM 720p | -0.57% | -3.16% | -3.03% | 117% | 101% |
| Animation | -0.36% | -4.20% | -3.00% | 114% | 99% |
| Mixed content | 0.10% | -5.97% | -6.70% | 123% | 103% |
| Camera-Captured content | 0.02% | -0.03% | 0.00% | 111% | 99% |
| Overall | -0.37% | -3.72% | -3.56% | 117% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -4.22% | -7.28% | -7.56% | 113% | 97% |
| TGM 720p | 0.60% | -8.31% | -8.47% | 111% | 101% |
| Animation | -2.45% | -8.68% | -7.77% | 110% | 98% |
| Mixed content | 1.41% | -18.67% | -19.61% | 109% | 96% |
| Camera-Captured content | -0.03% | -0.09% | -0.43% | 106% | 100% |
| Overall | -1.10% | -9.04% | -9.19% | 110% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -4.48% | -6.79% | -6.91% | 108% | 98% |
| TGM 720p | -0.77% | -5.23% | -4.41% | 108% | 101% |
| Animation | -3.37% | -6.88% | -6.35% | 109% | 98% |
| Mixed content | -0.79% | -10.60% | -11.30% | 107% | 99% |
| Camera-Captured content | -0.03% | -0.08% | -0.23% | 102% | 97% |
| Overall | -2.10% | -6.29% | -6.17% | 107% | 99% |

Conclusion

It is recommended to adopt proposed changes into the next VTM/VVC WD.

Reference

[1] F. Henry, G. Clare, “CE8-related: BDPCM for chroma,” JVET-O0166, July 2019

[2] T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki, “CE8-3.2: Chroma Transform Skip,” JVET-O0081, July 2019

[3] M. Coban, M. Karczewicz, “CE8(7)-related: Alignment of number of context coded bins per coefficients for TS residual coding and transform coefficient coding,” JVET-O1169, July 2019

[4] X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” JVET-O2028, July 2019

Patent rights declaration(s)

Orange and b<>com may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).