| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0074-v1 |
| --- | --- |

| Title: | CE4: Geometric merge mode (GEO) with partial transform (tests CE4-1.5 and CE4-1.6) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Filippov, V. Rufitskiy | Email: | alexey.filippov@huawei.com vasily.rufitskiy@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This document presents the results of two tests, namely CE4-1.5 and CE4-1.6. In test CE4-1.5, a combination of geometric merge mode (GEO) with partial transform is evaluated. Test CE4-1.5 is implemented on top of test CE4-1.2 with 80 GEO modes. In this test, partial transform adaptively uses partitions of ¼ and ½ of a block. Over VTM-6.0, test CE4-1.5 reportedly shows the overall results of - 0.35% (Y), -0.41% (U), -0.49% (V) with encoding time of 104% for RA configuration and -0.83% (Y), - 0.67% (U), -0.67% (V) with encoding time of 105% for LDB configuration. In test CE4-1.6, partial transform uses fixed-size partitions of ½ a block. Over VTM-6.0, test CE4- 1.6 reportedly provides the overall results of -0.34% (Y), -0.43% (U), -0.49% (V) with encoding time of 104% for RA configuration and -0.87% (Y), -0.56% (U), -0.60% (V) with encoding time of 105% for LDB configuration. Decoding time is appr. 100% in both tests.

# Introduction

Core Experiment 4 “Inter prediction” (CE4) specifies two tests (namely, CE4-1.5 and CE4-1.6) where partial transform (PT) method applied on the top of TPM and GEO partitioning is studied. GEO merge mode is also being studied in this CE4 test [1]. This document describes tests CE4-1.5 and CE4-1.6 and reports the results of the objective evaluation obtained in compliance with the JVET common test conditions [2].

Both tests CE4-1.5 and CE4-1.6 have the same parameters of GEO method as in test CE4-1.2 with 80 allowed values of geo_merge_idx. The difference between these two tests is that CE4-1.5 allows to have a dimension of a partial transform area that is a half or a quarter of a corresponding PU size, while in CE4-1.6 PT may have a width or a height that is a half of a width or a height of the PU, which partial transform is applied to.

# Proposed method

The basic idea behind partial transform is to align residual signal sampling with residual transform so that the residual signal is localized within a PU by the means of a PT area. The method is close to the one known as sub-block transform (SBT). However, the conceptual difference between SBT and partial transform is in the fact that a PT area may have sides that are non-orthogonal to any side of the PU. The skew of sides of a PT area is defined by the line that partitions the PU, thus aligning transform direction with the direction of the partitioning line.

In both tests (CE4-1.5 and CE4-1.6), we propose to apply partial transform to the PUs where the partial transform is available and is indicated to be performed. Partial transform is applied only to luma component and consists in transforming only an area of a PU. This area is defined to have sides that are not necessarily orthogonal or parallel to the sides of a PU. Instead, the sides of the area may have a skew that is determined by a splitting line angle, wherein this splitting line is an inherent part of a corresponding splitting method, namely GEO or TPM.

Availability of the partial transform is estimated by determining whether resulting PT area have both width and height that are not shorter than 4 samples.

Indication of whether a residual of a PU is subject to be processed by partial transform is done in residual coding, after “tu_cbf_luma” flag is determined to be true:

| if( tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| --- | --- |
| if (partial_transform enabled && (MergeTriangleFlag ¦¦ geo_flag[ x0 ][ y0 ] ) && !cu_skip_flag[ x0 ][ y0 ] && !cu_skip_flag[ x0 ][ y0 ] && !cu_transquant_bypass_flag) { |  |
| part_trans_flag[ x0 ][ y0 ] | ae(v) |
| if (part_trans_flag) |  |
| part_trans_idx[ x0 ][ y0 ] | ae(v) |
| } |  |

Starting and ending point of the PT line that defines a partial transform area (PT area) is derived from the parameters of the splitting line and the shape of the PU. The PT area is specified by this line. In particular, for each position along this line, the number of samples which is equal to ½ or ¼ of the shorter side are selected column-wise in case the shorter side is the left or right side of the PU, or row-wise in case the shorter side is the top side or bottom side of the PU. The center position of the selected samples is the position on the PT line.

The process of inverse transform is modified to position the reconstructed residual signal into the PT area of the PU. Samples outside the PT are set equal to zero. In order to avoid visual artifacts the following residual propagation is performed after the PT area is populated with residual samples. Depending on whether a scan of a PT area is row-wise or columnwise, two outmost samples are selected for each row or each column, respectively. Each of these samples is propagated outside the PT area (correspondingly, within a row or a column), so that a sample value outside the PT area is redefined:

p[x,y] = s >> d,

wherein s is the luminance value of a selected outmost PT area residual sample, d is the number of samples from the selected outmost PT area residual sample to the sample value outside the PT area that is being redefined. The sample value outside the PT area that have the value of d greater than 3 are not redefined, i.e. set equal to 0.

# Experimental results

## The simulation results reportedly obtained for test CE4-1.5 and CE4-1.6 using the JVET common test conditions [2] over VTM-6.0 are presented in Table 1 and Table 2 for RA and LDB configurations.

Table 1. Simulation results for test CE4-1.5

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.19% | -0.22% | 104% | 100% |
| Class A2 | -0.23% | -0.21% | -0.23% | 104% | 100% |
| Class B | -0.23% | -0.18% | -0.32% | 104% | 100% |
| Class C | -0.76% | -1.01% | -1.10% | 105% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.35% | -0.41% | -0.49% | 104% | 100% |
| Class D | -0.52% | -0.91% | -0.68% | 105% | 104% |
| Class F (optional) | -0.34% | -0.45% | -0.40% | 103% | 102% |
|  | Low delay B |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.50% | -0.55% | -0.44% | 106% | 101% |
| Class C | -0.96% | -1.05% | -1.18% | 106% | 100% |
| Class E | -1.19% | -0.36% | -0.38% | 104% | 100% |
| Overall | -0.83% | -0.67% | -0.67% | 105% | 100% |
| Class D | -0.85% | -0.92% | -0.62% | 107% | 100% |
| Class F (optional) | -0.52% | -0.59% | 0.15% | 103% | 100% |

Table 2. Simulation results for test CE4-1.6

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.25% | -0.26% | 104% | 99% |
| Class A2 | -0.23% | -0.26% | -0.25% | 103% | 99% |
| Class B | -0.22% | -0.16% | -0.22% | 104% | 100% |
| Class C | -0.72% | -1.03% | -1.18% | 105% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.34% | -0.43% | -0.49% | 104% | 100% |
| Class D | -0.49% | -0.87% | -0.60% | 105% | 103% |
| Class F | -0.33% | -0.46% | -0.41% | 102% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.50% | -0.39% | -0.55% | 105% | 100% |
| Class C | -0.98% | -0.82% | -0.93% | 106% | 99% |
| Class E | -1.34% | -0.48% | -0.25% | 104% | 99% |
| Overall | -0.87% | -0.56% | -0.60% | 105% | 100% |
| Class D | -0.82% | -0.60% | -0.81% | 106% | 99% |
| Class F | -0.48% | -0.71% | 0.82% | 103% | 99% |

# Conclusion

In this contribution, the results of CE4-1.5 and CE4-1.6 are presented. The obtained simulation results for these tests demonstrate that the compression performance improvements for partial transform with adaptively selected size and fixed-size of partitions differ insignificantly. Moreover, the BD-rate gain for LDB configuration is higher for fixed-size partitions than for adaptively selected size. Thus, the method evaluated in test CE4-1.6 looks more preferable than the method from test CE4-1.5.

# References

- H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, M. Bläser, J. Sauer, “CE4: CE4-1.1, CE4-1.2 and CE4-1.14: Geometric Merge Mode (GEO)”, JVET-P0068, 16th JVET Meeting, Geneva, Switzerland, October 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th JVET Meeting, Geneva, Switzerland, March 2019.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).