| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0082-v4 |
| --- | --- |

| Title: | AHG18: BDPCM for Lossless |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alican Nalci, Hongtao Wang, Hilmi E. Egilmez, Muhammed Coban, and Marta Karczewicz 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | analci@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In AHG18 VTM-6.0-lossless branch, trans quant bypass (QB) mode provides a lossless alternative for VVC. This contribution proposes to enable block DPCM (BDPCM) coding when QB mode is used for lossless. Here, 4 different proposals are presented with varying bit-rate savings for lossless.

For proposal #1, simulation results show overall bit-rate savings of -1.34% AI, -0.30% RA with Class F: -3.80% AI, -2.75% RA, Class TGM: -3.54% AI, -2.67 % RA.

For proposal #2, simulation results show overall bit-rate savings of -1.28% AI, -0.26% RA, with Class F: X.xx% AI, X.xx% RA, Class TGM: X.xx% AI, X.xx% RA, X.xx%

For proposal #3, simulation results show overall bit-rate savings of X.xx% AI, X.xx% RA, X.xx% with Class F: X.xx% AI, X.xx% RA, Class TGM: X.xx% AI, X.xx% RA, X.xx%

For proposal #4, simulation results show overall bit-rate savings of X.xx% AI, X.xx% RA, X.xx% with Class F: X.xx% AI, X.xx% RA, Class TGM: X.xx% AI, X.xx% RA, X.xx%

# Introduction

Block DPCM (BDPCM) mode proposed in  [1, 2] uses either a vertical or horizontal prediction of intra samples combined with DPCM residual coding. This mode in VVC utilizes only transform skip (TS) for residual coding. BDPCM signaling is done at the CU level with a bdpcm_flag to indicate whether regular intra-coding or BDPCM mode is used. Additionally, a bdpcm_dir flag is also sent to indicate whether the vertical or horizontal mode is selected when BDPCM prediction is used. In AHG18 VTM-6.0-lossless branch [3, 4], BDPCM related flags are signalled and TS residual coding is implicitly used when bdpcm_flag is non-zero.

# Proposals

- In proposal #1, we enable BDPCM both for TS residual coding and transform residual (TR) coding for lossless. As in VTM-6.0, non-BDPCM TS and TR residual coding are also allowed.

- In proposal #2, we enable BDPCM only for TR residual coding and disable TS residual coding for lossless.

- In proposal #3, we enable BDPCM only for TS residual coding. Non-BDPCM TS and TR residual coding are also allowed as in VTM-6.0.

- In proposal #4, chroma BDPCM is enabled for TS residual coding on top of proposal #1.

Our results that enabling BDPCM in QB mode provides considerable bit-rate savings for both natural sequences and for Class F and Class TGM. These results are detailed below.

# Results

Anchor is AHG18 VTM-6.0-lossless branch. The following settings are used to run both the anchor and proposals: TransquantBypassEnable : 1, CUTransquantBypassFlagForce : 1, CostMode : lossless, InternalBitDepth : 0. Moreover, QP value was also set as 0. Additionally, for proposals #1, #2, and #3 BDPCM : 1 configuration was used for all sequences e.g. A1, A2,…TGM. For proposal #4 we use BDPCM: 2 configuration for all sequences.

## Proposal 1: Enable BDPCM both for TS and Transform Residual Coding

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 LL | Proposal 1 |  | AHG18 LL | Proposal 1 |  | AHG18 LL | Proposal 1 |  |
| Class A1 | 2.2 | 2.2 | -0.87% | 2.3 | 2.3 | -0.36% |  |  |  |
| Class A2 | 1.7 | 1.8 | -1.18% | 1.9 | 1.9 | -0.27% |  |  |  |
| Class B | 2.1 | 2.1 | -1.34% | 2.3 | 2.3 | -0.26% | 2.3 | 2.3 | -0.29% |
| Class C | 2.1 | 2.1 | -1.45% | 2.6 | 2.6 | -0.34% | 2.6 | 2.6 | -0.38% |
| Class D | 1.9 | 2.0 | -2.66% | 2.8 | 2.8 | -0.52% | 2.7 | 2.8 | -0.47% |
| Class E | 2.9 | 3.0 | -1.86% |  |  |  | 3.3 | 3.4 | -0.99% |
| Class F | 5.5 | 5.7 | -3.80% | 39.4 | 41.3 | -2.75% | 60.7 | 64.4 | -3.16% |
| TGM | 12.3 | 12.7 | -3.54% | 112.6 | 115.7 | -2.67% | 142.4 | 155.1 | -36.20% |
| Overall | 2.2 | 2.2 | -1.34% | 2.3 | 2.3 | -0.30% | 2.6 | 2.7 | -0.49% |
| Enc Time[%] | 113% |  |  | 110% |  |  | 117% |  |  |
| Dec Time[%] | 91% |  |  | 99% |  |  | 103% |  |  |

Table 1. Results with BDPCM enabled both for TS and transform residual coding.

(AI and RA results are complete, pending full LDB results)

## Proposal 2: Enable BDPCM only for Transform Residual Coding

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 LL | Proposal 2 |  | AHG18 LL | Proposal 2 |  | AHG18 LL | Proposal 2 |  |
| Class A1 | 2.2 | 2.2 | -0.67% | 2.3 | 2.3 | -0.25% |  |  |  |
| Class A2 | 1.7 | 1.8 | -1.14% | 1.9 | 1.9 | -0.25% |  |  |  |
| Class B | 2.1 | 2.1 | -1.31% | 2.3 | 2.3 | -0.24% | 2.3 | 0.0 | -500.00% |
| Class C | 2.1 | 2.1 | -1.38% | 2.6 | 2.6 | -0.31% | 2.6 | 2.6 | -0.33% |
| Class D | 1.9 | 2.0 | -2.54% | 2.8 | 2.8 | -0.50% | 2.7 | 2.8 | -0.45% |
| Class E | 2.9 | 3.0 | -1.85% |  |  |  | 3.3 | 3.4 | -0.99% |
| Class F | 5.5 | 5.7 | -2.75% | 39.4 | 40.2 | -1.14% | 60.7 | 79.6 | -34.24% |
| TGM | 12.3 | 12.5 | -1.58% | 112.6 | 113.4 | -0.57% | 142.4 | 142.8 | -0.26% |
| Overall | 2.2 | 2.2 | -1.28% | 2.3 | 2.3 | -0.26% | 2.6 | 1.7 | -42.03% |
| Enc Time[%] | 102% |  |  | 102% |  |  | #NUM! |  |  |
| Dec Time[%] | 93% |  |  | 100% |  |  | #NUM! |  |  |

Table 2. Results with BDPCM enabled only for transform residual coding.

(AI and RA results are complete, pending full LDB results)

## Proposal 3: Enable BDPCM for TS residual coding

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 LL | Proposal 3 |  | AHG18 LL | Proposal 3 |  | AHG18 LL | Proposal 3 |  |
| Class A1 | 2.2 | 2.2 | -0.32% | 2.3 | 0.0 | -300.00% |  |  |  |
| Class A2 | 1.7 | 1.7 | -0.30% | 1.9 | 0.0 | -300.00% |  |  |  |
| Class B | 2.1 | 2.1 | -0.25% | 2.3 | 2.2 | -25.09% | 2.3 | 0.0 | -500.00% |
| Class C | 2.1 | 2.1 | -0.60% | 2.6 | 2.6 | -0.20% | 2.6 | 2.4 | -100.41% |
| Class D | 1.9 | 2.0 | -1.53% | 2.8 | 2.8 | -0.35% | 2.7 | 2.8 | -0.36% |
| Class E | 2.9 | 3.0 | -0.92% |  |  |  | 3.3 | 0.0 | -300.00% |
| Class F | 5.5 | 5.7 | -3.15% | 39.4 | 41.1 | -2.52% | 60.7 | 124.0 | -105.17% |
| TGM | 12.3 | 12.7 | -3.16% | 112.6 | 115.5 | -2.54% | 142.4 | 191.2 | -35.79% |
| Overall | 2.2 | 2.2 | -0.46% | 2.3 | 1.3 | -46.74% | 2.6 | 0.4 | -83.40% |
| Enc Time[%] | 105% |  |  | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | 95% |  |  | #NUM! |  |  | #NUM! |  |  |

Table 3. Results with BDPCM enabled only for TS residual coding and TS is enabled for lossless.

(pending results)

## Proposal 4: Enable Chroma BDPCM on top of Proposal #1

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 LL | Proposal 4 |  | AHG18 LL | Proposal 4 |  | AHG18 LL | Proposal 4 |  |
| Class A1 | 2.2 | 2.3 | -1.69% | 2.3 | 2.3 | -0.72% |  |  |  |
| Class A2 | 1.7 | 1.8 | -1.64% | 1.9 | 1.9 | -50.44% |  |  |  |
| Class B | 2.1 | 2.1 | -1.98% | 2.3 | 2.3 | -0.39% | 2.3 | 0.0 | -500.00% |
| Class C | 2.1 | 2.1 | -2.22% | 2.6 | 2.6 | -0.50% | 2.6 | 2.6 | -0.50% |
| Class D | 1.9 | 2.0 | -3.81% | 2.8 | 2.8 | -0.74% | 2.7 | 2.8 | -0.64% |
| Class E | 2.9 | 3.0 | -3.66% |  |  |  | 3.3 | 3.4 | -1.15% |
| Class F | 5.5 | 5.9 | -5.37% | 39.4 | 41.8 | -3.53% | 60.7 | 84.5 | -37.59% |
| TGM | 12.3 | 13.2 | -6.63% | 112.6 | 117.0 | -3.68% | 142.4 | 147.3 | -3.30% |
| Overall | 2.2 | 2.2 | -2.21% | 2.3 | 2.2 | -7.13% | 2.6 | 1.7 | -42.12% |
| Enc Time[%] | 115% |  |  | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | 88% |  |  | #NUM! |  |  | #NUM! |  |  |

Table 4. Results with BDPCM enabled both for TS and transform residual coding. Additionally chroma BDPCM is enabled for TS residual coding.

## Additional Results: Enable TS coding in QB mode

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 LL | TS Only |  | AHG18 LL | TS Only |  | AHG18 LL | TS Only |  |
| Class A1 | 2.2 | 2.2 | -0.19% | 2.3 | 2.3 | -0.10% |  |  |  |
| Class A2 | 1.7 | 1.7 | 0.00% | 1.9 | 1.9 | 0.00% |  |  |  |
| Class B | 2.1 | 2.1 | -0.02% | 2.3 | 2.3 | -0.02% | 2.3 | 2.3 | -25.01% |
| Class C | 2.1 | 2.1 | -0.04% | 2.6 | 2.6 | -0.03% | 2.6 | 2.6 | -0.03% |
| Class D | 1.9 | 2.0 | -0.04% | 2.8 | 2.8 | -0.01% | 2.7 | 2.7 | -0.04% |
| Class E | 2.9 | 2.9 | -0.05% |  |  |  | 3.3 | 3.3 | 0.02% |
| Class F | 5.5 | 5.5 | -0.69% | 39.4 | 40.5 | -1.55% | 60.7 | 63.7 | -2.33% |
| TGM | 12.3 | 12.5 | -2.00% | 112.6 | 114.9 | -2.11% | 142.4 | 146.1 | -2.44% |
| Overall | 2.2 | 2.2 | -0.05% | 2.3 | 2.3 | -0.03% | 2.6 | 2.5 | -8.34% |
| Enc Time[%] | 107% |  |  | 108% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  | 92% |  |  |

Table 5. Additional results with only allowing TS in lossless (without BDPCM).

(pending results)

# Conclusion

In this contribution we proposed to enable BDPCM coding for lossless. We presented 4 sub-proposals with different bit-rate savings.

It is recommended to enable BDPCM for lossless coding in VVC.

# Spec Changes

## Spec Changes to Enable BDPCM for lossless

#### 7.3.8.10 Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) && ChromaArrayType != 0 ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex = = NumIntraSubPartitions − 1 ) ) ) { |  |
| tu_cbf_cb[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr[ x0 ][ y0 ] | ae(v) |
| } |  |
| …. |  |
| if( sps_joint_cbcr_enabled_flag && ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && ( tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) ) ¦ ¦ ( tu_cbf_cb[ x0 ][ y0 ] && tu_cbf_cr[ x0 ][ y0 ] ) ) ) |  |
| tu_joint_cbcr_residual_flag[ x0 ][ y0 ] | ae(v) |
| if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) ) { |  |
| if( sps_transform_skip_enabled_flag && (cu_transquant_bypass_flag ¦ ¦ !BdpcmFlag[ x0 ][ y0 ] ) && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize ) |  |
| transform_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) && ( !transform_skip_flag[ x0 ][ y0 ] ) ) |  |
| tu_mts_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| if( !transform_skip_flag[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| … |  |

#### 7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) && ChromaArrayType != 0 ) { |  |
| if ( pred_mode_plt_flag && treeType = = DUAL_TREE_CHROMA ) |  |
| palette_coding( x0, y0, cbWidth / SubWidthC, cbHeight / SubHeightC, 1, 2 ) |  |
| else { |  |
| if( sps_bdpcm_enabled_flag && cu_transquant_bypass_flag && cbWidth <= MaxTsSize && cbHeight <= MaxTsSize ) |  |
| intra_bdpcm_flag | ae(v) |
| if( intra_bdpcm_flag ) |  |
| intra_bdpcm_dir_flag | ae(v) |
| else { |  |
| if( CclmEnabled ) |  |
| cclm_mode_flag | ae(v) |
| if( cclm_mode_flag ) |  |
| cclm_mode_idx | ae(v) |
| else |  |
| intra_chroma_pred_mode | ae(v) |
| } |  |
| } |  |
| } |  |

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| …. |  |
| if( sps_joint_cbcr_enabled_flag && ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && ( tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) ) ¦ ¦ ( tu_cbf_cb[ x0 ][ y0 ] && tu_cbf_cr[ x0 ][ y0 ] ) ) ) |  |
| tu_joint_cbcr_residual_flag[ x0 ][ y0 ] | ae(v) |
| if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) ) { |  |
| if( sps_transform_skip_enabled_flag && (cu_transquant_bypass_flag ¦ ¦ !BdpcmFlag[ x0 ][ y0 ] ) && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize ) |  |
| transform_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) && ( !transform_skip_flag[ x0 ][ y0 ] ) ) |  |
| tu_mts_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| if( !transform_skip_flag[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| … |  |

## Spec Changes to Enable Chroma BDPCM

7.3.2.3 Sequence parameter set RBSP syntax

| … | u(1) |
| --- | --- |
| if( sps_transform_skip_enabled_flag ) |  |
| sps_bdpcm_enabled_flag | u(1) |
| if ( sps_bdpcm_enabled_flag ) |  |
| sps_bdpcm_chroma_enabled_flag | u(1) |
| … | u(1) |

#### 7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) && ChromaArrayType != 0 ) { |  |
| if ( pred_mode_plt_flag && treeType = = DUAL_TREE_CHROMA ) |  |
| palette_coding( x0, y0, cbWidth / SubWidthC, cbHeight / SubHeightC, 1, 2 ) |  |
| else { |  |
| if(cbWidth <= MaxTsSize && cbHeight <= MaxTsSize && ( sps_bdpcm_chroma_enable_flag && cu_transquant_bypass_flag ) ) { |  |
| intra_bdpcm_chroma_flag[ x0 ][ y0 ] | ae(v) |
| if(intra_bdpcm_chroma_flag[ x0 ][ y0 ] ) |  |
| intra_bdpcm_chroma_dir_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( CclmEnabled ) |  |
| cclm_mode_flag | ae(v) |
| if( cclm_mode_flag ) |  |
| cclm_mode_idx | ae(v) |
| else |  |
| intra_chroma_pred_mode | ae(v) |
| } |  |
| } |  |
| } |  |
| … |  |

#### 7.3.8.5 Residual coding syntax

| residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| … |  |
| /* remainder scan pass */ |  |
| for( n = 0; n <= numSbCoeff − 1; n++ ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( abs_level_gtx_flag[ n ][ 4 ] ) |  |
| abs_remainder[ n ] | ae(v) |
| if( ( ( cIdx==0 ¦ ¦ !cu_transquant_bypass_flag ) ? ( intra_bdpcm_flag = = 0 ) : (intra_bdpcm_chroma_flag == 0)) ) { |  |
| absRightCoeff = abs( TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC − 1 ][ yC ] ) |  |
| … |  |

#### 7.4.3.3 Sequence parameter set RBSP semantics

…

sps_bdpcm_chroma_enabled_flag equal to 1 specifies that intra_bdpcm_chroma_flag may be present in the coding unit syntax for intra coding units. sps_bdpcm_chroma_enabled_flag equal to 0 specifies that intra_bdpcm_chroma_flag is not present in the coding unit syntax for intra coding units.

…

#### 7.4.9.5 Coding unit semantics

…

intra_bdpcm_chroma_flag[ x0 ][ y0 ] equal to 1 specifies that BDPCM is applied to the current chroma coding blocks at the location ( x0, y0 ), i.e. the transform is skipped, the intra chroma prediction mode is specified by intra_bdpcm_chroma_dir_flag[ x0 ][ y0 ]. intra_bdpcm_chroma_flag[ x0 ][ y0 ] equal to 0 specifies that BDPCM is not applied to the current chroma coding blocks at the location ( x0, y0 ).

When intra_bdpcm_chroma_flag[ x0 ][ y0 ] is not present it is inferred to be equal to 0.

The variable BdpcmChromaFlag[ x ][ y ] is set equal to intra_bdpcm_chroma_flag[ x0 ][ y0 ] for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1.

intra_bdpcm_chroma_dir_ flag [ x0 ][ y0 ] equal to 0 specifies that the BDPCM prediction direction is horizontal. intra_bdpcm_chroma_dir_flag[ x0 ][ y0 ] equal to 1 specifies that the BDPCM prediction direction is vertical.

The variable BdpcmChromaDir[ x ][ y ] is set equal to intra_bdpcm_chroma_dir_flag[ x0 ][ y0 ] for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1.

…

### 8.4.1 General decoding process for coding units coded in intra prediction mode

…

- Otherwise (pred_mode_plt_flag is equal to 0), the following applies:

- The derivation process for the chroma intra prediction mode as specified in clause 8.4.3 is invoked with the luma location ( xCb, yCb ) , the width of the current coding block in luma samples cbWidth and the height of the current coding block in luma samples cbHeight as input.

- The variable predModeIntraChroma is derived as follows:

- If BdpcmFlagChroma [ xCb ][ yCb ] is equal to 1, predModeIntraChroma is derived as follows:

predModeIntraChroma = BdpcmDirChroma [ xCb ][ yCb ]  ?  50  :  18	(8-X)

- Otherwise, predModeIntraChroma is set equal to IntraPredModeC [ xCb ][ yCb ].

### 8.4.3 Derivation process for chroma intra prediction mode

…

If BdpcmChromaFlag[ xCb ][ yCb ] is equal to 1, IntraPredModeC[ xCb ][ yCb ] is set equal to INTRA_PLANAR, otherwise, the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived using cclm_mode_mode_flag, cclm_mode_idx, intra_chroma_pred_mode and lumaIntraPredMode as specified in Table 8-2.

…

##### 8.4.5.2.5 General intra sample prediction

…

When all of the following conditions are true, the position-dependent prediction sample filtering process specified in clause 8.4.5.2.14 is invoked with the intra prediction mode predModeIntra, the transform block width nTbW, the transform block height nTbH, the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1, the reference sample width refW, the reference sample height refH, the reference samples p[ x ][ y ], with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1, and the colour component index cIdx as inputs, and the output is the modified predicted sample array predSamples:

- nTbW is greater than or equal to 4 and nTbH is greater than or equal to 4 or cIdx is not equal to 0

- refIdx is equal to 0 or cIdx is not equal to 0

- cIdx is equal to 0 and BdpcmFlag[ xTbCmp ][ xTbCmp ] is equal to 0

- cIdx is not equal to 0 and BdpcmChromaFlag[ xTbCmp ][ yTbCmp ] is equal to 0

| Table 9-4 – Association of ctxIdx and syntax elements for each initializationType in the initialization process |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax structure | Syntax element | ctxTable | initType |  |  |  |
|  |  |  | 0 | 1 | 2 |  |
| coding_unit( ) | … | … | … | … | … |  |
|  | intra_bdpcm_chroma_flag[ ][ ] | Table 9-2X | 0 | 1 | 2 |  |
|  | intra_bdpcm_ chroma_dir_flag[ ][ ] | Table 9-2Y | 0 | 1 | 2 |  |
|  | … | … | … | … | … |  |

Table 9-2X – Specification of initValue and shiftIdx for ctxInc of intra_bdpcm_chroma_flag

| Initialization variable | ctxIdx of intra_bdpcm_chroma_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | EP | EP | EP |
| shiftIdx | 0 | 0 | 0 |

Table 9-2Y – Specification of initValue and shiftIdx for ctxInc of intra_bdpcm_chroma_dir_flag

| Initialization variable | ctxIdx of intra_bdpcm_chroma_dir_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | EP | EP | EP |
| shiftIdx | 0 | 0 | 0 |

| Table 9-77 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| coding_unit( ) | … | … | … |
|  | intra_bdpcm_chroma_flag | FL | cMax = 1 |
|  | Intra_bdpcm_chroma_dir_flag | FL | cMax = 1 |
| coding_unit( ) | … | … | … |

| Table 9-82 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| … | … | … | … | … | … | … |
| intra_bdpcm_chroma_flag | 0 | na | na | na | na | na |
| intra_bdpcm_chroma_dir_flag | 0 | na | na | na | na | na |
| … | … | … | … | … | … | … |
| tu_cbf_cb[ ][ ][ ] | (intra_bdpcm_chroma_flag[][] && cu_transquant_bypass_flag ) ? 1 : 0 | na | na | na | na | na |
| tu_cbf_cr[ ][ ][ ] | (intra_bdpcm_chroma_flag[][] && cu_transquant_bypass_flag ) ? 2 : tu_cbf_cb[ ][ ][ ] | na | na | na | na | na |

##### 9.3.4.2.9 Derivation process of ctxInc for the syntax elements par_level_flag and abs_level_gtx_flag

Inputs to this process are the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the variable ctxInc.

The variable ctxInc is derived as follows:

- If transform_skip_flag[ xS ][ yS ] is equal to 1, the following applies:

- If the syntax element is par_level_flag, the following applies:

ctxInc = 33			(9-54)

- Otherwise, if the syntax element is abs_level_gtx_flag[ n ][ 0 ], the following applies:

- If intra_bdpcm_flag is equal to 1 and cIdx is equal to 0,  or intra_bdpcm_chroma_flag is equal to 1 and cIdx is not equal to 0, ctxInc is derived as follows:

ctxInc = 68								(9-55)

##### 9.3.4.2.10 Derivation process of ctxInc for the syntax element coeff_sign_flag for transform skip mode

Inputs to this process are the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC )

Output of this process is the variable ctxInc.

The variables rightSign and belowSign are set as follows:

rightSign = ( xC = = 0 ) ? 0 : CoeffSignLevel[ xC − 1 ][ yC ]	(9-64)

belowSign = ( yC = = 0 ) ? 0 : CoeffSignLevel[ xC ][ yC − 1 ]	(9-65)

The variable ctxInc is derived as follows:

- If  rightSign is equal to 0 and belowSign is equal to 0, or if rightSign is equal to  −belowSign, the following applies:

ctxInc =  (cIdx==0 | | !cu_transquant_bypass_flag ) ? ( intra_bdpcm_flag  = =  0  ?  0  :  3 )  : ( intra_bdpcm_chroma_flag == 0 ? 0 : 3)		(9-66)

- Otherwise, if rightSign is greater than or equal to 0 and belowSign is greater than or equal to 0, the following applies:

ctxInc = (cIdx==0 | | !cu_transquant_bypass_flag ) ? ( intra_bdpcm_flag  = =  0  ?  1  :  4 ) : ( intra_bdpcm_chroma_flag == 0 ? 1 : 4)		(9-67)

- Otherwise, the following applies:

ctxInc = (cIdx==0 | | !cu_transquant_bypass_flag ) ?  ( intra_bdpcm_flag  = =  0  ?  2  :  5 ) : ( intra_bdpcm_chroma_flag == 0 ? 2 : 5)		(9-68)

# References

- JVET-N0413 	CE8-related: Quantized residual BDPCM 	M. Karczewicz, M. Coban (Qualcomm)

- F. Henry, M. Abdoli, P. Philippe, G. Clare, “CE8: BDPCM with horizontal/vertical predictor and independently decodable areas (test 8.3.1b)”, JVET-M0057, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, January 2019.

- AHG18 VTM-6.0-lossless, https://vcgit.hhi.fraunhofer.de/jvet-o-ahg-18/VVCSoftware_VTM

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001, 15th Meeting, Gothenburg, SE, June 2019.

# Patent rights declaration

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).