| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0093 |
| --- | --- |

| Title: | Fix on the value ranges of cu_qp_delta_sub_div and cu_chroma_qp_offset_subdiv |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Eiichi Sasaki Takeshi Chujoh Tomohiro Ikai | Tel: Email: | +81 50-5123-6336 eiichi.sasaki@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes a fix on the value range of PPS syntax, cu_qp_delta_subdiv and cu_chroma_qp_offset_subdiv in order not to refer slice level syntaxes. It is proposed to use variables, CtbLog2SizeY and MinCbLog2SizeY specified in SPS level to specify the value range of cu_qp_delta_subdiv and cu_chroma_qp_offset_subdiv.

Problem statement

In VVC Draft 6 [1], cu_qp_delta_sub_div and cu_chroma_qp_offset_sub_div are signaled in PPS to control minimum size of quantization group. However, the maximum values of these syntax are specified depending on minimum QT size and maximum MTT depth informations which are signalled in slice header syntax. This order makes confusions and complicated.

Proposed specification (WD changes)

It is proposed to use variables, CtbLog2SizeY and MinCbLog2SizeY specified in SPS level to specify the range of cu_qp_delta_subdiv and cu_chroma_qp_offset_subdiv, i.e. we use sequence-level maximum range instead of slice-level maximum range.

It is noted larger cu_qp_delta_subdiv and cu_chroma_qp_offset_subdiv means smaller / finer blocks can be used in qp changes. So even if we signal a value larger than the maximum value in a specific slice, just every unit can signals the qp changes that is not a problem.

The text changes are highlighted with Turquoise.

7.4.3.4 Picture parameter set RBSP semantics

cu_qp_delta_subdiv specifies the maximum cbSubdiv value of coding units that convey cu_qp_delta_abs and cu_qp_delta_sign_flag. The value range of cu_qp_delta_subdiv shall be in the range of 0 to 2 * (CtbLog2SizeY – MinCbLog2SizeY), inclusive. is specified as follows:

[Ed. (BB): The issue here is that MaxMttDepthY is derived on slice level. In case of partition_constraints_override_enabled_flag equal to 1, one would need to parse the slice header in order to know the value of MaxMttDepthY.]

- If slice_type is equal to I, the value of cu_qp_delta_subdiv shall be in the range of 0 to 2 * ( log2_ctu_size_minus2 − log2_min_qt_size_intra_slice_minus2 + MaxMttDepthY ), inclusive.

- Otherwise (slice_type is not equal to I), the value of cu_qp_delta_subdiv shall be in the range of 0 to 2* ( log2_ctu_size_minus2 − log2_min_qt_size_inter_slice_minus2 + MaxMttDepthY ), inclusive.

When not present, the value of cu_qp_delta_subdiv is inferred to be equal to 0.

cu_chroma_qp_offset_subdiv specifies the maximum cbSubdiv value of coding units that convey cu_chroma_qp_offset_flag. The value range of cu_qp_delta_subdiv shall be in the range of 0 to 2 * (CtbLog2SizeY – MinCbLog2SizeY), inclusive. is specified as follows:

- If slice_type is equal to I, the value of cu_chroma_qp_offset_subdiv shall be in the range of 0 to 2 * ( log2_ctu_size_minus2 − log2_min_qt_size_intra_slice_minus2 + MaxMttDepthY ), inclusive.

- Otherwise (slice_type is not equal to I), the value of cu_chroma_qp_offset_subdiv shall be in the range of 0 to 2* ( log2_ctu_size_minus2 − log2_min_qt_size_inter_slice_minus2 + MaxMttDepthY ), inclusive.

When not present, the value of cu_chroma_qp_offset_subdiv is inferred to be equal to 0.

[Ed. (BB): The issue here is that MaxMttDepthY is derived on slice level. In case of partition_constraints_override_enabled_flag equal to 1, one would need to parse the slice header in order to know the value of MaxMttDepthY.]

Related specification (information)

7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| … | ue(v) |
| cu_qp_delta_enabled_flag | u(1) |
| if( cu_qp_delta_enabled_flag ) |  |
| cu_qp_delta_subdiv | ue(v) |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_joint_cbcr_qp_offset | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| cu_chroma_qp_offset_enabled_flag | u(1) |
| if( cu_chroma_qp_offset_enabled_flag ) { |  |
| cu_chroma_qp_offset_subdiv | ue(v) |
| chroma_qp_offset_list_len_minus1 | ue(v) |
| for( i = 0; i <= chroma_qp_offset_list_len_minus1; i++ ) { |  |
| cb_qp_offset_list[ i ] | se(v) |
| cr_qp_offset_list[ i ] | se(v) |
| joint_cbcr_qp_offset_list[ i ] | se(v) |
| } |  |
| } |  |
| … |  |

7.4.3.3 Sequence parameter set RBSP semantics

CtbLog2SizeY = log2_ctu_size_minus5 + 5		(7-15)

CtbSizeY = 1  <<  CtbLog2SizeY		(7-16)

MinCbLog2SizeY = log2_min_luma_coding_block_size_minus2 + 2	(7-17)

MinCbSizeY = 1  <<  MinCbLog2SizeY		(7-18)

7.4.7.1 General slice header semantic

…

MaxMttDepthY  =  slice_max_mtt_hierarchy_depth_luma	(7-109)

MaxMttDepthC  =  slice_max_mtt_hierarchy_depth_chroma	(7-110)

7.3.8.2 Coding tree unit syntax

| dual_tree_implicit_qt_split( x0, y0, cbSize, cqtDepth ) { | Descriptor |
| --- | --- |
| cbSubdiv = 2 * cqtDepth |  |
| if( cbSize > 64 ) { |  |
| if( cu_qp_delta_enabled_flag && cbSubdiv <= cu_qp_delta_subdiv ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDeltaVal = 0 |  |
| CuQgTopLeftX = x0 |  |
| CuQgTopLeftY = y0 |  |
| } |  |
| if( cu_chroma_qp_offset_enabled_flag && cbSubdiv <= cu_chroma_qp_offset_subdiv ) |  |
| IsCuChromaQpOffsetCoded = 0 |  |
| x1 = x0 + ( cbSize / 2 ) |  |
| y1 = y0 + ( cbSize / 2 ) |  |
| … |  |

7.3.8.4 Coding tree syntax

| coding_tree( x0, y0, cbWidth, cbHeight, qgOnY, qgOnC, cbSubdiv, cqtDepth, mttDepth, depthOffset, partIdx, treeTypeCurr, modeTypeCurr ) { | Descriptor |
| --- | --- |
| if( ( allowSplitBtVer ¦ ¦ allowSplitBtHor ¦ ¦ allowSplitTtVer ¦ ¦ allowSplitTtHor ¦ ¦ allowSplitQT ) &&( x0 + cbWidth <= pic_width_in_luma_samples ) && (y0 + cbHeight <= pic_height_in_luma_samples ) ) |  |
| split_cu_flag | ae(v) |
| if( cu_qp_delta_enabled_flag && qgOnY && cbSubdiv <= cu_qp_delta_subdiv ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDeltaVal = 0 |  |
| CuQgTopLeftX = x0 |  |
| CuQgTopLeftY = y0 |  |
| } |  |
| if( cu_chroma_qp_offset_enabled_flag && qgOnC && cbSubdiv <= cu_chroma_qp_offset_subdiv ) |  |
| IsCuChromaQpOffsetCoded = 0 |  |

References

[1] B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-O2001, July 2019

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).