| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0096 |
| --- | --- |

| Title: | AHG12: On signalling of tile and brick partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes having a single signalling mode instead of separate uniform and explicit tile/brick spacing modes. The contribution is asserted to reduce:

- the number of syntax elements from 13 to 8,

- the number of syntax table rows from 28 to 17,

- the length of the semantics, and

- the length of the scanning process in clause 6.5.1.

The contribution is asserted to require a similar amount or fewer bits compared to VVC Draft 6 depending on the partitioning scheme.

# Introduction

There are two modes of indicating the tile grid and splitting of tiles to bricks:

- Uniform spacing (indicated with the yellow highlighting in the syntax table below) where:

  - Tiles have equal width with the possible exception of the rightmost tile column and equal height with the possible exception of the bottom-most tile row.

  - Bricks of a tile have equal height with the possible exception of the bottom-most brick of the tile.

- Explicit spacing (indicated with the green highlighting in the syntax table below) where:

  - The widths and heights of the tile columns and rows (respectively) are indicated (in CTUs) except for the rightmost column and bottom-most row (respectively).

  - The brick heights are indicated (in CTUs) except for the bottom-most brick of a tile.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| uniform_tile_spacing_flag | u(1) |
| if( uniform_tile_spacing_flag ) { |  |
| tile_cols_width_minus1 | ue(v) |
| tile_rows_height_minus1 | ue(v) |
| } else { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| brick_splitting_present_flag | u(1) |
| if( uniform_tile_spacing_flag && brick_splitting_present_flag ) |  |
| num_tiles_in_pic_minus1 | ue(v) |
| for( i = 0; brick_splitting_present_flag && i <= num_tiles_in_pic_minus1 + 1; i++ ) { |  |
| if( RowHeight[ i ] > 1 ) |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| if( RowHeight[ i ] > 2 ) |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus2[ i ] | ue(v) |
| for( j = 0; j <= num_brick_rows_minus2[ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |

This contribution proposes having a single signalling mode instead of separate uniform and explicit tile/brick spacing modes.

# Proposal

## Overview of the proposed syntax

Tile columns are specified as follows:

- The number of explicitly provided tile column widths is indicated (num_exp_tile_columns_minus1)

- Tile column widths are explicitly provided from left to right (tile_column_width_minus1[ i ]).

- The last explicitly provided tile column width (tile_column_width_minus1[ num_exp_tile_columns_minus1 ]) is repeated until no further tile columns of that width fit within the picture boundaries.

- The remaining CTUs not yet allocated to any tile column, if any, are assigned to the right-most tile column.

The uniform and explicit spacing modes can be regarded as special cases in the proposed design:

- When num_exp_tile_columns_minus1 is equal to 0, the signalling is similar to the uniform tile spacing mode.

- When num_exp_tile_columns_minus1 plus 1 is equal to the number of tile columns of the picture, the signalling is similar to the explicit tile spacing mode.

Other values of num_exp_tile_columns_minus1 decrease the bit count when there are two or more distinct column widths within the picture and two or more right-most tile columns (possibly excluding the last tile column) have an equal width.

Tile row heights and brick rows are specified similarly to tile rows.

## Proposed syntax

The proposed syntax with change marks is presented below:

| if( !single_tile_in_pic_flag ) { |  |
| --- | --- |
| log2_pps_ctu_size_minus5 | u(2) |
| uniform_tile_spacing_flag | u(1) |
| if( uniform_tile_spacing_flag ) { |  |
| tile_cols_width_minus1 | ue(v) |
| tile_rows_height_minus1 | ue(v) |
| } else { |  |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| brick_splitting_present_flag | u(1) |
| if( uniform_tile_spacing_flag && brick_splitting_present_flag ) |  |
| num_tiles_in_pic_minus1 | ue(v) |
| for( i = 0; brick_splitting_present_flag && i <= num_tiles_in_pic_minus1 + 1NumTilesInPic; i++ ) { |  |
| if( RowHeight[ i / NumTileColumns ] > 1 ) |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| if( RowHeight[ i / NumTileColumns ] > 2 ) |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_exp_brick_rows_minus12[ i ] | ue(v) |
| for( j = 0; j <= num_exp_brick_rows_minus12[ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |

The proposed syntax without change marks is presented below:

| if( !single_tile_in_pic_flag ) { |  |
| --- | --- |
| log2_pps_ctu_size_minus5 | u(2) |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| brick_splitting_present_flag | u(1) |
| for( i = 0; brick_splitting_present_flag && i < NumTilesInPic; i++ ) { |  |
| if( RowHeight[ i / NumTileColumns ] > 1 ) |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| if( RowHeight[ i / NumTileColumns ] > 2 ) |  |
| num_exp_brick_rows_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_exp_brick_rows_minus1[ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |

# Asserted benefits

The following properties were derived from VVC Draft 6 and this contribution:

- The number of syntax elements for indicating tile and brick partitioning

- The number of syntax table rows for indicating tile and brick partitioning

- The number of bits required for indicating the tile and brick partitioning for the schemes presented in Figure 1.

The properties derived with 128×128 luma CTB size are presented in Table 1 and more details can be observed from the accompanying Excel file. It is also remarked that with the proposal clause 6.5.1 become some lines shorter and the semantics became almost half a page shorter.

Table 1. Comparison of tile and brick signalling in VVC Draft 6 and in this contribution.

|  |  |  | a | b | c | d | e | f |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | # syntax elements | # syntax table rows | # bits | # bits | # bits | # bits | # bits | # bits |
| VVC draft 6 | 13 | 28 | 70 | 54 | 85 | 62 | 85 | 83 |
| Proposal | 8 | 17 | 67 | 55 | 84 | 47 | 60 | 70 |

Figure 1a and Figure 1b present the tile and brick partitionings that achieve the 6K effective ERP and CMP schemes (respectively) that are described in OMAF clauses D.6.3 and D.6.4 (respectively) [1]. These schemes have been recommended in the VR Industry Forum Guidelines [2]. The scheme presented in Figure 1c is otherwise equivalent to that in Figure 1b but uses a different picture aspect ratio. To test the robustness of the proposal for different tile row and column selection order, Figure 1d, Figure 1e, and Figure 1f are the same as Figure 1a, Figure 1b, and Figure 1c, respectively, but with a reversed order of tile columns and tile rows.

| a) 6K effective ERP scheme of OMAF clause D.6.3. 6×2 tile grid with tile column widths of 768, 768, 768, 768, 384, and 384 luma samples and tile row heights of 2048 and 256 luma samples | b) 6K effective CMP scheme of OMAF clause D.6.4. 3×6 tile grid with tile column widths of 768, 768, and 384 luma samples and all tile row heights equal to 768 luma samples. | c) 6K effective CMP scheme. 6×3 tile grid with tile column widths of 768, 768, 768, 768, 384, and 384 luma samples and all tile row heights equal to 768 luma samples. |
| --- | --- | --- |
| d) 6K effective ERP scheme of OMAF clause D.6.3 with reversed tile column and row order. | e) 6K effective CMP scheme of OMAF clause D.6.4 with reversed tile column and row order. | f) 6K effective CMP scheme with reversed tile column and row order (compared to c). |

Figure 1. VVC tile and brick partitioning to achieve 6K effective resolutions for 360° video.

The following syntax table rows are excluded from the results of VVC Draft 6:

| if( uniform_tile_spacing_flag && brick_splitting_present_flag ) |  |
| --- | --- |
| num_tiles_in_pic_minus1 | ue(v) |

The following syntax table row is excluded from the results of this proposal:

| log2_pps_ctu_size_minus5 | u(2) |
| --- | --- |

The motivation for excluding the syntax table rows is the PPS parsing dependency on SPS, as documented in the following editor's note:

[Ed. (HD/YK): PPS parsing dependency on SPS is introduced by adding the syntax condition "if( RowHeight[ i ] > 1 )". Similarly for uniform_brick_spacing_flag[ i ].]

One way to fix the parsing dependency would be to introduce log2_pps_ctu_size_minus5 and removing the conditional signalling of num_tiles_in_pic_minus1. Thus, any potential bit count differences caused by the above-cited syntax table rows were regarded as insignificant for the decision-making.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).