| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0097 |
| --- | --- |

| Title: | AHG8/AHG17: Removing dependencies on VPS from the decoding process of a non-scalable bitstream |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution removes dependencies on VPS from the decoding process of a non-scalable bitstream. It is asserted that the proposed changes make the presence of VPS optional in a non-scalable bitstream.

- An additional constraint is proposed: when sps_video_parameter_set_id is equal to 0, it is a requirement for bitstream conformance that there is only a single layer in the bitstream.

- lIdTargetList is proposed to be an optional input parameter of the sub-bitstream extraction process (rather than being a mandatory input parameter as in VVC Draft 6). When not given as input, the extraction prunes NAL units only based on the TemporalId values.

- The derivation of TargetLayerIdList and its use as input for sub-bitstream extraction are proposed to take place only when sps_video_parameter_set_id is greater than 0.

- References to VPS syntax elements are proposed to be conditional on sps_video_parameter_set_id being greater than 0.

# Introduction

## Agreed design goal

JVET-O_Notes state the following agreement regarding the need of VPS in a non-scalable bitstream:

- VPS needs to be included in non-scalable bitstreams. Track A agreed to start with approach 1 as the goal and move to approach 2 if we see a good reason, and then to approach 3 if there’s a good reason.

  - Approach 1: Does not need to be in non-scalable bitstream

  - Approach 2 (like HEVC): Needs to be there, but not needed by decoder

  - Approach 3: Needed by non-scalable and scalable decoders

## sps_video_parameter_set_id equal to 0 indicating absence of VPS in VVC Draft 6

In VVC Draft 6, sps_video_parameter_set_id equal to 0 is reserved for the purpose of no VPS being needed for decoding. The semantics of sps_video_parameter_set_id are the following:

sps_video_parameter_set_id, when greater than 0, specifies the value of vps_video_parameter_set_id for the VPS referred to by the SPS. When sps_video_parameter_set_id is equal to 0, the SPS does not refer to a VPS and no VPS is referred to when decoding each CVS referring to the SPS.

The presence of VPS is mandatory prior to is being referred, as stated in the semantics of the VPS RBSP:

A VPS RBSP shall be available to the decoding process prior to it being referred, included in at least one access unit with TemporalId equal to 0 or provided through external means, and the VPS NAL unit containing the VPS RBSP shall have nuh_layer_id equal to vps_layer_id[ 0 ].

## Problem 1: use of VPS syntax elements in general decoding process

The syntax elements of VPS are used in the general decoding process in clause 8.1.1:

For each CVS in the bitstream, the list TargetLayerIdList, which identifies the list of target layers to be decoded, and the variable HighestTid, which identifies the highest temporal sub-layer to be decoded, are specified as follows:

–	If some external means, not specified in this Specification, is available to set TargetLayerIdList, TargetLayerIdList is set by the external means.

–	Otherwise, if vps_all_independent_layers_flag is equal to 1, TargetLayerIdList is set to contain only vps_layer_id[ 0 ].

–	Otherwise, TargetLayerIdList is set to contain vps_layer_id[ i ] for i in the range of 0 to vps_max_layers_minus1, inclusive.

## Problem 2: use of VPS syntax elements in deriving PictureOutputFlag

In clause 8.1.2, PictureOutputFlag is derived by using VPS syntax elements:

–	PictureOutputFlag is set as follows:

–	If one of the following conditions is true, PictureOutputFlag is set equal to 0:

–	the current picture is a RASL picture and NoIncorrectPicOutputFlag of the associated IRAP picture is equal to 1.

–	gdr_enabled_flag is equal to 1 and the current picture is a GDR picture with NoIncorrectPicOutputFlag equal to 1.

–	gdr_enabled_flag is equal to 1, the current picture is associated with a GDR picture with NoIncorrectPicOutputFlag equal to 1, and PicOrderCntVal of the current picture is less than RpPicOrderCntVal of the associated GDR picture.

–	vps_output_layer_mode is equal to 0 and the current access unit contains a picture that has pic_output_flag equal to 1, has nuh_layer_id nuhLid greater than that of the current picture, and belongs to an output layer (i.e., OutputLayerFlag[ GeneralLayerIdx[ nuhLid ] ] is equal to 1).

–	vps_output_layer_mode is equal to 2 and OutputLayerFlag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0.

–	Otherwise, PictureOutputFlag is set equal to pic_output_flag.

## Problem 3: use of VPS syntax elements for a conformance constraint

VPS syntax elements are also in the conformance constraint in the sub-bitstream extraction process in clause 10:

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream that is the output of the process specified in this clause with the bitstream, lIdTargetList containing of any particular nuh_layer_id value in the range of vps_layer_id[ 0 ] to vps_layer_id[ vps_max_layers_minus1 ], inclusive, when vps_all_independent_layers_flag is equal to 1, or lIdTargetList containing all nuh_layer_id value in the range of vps_layer_id[ 0 ] to vps_layer_id[ vps_max_layers_minus1 ], inclusive, when vps_all_independent_layers_flag is equal to 0, and tIdTarget equal to any value in the range of 0 to 6, inclusive, as inputs, and that satisfies the following conditions shall be a conforming bitstream:

–	The output sub-bitstream contains at least one VCL NAL unit with nuh_layer_id equal to each of the nuh_layer_id values in the lIdTargetList.

–	The output sub-bitstream contains at least one VCL NAL unit with TemporalId equal to tIdTarget.

NOTE – A conforming bitstream contains one or more coded slice NAL units with TemporalId equal to 0.

# Proposal

## Overview

This contribution proposes the following:

- An additional constraint is proposed: when sps_video_parameter_set_id is equal to 0, it is a requirement for bitstream conformance that there is only a single layer in the bitstream.

- lIdTargetList is proposed to be an optional input parameter of the sub-bitstream extraction process (rather than being a mandatory input parameter as in VVC Draft 6). When not given as input, the extraction prunes NAL units only based on the TemporalId values.

- The derivation of TargetLayerIdList and its use as input for sub-bitstream extraction are proposed to take place only when sps_video_parameter_set_id is greater than 0.

- References to VPS syntax elements are proposed to be conditional on sps_video_parameter_set_id being greater than 0.

## Proposed specification text changes

### 7.4.3.3	Sequence parameter set RBSP semantics

…

sps_video_parameter_set_id, when greater than 0, specifies the value of vps_video_parameter_set_id for the VPS referred to by the SPS. When sps_video_parameter_set_id is equal to 0, the SPS does not refer to a VPS and no VPS is referred to when decoding each CVS referring to the SPS. When sps_video_parameter_set_id is equal to 0 is any SPS referred to by coded pictures in a CVS, it is a requirement for bitstream conformance that nuh_layer_id shall have the same value in all NAL units of the CVS.

    - General

…

For each CVS in the bitstream, the list TargetLayerIdList, which identifies the list of target layers to be decoded, is specified as follows when sps_video_parameter_set_id is greater than 0 and the variable HighestTid, which identifies the highest temporal sub-layer to be decoded, are specified as follows:

–	If some external means, not specified in this Specification, is available to set TargetLayerIdList, TargetLayerIdList is set by the external means.

–	Otherwise, if vps_all_independent_layers_flag is equal to 1, TargetLayerIdList is set to contain only vps_layer_id[ 0 ].

–	Otherwise, TargetLayerIdList is set to contain vps_layer_id[ i ] for i in the range of 0 to vps_max_layers_minus1, inclusive.

For each CVS in the bitstream, the variable HighestTid, which identifies the highest temporal sub-layer to be decoded, is specified as follows:

–	If some external means, not specified in this Specification, is available to set HighestTid, HighestTid is set by the external means.

–	Otherwise, HighestTid is set equal to sps_max_sub_layers_minus1.

The variable DecodingUnitHrdFlag is specified as follows:

–	If the decoding process is invoked in a bitstream conformance test as specified in clause C.1, DecodingUnitHrdFlag is set as specified in clause C.1.

–	Otherwise, DecodingUnitHrdFlag is set equal to decoding_unit_hrd_params_present_flag.

For each CVS in the bitstream, the sub-bitstream extraction process as specified in clause 10 is applied with the CVS, TargetLayerIdList (when sps_video_parameter_set_id is greater than 0), and HighestTid as inputs, and the output is assigned to a bitstream referred to as CvsToDecode. After that, the instances of CvsToDecode of all the CVSs are concatenated, in decoding order, and the result is assigned to the bitstream BitstreamToDecode.

Clause 8.1.2 is repeatedly invoked for each coded picture in BitstreamToDecode in decoding order.

    - Decoding process for a coded picture

…

–	PictureOutputFlag is set as follows:

–	If one of the following conditions is true, PictureOutputFlag is set equal to 0:

–	the current picture is a RASL picture and NoIncorrectPicOutputFlag of the associated IRAP picture is equal to 1.

–	gdr_enabled_flag is equal to 1 and the current picture is a GDR picture with NoIncorrectPicOutputFlag equal to 1.

–	gdr_enabled_flag is equal to 1, the current picture is associated with a GDR picture with NoIncorrectPicOutputFlag equal to 1, and PicOrderCntVal of the current picture is less than RpPicOrderCntVal of the associated GDR picture.

–	sps_video_parameter_set_id is greater than 0, vps_output_layer_mode is equal to 0 and the current access unit contains a picture that has pic_output_flag equal to 1, has nuh_layer_id nuhLid greater than that of the current picture, and belongs to an output layer (i.e., OutputLayerFlag[ GeneralLayerIdx[ nuhLid ] ] is equal to 1).

–	sps_video_parameter_set_id is greater than 0, vps_output_layer_mode is equal to 2 and OutputLayerFlag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0.

–	Otherwise, PictureOutputFlag is set equal to pic_output_flag.

Note: editorial changes for creating bullet lists not highlighted below.

# 10	Sub-bitstream extraction process

Inputs to this process are:

–	a bitstream,

–	optionally, a target nuh_layer_id list lIdTargetList,

–	a target highest TemporalId value tIdTarget.

Output of this process is a sub-bitstream.

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream that is the output of the process specified in this clause with:

–	the bitstream,

–	at most one of the following:

–	when sps_video_parameter_set_id is greater than 0 and vps_all_independent_layers_flag is equal to 1, lIdTargetList containing of any particular nuh_layer_id value in the range of vps_layer_id[ 0 ] to vps_layer_id[ vps_max_layers_minus1 ], inclusive,

–	when sps_video_parameter_set_id is greater than 0 and vps_all_independent_layers_flag is equal to 0, lIdTargetList containing all nuh_layer_id value in the range of vps_layer_id[ 0 ] to vps_layer_id[ vps_max_layers_minus1 ], inclusive,

and

–	tIdTarget equal to any value in the range of 0 to 6, inclusive,

as inputs, and that satisfies the following conditions shall be a conforming bitstream:

–	When sps_video_parameter_set_id is greater than 0, the output sub-bitstream contains at least one VCL NAL unit with nuh_layer_id equal to each of the nuh_layer_id values in the lIdTargetList.

–	The output sub-bitstream contains at least one VCL NAL unit with TemporalId equal to tIdTarget.

NOTE – A conforming bitstream contains one or more coded slice NAL units with TemporalId equal to 0.

The output sub-bitstream is derived as follows:

–	Remove all NAL units with TemporalId greater than tIdTarget.

–	When lIdTargetList is given as input to this process, remove all NAL units with nuh_layer_id not included in lIdTargetList.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).