| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0112 |
| --- | --- |

| Title: | CE2: Test 2-3 Wavefront-Based Gradual Decoding Refresh (GDR) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Limin Wang Seungwook Hong Krit Panusopone | Tel: Email: | +1 858-335-9196 limin.2.wang@nokia.com seungwook.hong@nokia.com krit.panusopone@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution presents the test results of CE2-3 on wavefront-based GDR (Gradual Decoding Refresh), which is based upon contributions of JVET-O976 and JVET-O979. In general, a GDR picture consists of three areas; clean area, intra coded area and dirty area. Clean area and intra coded area can be considered as refresh area. With wavefront-based GDR approach, the reference samples for CUs in intra coded area can be either in clean area or in intra coded area. Hence, no restriction needs to be imposed on intra prediction for CUs in intra coded area. For CUs in clean area, if in inter mode, encoder makes sure the associated prediction blocks not use any pixels in dirty area. In addition, the in-loop filtering is disabled along the boundaries between the refresh area and the dirty area.

The test results demonstrated that the bitrates of CE2-3 vary smoothly from picture to picture with fluctuation only due to content. The simulation results of CE2-3 are also compared with the GDR anchor in terms of BD rate. With ALF, LMCS and local duel tree off for CE2-3 tests, CE2-3 shows a loss of 8.31% for class B, 9.69% for class C, 15.02% for class D, 23.08% for class E, and 29.72% for class F, respectively.

# Introduction

Instead of coding an intra picture, GDR gradually refreshes pictures from random access point to recovery point. In general, a GDR picture consists of three areas; clean area, intra coded area and dirty area. Clean area and intra coded area can be considered as refresh area. As examples, Fig. 1 illustrates the concept of vertical GDR, where intra coded area (green) moves and clean area (yellow) expands from left to right over pictures.

Fig. 1. Vertical GDR.

With the current VVC design, vertical GDR may have some reference samples for CUs in intra coded area in dirty area. Hence, restrictions may need to be imposed on intra prediction for CUs in intra coded area, which likely affect the coding performance.

# Proposed Solutions

JVET-O976 and JVET-O979 proposed a wavefront-based GDR approach, where intra coded areas (green) are spread over pictures and clean area is gradually expanded diagonally from the top-left corner to the bottom-right corner, as shown in Fig. 2.

Fig. 2. Wavefront GDR.

The main advantage of the wavefront-based GDR approach is that it can have the reference samples for CUs in intra coded area in either clean or intra coded areas. Therefore, no restriction needs to be imposed on intra prediction for CUs in intra coded area.

JVET-O979 further introduced the concept of intra GDR block in intra coded area. GDR blocks and CTUs are not necessarily the same. For example, CTU size can be of 128x128 as in JVET CTC while intra GDR block size can be set based upon picture size and frame rate, e.g. 16x16 for JVET class D sequences. Fig. 3 shows an example, where a current picture is divided into CTUs (white) and a set of intra GDR blocks (green) is placed along a diagonal line. Both CTUs and intra GDR blocks are square, and intra GDR block should not be greater than CTU.

Fig. 3. GDR blocks and CTUs.

JVET-O979 also proposed a set of rules for block partitioning as follows.

- If a block does not contain any intra GDR block, like CTU 1 and 6 in Fig. 3, a regular partition procedure is performed, that is, QT, BT and TT partitions are all allowed.

- If a block contains at least one intra GDR block, like CTU 2, 3, 4 and 5 in Fig. 3, some restrictions on partition are imposed. Specifically, partition line should not cut through intra GDR blocks.

- GDR blocks can be further partitioned into (smaller) CUs.

- CUs outside GDR blocks can be coded in either intra or inter mode.

- CUs inside GDR blocks are all coded in intra mode.

In HEVC, when exact_match_flag is equal to 1, it is required that the decoded pictures at and subsequent to the specified recovery point in output order derived by starting the decoding process at the access unit associated with the recovery point SEI message shall be an exact match to the pictures that would be produced by starting the decoding process at the location of a previous IRAP access unit, if any, in the bitstream.

If “exact_match” is required as in HEVC, only the coding information associated with clean areas and intra coded areas can be used for decoding the pictures between the random access point and the recovery point. Specifically, for CUs in intra coded area, the intra prediction modes cannot use any reference samples (or pixels) in dirty area. For CUs in clean area, if in inter mode, the associated prediction block cannot use any reconstructed pixels in dirty areas of reference pictures. In addition, the in-loop filtering (deblocking, SAO, etc) needs to be disabled for the boundary pixels next to the dirty areas.

The information about the boundary between the refresh area (clean area and intra coded area) and dirty area is signaled at pps and slice header, as proposed in JVET-O976. The related syntax are shown in the following PPS syntax table and slice header syntax table.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_loop_filter_across_virtual_boundaries_disabled_flag | u(1) |
| if( pps_loop_filter_across_virtual_boundaries_disabled_flag ) { |  |
| pps_virtual_boundary_type | ue(v) |
| if( pps_virtual_boundary_type = = 0 ) { |  |
| pps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_ver_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_x[ i ] | u(v) |
| pps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_hor_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_y[ i ] | u(v) |
| } else if( pps_virtual_boundary_type >= 1 && pps_virtual_boundary_type <= 3 ) |  |
| pps_virtual_boundary_granularity | ue(v) |
| } |  |
| … | u(1) |

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( pps_virtual_boundary_type >= 1 && pps_virtual_boundary_type <= 3 ) { |  |
| num_virtual_boundaries | u(2) |
| for( i = 0; i < num_virtual_boundaries; i++ ) { |  |
| virtual_boundary_pos[ i ] | u(v) |
| if( pps_virtual_boundary_type = = 3 ) |  |
| virtual_boundary_len[ i ] | ue(v) |
| } |  |
| } |  |
| … |  |

pps_virtual_boundary_type specifies GDR type. For example, pps_virtual_boundary_type equal to 1, 2, 3 may be assigned to vertical, horizontal, and diagonal GDR, respectively.

pps_virtual_boundary_granularity specifies the GDR blocks size. For example, pps_virtual_boundary_granularity equal to 0, 1,2, and 3 may be assigned to GDR block of 128x128, 64x64, 32x32 and 16x16, respectively.

num_virtual_boundaries specifies the number of virtual boundaries across which loop filtering is turned off.

virtual_boundary_pos[ i ] specifies the start address of the virtual boundary across which loop filtering is turned off. For wavefront-based GDR, the address is represented by an index along with the diagonal scan path, as shown in Fig. 5.

virtual_boundary_len[ i ] can be used to specify the length of intra coded area in terms of the number of GDR blocks.

Fig. 5. A GDR picture, where GDR blocks of 16x16 are indexed along with wavefront scan path.

As example, Fig. 5 shows a GDR picture, where the GDR block is of size 16x16, and the intra coded area has a start address of 45 and length of 6. Hence, for the example of Fig. 5, pps_virtual_boundary_granularity is equal to 3, virtual_boundary_pos[0] is equal to 45, and  virtual_boundary_len[0] is equal to 6, respectively. The set of parameters (3, 45, 6) can be used to describe the virtual boundary between the refresh area and the dirty area (red) of the GDR picture.

# Experimental Results

Simulations were conducted for CE2-3 over JVET test sequences. Each test is conducted with the following conditions:

- Use low-delay B configuration in current common test condition (CTC) (i.e., encoder_lowdelay_vtm.cfg) with the following changes:

  - The first picture is coded as an IDR picture,

  - The GOP size is 1. Each picture refers up to 4 immediately preceding pictures in decoding order,

  - For test sequences with 60 fps, a GDR picture is inserted every 60 pictures,

  - For test sequences with 50 fps, a GDR picture is inserted every 50 pictures,

  - For test sequences with 30 fps, a GDR picture is inserted every 30 pictures,

  - For test sequences with 20 fps, an GDR picture is inserted every 20 pictures,

  - IntraQPOffset and QPoffset are set equal to 0.

- Maximum GDR refresh period is 1 second.

- Number of forced GDR blocks per picture is set to (pic_width + GDR_block_size – 1) / GDR_block_size * (pic_height + GDR_block_size -1) / GDR_block_size) / frame_rate.

- SEIDecodedPictureHash is set to 1.

- ALF, LMCS and local dual tree are turned off.

- IBC, HashME and BDPCM are also off for class F.

The test results are compared with GDR anchor. Anchor bitstreams are encoded with the following conditions:

- Use low-delay B configuration in current common test condition (CTC) (i.e., encoder_lowdelay_vtm.cfg) with the following changes:

  - The first picture is coded as an IDR picture,

  - The GOP size is 1. Each picture refers up to 4 immediately preceding pictures in decoding order,

  - For test sequences with 60 fps, a CRA picture is inserted every 60 pictures,

  - For test sequences with 50 fps, a CRA picture is inserted every 50 pictures,

  - For test sequences with 30 fps, a CRA picture is inserted every 30 pictures,

  - For test sequences with 20 fps, a CRA picture is inserted every 20 pictures.

  - IntraQPOffset and Qpoffset are set equal to 0

- SEIDecodedPictureHash is set to 1.

The comparison results in terms of BD rate are shown in Table 1, where the pictures of the first second of sequences are excluded.

Table 1. ALF, LMCS and local dual tree are on for GDR anchor, but not for CE2-3 test.

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over O-CE2 Anchor (VTM6.0) on Linux |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 8.31% | 13.55% | 18.96% | 167% | 147% |
| Class C | 9.69% | 10.88% | 11.43% | 152% | 156% |
| Class E | 23.08% | 27.06% | 29.26% | 184% | 161% |
| Overall | 12.46% | 16.04% | 19.03% | 166% | 153% |
| Class D | 15.02% | 16.99% | 19.54% | 138% | 147% |
| Class F | 29.72% | 30.19% | 31.45% | 158% | 173% |

The bitrate fluctuations for CE2-3 are presented in the associated doc “JVET-P0112-CE2-3-BitFluc.xlsx”. As expected, the bitrates of CE2-3 tests vary smoothly with fluctuation only due to content.

# Conclusions

This contribution presents the test results for CE2-3 of wavefront-based GDR method. The test results demonstrate that the bitrates of CE2-3 vary smoothly with fluctuation only due to content. With ALF, LMCS and local dual tree off for CE2-3 tests, CE2-3 shows a loss of 8.31% for class B, 9.69% for class C, 15.02% for class D, 23.08% for class E, and 29.72% for class F.

.

# References

[1] L. Wang and M. Hannuksela, “Wavefront-based GRA and related syntax”, JVET O0976, July 2019.

[2] L. Wang, “Wavefront-based GRA method”, JVET O-979, July 2019.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).