| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0115-v2 |
| --- | --- |

| Title: | AHG8: Scalability - general and output layer sets |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang San Diego, CA, USA | Email: | ye-kui.wang@futurewei.com |
| Source: | Futurewei Technologies, Inc. |  |  |

_____________________________

# Abstract

It is asserted that the scalability design in VVC version 1 should be as simple as possible as long as main scalability use cases are addressed, while those fancy features that are not necessary for main scalability use cases, whenever they complicates the design and the specification, should not be included in VVC version 1.

This contribution proposes signalling of output layer sets.

The proposed specification text changes are included in the accompanying specification text document with change marks. Only those changes tagged with "JVET-P0115 OLS" in that document are for this contribution.

In v2 of this contribution, a text for the general decoding process in clause 8.1.1, as an implementation of the direction agreed at the initial discussion of scalability general and OLS aspects, is additionally proposed.

# Introduction

In the latest VVC draft, when some layers use inter-layer prediction (ILP), the entire bitstream consisting of all layers is specified to be decoded, and certain layers among the layers are specified to be output layers. The output layers may be indicated to be 1) only the highest layer, 2) all the layers, or 3) the highest layer plus a set of indicated lower layers.

For simplicity for describing the problem, assume that there are two layers, and the upper layer uses the lower layer for ILP reference. For typical spatial scalability and SNR scalability, it is necessary to specify the use of only the lower layer (i.e., decoding and output of the lower layer only) as well as the use of both layers (i.e., decoding both layers and output the higher layer). For typical multiview scalability, it is necessary to specify the use of only the lower layer (i.e., decoding and output of the lower layer only) as well as the use of both layers (i.e., decoding and output both layers). Unfortunately, both are not possible with the latest VVC draft.

Therefore, it is proposed that output layer sets (OLSs) should be specified.

In v2 of this contribution, a text for the general decoding process in clause 8.1.1 text as an implementation of the following JVET meeting minutes noted under this contribution is additionally proposed:

At least as a starting point, the following was agreed:

A bitstream fed to a decoder would have only one specified conforming decoded output.

Metadata may be present in a bitstream to indicate how to convert the bitstream to a different bitstream with a different conforming decoded output.

There may be a larger set of data stored on a server for which there is no conformance point specified as the decoded output, without first applying a conversion/extraction/rewriting process to that set of data.

This is different from the current spec in regard to temporal sublayering , as the current draft has a concept of an indication to a decoder of the highest sublayer to decode. Similarly, the current spec has a concept of a decoder receiving an indication of a layer to decode within a bitstream that contains more than one layer (for independent simulcast layers). Decision: Agreed. This affects only subclause 8.1.1 of the draft text.

The extraction process is not conceptually part of the decoding process; it is something that happens before the creation of a bitstream to be consumed by a decoder. It becomes part of the specification of the semantics of the syntax used for the extraction.

# Proposal

## General decoding process

The proposed text of the general decoding process text is provided below.

    - General

Input to this process is a bitstream BitstreamToDecode. Output of this process is a list of decoded pictures.

The decoding process is specified such that all decoders that conform to a specified profile and level will produce numerically identical cropped decoded output pictures when invoking the decoding process associated with that profile for a bitstream conforming to that profile and level. Any decoding process that produces identical cropped decoded output pictures to those produced by the process described herein (with the correct output order or output timing, as specified) conforms to the decoding process requirements of this Specification.

For each IRAP AU in the bitstream, the following applies:

–	If the AU is the first AU in the bitstream in decoding order, each picture is an IDR picture, or each picture is the first picture of the layer that follows an end of sequence NAL unit in decoding order, the variable NoIncorrectPicOutputFlag is set equal to 1.

–	Otherwise, if some external means not specified in this Specification is available to set the variable HandleCraAsCvsStartFlag to a value for the AU, HandleCraAsCvsStartFlag is set equal to the value provided by the external means and NoIncorrectPicOutputFlag is set equal to HandleCraAsCvsStartFlag.

–	Otherwise, HandleCraAsCvsStartFlag and NoIncorrectPicOutputFlag are both set equal to 0.

For each GDR AU in the bitstream, the following applies:

–	If the AU is the first AU in the bitstream in decoding order or each picture is the first picture of the layer that follows an end of sequence NAL unit in decoding order, the variable NoIncorrectPicOutputFlag is set equal to 1.

–	Otherwise, if some external means not specified in this Specification is available to set the variable HandleGdrAsCvsStartFlag to a value for the AU, HandleGdrAsCvsStartFlag is set equal to the value provided by the external means and NoIncorrectPicOutputFlag is set equal to HandleGdrAsCvsStartFlag.

–	Otherwise, HandleGdrAsCvsStartFlag and NoIncorrectPicOutputFlag are both set equal to 0.

NOTE – The above operations, for both IRAP pictures and GDR pictures, are needed for identification of the CVSs in the bitstream.

Clause 8.1.2 is repeatedly invoked for each coded picture in BitstreamToDecode in decoding order.

## OLS signalling

The proposed definitions, syntax, and semantics for signalling and specifying OLSs are provided below. The proposed specification text changes are also included in the accompanying specification text document with change marks. Only those changes tagged with "JVET-P0115 OLS" in that document are for this contribution.

#### Definitions

output layer: A layer of an output layer set that is output.

output layer set (OLS): A set of layers consisting of a specified set of layers, where one or more layers in the set of layers are specified to be output layers.

output layer set (OLS) layer index: An index, of a layer in an OLS, to the list of layers in the OLS.

#### Video parameter set RBSP syntax

| video_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| vps_video_parameter_set_id | u(4) |  |
| vps_max_layers_minus1 | u(6) |  |
| if( vps_max_layers_minus1 > 0 ) |  |  |
| vps_all_independent_layers_flag | u(1) |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |  |
| vps_layer_id[ i ] | u(6) |  |
| if( i > 0 && !vps_all_independent_layers_flag ) { |  |  |
| vps_independent_layer_flag[ i ] | u(1) |  |
| if( !vps_independent_layer_flag[ i ] ) |  |  |
| for( j = 0; j < i; j++ ) |  |  |
| vps_direct_dependency_flag[ i ][ j ] | u(1) |  |
| } |  |  |
| } |  |  |
| if( vps_max_layers_minus1 > 0 ) { |  |  |
| if( vps_all_independent_layers_flag ) |  |  |
| each_layer_is_an_ols_flag | u(1) |  |
| if( !each_layer_is_an_ols_flag ) { |  |  |
| if( !vps_all_independent_layers_flag ) |  |  |
| ols_mode_idc | u(2) |  |
| if( ols_mode_idc = = 2 ) { |  |  |
| num_output_layer_sets_minus1 | u(8) |  |
| for( i = 1; i < num_output_layer_sets_minus1 + 1; i ++) { |  |  |
| for( j = 0; j <= vps_max_layers_minus1; j++ ) |  |
| layer_included_flag[ i ][ j ] | u(1) |
| if( !vps_all_independent_layers_flag ) |  |
| for( j = 0; j < NumLayersInOls[ i ] − 1; j++ ) |  |  |
| vps_output_layer_flag[ i ][ j ] | u(1) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| vps_constraint_info_present_flag | u(1) |  |
| vps_reserved_zero_7bits | u(7) |  |
| if( vps_constraint_info_present_flag ) |  |  |
| general_constraint_info( ) |  |  |
| vps_extension_flag | u(1) |  |
| if( vps_extension_flag ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| vps_extension_data_flag | u(1) |  |
| rbsp_trailing_bits( ) |  |  |
| } |  |  |

#### Video parameter set RBSP semantics

A VPS RBSP shall be available to the decoding process prior to it being referenced, included in at least one access unit with TemporalId equal to 0 or provided through external means, and the VPS NAL unit containing the VPS RBSP shall have nuh_layer_id equal to vps_layer_id[ 0 ].

All VPS NAL units with a particular value of vps_video_parameter_set_id in a CVS shall have the same content.

vps_video_parameter_set_id provides an identifier for the VPS for reference by other syntax elements.

vps_max_layers_minus1 plus 1 specifies the maximum allowed number of layers in each CVS referring to the VPS.

vps_all_independent_layers_flag equal to 1 specifies that all layers in the CVS are independently coded without using inter-layer prediction. vps_all_independent_layers_flag equal to 0 specifies that one or more of the layers in the CVS may use inter-layer prediction. When not present, the value of vps_all_independent_layers_flag is inferred to be equal to 1. When vps_all_independent_layers_flag is equal to 1, the value of vps_independent_layer_flag[ i ] is inferred to be equal to 1. When vps_all_independent_layers_flag is equal to 0, the value of vps_independent_layer_flag[ 0 ] is inferred to be equal to 1.

vps_layer_id[ i ] specifies the nuh_layer_id value of the i-th layer. For any two non-negative integer values of m and n, when m is less than n, the value of vps_layer_id[ m ] shall be less than vps_layer_id[ n ].

vps_independent_layer_flag[ i ] equal to 1 specifies that the layer with index i does not use inter-layer prediction. vps_independent_layer_flag[ i ] equal to 0 specifies that the layer with index i may use inter-layer prediction and vps_layer_dependency_flag[ i ] is present in VPS. When not present, the value of vps_independent_layer_flag[ i ] is inferred to be equal to 1.

vps_direct_dependency_flag[ i ][ j ] equal to 0 specifies that the layer with index j is not a direct reference layer for the layer with index i. vps_direct_dependency_flag [ i ][ j ] equal to 1 specifies that the layer with index j is a direct reference layer for the layer with index i. When vps_direct_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to vps_max_layers_minus1, inclusive, it is inferred to be equal to 0.

The variable DirectDependentLayerIdx[ i ][ j ], specifying the j-th direct dependent layer of the i-th layer, is derived as follows:

for( i = 1; i < vps_max_layers_minus1; i++ )
	if( !vps_independent_layer_flag[ i ] )
		for( j = i, k = 0; j  >=  0; j− − )		(7-2)
			if( vps_direct_dependency_flag[ i ][ j ] )
				DirectDependentLayerIdx[ i ][ k++ ] = j

The variable GeneralLayerIdx[ i ], specifying the layer index of the layer with nuh_layer_id equal to vps_layer_id[ i ], is derived as follows:

for( i = 0; i  <=  vps_max_layers_minus1; i++ )		(7-3)
	GeneralLayerIdx[ vps_layer_id[ i ] ] = i

each_layer_is_an_ols_flag equal to 1 specifies that each output layer set contains only one layer and each layer itself in the bitstream is an outpu layer set with the single included layer being the only output layer. each_layer_is_an_ols_flag equal to 0 that an outpu layer set may contain more than one layer. If vps_max_layers_minus1 is equal to 0, the vlaue of the value of each_layer_is_an_ols_flag is inferred to be equal to 1. Otherwise, when vps_all_independent_layers_flag is equal to 0, the value of each_layer_is_an_ols_flag is inferred to be equal to 0.

ols_mode_idc equal to 0 specifies that the total number of OLSs specified by the VPS is equal to vps_max_layers_minus1 + 1, the i-th OLS includes the layers with layer indices from 0 to i, inclusive, and for each OLS only the highest layer in the OLS is output. ols_mode_idc equal to 1 specifies that the total number of OLSs specified by the VPS is equal to vps_max_layers_minus1 + 1, the i-th OLS includes the layers with layer indices from 0 to i, inclusive, and for each OLS all layers in the OLS are output. ols_mode_idc equal to 2 specifies that the total number of OLSs specified by the VPS is explicitly signalled and for each OLS the highest layer and an explicitly signalled set of lower layers in the OLS are output. The value of ols_mode_idc shall be in the range of 0 to 2, inclusive. The value 3 of ols_mode_idc is reserved for future use by ITU-T | ISO/IEC. When vps_all_independent_layers_flag is equal to 1 and each_layer_is_an_ols_flag is equal to 0, the value of ols_mode_idc is inferred to be equal to 2.

num_output_layer_sets_minus1 plus 1 specifies the total number of OLSs specified by the VPS when ols_mode_idc is equal to 2.

The variable TotalNumOlss, specifying the total number of OLSs specified by the VPS, is derived as follows:

if( vps_max_layers_minus1  = =  0 )
	TotalNumOlss = 1
else if( each_layer_is_an_ols_flag  | |  ols_mode_idc  = =  0  | |  ols_mode_idc  = =  1 )
	TotalNumOlss = vps_max_layers_minus1 + 1	(7-4)
else if( ols_mode_idc  = = 2 )
	TotalNumOlss = num_output_layer_sets_minus1 + 1

layer_included_flag[ i ][ j ] specifies whether the j-th layer (i.e., the layer with nuh_layer_id equal to vps_layer_id[ j ]) is included in the i-th OLS when ols_mode_idc is equal to 2. layer_included_flag[ i ][ j ] equal to 1 specifies that the j-th layer is included in the i-th OLS. layer_included_flag[ i ][ j ] equal to 0 specifies the j-th layer is not included in the i-th OLS.

The variable NumLayersInOls[ i ], specifying the number of layers in the i-th OLS, and the variable LayerIdInOls[ i ][ j ], specifying the nuh_layer_id value of the j-th layer in the i-th OLS, are derived as follows:

NumLayersInOls[ 0 ] = 1
LayerIdInOls[ 0 ][ 0 ] = vps_layer_id[ 0 ]
for( i = 1, i < TotalNumOlss; i++ ) {
	if( each_layer_is_an_ols_flag ) {
		NumLayersInOls[ i ] = 1
		LayerIdInOls[ i ][ 0 ] = vps_layer_id[ i ]		(7-4)
	} else if( ols_mode_idc  = =  0  | |  ols_mode_idc  = =  1 ) {
		NumLayersInOls[ i ] = i + 1
		for( j = 0; j < NumLayersInOls[ i ]; j++ )
			LayerIdInOls[ i ][ j ] = vps_layer_id[ j ]
	} else if( ols_mode_idc  = =  2 ) {
		for( k = 0, j = 0; k  <=  vps_max_layers_minus1; k++ )
			if( layer_included_flag[ i ][ k ] )
				LayerIdInOls[ i ][ j++ ] = vps_layer_id[ k ]
		NumLayersInOls[ i ] = j
	}
}

The variable OlsLayeIdx[ i ][ j ], specifying the OLS layer index of the layer with nuh_layer_id equal to LayerIdInOls[ i ][ j ], is derived as follows:

for( i = 0, i < TotalNumOlss; i++ )
	for j = 0; j < NumLayersInOls[ i ]; j++ )		(7-4)
		OlsLayeIdx[ i ][ LayerIdInOls[ i ][ j ] ] = j

The lowest layer in each OLS shall be an independent layer. In other words, for each i in the range of 0 to TotalNumOlss − 1, inclusive, the value of vps_independent_layer_flag[ GeneralLayerIdx[ LayerIdInOls[ i ][ 0 ] ] ] shall be equal to 1.

Each layer shall be included in at least one OLS specified by the VPS. In other words, for each layer with a particular value of nuh_layer_id nuhLayerId, equal to one of vps_layer_id[ k ] for k in the range of 0 to vps_max_layers_minus1, inclusive,  there shall be at least one pair of values of i and j, where i is in the range of 0 to TotalNumOlss − 1, inclusive, and j is in the range of NumLayersInOls[ i ] − 1, inclusive, such that the value of LayerIdInOls[ i ][ j ] is equal to nuhLayerId.

Any layer in an OLS shall be an output layer of the OLS or a (direct or indirect) reference layer of an output layer of the OLS.

vps_output_layer_flag[ i ][ j ] specifies whether the j-th layer in the i-th OLS is output when ols_mode_idc is equal to 2. vps_output_layer_flag[ i ] equal to 1 specifies that the j-th layer in the i-th OLS is output. vps_output_layer_flag[ i ] equal to 0 specifies that the j-th layer in the i-th OLS is not output. When vps_all_independent_layers_flag is equal to 1 and each_layer_is_an_ols_flag is equal to 0, the value of vps_output_layer_flag[ i ] is inferred to be equal to 1.

The variable OutputLayerFlag[ i ][ j ], for which the value 1 specifies that the j-th layer in the i-th OLS is output and the value 0 specifies that the j-th layer in the i-th OLS is not output, is derived as follows:

for( i = 0, i < TotalNumOlss; i++ ) {
	OutputLayerFlag[ i ][ NumLayersInOls[ i ] − 1 ] = 1
	for( j = 0; j < NumLayersInOls[ i ] − 1; j++ )
		if( ols_mode_idc[ i ]  = =  0 )
			OutputLayerFlag[ i ][ j ] = 0		(7-4)
		else if( ols_mode_idc[ i ]  = =  1 )
			OutputLayerFlag[ i ][ j ] = 1
		else if( ols_mode_idc[ i ]  = =  2 )
			OutputLayerFlag[ i ][ j ] = vps_output_layer_flag[ i ][ j ]
}

NOTE 1 – The 0-th OLS contains only the lowest layer (i.e., the layer with nuh_layer_id equal to vps_layer_id[ 0 ]) and for the 0-th OLS the only included layer is output.

vps_constraint_info_present_flag equal to 1 specifies that the general_constraint_info( ) syntax structure is present in the VPS. vps_constraint_info_present_flag equal to 0 specifies that the general_constraint_info( ) syntax structure is not present in the VPS.

vps_reserved_zero_7bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for vps_reserved_zero_7bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of vps_reserved_zero_7bits.

vps_extension_flag equal to 0 specifies that no vps_extension_data_flag syntax elements are present in the VPS RBSP syntax structure. vps_extension_flag equal to 1 specifies that there are vps_extension_data_flag syntax elements present in the VPS RBSP syntax structure.

vps_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in this version of this Specification. Decoders conforming to this version of this Specification shall ignore all vps_extension_data_flag syntax elements.

# Patent rights declaration(s)

Futurewei Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).