| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0121-v1 |
| --- | --- |

| Title: | On padding process of adaptive loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen Hendry 2330 Central Expy Santa Clara, CA, 95050 USA | Email: | Jianle.chen@futurewei.com fhendry@futurewei.com |
| Source: | Futurewei Technologies Inc. |  |  |

_____________________________

# Abstract

It was identified that the ALF sampling padding process in VTM6 does not work properly at slice boundaries in case of raster-scan slice mode. This contribution provides two options to fix the identified problem as follows:

Option 1: Apply a constraint such that raster scan slice mode shall be disallowed when ALF across brick boundaries is enabled but ALF across slice boundaries is disabled. The following constraint is proposed:

It is a requirement of bitstream conformance that when the value of loop_filter_across_bricks_enabled_flag is equal to 1 and loop_filter_across_slice_enabled_flag is equal to 0, the value of rect_slice_flag shall be equal to 1.

Option 2: Checking process is added to handle the raster-scan slice mode during ALF boundary position derivation process. For this process, in addition to check whether the top, bottom, left and right coding tree blocks (CTBs) are from the same slice, the top-left and bottom-right CTBs are also checked

# Problem statement

In VTM6, adaptive loop filter applies specific sample padding process at slice, brick and tile boundaries when filter across the boundaries is disabled. For the ALF block classification, only the samples which are in the same slice, brick or tile are used and the activity value is scaled accordingly. For the ALF filtering, symmetric padding is applied both in horizontal and vertical directions for horizontal and vertical boundaries respectively.

The padding process is shown in Figure 1

Figure 1. Padding methods in ALF for luma samples at slice/tile/brick boundaries.

There are two modes of slices are supported, namely the raster-scan slice mode and the rectangular slice mode. In the raster-scan slice mode, a slice contains a sequence of tiles in a tile raster scan of a picture. In the rectangular slice mode, a slice contains a number of bricks of a picture that collectively form a rectangular region of the picture. In current VVC draft, the defined process for disabling ALF across slice boundaries only works correctly in rectangular slice mode. While in raster-scan slice mode, the samples from another slice might still be used in the filtering process for a sample in the current slice.

# Proposal

In order to solve the identified problem, the following two optionals are proposed.

Option 1: Apply a constraint such that raster scan slice mode shall be disallowed when ALF across brick boundaries is enabled but ALF across slice boundaries is disabled. The following constraint is proposed:

It is a requirement of bitstream conformance that when the value of loop_filter_across_bricks_enabled_flag is equal to 1 and loop_filter_across_slice_enabled_flag is equal to 0, the value of rect_slice_flag shall be equal to 1.

Option 2: Checking process is added to handle the raster-scan slice mode during ALF boundary position derivation process.

In this proposed solution, in addition to check whether the top, bottom, left and right coding tree blocks (CTBs) are from the same slice, the top-left and bottom-right CTBs are also checked. When the top-left or bottom-right CTB is from a different slice, same sample padding process is applied when the filtering process involves the samples from a different slice. The proposed changes are as described below:

      - ALF boundary position derivation process

Inputs of this process are:

  - a luma location ( xCtb, yCtb ) specifying the top-left sample of the current luma coding tree block relative to the top left sample of the current picture,

  - a luma location ( x, y ) specifying the current sample relative to the top-left sample of the current luma coding tree block.

Output of this process are:

  - the left vertical boundary position clipLeftPos,

  - the right vertical boundary position clipRightPos,

  - the above horizontal boundary position clipTopPos,

  - the below horizontal boundary position clipBottomPos.

The variables clipLeftPos, clipRightPos, clipTopPos and clipBottomPos are set equal to −128.

The variable clipTopPos is modified as follows:

  - If the bottom boundary of the current coding tree block is not the bottom boundary of the picture and y − ( CtbSizeY − 4 ) is greater than or equal to 0, the variable clipTopPos is set equal to yCtb + CtbSizeY − 4.

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is equal to 0, and yCtb + y − PpsVirtualBoundariesPosY[ n ] is greater than or equal to 0 and less than 3 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

clipTopPos =  PpsVirtualBoundariesPosY[ n ]	(5-74)

  - Otherwise, if y is less than 3, and the top boundary of the current coding tree block is not the top boundary of the picture, and one or more of the following conditions are true, the variable clipTopPos is set equal to yCtb:

    - If the top boundary of the current coding tree block is the top boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the top boundary of the current coding tree block is the top boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If the top boundary of the current coding tree block is the top boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

- Otherwise, if ( x + y ) is less than 3, and both the top coding tree block and the left coding tree block of the current coding tree block are belong to the current slice , and the top-left top coding tree block of the current coding tree block is not belong to the current slice, and loop_filter_across_slices_enabled_flag is equal to 0, the variable clipTopPos is set equal to yCtb:

The variable clipBottomPos is modified as follows:

  - If the bottom boundary of the current coding tree block is not the bottom boundary of the picture and CtbSizeY − 4 − y is greater than 0 and is less than 4, the variable clipBottomPos is set equal to yCtb + CtbSizeY − 4.

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, PpsVirtualBoundariesPosY[ n ] % CtbSizeY is equal to 0, PpsVirtualBoundariesPosY[ n ] is not equal to pic_height_in_luma_samples − 1 or 0, and PpsVirtualBoundariesPosY[ n ] − yCtb − y is greater than 0 and less than 4 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

clipBottomPos =  PpsVirtualBoundariesPosY[ n ]	(5-75)

  - Otherwise, if CtbSizeY − y is less than 4, and the bottom boundary of the current coding tree block is not the bottom boundary of the picture, and one or more of the following conditions are true, the variable clipBottomPos is set equal to yCtb + CtbSizeY:

    - If the bottom boundary of the current coding tree block is the bottom boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the bottom boundary of the current coding tree block is the bottom boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If the bottom boundary of the current coding tree block is the bottom boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

- Otherwise, if ( 2 * ( CtbSizeY − 1 ) − x − y ) is less than 3, and both the bottom coding tree block and the right coding tree block of the current coding tree block are belong to the current slice , and the bottom-right coding tree block of the current coding tree block is not belong to the current slice, and loop_filter_across_slices_enabled_flag is equal to 0, the variable clipBottomPos is set equal to yCtb + CtbSizeY:

The variable clipLeftPos is modified as follows:

  - If pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is equal to 0, and xCtb + x − PpsVirtualBoundariesPosX[ n ] is greater than or equal to 0 and less than 3 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

clipLeftPos = PpsVirtualBoundariesPosX[ n ]	(5-76)

  - Otherwise, if x is less than 3, the left boundary of the current coding tree block is not the left boundary of the picture and one or more of the following conditions are true, the variable clipLeftPos is set equal to xCtb:

    - If the left boundary of the current coding tree block is the left boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the left boundary of the current coding tree block is the left boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If the left boundary of the current coding tree block is the left boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

- Otherwise, if ( x + y ) is less than 3, and both the top coding tree block and the left coding tree block of the current coding tree block are belong to the current slice , and the top-left top coding tree block of the current coding tree block is not belong to the current slice, and loop_filter_across_slices_enabled_flag is equal to 0, the variable clipLeftPos is set equal to xCtb.

The variable clipRightPos is modified as follows:

  - If pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is equal to 0, and PpsVirtualBoundariesPosX[ n ]− xCtb − x is greater than 0 and less than 4 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

clipRightPos = PpsVirtualBoundariesPosX[ n ]	(5-77)

  - Otherwise, if CtbSizeY − x is less than 4, and the right boundary of the current coding tree block is not the right boundary of the picture, and one or more of the following conditions are true, the variable clipRightPos is set equal to xCtb + CtbSizeY:

    - If the right boundary of the current coding tree block is the right boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the right boundary of the current coding tree block is the right boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - if the right boundary of the current coding tree block is the right boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

- Otherwise, if ( 2 * ( CtbSizeY − 1 ) − x − y ) is less than 3, and both the bottom coding tree block and the right coding tree block of the current coding tree block are belong to the current slice , and the bottom-right coding tree block of the current coding tree block is not belong to the current slice, and loop_filter_across_slices_enabled_flag is equal to 0, the variable clipRightPos is set equal to xCtb + CtbSizeY.

# Patent rights declaration(s)

Futurewei Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).