| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0128_v2 |
| --- | --- |

| Title: | AHG8: Scalability - GDR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Ahmed Hamza 9276 Scranton Road Suite 300 San Diego, CA 92121, USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes constraints on GDR scalability as the requirement for conformance. The constraints can be summarized as follows.

- An enhancement layer picture whose inter-layer reference picture is a GDR picture shall be a GDR picture

- An enhancement layer GDR picture shall have the same recovery_poc_cnt as its inter-layer GDR reference picture if available.

# Proposal

It is a requirement of bitstream conformance that, when present, each layer access unit in the next access unit after an access unit that contains an end of sequence NAL unit or an end of bitstream NAL unit shall be an IRAP layer access unit, which may be an IDR layer access unit or a CRA layer access unit, or a GDR layer access unit.

For layer coding structure, an enhancement layer picture may predict from a GDR reference picture of its direct dependent layer as shown in Figure 1.

Figure 1 GDR based inter-layer prediction structure example

VVC specifies gradual decoding refresh (GDR) access unit as an access unit in which the coded picture in each present layer access unit is a GDR picture.

Since the recovery_poc_cnt of inter-layer reference picture may not apply to enhancement layer pictures, a picture whose inter-layer reference picture is GDR picture shall be a GDR picture.

As a picture follows the current GDR picture and has PicOrderCntVal equal to the PicOrderCntVal of the current GDR picture plus the value of recovery_poc_cnt is referred as the recovery point picture, constraint is proposed for the enhancement layer GDR picture to have the same recovery_poc_cnt alue as its inter-layer GDR reference picture if its interlayer-reference picture is a GDR picture.

Figure 2 Proposed conformance requirement

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).