| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0129 |
| --- | --- |

| Title: | AHG12: On subpicture grid syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Ahmed Hamza 9276 Scranton Road Suite 300 San Diego, CA 92121, USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following modification for subpicture related syntax elements.

- Simple modification to replace max_subpics_minus1 with max_subpics_minus2

- Propose single_subpic_per_grid_flag to avoid signaling grid index for each subpicture when the grid is at the subpicture granularity.

# Proposal

This contribution proposes simple modification for signaling maximum number of sub-pictures syntax element. If subpics_present_flag is equal to 1, a picture is composed of two or more subpictures, which means number of minimum subpictures is 2 in a picture when subpics_present_flag is equal to 1.

Table 1 shows the proposed syntax max_subpics_minus2.

Table 1 Proposed subpicture syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus2 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |

max_subpics_minus2 plus 2 specifies the maximum number of subpictures that may be present in the CVS. max_subpics_minus1 shall be in the range of 0 to 254. The value of 255 is reserved for future use by ITU-T | ISO/IEC.

A subpicture may consist of one or multiple slices, a slice may consist of one or multiple bricks, a picture may be divided into multiple tiles uniformly. As a result, there is common scenario that a uniformly distributed tile/brick (uniform_tile_spacing_flag is equal to 1) may form a slice (single_brick_per_slice_flag is equal to 1) and a slice may form a subpicture.

Figure 1 single subpicture per uniformly distributed tile example

Figure 1 shows such an example for common CMP projection format where each face may be a uniformly distributed tile/brick. In such case, it is not necessary to signal subpic_grid_idx for each grid/subpicture, a subpic_per_grid_flag is proposed in Table 2.

Table 2 proposed SPS syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| subpic_per_grid_flag | u(1) |
| if ( !subpic_per_grid_flag ) |  |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |

subpic_per_grid_flag equal to 1 specifies that the syntax element subpic_grid_idx is not present in SPS. subpic_per_grid_flag equal to 0 specifies that syntax element subpic_grid_idx is present in SPS.

When subpic_per_grid_flag is equal to 1, subpic_grid_idx[ i ][ j ] can be derived as follows.

for( i = 0; i < NumSubPicGridRows; i++ )

for( j = 0; j < NumSubPicGridCols; j++ )

subpic_grid_idx[ i ][ j ] = i * NumSubPicGridRows + NumSubPicGridCols

The value of subpic_grid_idx shall be in the range of 0 to max_subpics_minus1, inclusive. With proposed signalling condition, the bit counts can be reduced to 30 bits for 6, 24 and 96 sub-pictures.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).