| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0130_v2 |
| --- | --- |

| Title: | AHG12: On associating slices with subpictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Ahmed Hamza 9276 Scranton Road Suite 300 San Diego, CA 92121, USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes subpicture related signaling as follows.

- The contribution supports rectangular slices as the units for subpicture as proposed in JVET-O0394 and JVET-N0109.

- Propose to associate slices to subpictures in PPS.

- Propose to use single_slice_per_subpic_flag to skip subpicture layout signaling.

- Propose to use subpicture index to identify specific subpicture for extraction and merging process.

# Introduction

It was agreed in 15th JVET meeting to use the SPS syntax proposed in O0555 for signaling sub-picture location and size (Table 1) as a starting point w/o considering scalability, RPR and GDR. Subpicture ID is not specified and there is no explicit mapping between the slice header and subpicture. The approach does not update slice header syntax elements for sub-bitstream extraction and reposition, while additional operations needed to identify the corresponding slices consisting a subpicture.

Table 1 SPS syntax in VVC D6VE

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |

Each subpicture may consist of one or more rectangular mode slices collectively form a rectangular region, and a slice may consist of one or more tiles or bricks. Since VVC already specifies lcation and size of tile/brick and rectangular slice in PPS (Table 2), and rectangular slice ID is carried in slice header slice_address (Table 3). It may be more efficient to use slice ID to indicate subpicture. Additional benefit is to facilitate the sub-bitstream extraction straighforwardly.

Table 2 Rectangular slice syntax in PPS

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( rect_slice_flag ) { |  |
| signalled_slice_id_flag | u(1) |
| if( signalled_slice_id_flag ) { |  |
| signalled_slice_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) |  |
| slice_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

Table 3 slice_address syntax in slice header

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && !single_brick_per_slice_flag ) |  |
| num_bricks_in_slice_minus1 | ue(v) |
| … |  |
| } |  |

# Proposal

It is proposed to map subpicture to corresponding slices in PPS since the partitioning to tiles, bricks, and rectangular slices is currently included in the PPS RBSP syntax. Figure 1 shows an example of a picture partitioned into tiles, bricks, and rectangular slices, where the picture is divided into 4 tiles (2 tile columns and 2 tile rows), 11 bricks (the top-left tile contains 1 brick, the top-right tile contains 5 bricks, the bottom-left tile contains 2 bricks, and the bottom-right tile contain 3 bricks), and 4 rectangular slices. There are 3 subpictures, the left and bottom-right subpictures contain a single rectangular slice, and the top-right subpicture contains 2 rectangular slices.

Figure 1 subpicture, rectangular mode slice, tile and brick example

Picture level RPR may be supported at sub-picture level to minimize viewport switching latency for viewport-dependent 360-degree streaming. Figure 2 shows such an example where each subpicture may represent a viewport face. When viewing orientation switches from viewport #0 to viewport #1, RPR allows high resolution viewport matching the viewing orientation to be presented instantly w/o introducing an IRAP subpicture.  As a result, the width, height and location of specific subpicture may change throughout the CVS.

Figure 2 subpicture based RPR example

Table 4 illustrates the proposed subpicture syntax elements in PPS to map a subpicture to one or more rectangular slices.

Table 4 Proposed sub-picture syntax in PPS

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if (rect_slice_flag) { |  |
| subpics_present_flag | u(1) |
| if ( subpics_present_flag) { |  |
| single_slice_per_subpic_flag | u(1) |
| if (!single_slice_per_subpic_flag) { |  |
| max_subpics_minus2 | u(8) |
| for ( i = 0; i < NumSubPics; i++ ) { |  |
| num_slices_in_subpic_minus1[ i ] | ue(v) |
| for ( j = 0; j <= num_slices_minus1[ i ]; j++ ) |  |
| subpic_slice_id[ i ][ j ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

subpics_present_flag equal to 1 indicates that subpicture parameters are present in the present in the PPS RBSP syntax. subpics_present_flag equal to 0 indicates that subpicture parameters are not present in the present in the PPS RBSP syntax.

single_slice_per_subpic_flag equal to 1 specifies that each subpicture that refers to this PPS includes one slice. Single_slice_per_subpic_flag equal to 0 specifies that a subpicture that refers to this PPS may include more than one slices. When not present, the value of single_slice_per_subpic_flag is inferred to be equal to 0.

max_subpics_minus1 plus 1 specifies the maximum number of subpictures that may be present in the CVS. max_subpics_minus1 shall be in the range of 0 to 254. The value of 255 is reserved for future use by ITU-T | ISO/IEC.

num_slices_in_subpic_minus1[ i ] plus 1 specifies the number of slices in i-th subpicture referring to the PPS. The value of num_slices_in_subpic_minus1 shall be in the range of 0 to NumBricksInPic − 1, inclusive. When not present and single_slice_per_subpic_flag is equal to 1, the value of num_slices_in_subpic_minus1[ i ] is inferred to be equal to NumBricksInPic − 1.

subpic_slice_id[ i ][ j ] specifies the slice ID of the j-th slice in i-th subpicture. The length of the subpic_slice_id[ i ][ j ] syntax element is signalled_slice_id_length_minus1 + 1 bits when signalled_slci_id_flag is equal to 1. Otherwise, when not present, the value of subpic_slice_id[ i ] is inferred to be equal to i, for each i in the range of 0 to num_slices_in_pic_minus1, inclusive.

The subpicture index can be used to identify specific subpicture associated with one or multiple slices for subpicture extraction and merging.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).