| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0135_r3 |
| --- | --- |

| Title: | AHG8/AHG17: On inter-layer reference picture signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Ma Haitao Yang | Tel: Email: | maxiang6@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

In the current VVC draft specification, some syntax elements for inter-layer reference information are signaled always, without considering the index of current layer, or the number of the reference layers. This contribution propose to add some conditions to the syntax elements to remove the redundancy.

Introduction

Propose 1: conditional signaling of vps_direct_dependency_flag[ i ][ j ]

The current syntax for signalling of vps_direct_dependency_flag[ i ][ j ] is as follows:

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_max_layers_minus1 | u(6) |
| if( vps_max_layers_minus1 > 0 ) |  |
| vps_all_independent_layers_flag | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_layer_id[ i ] | u(6) |
| if( i > 0 && !vps_all_independent_layers_flag ) { |  |
| vps_independent_layer_flag[ i ] | u(1) |
| if( !vps_independent_layer_flag[ i ] ) |  |
| for( j = 0; j < i; j++ ) |  |
| vps_direct_dependency_flag[ i ][ j ] | u(1) |
| } |  |
| } |  |
| if( vps_max_layers_minus1 > 0 ) { |  |
| … |  |

Actually, when the layer index i is equal to 1, and the vps_independent_layer_flag[ i ] is equal to 0, which means, the layer with index equal to 1 is a dependent layer. In this case, there is only one layer can be its reference layer, and the layer index of the reference layer must be 0. So, there is no need to signal vps_direct_dependency_flag[ i ][ j ], and it can be inferred to be 1.

In other words, when i is equal to 1, and vps_independent_layer_flag[ 1 ] is equal to 0, then vps_direct_dependency_flag[ 1 ][ 0 ] must be 1.

Propose 2: conditional signaling of inter_layer_ref_pics_present_flag

The current syntax for signalling of inter_layer_ref_pics_present_flag is as follows:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| long_term_ref_pics_flag | u(1) |
| inter_layer_ref_pics_present_flag | u(1) |
| sps_idr_rpl_present_flag | u(1) |
| rpl1_same_as_rpl0_flag | u(1) |
| … |  |

inter_layer_ref_pics_present_flag equal to 0 specifies that no ILRP is used for inter prediction of any coded picture in the CVS. inter_layer_ref_pics_flag equal to 1 specifies that ILRPs may be used for inter prediction of one or more coded pictures in the CVS. When sps_video_parameter_set_id is equal to 0, the value of inter_layer_ref_pics_present_flag is inferred to be equal to 0.

Here, inter_layer_ref_pics_present_flag is used to indicate whether ILRP is used for inter prediction of any coded picture in the CVS.

If current layer is a independent layer (vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] equal to 1), then there is no layer can be a reference layer, inter_layer_ref_pics_present_flag must be 0.

Propose 3: conditional signaling of ilrp_idc[ listIdx ][ rplsIdx ][ i ]

| ref_pic_list_struct( listIdx, rplsIdx ) { | Descriptor |
| --- | --- |
| num_ref_entries[ listIdx ][ rplsIdx ] | ue(v) |
| if( long_term_ref_pics_flag ) |  |
| ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] | u(1) |
| for( i = 0, j = 0; i < num_ref_entries[ listIdx ][ rplsIdx ]; i++) { |  |
| if( inter_layer_ref_pics_present_flag ) |  |
| inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( !inter_layer_ref_pics_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| if( long_term_ref_pics_flag ) |  |
| st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| if( AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] > 0 ) |  |
| strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| } else if( !ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] ) |  |
| rpls_poc_lsb_lt[ listIdx ][ rplsIdx ][ j++ ] | u(v) |
| } else |  |
| ilrp_idc[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| } |  |
| } |  |

ilrp_idc[ listIdx ][ rplsIdx ][ i ] specifies the index, to the list of directly dependent layers, of the ILRP of i-th entry in ref_pic_list_struct( listIdx, rplsIdx ) syntax structure to the list of directly dependent layers. The value of ilrp_idc[ listIdx ][ rplsIdx ][ i ] shall be in the range of 0 to the GeneralLayerIdx[ nuh_layer_id ] − 1, inclusive.

Actually, when GeneralLayerIdx[ nuh_layer_id ] is equal to 1, and if there is a referecce layer, the index of the reference layer must be 0, so in this case, there is no need to signal ilrp_idc[ listIdx ][ rplsIdx ][ i ].

Proposal text

Propose 1: conditional signaling of vps_direct_dependency_flag[ i ][ j ]

Option A: constraint

vps_direct_dependency_flag[ i ][ j ] equal to 0 specifies that the layer with index j is not a direct reference layer for the layer with index i. vps_direct_dependency_flag [ i ][ j ] equal to 1 specifies that the layer with index j is a direct reference layer for the layer with index i. When vps_direct_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to vps_max_layers_minus1, inclusive, it is inferred to be equal to 0. Here, for i and j in the range of 0 to i − 1, inclusive, when vps_independent_layer_flag[ i ] equal to 0, at least one value of vps_direct_dependency_flag[ i ][ j ] equal to 1.

Option B: constraint + conditional signaling

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_max_layers_minus1 | u(6) |
| if( vps_max_layers_minus1 > 0 ) |  |
| vps_all_independent_layers_flag | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_layer_id[ i ] | u(6) |
| if( i > 0 && !vps_all_independent_layers_flag ) { |  |
| vps_independent_layer_flag[ i ] | u(1) |
| if(!vps_independent_layer_flag[ i ] ) { |  |
| for( j = 0, SumDependencyFlag = 0; j < i; j++ ) { |  |
| if( !( j = = i − 1 && SumDependencyFlag = = 0) ) |  |
| vps_direct_dependency_flag[ i ][ j ] | u(1) |
| SumDependencyFlag += vps_direct_dependency_flag[ i ][ j ] |  |
| } |  |
| } |  |
| } |  |
| } |  |
| if( vps_max_layers_minus1 > 0 ) { |  |
| … |  |

vps_direct_dependency_flag[ i ][ j ] equal to 0 specifies that the layer with index j is not a direct reference layer for the layer with index i. vps_direct_dependency_flag [ i ][ j ] equal to 1 specifies that the layer with index j is a direct reference layer for the layer with index i. When vps_direct_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to vps_max_layers_minus1, inclusive, if vps_independent_layer_flag[ i ] equal to 0, and j equal to i − 1 , and the value of SumDependencyFlag equal to 0, then vps_direct_dependency_flag[ i ][ j ] is inferred to be equal to 1, otherwise, it is inferred to be equal to 0. Here, for i and j in the range of 0 to i − 1, inclusive, when vps_independent_layer_flag[ i ] equal to 0, at least one value of vps_direct_dependency_flag[ i ][ j ] equal to 1.

Propose 2: conditional signaling of inter_layer_ref_pics_present_flag

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| long_term_ref_pics_flag | u(1) |
| if( !vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] ) |  |
| inter_layer_ref_pics_present_flag | u(1) |
| sps_idr_rpl_present_flag | u(1) |
| rpl1_same_as_rpl0_flag | u(1) |
| … |  |

inter_layer_ref_pics_present_flag equal to 0 specifies that no ILRP is used for inter prediction of any coded picture in the CVS. inter_layer_ref_pics_flag equal to 1 specifies that ILRPs may be used for inter prediction of one or more coded pictures in the CVS. When sps_video_parameter_set_id is equal to 0 inter_layer_ref_pics_flag is not present, the value of inter_layer_ref_pics_present_flag it is inferred to be equal to 0.

Propose 3: conditional signaling of ilrp_idc[ listIdx ][ rplsIdx ][ i ]

| ref_pic_list_struct( listIdx, rplsIdx ) { | Descriptor |
| --- | --- |
| num_ref_entries[ listIdx ][ rplsIdx ] | ue(v) |
| if( long_term_ref_pics_flag ) |  |
| ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] | u(1) |
| for( i = 0, j = 0; i < num_ref_entries[ listIdx ][ rplsIdx ]; i++) { |  |
| if( inter_layer_ref_pics_present_flag ) |  |
| inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( !inter_layer_ref_pics_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| if( long_term_ref_pics_flag ) |  |
| st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| if( AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] > 0 ) |  |
| strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| } else if( !ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] ) |  |
| rpls_poc_lsb_lt[ listIdx ][ rplsIdx ][ j++ ] | u(v) |
| } else if( GeneralLayerIdx[ nuh_layer_id ] > 1 ) |  |
| ilrp_idc[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| } |  |
| } |  |

ilrp_idc[ listIdx ][ rplsIdx ][ i ] specifies the index, to the list of directly dependent layers, of the ILRP of i-th entry in ref_pic_list_struct( listIdx, rplsIdx ) syntax structure to the list of directly dependent layers. The value of ilrp_idc[ listIdx ][ rplsIdx ][ i ] shall be in the range of 0 to the GeneralLayerIdx[ nuh_layer_id ] − 1, inclusive. When GeneralLayerIdx[ nuh_layer_id ] is equal to 1, the value of ilrp_idc[ listIdx ][ rplsIdx ][ i ] is inferred to be equal to 0.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).