| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0141-v1 |
| --- | --- |

| Title: | AHG17/AHG12: On signalling the subpicture IDs |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen, Chih-Wei Hsu, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {lulin.chen, cw.hsu, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

Signalling the subpicture IDs is proposed.

- Signalling in the PPS and the slice header, or

- Signalling in the SPS and the slice header, when assuming the subpicure IDs do not change for the CVS.

# Introduction

In the current VVC draft [1], the subpicture information is specified in the SPS.

      - 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

# Proposal

The subpicures are only indexed in the SPS according to the current VVC draft. This contributin proposed to add signalling the subpicture ID for each subpicture within a picture. The signalled subpicture ID, subpic_id[ i ], for the i-th subpicture may be added in the PPS and the signalled subpicture ID, subpicure_id, mapping to subpic_id[ i ] in the PPS, may be added in the slice headers. See below as highlighted.

| Video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_max_layers_minus1 | u(6) |
| vps_max_subpics_minus1 | u(8) |
| … |  |
| rbsp_trailing_bits( ) |  |
| } |  |

vps_max_subpics_minus1 plus 1 specifies the maximum allowed number of subpictures in each CVS referring to the VPS. vps_max_subpics_minus1 shall be in the range of 0 to 254. The value of 255 is reserved for future use by ITU-T | ISO/IEC.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| maxnum_subpics_minus1 | u(8) |
| … |  |
| } |  |
| … |  |
| rbsp_trailing_bits( ) |  |
| } |  |

num_subpics_minus1 plus 1 specifies the number of subpictures that may be present in the CVS. num_subpics_minus1 shall be in the range of 0 to 254. The value of 255 is reserved for future use by ITU-T | ISO/IEC.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| … |  |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| single_tile_in_pic_flag | u(1) |
| … |  |
| if( rect_slice_flag ) { |  |
| signalled_slice_id_flag | u(1) |
| if( signalled_slice_id_flag ) { |  |
| signalled_slice_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) |  |
| slice_id[ i ] | u(v) |
| } |  |
| } |  |
| if(subpics_present_flag ) { |  |
| signalled_subpic_id_flag | u(1) |
| if( signalled_subpic_id_flag ) { |  |
| signalled_subpic_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_subpics_minus1; i++ ) |  |
| subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| entropy_coding_sync_enabled_flag | u(1) |
| … |  |
| slice_header_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

| slice_header( ) { | Descriptor |  |
| --- | --- | --- |
| slice_pic_parameter_set_id | ue(v) |  |
| if( subpics_present_flag ) |  |  |  |
| subpicture_id |  | u(v) |  |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |  |
| slice_address | u(v) |  |
| if( !rect_slice_flag && !single_brick_per_slice_flag ) |  |  |
| num_bricks_in_slice_minus1 | ue(v) |  |
| non_reference_picture_flag | u(1) |  |
| slice_type | ue(v) |  |
| … |  |  |
| if( slice_header_extension_present_flag ) { |  |  |
| slice_header_extension_length | ue(v) |  |
| for( i = 0; i < slice_header_extension_length; i++) |  |  |
| slice_header_extension_data_byte[ i ] | u(8) |  |
| } |  |  |
| byte_alignment( ) |  |  |
| } |  |  |

signalled_subpic_id_flag equal to 1 specifies that the subpicture ID for each subpicture is signalled. signalled_subpic_id_flag equal to 0 specifies that subpicture IDs are not signalled. When not present, the value of signalled_subpic_id_flag is inferred to be equal to 0.

signalled_subpic_id_length_minus1 plus 1 specifies the number of bits used to represent the syntax element subpic_id[ i ] when present, and the syntax element subpicture_id in slice headers. The value of signalled_subpic_id_length_minus1 shall be in the range of 0 to 7, inclusive. When not present, the value of signalled_subpic_id_length_minus1 is inferred to be equal to Ceil( Log2( Max( 2, num_subpics_minus1 + 1 ) ) ) − 1.

subpic_id[ i ] specifies the subpicture ID of the i-th subpicture. The length of the subpuic_id[ i ] syntax element is signalled_subpic_id_length_minus1 + 1 bits.  When not present, the value of subpic_id[ i ] is inferred to be equal to i, for each i in the range of 0 to num_subpics_minus1, inclusive. The slices with the same subpicture ID collectively form the subpicture region.

subpicture_id specifies the subpicture ID to which the the current slice belongs. The length of the subpic_id syntax element is Ceil( Log2( num_subpics_minus1 + 1 ) ) bits. The value of subpicture_id is a mappting to subpic_id[ i ] specified in the PPS.

When assuming the subpicure IDs do not change for the CVS, the signalled subpicture ID, subpic_id[ i ], for the i-th subpicture may be added in the SPS instead of the PPS as follows.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| signalled_subpic_id_flag | u(1) |
| if( signalled_subpic_id_flag ) { |  |
| signalled_subpic_id_length_minus1 | ue(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_id[ i ] | u(v) |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

# Conclusions

Signalling the subpicture IDs is proposed.

- Signalling in the PPS and the slice header, or

- Signalling in the SPS and the slice header, when assuming the subpicure IDs do not change for the CVS.

# References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team, JVET-O2001

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).