| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0142-v1 |
| --- | --- |

| Title: | AHG17/AHG12: Comments on the subpicture grid in the SPS |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen, Chih-Wei Hsu, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {lulin.chen, cw.hsu, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

Considering the picture size is an integer of multiple of 8 samples and brick/tile/slice is in units of CTBs, two more efficient options for discussions are proposed.

- Option 1: Using the grid in units of 8 samples to ensure the subpicture size is also in integer of multiple of 8 samples. However it needs to take care that the subpicture size is also aligned with CTBs.

- Option 2: Using the grid of units of CTBs to ensure the subpicture size is the same in principle as that of the picture (as the subpicture may be extracted as a picture).

# Introduction

In the current VVC draft [1], the subpicture structure with a grid approach is added in the SPS.

      - 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

# Proposal

In the current VVC draft, the subpicture grid approach is using a grid in units of 4 samples. Considering the picture size is an integer of multiple of 8 samples and brick/tile/slice is in units of CTBs, two more efficient options for discussions if the group still think the grid approach is the right way to specify the subpicture structure. The proponent prefers the option 2 in this aspect.

- Option 1: Using the grid in units of 8 samples to ensure the subpicture size is also in integer of multiple of 8 samples. However it needs to take care that the subpicture size is also aligned with CTBs.

- Option 2: Using the grid of units of CTBs to ensure the subpicture size is the same in principle as that of the picture (as the subpicture may be extracted as a picture).

The proposed text changes are as follows (highlighted in yellow).

Option 1: the grid in units of 8 samples

Change 4 to 8 in the related semantice as below.

subpic_grid_col_width_minus1 plus 1 specifies the width of each element of the subpicture identifier grid in units of 8 samples. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples / 8) ) bits.

The variable NumSubPicGridCols is derived as follows:

NumSubPicGridCols = ( pic_width_max_in_luma_samples + subpic_grid_col_width_minus1 * 8 + 7 ) /

( subpic_grid_col_width_minus1 * 8 + 8 )                                        (7-5)

subpic_grid_row_height_minus1 plus 1 specifies the height of each element of the subpicture identifier grid in units of 8 samples. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples / 8) ) bits.

The variable NumSubPicGridRows is derived as follows:

NumSubPicGridRows = (pic_height_max_in_luma_samples + subpic_grid_row_height_minus1 * 8 + 7) /
			( subpic_grid_row_height_minus1 * 8 + 8)                                        (7-6)

The scanning process as in 6.5.1 may be modified as:

The list CtbToSubPicIdx[ ctbAddrRs ] for ctbAddrRs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in picture raster scan to a subpicture index, is derived as follows:

for( ctbAddrRs = 0; ctbAddrRs < PicSizeInCtbsY; ctbAddrRs++ ) {
	posX = ctbAddrRs % PicWidthInCtbsY * CtbSizeY 
	posY = ctbAddrRs / PicWidthInCtbsY * CtbSizeY 
	CtbToSubPicIdx[ ctbAddrRs ] = −1
	for( i = 0; CtbToSubPicIdx[ ctbAddrRs ] < 0  && i < NumSubPics; i++ ) {
		if( ( posX  >=  SubPicLeft[ i ] * ( subpic_grid_col_width_minus1 + 1 ) * 8 )  &&
				( posX < ( SubPicLeft[ i ] + SubPicWidth[ i ] ) *
						( subpic_grid_col_width_minus1 + 1 ) * 8 )  &&	(6-11)
				( posY  >=  SubPicTop[ i ] *
						( subpic_grid_row_height_minus1 + 1 ) * 8 )  &&
				( posY < ( SubPicTop[ i ] + SubPicHeight[ i ] ) *
						( subpic_grid_row_height_minus1 + 1 ) * 8 ) )
			CtbToSubPicIdx[ ctbAddrRs ] = i
	}
}

Option 2: the grid in units of CTBs

log2_ctu_size_minus5 is moved up to be used in the derivations.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| log2_ctu_size_minus5 | u(2) |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| min_qp_prime_ts_minus4 | ue(v) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| if( sps_max_sub_layers_minus1 > 0 ) |  |
| sps_sub_layer_ordering_info_present_flag | u(1) |
| … |  |
| if( ChromaArrayType != 0 ) |  |
| qtbtt_dual_tree_intra_flag | u(1) |
| log2_ctu_size_minus5 | u(2) |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

subpic_grid_col_width_minus1 plus 1 specifies the width of each element of the subpicture identifier grid excluding the right-most grid column of the picture in units of CTBs. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples / CtbSizeY) ) bits.

The variable NumSubPicGridCols is derived as follows:

NumSubPicGridCols = (pic_width_max_in_luma_samples + subpic_grid_col_width_minus1 * CtbSizeY + CtbSizeY – 1) /

( subpic_grid_col_width_minus1 * CtbSizeY + CtbSizeY )                          (7-5)

subpic_grid_row_height_minus1 plus 1 specifies the height of each element of the subpicture identifier grid excluding the bottom grid row of the picture in units of CTBs. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples / CtbSizeY) ) bits.

The variable NumSubPicGridRows is derived as follows:

NumSubPicGridRows = (pic_height_max_in_luma_samples + subpic_grid_row_height_minus1 * CtbSizeY + CtbSizeY – 1 ) /
		( subpic_grid_row_height_minus1 * CtbSizeY + CtbSizeY)                         (7-6)

The scanning process as in 6.5.1 may be modified accordingly as:

The list CtbToSubPicIdx[ ctbAddrRs ] for ctbAddrRs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in picture raster scan to a subpicture index, is derived as follows:

for( ctbAddrRs = 0; ctbAddrRs < PicSizeInCtbsY; ctbAddrRs++ ) {
	posX = ctbAddrRs % PicWidthInCtbsY * CtbSizeY 
	posY = ctbAddrRs / PicWidthInCtbsY * CtbSizeY 
	CtbToSubPicIdx[ ctbAddrRs ] = −1
	for( i = 0; CtbToSubPicIdx[ ctbAddrRs ] < 0  && i < NumSubPics; i++ ) {
		if( ( posX  >=  SubPicLeft[ i ] * ( subpic_grid_col_width_minus1 + 1 ) * CtbSizeY )  &&
				( posX < ( SubPicLeft[ i ] + SubPicWidth[ i ] ) *
						( subpic_grid_col_width_minus1 + 1 ) * CtbSizeY )  &&	(6-11)
				( posY  >=  SubPicTop[ i ] *
						( subpic_grid_row_height_minus1 + 1 ) * CtbSizeY )  &&
				( posY < ( SubPicTop[ i ] + SubPicHeight[ i ] ) *
						( subpic_grid_row_height_minus1 + 1 ) * CtbSizeY ) )
			CtbToSubPicIdx[ ctbAddrRs ] = i
	}
}

Note: Similar to a picture that the right-most column and the bottom row of CTU may be incomplete, the grid elements in the right-most grid column of the picture may be incomplete and the grid width may be smaller than (subpic_grid_row_width_minus1 + 1) * CtbSizeY. The grid elements in the bottom grid row of the picture may be incomplete and the grid height may be smaller than (subpic_grid_row_height_minus1 + 1) * CtbSizeY.

# Conclusions

Two more efficient options for the subpicture grid and the proponent prefers the option 2 if the grid remains in the SPS after discussion:

- Option 1: Using the grid in units of 8 samples to ensure the subpicture size is also in integer of multiple of 8 samples. However it needs to take care that the subpicture size is also aligned with CTBs.

- Option 2: Using the grid of units of CTBs to ensure the subpicture size is the same in principle as that of the picture (as the subpicture may be extracted as a picture).

# References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team, JVET-O2001

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).