| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0146-v1 |
| --- | --- |

| Title: | AHG17/AHG12: On enabling subpicture with random access properties |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {lulin.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

A method is proposed to specify subpictures with random access properties in the AUD. au_subpic_type_signalled_flag, au_subpic_type_flag[ i ], and au_subpic_type[ i ] are introduced in the AUD syntax.

# Introduction

In the VVC Draft 6 [1], the subpicture information is signalled in the SPS and the access unit delimiter (AUD) is the start of an AU, see below as highlighted.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

        - The 7.4.2.4.3 clause in the current draft specifies the order of NAL units and coded pictures and their association to layer access units and access units for CVSs.

A layer access unit consists of one coded picture, zero or more VCL NAL units and zero or more non-VCL NAL units. The association of VCL NAL units to coded pictures is described in clause 7.4.2.4.4.

An access unit consists of an access unit delimiter NAL unit and one or more layer access units in increasing order of nuh_layer_id.

The first access unit in the bitstream starts with the first NAL unit of the bitstream.

Each acess unit starts with an access unit delimiter NAL unit. There shall be at most one access unit delimiter NAL unit in any layer access unit.

The access unit delimiter RBSP syntax and semantics are as below.

| access_unit_delimiter_rbsp( ) { | Descriptor |
| --- | --- |
| pic_type | u(3) |
| rbsp_trailing_bits( ) |  |
| } |  |

The access unit delimiter is used to indicate the start of an access unit and the type of slices present in the coded pictures in the access unit containing the access unit delimiter NAL unit. There is no normative decoding process associated with the access unit delimiter.

pic_type indicates that the slice_type values for all slices of the coded pictures in the access unit containing the access unit delimiter NAL unit are members of the set listed in Table 7-3 for the given value of pic_type. The value of pic_type shall be equal to 0, 1 or 2 in bitstreams conforming to this version of this Specification. Other values of pic_type are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this version of this Specification shall ignore reserved values of pic_type.

Table 7-3 – Interpretation of pic_type

| pic_type | slice_type values that may be present in the coded picture |
| --- | --- |
| 0 | I |
| 1 | P, I |
| 2 | B, P, I |

# Proposal

As a desired feature that random access to subpictures may help in some use cases such as viewport dependent streaming, subpictures may have their own random access properties rather than that of the whole picture. This contribution proposes to use the access unit delimiter (AUD) to further provide specific subpicture information for spatial and temporal random access. The AUD may serve as the picture header and the decoding process becomes a normative part to the subpicture support.

Currently the subpicture related information is mainly in the SPS [1] and the number of subpictures and the indexes may be derived. Some improvements on that may be in a separate contribution.

Generally a subpicture is an rectangular region of one or more slices within a picture. For pic_type = 0, 1, or 2 as indicated in the AUD, there may be some I slices within the picure forming subpictures and those may be extracted as IRAP picutures in some use cases for example for viewport dependent streaming applications, when it is specified that subpicure is treated as picture ( subpic_treated_as_pic_flag[ i ] = 1 ) and loop filtering across subpictures is not enabled ( loop_filter_across_subpic_enabled_flag[ i ] = 0 ). Thus the subpicture, when extracted, may be served as an IDR, CRA, or GDR picture and the corresponding signalling may be added in the AUD syntax. A method is proposed, see below as highlighted.

| Access_unit_delimiter_rbsp( ) { | Descriptor |
| --- | --- |
| pic_type | u(3) |
| if( subpics_present_flag ) { |  |
| au_subpic_type_signalled_flag | u(1) |
| if( au_subpic_type_signalled_flag ) { |  |
| for( i = 0; i <= NumSubPics; i++ ) |  |
| if( subpic_treated_as_pic_flag[ i ] == 1 && loop_filter_across_subpic_enabled_flag[ i ] == 0 ) { |  |
| au_subpic_type_flag[ i ] | u(1) |
| if ( au_subpic_type_flag[ i ] ) |  |
| au_subpic_type[ i ] | u(2) |
| } |  |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

au_subpic_type_signalled_flag equal to 1 indicates that subpicture type signalling is present in the current AUD RBSP syntax. au_subpic_type_signalled_flag equal to 0 indicates that subpicture type signalling is not present in the current AUD RBSP syntax.

au_subpic_type_flag[ i ] equal to 1 indicates the value of au_subpic_type for the i-th subpicture is present. au_subpic_type_flag[ i ] equal to 0 indicates the value of au_subpic_type for the i-th subpicture is not present.

au_subpic_type[ i ] indicates the types of slices of the subpicture in the AU as specified in Table 7-Y.

Table 7–Y - Interpretation of au_subpic_type

| au_subpic_type | The types of slices of the subpicture in the (layer) access unit that may be present |
| --- | --- |
| 0 | Slices are coded slices of a non-IRAP picture |
| 1 | Slices are coded slices of an IDR picture with nal_unit_type = IDR_W_RADL |
| 2 | Slices are coded slices of an IDR picture with nal_unit_type = IDR_N_LP |
| 3 | Slices are coded slices of a CRA picture |

NOTE 1 – when a bitstream is the result of a sub-bitstream extraction process and contains slices as in Table 7-Y for the subpictures, it might be required to set the value of nal_unit_type accordingly. The corresponding syntax re-writing may apply for the relevant parameter sets and slice headers etc. When au_subpic_type != 0, the subpicure may be considered as a random access point.

NOTE 2 –  a subpicture formed by a refreshed region in a GDR picture and the following pictures before the recovery point, the value of au_subpic_type for the subpicture may be set equal to 2, or 3.

The value 0 of au_subpic_type in Table 7-Y may be reserved. The subpicture that the slices are coded slices of a non-IRAP picture is implied by setting au_subpic_type_flag[ i ] equal to 0.

Generally the subpictures are indexed with associated size and position information and each of them may be mapped with a subpicture ID.

# Conclusions

A method is proposed to specify subpictures with random access properties in the AUD. au_subpic_type_signalled_flag, au_subpic_type_flag[ i ], and au_subpic_type[ i ] are introduced in the AUD syntax.

# References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team, JVET-O2001

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).