| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0148-v2 |
| --- | --- |

| Title: | AHG18: Disabling dependent quantization in lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei Tsung-Chuan Ma, Yi-Wen Chen, Xiaoyu Xiu, Hong-jheng Jhu, Xianglin Wang | Email: | {zhi-yi.lin, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com {tsung-chuanma, yiwenchen, xiaoyuxiu, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | MediaTek Inc., Kwai Inc. |  |  |

_____________________________

# Abstract

When enabling dependent quantization (DQ) at slice level, the conversion from coefficient levels to quantized coefficients and the quantization state (Q-state) dependent coefficient coding is performed in the coefficient parsing stage. The conversion from coefficient levels to quantized coefficients always changes the received residuals, which makes lossless coding impossible. The Q-state dependent coefficient coding was designed under lossy coding conditions and may not be beneficial for lossless coding. Therefore, Method 1 is proposed to achieve lossless coding by removing the conversion from coefficient levels to quantized coefficients, and has no impact on the VTM6.0-lossless software in BD-rate and runtime. Based on Method 1, Method 2 further disables the Q-state transition and fix the Q-state to state 0 during the coefficient coding. It reportedly shows minor bit saving compared to the VTM6.0-lossless software under AI and RB, respectively, with negligible runtime changes. Based on Method 2, Method 3 further removes the signalling of the last significant coefficient position in lossless blocks. Results of Method 3 show 0.62 % and X.XX% bit savings compared to the VTM6.0-lossless software under AI and RB, respectively, with negligible runtime changes.

# Introduction

In VVC Draft 6 [1], DQ comprises two parts. The first part is using different quantizers in quantization, and the second part is Q-state dependent coefficient coding. When DQ is enabled, a Q-state transition is performed according to each coefficient’s parity bit. During the coefficient coding, both the context variable set for each coefficient’s significant flag and the Rice parameter derivation are dependent on the Q-state. After the sign flags in one coding group are decoded, the coefficient levels are converted to quantized coefficients. These quantized coefficients will be further dequantized in the inverse quantization stage. However, when a block is coded in lossless mode, the conversion from coefficient levels to quantized coefficients will make lossless coding impossible since the original residuals are changed by the conversion.

# Proposed method

## Method 1: No conversion from coefficient levels to quantized coefficients in lossless mode

It is proposed to disable the conversion from coefficient levels to quantized coefficients when the current block is coded with lossless mode. The specification changes are as follows.

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| … |  |
| QState = 0 |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| startQStateSb = QState |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 1 ] |  |
| inferSbDcSigCoeffFlag = 0 |  |
| if( ( i < lastSubBlock ) && ( i > 0 ) ) { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| inferSbDcSigCoeffFlag = 1 |  |
| } |  |
| firstSigScanPosSb = numSbCoeff |  |
| lastSigScanPosSb = −1 |  |
| firstPosMode0 = ( i = = lastSubBlock ? lastScanPos : numSbCoeff − 1 ) |  |
| firstPosMode1 = −1 |  |
| for( n = firstPosMode0; n >= 0 && remBinsPass1 >= 4; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| … |  |
| AbsLevelPass1[ xC ][ yC ] = sig_coeff_flag[ xC ][ yC ] + par_level_flag[ n ] + abs_level_gtx_flag[ n ][ 0 ] + 2 * abs_level_gtx_flag[ n ][ 1 ] |  |
| if( dep_quant_enabled_flag) |  |
| QState = QStateTransTable[ QState ][ AbsLevelPass1[ xC ][ yC ] & 1 ] |  |
| if( remBinsPass1 < 4 ) |  |
| firstPosMode1 = n − 1 |  |
| } |  |
| for( n = numSbCoeff − 1; n >= firstPosMode1; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( abs_level_gtx_flag[ n ][ 1 ] ) |  |
| abs_remainder[ n ] | ae(v) |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] +2 * abs_remainder[ n ] |  |
| } |  |
| for( n = firstPosMode1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| dec_abs_level[ n ] | ae(v) |
| if(AbsLevel[ xC ][ yC ] > 0 ) |  |
| firstSigScanPosSb = n |  |
| if( dep_quant_enabled_flag) |  |
| QState = QStateTransTable[ QState ][ AbsLevel[ xC ][ yC ] & 1 ] |  |
| } |  |
| if( dep_quant_enabled_flag ¦ ¦ cu_transquant_bypass_flag[ x0 ][ y0 ] ¦ ¦ !sign_data_hiding_enabled_flag ) |  |
| signHidden = 0 |  |
| else |  |
| signHidden = ( lastSigScanPosSb − firstSigScanPosSb > 3 ? 1 : 0 ) |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( ( AbsLevel[ xC ][ yC ] > 0 ) && ( !signHidden ¦ ¦ ( n != firstSigScanPosSb ) ) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| if( dep_quant_enabled_flag && ! cu_transquant_bypass_flag[ x0 ][ y0 ] ) { |  |
| QState = startQStateSb |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( AbsLevel[ xC ][ yC ] > 0 ) |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 2 * AbsLevel[ xC ][ yC ] − ( QState > 1 ? 1 : 0 ) ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| QState = QStateTransTable[ QState ][ par_level_flag[ n ] ] |  |
| } else { |  |
| sumAbsLevel = 0 |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( AbsLevel[ xC ][ yC ] > 0 ) { |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = AbsLevel[ xC ][ yC ] * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( signHidden ) { |  |
| sumAbsLevel += AbsLevel[ xC ][ yC ] |  |
| if( ( n = = firstSigScanPosSb ) && ( sumAbsLevel % 2 ) = = 1 ) ) |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = −TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |  |

## Method 2: Disable DQ in lossless mode

Since the Q-state dependent coefficient coding is designed for coefficients quantized using DQ, it may not be beneficial in lossless coding. Therefore, based on Method 1, it is further proposed to disable Q-state transition and fix the Q-state to state 0 in blocks coded with lossless mode during the coefficient coding.Therefore, Method 2 is equivalent to disabling DQ in blocks coded with lossless mode. The specification changes are as follows.

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| … |  |
| QState = 0 |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| startQStateSb = QState |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 1 ] |  |
| inferSbDcSigCoeffFlag = 0 |  |
| if( ( i < lastSubBlock ) && ( i > 0 ) ) { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| inferSbDcSigCoeffFlag = 1 |  |
| } |  |
| firstSigScanPosSb = numSbCoeff |  |
| lastSigScanPosSb = −1 |  |
| firstPosMode0 = ( i = = lastSubBlock ? lastScanPos : numSbCoeff − 1 ) |  |
| firstPosMode1 = −1 |  |
| for( n = firstPosMode0; n >= 0 && remBinsPass1 >= 4; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| … |  |
| AbsLevelPass1[ xC ][ yC ] = sig_coeff_flag[ xC ][ yC ] + par_level_flag[ n ] + abs_level_gtx_flag[ n ][ 0 ] + 2 * abs_level_gtx_flag[ n ][ 1 ] |  |
| if( dep_quant_enabled_flag && ! cu_transquant_bypass_flag[ x0 ][ y0 ]) |  |
| QState = QStateTransTable[ QState ][ AbsLevelPass1[ xC ][ yC ] & 1 ] |  |
| if( remBinsPass1 < 4 ) |  |
| firstPosMode1 = n − 1 |  |
| } |  |
| for( n = numSbCoeff − 1; n >= firstPosMode1; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( abs_level_gtx_flag[ n ][ 1 ] ) |  |
| abs_remainder[ n ] | ae(v) |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] +2 * abs_remainder[ n ] |  |
| } |  |
| for( n = firstPosMode1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| dec_abs_level[ n ] | ae(v) |
| if(AbsLevel[ xC ][ yC ] > 0 ) |  |
| firstSigScanPosSb = n |  |
| if( dep_quant_enabled_flag && ! cu_transquant_bypass_flag[ x0 ][ y0 ]) |  |
| QState = QStateTransTable[ QState ][ AbsLevel[ xC ][ yC ] & 1 ] |  |
| } |  |
| if( dep_quant_enabled_flag ¦ ¦ cu_transquant_bypass_flag[ x0 ][ y0 ] ¦ ¦ !sign_data_hiding_enabled_flag ) |  |
| signHidden = 0 |  |
| else |  |
| signHidden = ( lastSigScanPosSb − firstSigScanPosSb > 3 ? 1 : 0 ) |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( ( AbsLevel[ xC ][ yC ] > 0 ) && ( !signHidden ¦ ¦ ( n != firstSigScanPosSb ) ) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| if( dep_quant_enabled_flag && ! cu_transquant_bypass_flag[ x0 ][ y0 ] ) { |  |
| QState = startQStateSb |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( AbsLevel[ xC ][ yC ] > 0 ) |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 2 * AbsLevel[ xC ][ yC ] − ( QState > 1 ? 1 : 0 ) ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| QState = QStateTransTable[ QState ][ par_level_flag[ n ] ] |  |
| } else { |  |
| sumAbsLevel = 0 |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( AbsLevel[ xC ][ yC ] > 0 ) { |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = AbsLevel[ xC ][ yC ] * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( signHidden ) { |  |
| sumAbsLevel += AbsLevel[ xC ][ yC ] |  |
| if( ( n = = firstSigScanPosSb ) && ( sumAbsLevel % 2 ) = = 1 ) ) |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = −TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |  |

## Method 3: Removal of the signalling of the position of the last significant coefficient in lossless blocks

Since in lossless mode, most of the coefficients are nonzero coefficients, the signalling of the last significant coefficient’s position may not be necessary and even may introduce redundancy. Therefore, based on Method 2, it is further proposed to remove the signalling of the position of the last significant coefficient. The specification changes are as follows.

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| if( ( tu_mts_idx[ x0 ][ y0 ] > 0 ¦ ¦ ( cu_sbt_flag && log2TbWidth < 6 && log2TbHeight < 6 ) ) && cIdx = = 0 && log2TbWidth > 4 ) |  |  |
| log2ZoTbWidth = 4 |  |  |
| else |  |  |
| log2ZoTbWidth = Min( log2TbWidth, 5 ) |  |  |
| MaxCcbs = 2 * ( 1 << log2TbWidth ) * ( 1<< log2TbHeight ) |  |  |
| if( tu_mts_idx[ x0 ][ y0 ] > 0 ¦ ¦ ( cu_sbt_flag && log2TbWidth < 6 && log2TbHeight < 6 ) ) && cIdx = = 0 && log2TbHeight > 4 ) |  |  |
| log2ZoTbHeight = 4 |  |  |
| else |  |  |
| log2ZoTbHeight = Min( log2TbHeight, 5 ) |  |  |
| if( log2TbWidth > 0 && !cu_transquant_bypass_flag[ x0 ][ y0 ]) |  |  |
| last_sig_coeff_x_prefix | ae(v) |
| if( log2TbHeight > 0 && !cu_transquant_bypass_flag[ x0 ][ y0 ]) |  |
| last_sig_coeff_y_prefix | ae(v) |
| if( last_sig_coeff_x_prefix > 3 && !cu_transquant_bypass_flag[ x0 ][ y0 ]) |  |
| last_sig_coeff_x_suffix | ae(v) |
| if( last_sig_coeff_y_prefix > 3 && !cu_transquant_bypass_flag[ x0 ][ y0 ]) |  |
| last_sig_coeff_y_suffix | ae(v) |
| … |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| … |  |
| if( ( ( i < lastSubBlock ) ¦¦ cu_transquant_bypass_flag[ x0 ][ y0 ]) && ( i > 0 ) ) { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| inferSbDcSigCoeffFlag = 1 |  |
| } |  |
| } |  |
| … |  |
| } |  |

last_sig_coeff_x_prefix specifies the prefix of the column position of the last significant coefficient in scanning order within a transform block. The values of last_sig_coeff_x_prefix shall be in the range of 0 to ( log2ZoTbWidth  <<  1 ) − 1, inclusive.

When last_sig_coeff_x_prefix is not present, it is inferred as follows.

- If cu_transquant_bypass_flag[ x0 ][ y0 ] is equal to 1, it is inferred to be ( log2ZoTbWidth  <<  1 ) − 1.

- Otherwise, it is inferred to be 0.

last_sig_coeff_y_prefix specifies the prefix of the row position of the last significant coefficient in scanning order within a transform block. The values of last_sig_coeff_y_prefix shall be in the range of 0 to ( log2ZoTbHeight  <<  1 ) − 1, inclusive.

When last_sig_coeff_y_prefix is not present, it is inferred as follows.

- If cu_transquant_bypass_flag[ x0 ][ y0 ] is equal to 1, it is inferred to be ( log2ZoTbHeight  <<  1 ) − 1.

- Otherwise, it is inferred to be 0.

# Experiment results

Both methods are implemented on VTM-6.0-lossless software [2]. Method 1 does not change BD-rates or runtimes. The results of Method 2 and Method 3 are summarized in Table 1 and Table 2, respectively.

Table 1.  Results of Method 2 (to be updated)

|  |  | All Intra |  |  | Random Access |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  |  | ratio |  |  |
|  | VTM6 | Test | bit-rate savings | VTM6 | Test | bit-rate savings |
| Class A1 | 2.2 | 0.0 | 0.00% | 62.2 | 0.0 | -100.00% |
| Class A2 | 1.7 | 0.0 | 0.00% | 52.3 | 0.0 | -100.00% |
| Class B | 2.1 | 0.0 | 0.00% | 2.3 | 0.0 | -500.00% |
| Class C | 2.1 | 2.0 | -0.23% | 2.6 | 4.3 | -35.36% |
| Class D | 1.9 | 2.0 | -0.26% | 2.8 | 2.8 | -0.50% |
| Class E | 2.9 | 2.8 | -0.21% |  |  |  |
| Class F | 5.5 | 6.4 | -0.40% | 39.4 | 51.6 | -46.64% |
| TGM | 12.3 | 21.8 | -0.54% | 112.6 | 154.7 | -37.76% |
| Overall | 2.2 | 0.5 | -0.05% | 9.1 | 1.1 | -56.10% |
| Enc Time[%] |  | #NUM! |  |  | #NUM! |  |
| Dec Time[%] |  | #NUM! |  |  | #NUM! |  |

Table2: Results of Method 3 (disabling DQ + remove signalling last significant coefficient position)

(to be updated)

|  |  | All Intra |  |  | Random Access |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  |  | ratio |  |  |
|  | VTM6 | Test | bit-rate savings | VTM6 | Test | bit-rate savings |
| Class A1 | 2.2 | 2.2 | -0.47% | 2.3 | 6220800.0 | -100.00% |
| Class A2 | 1.7 | 1.8 | -0.41% | 1.9 | 7050240.0 | -100.00% |
| Class B | 2.1 | 2.1 | -0.60% | 2.3 | 1542758.4 | -100.00% |
| Class C | 2.1 | 2.1 | -0.68% | 2.6 | 2.6 | -0.93% |
| Class D | 1.9 | 2.0 | -0.63% | 2.8 | 2.8 | -0.96% |
| Class E | 2.9 | 3.0 | -0.95% |  |  |  |
| Class F | 5.5 | 5.5 | -0.33% | 39.2 | 373285.7 | -25.29% |
| TGM | 12.3 | 12.2 | 0.52% | 112.4 | 373357.4 | -24.86% |
| Overall | 2.2 | 2.2 | -0.62% | 2.3 | 3168461.5 | -73.58% |
| Enc Time[%] |  | 100% |  |  | 0% |  |
| Dec Time[%] |  | #NUM! |  |  | #NUM! |  |

# Conclusions

This contribution presents two methods to achieve lossless coding with DQ enabled in slice level. Method 1 achieves lossless coding by removing the conversions from coefficient levels to quantized coefficients in blocks coded with lossless mode. Based on Method 1, Method 2 further removes the unnecessary Q-state dependent coefficient coding in blocks coded with lossless mode. Based on Method 2, Method 3 further remove the signalling of the last significant coefficient in lossless blocks. Method 1 has no impact on VTM6.0-lossless software in terms of BD-rate and runtime. Method 2 reportedly shows minor bit saving. Method 3 reportedly shows 0.62 % and X.XX% bit savings compared to the VTM6.0-lossless software under AI and RB, respectively, with negligible runtime changes.

# References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team, JVET-O2001.

- AHG18 VTM-6.0-lossless, https://vcgit.hhi.fraunhofer.de/jvet-o-ahg-18/VVCSoftware_VTM

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).