| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0150-v1 |
| --- | --- |

| Title: | CE3-related: Simplification of reference luma intra prediction mode derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {zhi-yi.lin, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VVC Draft 6, the special settings for the reference luma intra prediction modes are not unified, and many checks are required to set the reference luma intra prediction mode. In this contribution, simplified reference luma intra prediction mode derivation is proposed. In the beginning of each coding tree unit (CTU), stored luma intra prediction modes of luma blocks are initialized to DC. If a luma block is coded with combined inter and intra prediction (CIIP), intra prediction without matrix-based intra prediction (MIP), or block-wise differential pulse code modulation (BDPCM), its stored luma intra prediction mode is updated to planar, the luma intra prediction mode of the luma block, and vertical/horizontal according to the BDPCM index, respectively. In the reference luma intra prediction mode derivation, if the reference luma block is available, its stored luma intra prediction mode is used, and only block availability checks are required. It is reported that the proposed method results in 0.00%/-0.08%/-0.07%, 0.00%/-0.10%/-0.13% and -0.03%/0.04%/0.14% Y/Cb/Cr BD-rates with negligible runtime changes for VTM6.0-AI, VTM6.0-RA and VTM6.0-LB, respectively.

# Introduction

In VTM6.0 [1] and VVC Draft 6 [2], when deriving a reference intra prediction mode, in some cases, the reference luma intra prediction mode is specially set. The settings are described as below:

In the most probable mode (MPM) derivation for luma intra prediction mode signalling, two reference luma intra prediction modes are derived from two neighbouring blocks. However, if one of the following conditions is true, the reference luma intra prediction mode is set to planar.

- The neighbouring block is not available.

- The neighbouring block is not coded with intra prediction mode.

- The neighbouring block is coded with MIP mode.

- The neighbouring block and the current block are not in the same CTU row.

In the chroma intra prediction mode derivation, the reference luma intra prediction mode is set to the luma intra prediction mode of the center collocated luma block. However, if the center collocated luma block is coded with MIP mode, the reference luma intra prediction mode is set to planar. If the center collocated luma block is coded with intra block copy (IBC) or palette mode, the reference luma intra prediction mode is set to DC.

In the reference luma intra prediction mode derivation of low frequency non-separable secondary transform (LFNST) set selection for a luma block, the reference luma intra prediction mode is set to the luma intra prediction mode of the luma block. However, if the luma block is coded with MIP mode, the reference luma intra prediction mode is set to planar.

It is observed that the special settings for the reference luma intra prediction modes are not unified, and many checks are required to set the reference luma intra prediction mode. In this contribution, a simplified reference luma intra prediction mode derivation is proposed.

# Proposed method

In the proposed method, stored luma intra prediction modes of luma blocks are initialized to DC in the beginning of each CTU, and the update process is as follows. If a luma block is coded with CIIP mode, its stored luma intra prediction mode is set to planar. If a luma block is coded with non-MIP intra prediction mode, its stored luma intra prediction mode is set to the luma intra prediction mode of the luma block. If a luma block is coded with BDPCM mode, its stored luma intra prediction mode is set to vertical or horizontal depending on the BDPCM index. When referencing a luma block’s luma intra prediction mode, if the luma block is available, its stored luma intra prediction mode is used. It is noted that although the initialization is an additional process, the above update process simplifies the update process in the current VVC Draft 6, which needs additional update when the luma block is coded with MIP. In the proposed derivation, DC is used as the initialized luma intra prediction mode because its complexity is the lowest among all intra prediction modes. With the proposed method, only block availability checks are required in the reference luma intra prediction mode derivation as follows with changes compared to VVC Draft 6 highlighted.

In the most probable mode (MPM) derivation for luma intra prediction mode signalling, two reference luma intra prediction modes are derived from two neighbouring blocks. However, if one of the following conditions is true, the reference luma intra prediction mode is set to DC.

- The neighbouring block is not available.

- The neighbouring block is not coded with intra prediction mode.

- The neighbouring block is coded with MIP mode.

- The neighbouring block and the current block are not in the same CTU row.

In the chroma intra prediction mode derivation, the reference luma intra prediction mode is set to the luma intra prediction mode of the center collocated luma block. However, if the center collocated luma block is coded with MIP mode, the reference luma intra prediction mode is set to planar. If the center collocated luma block is coded with intra block copy (IBC) or palette mode, the reference luma intra prediction mode is set to DC.

In the reference luma intra prediction mode derivation of low frequency non-separable secondary transform (LFNST) set selection for a luma block, the reference luma intra prediction mode is set to the luma intra prediction mode of the luma block. However, if the luma block is coded with MIP mode, the reference luma intra prediction mode is set to planar.

# Experiment results

The proposed method has been implemented on VTM6.0. The results of the proposed method under the CTC [3] are summarized in Table 1.

Table 1.  Results of the proposed method

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.10% | -0.14% | 102% | 101% |
| Class A2 | -0.02% | -0.07% | -0.05% | 101% | 96% |
| Class B | 0.00% | -0.01% | -0.09% | 100% | 99% |
| Class C | 0.00% | -0.14% | 0.02% | 99% | 98% |
| Class E | -0.02% | -0.10% | -0.13% | 103% | 104% |
| Overall | 0.00% | -0.08% | -0.07% | 101% | 99% |
| Class D | -0.01% | -0.03% | -0.04% | 98% | 100% |
| Class F | 0.01% | -0.06% | -0.08% | 100% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.24% | -0.21% | 101% | 103% |
| Class A2 | 0.00% | -0.09% | -0.08% | 101% | 103% |
| Class B | 0.00% | -0.11% | -0.13% | 100% | 100% |
| Class C | -0.01% | 0.00% | -0.09% | 100% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.10% | -0.13% | 100% | 102% |
| Class D | -0.08% | -0.14% | -0.01% | 100% | 99% |
| Class F | 0.02% | -0.08% | -0.03% | 100% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.04% | 0.17% | 100% | 102% |
| Class C | -0.04% | -0.34% | -0.03% | 101% | 103% |
| Class E | -0.10% | 0.54% | 0.33% | 100% | 102% |
| Overall | -0.03% | 0.04% | 0.14% | 100% | 102% |
| Class D | 0.02% | 0.05% | 0.26% | 101% | 101% |
| Class F | 0.01% | -0.34% | 0.31% | 100% | 101% |

# Conclusions

This contribution presents a simplified reference luma intra prediction mode derivation. The proposed method does not cause noticeable impact on coding efficiency.

# References

- VTM6.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team, JVET-O2001.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).