| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0152-v1 |
| --- | --- |

| Title: | CE4-related: On maximum number of subblock-based merging candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Olena Chubach, Chun-Chia Chen, Chih-Wei Hsu, Ching-Yeh Chen, Tzu-Der Chuang, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {olena.chubach, chunchia.chen, cw.hsu, chingyeh.chen, peter.chuang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

JVET-O0238 introduced a mechanism to enable signalling in picture parameter set (PPS) for a portion of slice parameters remaining constant in all pictures. This mechanism was adopted into VVC Draft 6. However, this mechanism is not complete and introduced a certain issue: indicating the maximum number of subblock-based merging candidates in PPS, pps_five_minus_max_num_subblock_merge_cand_plus1 only works properly when affine mode is enabled in sequence parameter set (SPS), i.e., sps_affine_enabled_flag=1. In this contribution, it is proposed to support the mechanism of JVET-O0238 for pps_five_minus_max_num_subblock_merge_cand_plus1 when affine mode is disabled in SPS. Moreover, in order to support the mechanism of JVET-O0238, the issue of subblock-based merge candidate list containing only one zero motion vector (MV) candidate should be avoided, as proposed in the adopted JVET-O0220. To fix the above two issues, this contribution proposes to update legal value ranges of pps_five_minus_max_num_subblock_merge_cand_plus1 under different conditions and update an inference method of five_minus_max_num_subblock_merge_cand when it is not present.

# Introduction

According to the adopted JVET-O0238 [1], constant_slice_header_params_enabled_flag has to be decoded in the PPS for a list of slice parameter syntax elements. This flag specifies if slice parameter syntax elements may be signalled in the PPS and slice headers or are always signalled in slice headers. The list of slice parameter syntax elements contains, among others, those to specify maximum number of subblock-based merging candidates in the slice, i.e., pps_five_minus_max_num_subblock_merge_cand_plus1 (the syntax element signalled in PPS) and five_minus_max_num_subblock_merge_cand (the syntax element signalled in slice header).

When five_minus_max_num_subblock_merge_cand is not present, following applies:

- If sps_affine_enabled_flag is equal to 0, the value of five_minus_max_num_subblock_merge_cand is inferred to be equal to 5 − ( sps_sbtmvp_enabled_flag  &&  slice_temporal_mvp_enabled_flag )

- Otherwise (sps_affine_enabled_flag is equal to 1), the value of five_minus_max_num_subblock_merge_cand is inferred to be equal to pps_five_minus_max_num_subblock_merge_cand_plus1 − 1.

In this contribution, two issues related to the inference of five_minus_max_num_subblock_merge_cand when sps_affine_enabled_flag is equal to 0 are asserted.

- When sps_affine_enabled_flag = 0, the value of five_minus_max_num_subblock_merge_cand is inferred by considering sps_sbtmvp_enabled_flag  and slice_temporal_mvp_enabled_flag. However, this rule is overlooked for pps_five_minus_max_num_subblock_merge_cand_plus1, which results in inconsistent infered values between slice header and PPS.

- When pps_five_minus_max_num_subblock_merge_cand_plus1 = 5 (meaning the number of subblock-based merging candidates is 1), slice_temporal_mvp_enabled_flag = 0, and sps_sbtmvp_enabled_flag =1, the slice disallows any subblock-based temporal motion vector prediction (SbTMVP) because the slice-level temporal motion vector prediction is disabled. In this case, the subblock-based merging candidate list only contains one zero MV candidate. This case should be avoided, according to the adopted JVET-O0220 [2].

# Proposed method to infer pps_five_minus_max_num_subblock_merge_cand_plus1

It is proposed to add following changes highlighted in yellow.

pps_five_minus_max_num_subblock_merge_cand_plus1 equal to 0 specifies that five_minus_max_num_subblock_merge_cand is present in slice header of slices referring to the PPS if sps_affine_enabled_flag is equal to 1. pps_five_minus_max_num_subblock_merge_cand_plus1 larger than 0 specifies that five_minus_max_num_subblock_merge_cand is not present in slice header of slices referring to the PPS.

If sps_affine_enabled_flag is equal to 1, the value of pps_five_minus_max_num_subblock_merge_cand_plus1 shall be in the range of 0 to 6, inclusive.

Otherwise, if all of the following conditions are true, the value of pps_five_minus_max_num_subblock_merge_cand_plus1 shall be 0.

- sps_temporal_mvp_enabled_flag is equal to 1

- sps_sbtmvp_enabled_flag is equal to 1

- pps_temporal_mvp_enabled_idc is equal to 0

Otherwise, if all of the following conditions are true, the value of pps_five_minus_max_num_subblock_merge_cand_plus1 shall be in the range of 5 to 6, inclusive

- sps_temporal_mvp_enabled_flag is equal to 1

- sps_sbtmvp_enabled_flag is equal to 1

- pps_temporal_mvp_enabled_idc is equal to 2

Otherwise, the value of pps_five_minus_max_num_subblock_merge_cand_plus1 shall be 6.

five_minus_max_num_subblock_merge_cand specifies the maximum number of subblock-based merging motion vector prediction (MVP) candidates supported in the slice subtracted from 5.

When five_minus_max_num_subblock_merge_cand is not present, the following applies:

- If sps_affine_enabled_flag is equal to 0 and pps_five_minus_max_num_subblock_merge_cand_plus1 is equal to 0, the value of five_minus_max_num_subblock_merge_cand is inferred to be equal to 5 − ( sps_sbtmvp_enabled_flag  &&  slice_temporal_mvp_enabled_flag )

- Otherwise (sps_affine_enabled_flag is equal to 1), the value of five_minus_max_num_subblock_merge_cand is inferred to be equal to pps_five_minus_max_num_subblock_merge_cand_plus1 − 1.

# References

[1] M. Pettersson, P. Wennersten, R. Sjöberg, and M. Damghanian, “AHG17: Parameters in SPS or slice headers,” Document of Joint Video Experts Team, JVET-O0238.

[2] R. Yu and D. Liu, “Non-CE4: On maximum number of subblock merge candidates,” Document of Joint Video Experts Team, JVET-O0220.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).