| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0155-v1 |
| --- | --- |

| Title: | CE4-related: An encoder switch for forcing temporal MV to zero in SbTMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Ling Hsiao, Chun-Chia Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {yuling.hsiao, chunchia.chen, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to add a high level switch for forcing the temporal motion vector (MV) to zero in the subblock-based temporal motion vector prediction (SbTMVP) to remove the clipping operations at encoder side. With the proposed method, most coding gain of SbTMVP can be preserved with lower complexity at encoder side. Results are summarized as follows.

Proposed method with setting the proposed switch to 1
VTM6.0-RA: {Y-BD-rate=0.06%, EncT=101%, DecT=100%}
VTM6.0-LB: {Y-BD-rate=0.05%, EncT=100%, DecT=99%}

# Introduction

In SbTMVP of Versatile Video Coding (VVC) Draft 6 [1], a coding unit (CU) is divided into 8x8 luma subblocks and corresponding 4x4 chroma subblocks, and MV data are derived for each 8x8 luma subblock separately. A temporal MV pointing from the current picture to the collocated picture could be zero or derived from a left neighbour of the current CU. A position clipping area is defined as “corresponding luma samples of the current luma coding tree block (CTB) and four luma sample columns of luma CTB height to the right of the current luma CTB in the collocated picture, but excluding samples outside the collocated picture.” The center position of the current luma coding block (CB) is shifted by the temporal MV pointing to the collocated picture, and then the shifted position is clipped within the position clipping area. If the shifted and clipped position from the center of the current luma CB is coded with intra or intra block copy (IBC), SbTMVP is unavailable; otherwise, the MV data of the shifted and clipped position from the center of the current luma CB are used to derive luma CB MV data. For each luma subblock, its center position is also shifted by the temporal MV pointing to the collocated picture, and then the shifted position is clipped within the position clipping area. If the shifted and clipped position from the center of the luma subblock is coded with intra or IBC, luma subblock MV data are set equal to the luma CB MV data; otherwise, the MV data of the shifted and clipped position from the center of the luma subblock are used to derive luma subblock MV data.

One position clipping operation needs four comparisons. For one 16x16 luma CB, there are five position clipping operations (one for luma CB and four for luma subblocks). For one 128x128 luma CB, there are 257 position clipping operations (one for luma CB and 256 for luma subblocks). This contribution aims at removing these operations at encoder side.

# Proposed method

It is proposed to add one high level switch for the encoder to allow forcing temporal MV to zero in SbTMVP. The new syntax design follows the design of constant_slice_header_params_enabled_flag in VVC Draft 6. In the proposed syntax, there is one picture parameter set (PPS) level indicator to specify whether the switch is determined from PPS or slice header. If the PPS level indicator is equal to two, that means the switch (for forcing temporal MV to zero) is turned on in the PPS; otherwise, if the PPS level indicator is equal to one, that means the switch is turned off in the PPS; otherwise, if the PPS level indicator is equal to zero, one slice level flag will be signalled to indicate the switch in the slice. In the SbTMVP process, if the switch is turned on, the temporal MV will be forced to zero; otherwise, the process will be the same as the original SbTMVP process. By the proposed switch, hardware encoder designers can simplify SbTMVP implementations by turning on the switch, so as to remove the position clipping operations.

# Experiment results

The proposed method has been implemented on top of VTM6.0. The common test conditions [2] are used for evaluation. Table 1 summarizes the results of the proposed method with setting the switch to 1.

Table 1. Results of the proposed method

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.01% | 0.04% | 101% | 100% |
| Class A2 | 0.06% | 0.13% | 0.03% | 101% | 100% |
| Class B | 0.01% | 0.07% | 0.04% | 101% | 100% |
| Class C | 0.10% | 0.14% | 0.07% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.08% | 0.04% | 101% | 100% |
| Class D | -0.01% | 0.03% | 0.07% | 100% | 100% |
| Class F | 0.03% | 0.01% | 0.02% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | 0.00% | -0.21% | 100% | 100% |
| Class C | 0.03% | 0.02% | -0.08% | 100% | 99% |
| Class E | 0.08% | 0.44% | 0.20% | 100% | 98% |
| Overall | 0.05% | 0.12% | -0.07% | 100% | 99% |
| Class D | 0.04% | 0.70% | 0.56% | 100% | 99% |
| Class F | -0.01% | -0.02% | 1.23% | 100% | 99% |

# Conclusion

In this contribution, a high level switch is proposed for forcing temporal MV to zero in SbTMVP. With the switch, SbTMVP with lower complexity at encoder side can be achieved with most coding gain of SbTMVP preserved.

# References

[1] B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team, JVET-O2001.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).