| `Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0157-v1 |
| --- | --- |

| Title: | CE5-related: Align ALF padding processes at picture and subpicture boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai, Olena Chubach, Lulin Chen, Ching-Yeh Chen, Tzu-Der Chuang, Chih-Wei Hsu Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {jenny.lai, olena.chubach, lulin.chen, chingyeh.chen, peter.chuang, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VVC Draft 6, two kinds of padding methods are used in adaptive loop filter (ALF) to generate unavailable neighbouring samples. One is the repetitive padding for picture boundaries, and the other is the ALF virtual boundary (VB) padding for slice/brick/subpicture boundaries. When a picture consists of independently decodable subpictures and the subpictures may be extracted as a picture, a mismatch may occur for the boundary padding if the subpicture boundaries are not the picture boundaries. Four methods are proposed in this contribution.

1. If picture boundaries are aligned with coding tree unit (CTU) boundaries, applying the ALF-VB padding to the picture boundaries; otherwise applying the repetitive padding to the picture boundaries.

2. If subpic_treated_as_pic_flag[ i ] is equal to 1, applying the repetitive padding to the subpicture boundary; otherwise, if loop_filter_across_subpic_enabled_flag[ i ] is equal to 0, applying the ALF-VB padding to the subpicture boundary.

3. Applying the repetitive padding to all subpicture boundaries.

4. Applying the repetitive padding to all slice/brick/360/subpicture/picture boundaries.

# Introduction

In VVC Draft 6 [1], there are two padding methods to handle the unavailable neighbouring samples in ALF. The unavailable neighbouring samples could be the samples outside the picture or the samples located at other segments, e.g., different slice/brick/subpicture segments, and the loop filter cannot reference the samples located at other segments, for example, loop_filter_across_bricks_enabled_flag is equal to 0, loop_filter_across_slices_enabled_flag is equal to 0, or loop_filter_across_subpic_enabled_flag[i] is equal to 0. If the current boundary is a picture boundary, the repetitive padding is applied to generate the unavailable neighbouring samples. If the current boundary is a slice/brick/subpicture boundary, the ALF-VB padding is used to avoid accessing the unavailable neighbouring samples. Figure 1 is an example. One picture is composed of four subpictures, and the loop filter cannot reference the samples located at different subpictures (loop_filter_across_subpic_enabled_flag[i]=0). Therefore, for samples near picture boundaries, unavailable samples are generated by the repetitive padding, but for samples near subpicture boundaries, the ALF-VB padding is used to avoid accessing samples from different subpictures.

Figure 1. ALF padding methods at picture and subpicture boundaries.

When independent subpicture decoding is enabled (subpic_treated_as_pic_flag[i]=1), one subpicure should be decoded without information from other subpictures. For example, subpicture2 extracted from the entire picture should be decoded independently. In this case, the left and bottom subpicture boundaries become picture boundaries, so the repetitive padding should be used at these boundaries. However, in the original bitstream, the ALF-VB padding is used instead of the repetitive padding, when the loop filter cannot reference samples located at other subpictures (loop_filter_across_subpic_enabled_flag[ i ]=0). One mismatch occurs. In order to solve this issue, four methods are proposed.

# Proposed methods

Method1 is applying the ALF-VB padding at picture boundaries, if the picture boundaries are also aligned with CTU boundaries; otherwise, the repetitive padding is applied. By applying this method, the selection of the ALF-VB padding and the repetitive padding at boundaries is unified. If a boundary, which can be a slice/brick/subpicture/360/picture boundary, is aligned with CTU boundaries, the ALF-VB padding is utilized; otherwise, the repetitive padding is used.

Method2 is conditionally selecting an ALF padding method at subpicture boundaries. For a subpicture boundary, if independent subpicture decoding is enabled (subpic_treated_as_pic_flag[i]=1), the repetitive padding is used; otherwise, if the loop filter cannot reference samples located at other subpictures (loop_filter_across_subpic_enabled_flag[ i ]=0), the ALF-VB padding is used.

Method3 is always applying the repetitive padding at subpicture boundaries. That is, if the current boundary is a picture boundary or a subpicture boundary, the repetitive padding is used.

Method4 is an extension of Method3. Method3 can be further extended to other boundaries, e.g., slice/brick/360 boundaries. That is, the repetitive padding is the only one padding method to generate unavailable neighbouring samples which are outside the picture or located at other segments (e.g., slice/brick/360/subpicture) with loop filter not allowed to reference samples from other segments.

Table 1 summarizes the proposed methods and current design in VVC Draft 6, when the neighbouring samples are unavailable. Compared to the current VVC, the change is highlighted in red color.

Table 1. Summary of different methods

| Method | Picture Boundary | Subpicture Boundaries | 360 Virtual Boundaries | Slice/Brick Boundaries |
| --- | --- | --- | --- | --- |
| VVC Draft6 | repetitive padding | CTU aligned: ALF-VB padding | CTU aligned: ALF-VB padding Not CTU-aligned: repetitive padding | CTU aligned: ALF-VB padding |
| Method1 | CTU aligned: ALF-VB padding Not CTU-aligned: repetitive padding | CTU aligned: ALF-VB padding | CTU aligned: ALF-VB padding Not CTU-aligned: repetitive padding | CTU aligned: ALF-VB padding |
|  | Rules can be described in the following simple way: If the boundary is aligned with CTU boundaries, use ALF-VB padding Otherwise, (the boundary is not aligned with CTU boundaries) repetitive padding |  |  |  |
| Method2 | repetitive padding | Independently decoding: repetitive padding Not independently decoding: ALF-VB padding | CTU aligned: ALF-VB padding Not CTU-aligned: repetitive padding | CTU aligned: ALF-VB padding |
| Method3 | repetitive padding | repetitive padding | CTU aligned: ALF-VB padding Not CTU-aligned: repetitive padding | CTU aligned: ALF-VB padding |
| Method4 | repetitive padding | repetitive padding | repetitive padding | repetitive padding |

# Corresponding spec changes

The corresponding text change of Method1 is provided and highlighted in yellow color in the following.

8.8.5.5	ALF boundary position derivation process

Inputs of this process are:

  - a luma location ( xCtb, yCtb ) specifying the top-left sample of the current luma coding tree block relative to the top left sample of the current picture,

  - a luma location ( x, y ) specifying the current sample relative to the top-left sample of the current luma coding tree block.

Output of this process are:

  - the left vertical boundary position clipLeftPos,

  - the right vertical boundary position clipRightPos,

  - the above horizontal boundary position clipTopPos,

  - the below horizontal boundary position clipBottomPos.

The variables clipLeftPos, clipRightPos, clipTopPos and clipBottomPos are set equal to −128.

The variable clipTopPos is modified as follows:

  - If the bottom boundary of the current coding tree block is not the bottom boundary of the picture and y − ( CtbSizeY − 4 ) is greater than or equal to 0, the variable clipTopPos is set equal to yCtb + CtbSizeY − 4.

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is equal to 0, and yCtb + y − PpsVirtualBoundariesPosY[ n ] is greater than or equal to 0 and less than 3 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

clipTopPos =  PpsVirtualBoundariesPosY[ n ]		(8-1292)

  - Otherwise, if y is less than 3, and the top boundary of the current coding tree block is not the top boundary of the picture, and one or more of the following conditions are true, the variable clipTopPos is set equal to yCtb:

    - If the top boundary of the current coding tree block is the top boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the top boundary of the current coding tree block is the top boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If the top boundary of the current coding tree block is the top boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

    - If the top boundary of the current coding tree block is the top boundary of the picture.

The variable clipBottomPos is modified as follows:

  - If the bottom boundary of the current coding tree block is not the bottom boundary of the picture and CtbSizeY − 4 − y is greater than 0 and is less than 4, the variable clipBottomPos is set equal to yCtb + CtbSizeY − 4.

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, PpsVirtualBoundariesPosY[ n ] % CtbSizeY is equal to 0, PpsVirtualBoundariesPosY[ n ] is not equal to pic_height_in_luma_samples − 1 or 0, and PpsVirtualBoundariesPosY[ n ] − yCtb − y is greater than 0 and less than 4 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

clipBottomPos =  PpsVirtualBoundariesPosY[ n ]		(8-1293)

  - Otherwise, if CtbSizeY − y is less than 4, and the bottom boundary of the current coding tree block is not the bottom boundary of the picture, and one or more of the following conditions are true, the variable clipBottomPos is set equal to yCtb + CtbSizeY:

    - If the bottom boundary of the current coding tree block is the bottom boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the bottom boundary of the current coding tree block is the bottom boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If the bottom boundary of the current coding tree block is the bottom boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

    - If the bottom boundary of the current coding tree block is the bottom boundary of the picture.

The variable clipLeftPos is modified as follows:

  - If pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is equal to 0, and xCtb + x − PpsVirtualBoundariesPosX[ n ] is greater than or equal to 0 and less than 3 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

clipLeftPos = PpsVirtualBoundariesPosX[ n ]		(8-1294)

  - Otherwise, if x is less than 3, the left boundary of the current coding tree block is not the left boundary of the picture and one or more of the following conditions are true, the variable clipLeftPos is set equal to xCtb:

    - If the left boundary of the current coding tree block is the left boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the left boundary of the current coding tree block is the left boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If the left boundary of the current coding tree block is the left boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

    - If the left boundary of the current coding tree block is the left boundary of the picture.

The variable clipRightPos is modified as follows:

  - If pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is equal to 0, and PpsVirtualBoundariesPosX[ n ]− xCtb − x is greater than 0 and less than 4 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

clipRightPos = PpsVirtualBoundariesPosX[ n ]		(8-1295)

  - Otherwise, if CtbSizeY − x is less than 4, and the right boundary of the current coding tree block is not the right boundary of the picture, and one or more of the following conditions are true, the variable clipRightPos is set equal to xCtb + CtbSizeY:

    - If the right boundary of the current coding tree block is the right boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the right boundary of the current coding tree block is the right boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - if the right boundary of the current coding tree block is the right boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

    - If the right boundary of the current coding tree block is the right boundary of the picture.

The corresponding text change of Method2 is provided and highlighted in green color in the following.

8.8.5.2	Coding tree block filtering process for luma samples

  - The locations ( hx + i, vy + j ) for each of the corresponding luma samples ( x, y ) inside the given array recPicture of luma samples with i, j = −3..3 are derived as follows:

  - If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the left boundary of the current coding tree block is the left boundary of the subpicture, and x is greater than or equal to 0 and less than 3, the following applies:

hx + i = Clip3( xCtb, pic_width_in_luma_samples − 1, xCtb + x + i )

  - Otherwise, if subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the right boundary of the current coding tree block is the right boundary of the subpicture, and CtbSizeY−x is greater than 0 and less than 4, the following applies:

hx + i = Clip3( 0, xCtb + CtbSizeY − 1, xCtb + x + i )

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is not equal to 0, and xCtb + x − PpsVirtualBoundariesPosX[ n ] is greater than or equal to 0 and less than 3 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

hx + i = Clip3( PpsVirtualBoundariesPosX[ n ], pic_width_in_luma_samples − 1, xCtb + x + i )	(8-1229)

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is not equal to 0, and PpsVirtualBoundariesPosX[ n ]− xCtb − x is greater than 0 and less than 4 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

hx + i = Clip3( 0, PpsVirtualBoundariesPosX[ n ] − 1, xCtb + x + i )	(8-1230)

  - Otherwise, the following applies:

hx + i = Clip3( 0, pic_width_in_luma_samples − 1, xCtb + x + i )	(8-1231)

  - If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the top boundary of the current coding tree block is the top boundary of the subpicture, and y is greater than or equal to 0 and less than 3, the following applies:

vy + j  = Clip3( yCtb, pic_height_in_luma_samples  − 1, yCtb + y + j )

  - Otherwise, if subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the bottom boundary of the current coding tree block is the bottom boundary of the subpicture, and yCtb − y is greater than 0 and less than 4, the following applies:

vy + j  = Clip3( 0, yCtb + CtbSizeY − 1, yCtb + y + j )

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is not equal to 0, and yCtb + y − PpsVirtualBoundariesPosY[ n ] is greater than or equal to 0 and less than 3 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

vy + j = Clip3( PpsVirtualBoundariesPosY[ n ], pic_height_in_luma_samples − 1, yCtb + y + j )	(8-1232)

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is not equal to 0, and PpsVirtualBoundariesPosY[ n ] − yCtb − y is greater than 0 and less than 4 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

vy + j = Clip3( 0, PpsVirtualBoundariesPosY[ n ] − 1, yCtb + y + j )	(8-1233)

  - Otherwise, the following applies:

vy + j = Clip3( 0, pic_height_in_luma_samples − 1, yCtb + y + j )	(8-1234)

8.8.5.3	Derivation process for ALF transpose and filter index for luma samples

The locations ( hx + i, vy + j ) for each of the corresponding luma samples ( x, y ) inside the given array recPicture of luma samples with i, j = −2..5 are derived as follows:

  - If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the left boundary of the current coding tree block is the left boundary of the subpicture, and x is greater than or equal to 0 and less than 2, the following applies:

hx + i = Clip3( xCtb, pic_width_in_luma_samples − 1, xCtb + x + i )

  - Otherwise, if subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the right boundary of the current coding tree block is the right boundary of the subpicture, and CtbSizeY−x is greater than 0 and less than 6, the following applies:

hx + i = Clip3( 0, xCtb + CtbSizeY − 1, xCtb + x + i )

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is not equal to 0, and xCtb + x − PpsVirtualBoundariesPosX[ n ] is greater than or equal to 0 and less than 2 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

hx + i = Clip3( PpsVirtualBoundariesPosX[ n ], pic_width_in_luma_samples − 1, xCtb + x + i )	(8-1239)

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is not equal to 0, and PpsVirtualBoundariesPosX[ n ] − xCtb − x is greater than 0 and less than 6 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

hx + i = Clip3( 0, PpsVirtualBoundariesPosX[ n ] − 1, xCtb + x + i )	(8-1240)

  - Otherwise, the following applies:

hx + i = Clip3( 0, pic_width_in_luma_samples − 1, xCtb + x + i )	(8-1241)

  - If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the top boundary of the current coding tree block is the top boundary of the subpicture, and y is greater than or equal to 0 and less than 2, the following applies:

vy + j  = Clip3( yCtb, pic_height_in_luma_samples  − 1, yCtb + y + j )

  - Otherwise, if subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the bottom boundary of the current coding tree block is the bottom boundary of the subpicture, and CtbSizeY−y is greater than 0 and less than 6, the following applies:

vy + j  = Clip3( 0, yCtb + CtbSizeY − 1, yCtb + y + j )

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is not equal to 0, and yCtb + y − PpsVirtualBoundariesPosY[ n ] is greater than or equal to 0 and less than 2 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

vy + j = Clip3( PpsVirtualBoundariesPosY[ n ], pic_height_in_luma_samples − 1, yCtb + y + j )	(8-1242)

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is not equal to 0, and PpsVirtualBoundariesPosY[ n ] − yCtb − y is greater than 0 and less than 6 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

vy + j = Clip3( 0, PpsVirtualBoundariesPosY[ n ] − 1, yCtb + y + j ) 	(8-1243)

  - Otherwise, the following applies:

vy + j = Clip3( 0, pic_height_in_luma_samples − 1, yCtb + y + j )	(8-1244)

8.8.5.4	Coding tree block filtering process for chroma samples

  - The locations ( hx + i, vy + j ) for each of the corresponding chroma samples ( x, y ) inside the given array recPicture of chroma samples with i, j = −2..2 are derived as follows:

  - If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the left boundary of the current coding tree block is the left boundary of the subpicture, and x is greater than or equal to 0 and less than 2, the following applies:

hx + i = Clip3( xCtbC, pic_width_in_luma_samples / SubWidthC  − 1, xCtbC + x + i )

  - Otherwise, if subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the right boundary of the current coding tree block is the right boundary of the subpicture, and CtbSizeY/ SubWidthC −x is greater than 0 and less than 3, the following applies:

hx + i = Clip3( 0, xCtbC + CtbSizeY/ SubWidthC  − 1, xCtbC + x + i )

  - Otherwise, if If pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is not equal to 0 and xCtbC + x − PpsVirtualBoundariesPosX[ n ] / SubWidthC is greater than or equal to 0 and less than 2 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

hx + i = Clip3( PpsVirtualBoundariesPosX[ n ] / SubWidthC,	(8-1280)
			pic_width_in_luma_samples / SubWidthC − 1, xCtbC + x + i )

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is not equal to 0 and PpsVirtualBoundariesPosX[ n ] / SubWidthC − xCtbC − x is greater than 0 and less than 3 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

hx + i = Clip3( 0, PpsVirtualBoundariesPosX[ n ] / SubWidthC − 1, xCtbC + x + i )	(8-1281)

  - Otherwise, the following applies:

hx + i = Clip3( 0, pic_width_in_luma_samples / SubWidthC − 1, xCtbC + x + i )	(8-1282)

  - If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the top boundary of the current coding tree block is the top boundary of the subpicture, and y is greater than or equal to 0 and less than 2, the following applies:

vy + j  = Clip3( yCtbC, pic_height_in_luma_samples / SubHeightC − 1, yCtbC/ SubHeightC + y + j )

  - Otherwise, if subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the bottom boundary of the current coding tree block is the bottom boundary of the subpicture, and CtbSizeY/ SubHeightC −y is greater than 0 and less than 3, the following applies:

vy + j  = Clip3( 0, yCtbC + CtbSizeY/ SubHeightC − 1, yCtbC + y + j )

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is not equal to 0 and yCtbC + y − PpsVirtualBoundariesPosY[ n ] / SubHeightC is greater than or equal to 0 and less than 2 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

vy + j = Clip3( PpsVirtualBoundariesPosY[ n ] / SubHeightC ,	(8-1283)
			pic_height_in_luma_samples / SubHeightC − 1, yCtbC + y + j )

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is not equal to 0 and PpsVirtualBoundariesPosY[ n ] / SubHeightC − yCtbC − y is greater than 0 and less than 3 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

vy + j = Clip3( 0, PpsVirtualBoundariesPosY[ n ] / SubHeightC − 1, yCtbC + y + j )	(8-1284)

  - Otherwise, the following applies:

vy + j = Clip3( 0, pic_height_in_luma_samples / SubHeightC − 1, yCtbC + y + j )	(8-1285)

8.8.5.5	ALF boundary position derivation process

Inputs of this process are:

  - a luma location ( xCtb, yCtb ) specifying the top-left sample of the current luma coding tree block relative to the top left sample of the current picture,

  - a luma location ( x, y ) specifying the current sample relative to the top-left sample of the current luma coding tree block.

Output of this process are:

  - the left vertical boundary position clipLeftPos,

  - the right vertical boundary position clipRightPos,

  - the above horizontal boundary position clipTopPos,

  - the below horizontal boundary position clipBottomPos.

The variables clipLeftPos, clipRightPos, clipTopPos and clipBottomPos are set equal to −128.

The variable clipTopPos is modified as follows:

  - If the bottom boundary of the current coding tree block is not the bottom boundary of the picture and y − ( CtbSizeY − 4 ) is greater than or equal to 0, the variable clipTopPos is set equal to yCtb + CtbSizeY − 4.

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is equal to 0, and yCtb + y − PpsVirtualBoundariesPosY[ n ] is greater than or equal to 0 and less than 3 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

clipTopPos =  PpsVirtualBoundariesPosY[ n ]		(8-1292)

  - Otherwise, if y is less than 3, and the top boundary of the current coding tree block is not the top boundary of the picture, and one or more of the following conditions are true, the variable clipTopPos is set equal to yCtb:

    - If the top boundary of the current coding tree block is the top boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the top boundary of the current coding tree block is the top boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If the top boundary of the current coding tree block is the top boundary of the subpicture, subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

The variable clipBottomPos is modified as follows:

  - If the bottom boundary of the current coding tree block is not the bottom boundary of the picture and CtbSizeY − 4 − y is greater than 0 and is less than 4, the variable clipBottomPos is set equal to yCtb + CtbSizeY − 4.

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, PpsVirtualBoundariesPosY[ n ] % CtbSizeY is equal to 0, PpsVirtualBoundariesPosY[ n ] is not equal to pic_height_in_luma_samples − 1 or 0, and PpsVirtualBoundariesPosY[ n ] − yCtb − y is greater than 0 and less than 4 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

clipBottomPos =  PpsVirtualBoundariesPosY[ n ]		(8-1293)

  - Otherwise, if CtbSizeY − y is less than 4, and the bottom boundary of the current coding tree block is not the bottom boundary of the picture, and one or more of the following conditions are true, the variable clipBottomPos is set equal to yCtb + CtbSizeY:

    - If the bottom boundary of the current coding tree block is the bottom boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the bottom boundary of the current coding tree block is the bottom boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If the bottom boundary of the current coding tree block is the bottom boundary of the subpicture, subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

The variable clipLeftPos is modified as follows:

  - If pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is equal to 0, and xCtb + x − PpsVirtualBoundariesPosX[ n ] is greater than or equal to 0 and less than 3 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

clipLeftPos = PpsVirtualBoundariesPosX[ n ]		(8-1294)

  - Otherwise, if x is less than 3, the left boundary of the current coding tree block is not the left boundary of the picture and one or more of the following conditions are true, the variable clipLeftPos is set equal to xCtb:

    - If the left boundary of the current coding tree block is the left boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the left boundary of the current coding tree block is the left boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If the left boundary of the current coding tree block is the left boundary of the subpicture, subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

The variable clipRightPos is modified as follows:

  - If pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is equal to 0, and PpsVirtualBoundariesPosX[ n ]− xCtb − x is greater than 0 and less than 4 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

clipRightPos = PpsVirtualBoundariesPosX[ n ]		(8-1295)

  - Otherwise, if CtbSizeY − x is less than 4, and the right boundary of the current coding tree block is not the right boundary of the picture, and one or more of the following conditions are true, the variable clipRightPos is set equal to xCtb + CtbSizeY:

    - If the right boundary of the current coding tree block is the right boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the right boundary of the current coding tree block is the right boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - if the right boundary of the current coding tree block is the right boundary of the subpicture, subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

The corresponding text change of Method3 is provided and highlighted in blue color in the following.

8.8.5.2	Coding tree block filtering process for luma samples

  - The locations ( hx + i, vy + j ) for each of the corresponding luma samples ( x, y ) inside the given array recPicture of luma samples with i, j = −3..3 are derived as follows:

  - If loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0, the left boundary of the current coding tree block is the left boundary of the subpicture, and x is greater than or equal to 0 and less than 3, the following applies:

hx + i = Clip3( xCtb, pic_width_in_luma_samples − 1, xCtb + x + i )

  - Otherwise, if loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0, the right boundary of the current coding tree block is the right boundary of the subpicture, and CtbSizeY−x is greater than 0 and less than 4, the following applies:

hx + i = Clip3( 0, xCtb + CtbSizeY − 1, xCtb + x + i )

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is not equal to 0, and xCtb + x − PpsVirtualBoundariesPosX[ n ] is greater than or equal to 0 and less than 3 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

hx + i = Clip3( PpsVirtualBoundariesPosX[ n ], pic_width_in_luma_samples − 1, xCtb + x + i )	(8-1229)

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is not equal to 0, and PpsVirtualBoundariesPosX[ n ]− xCtb − x is greater than 0 and less than 4 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

hx + i = Clip3( 0, PpsVirtualBoundariesPosX[ n ] − 1, xCtb + x + i )	(8-1230)

  - Otherwise, the following applies:

hx + i = Clip3( 0, pic_width_in_luma_samples − 1, xCtb + x + i )	(8-1231)

  - If loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0, the top boundary of the current coding tree block is the top boundary of the subpicture, and y is greater than or equal to 0 and less than 3, the following applies:

vy + j  = Clip3( yCtb, pic_height_in_luma_samples  − 1, yCtb + y + j )

  - Otherwise, if loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0, the bottom boundary of the current coding tree block is the bottom boundary of the subpicture, and yCtb − y is greater than 0 and less than 4, the following applies:

vy + j  = Clip3( 0, yCtb + CtbSizeY − 1, yCtb + y + j )

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is not equal to 0, and yCtb + y − PpsVirtualBoundariesPosY[ n ] is greater than or equal to 0 and less than 3 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

vy + j = Clip3( PpsVirtualBoundariesPosY[ n ], pic_height_in_luma_samples − 1, yCtb + y + j )	(8-1232)

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is not equal to 0, and PpsVirtualBoundariesPosY[ n ] − yCtb − y is greater than 0 and less than 4 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

vy + j = Clip3( 0, PpsVirtualBoundariesPosY[ n ] − 1, yCtb + y + j )	(8-1233)

  - Otherwise, the following applies:

vy + j = Clip3( 0, pic_height_in_luma_samples − 1, yCtb + y + j )	(8-1234)

8.8.5.3	Derivation process for ALF transpose and filter index for luma samples

The locations ( hx + i, vy + j ) for each of the corresponding luma samples ( x, y ) inside the given array recPicture of luma samples with i, j = −2..5 are derived as follows:

  - If loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0, the left boundary of the current coding tree block is the left boundary of the subpicture, and x is greater than or equal to 0 and less than 2, the following applies:

hx + i = Clip3( xCtb, pic_width_in_luma_samples − 1, xCtb + x + i )

  - Otherwise, if loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0, the right boundary of the current coding tree block is the right boundary of the subpicture, and CtbSizeY−x is greater than 0 and less than 6, the following applies:

hx + i = Clip3( 0, xCtb + CtbSizeY − 1, xCtb + x + i )

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is not equal to 0, and xCtb + x − PpsVirtualBoundariesPosX[ n ] is greater than or equal to 0 and less than 2 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

hx + i = Clip3( PpsVirtualBoundariesPosX[ n ], pic_width_in_luma_samples − 1, xCtb + x + i )	(8-1239)

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is not equal to 0, and PpsVirtualBoundariesPosX[ n ] − xCtb − x is greater than 0 and less than 6 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

hx + i = Clip3( 0, PpsVirtualBoundariesPosX[ n ] − 1, xCtb + x + i )	(8-1240)

  - Otherwise, the following applies:

hx + i = Clip3( 0, pic_width_in_luma_samples − 1, xCtb + x + i )	(8-1241)

  - If loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0, the top boundary of the current coding tree block is the top boundary of the subpicture, and y is greater than or equal to 0 and less than 2, the following applies:

vy + j  = Clip3( yCtb, pic_height_in_luma_samples  − 1, yCtb + y + j )

  - Otherwise, if loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0, the bottom boundary of the current coding tree block is the bottom boundary of the subpicture, and CtbSizeY−y is greater than 0 and less than 6, the following applies:

vy + j  = Clip3( 0, yCtb + CtbSizeY − 1, yCtb + y + j )

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is not equal to 0, and yCtb + y − PpsVirtualBoundariesPosY[ n ] is greater than or equal to 0 and less than 2 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

vy + j = Clip3( PpsVirtualBoundariesPosY[ n ], pic_height_in_luma_samples − 1, yCtb + y + j )	(8-1242)

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is not equal to 0, and PpsVirtualBoundariesPosY[ n ] − yCtb − y is greater than 0 and less than 6 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

vy + j = Clip3( 0, PpsVirtualBoundariesPosY[ n ] − 1, yCtb + y + j ) 	(8-1243)

  - Otherwise, the following applies:

vy + j = Clip3( 0, pic_height_in_luma_samples − 1, yCtb + y + j )	(8-1244)

8.8.5.4	Coding tree block filtering process for chroma samples

  - The locations ( hx + i, vy + j ) for each of the corresponding chroma samples ( x, y ) inside the given array recPicture of chroma samples with i, j = −2..2 are derived as follows:

  - If loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0, the left boundary of the current coding tree block is the left boundary of the subpicture, and x is greater than or equal to 0 and less than 2, the following applies:

hx + i = Clip3( xCtbC, pic_width_in_luma_samples / SubWidthC  − 1, xCtbC + x + i )

  - Otherwise, if loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0, the right boundary of the current coding tree block is the right boundary of the subpicture, and CtbSizeY/ SubWidthC −x is greater than 0 and less than 3, the following applies:

hx + i = Clip3( 0, xCtbC + CtbSizeY/ SubWidthC  − 1, xCtbC + x + i )

  - Otherwise, if If pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is not equal to 0 and xCtbC + x − PpsVirtualBoundariesPosX[ n ] / SubWidthC is greater than or equal to 0 and less than 2 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

hx + i = Clip3( PpsVirtualBoundariesPosX[ n ] / SubWidthC,	(8-1280)
			pic_width_in_luma_samples / SubWidthC − 1, xCtbC + x + i )

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is not equal to 0 and PpsVirtualBoundariesPosX[ n ] / SubWidthC − xCtbC − x is greater than 0 and less than 3 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

hx + i = Clip3( 0, PpsVirtualBoundariesPosX[ n ] / SubWidthC − 1, xCtbC + x + i )	(8-1281)

  - Otherwise, the following applies:

hx + i = Clip3( 0, pic_width_in_luma_samples / SubWidthC − 1, xCtbC + x + i )	(8-1282)

  - If loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0, the top boundary of the current coding tree block is the top boundary of the subpicture, and y is greater than or equal to 0 and less than 2, the following applies:

vy + j  = Clip3( yCtbC, pic_height_in_luma_samples / SubHeightC − 1, yCtbC/ SubHeightC + y + j )

  - Otherwise, if loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0, the bottom boundary of the current coding tree block is the bottom boundary of the subpicture, and CtbSizeY/ SubHeightC −y is greater than 0 and less than 3, the following applies:

vy + j  = Clip3( 0, yCtbC + CtbSizeY/ SubHeightC − 1, yCtbC + y + j )

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is not equal to 0 and yCtbC + y − PpsVirtualBoundariesPosY[ n ] / SubHeightC is greater than or equal to 0 and less than 2 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

vy + j = Clip3( PpsVirtualBoundariesPosY[ n ] / SubHeightC ,	(8-1283)
			pic_height_in_luma_samples / SubHeightC − 1, yCtbC + y + j )

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is not equal to 0 and PpsVirtualBoundariesPosY[ n ] / SubHeightC − yCtbC − y is greater than 0 and less than 3 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

vy + j = Clip3( 0, PpsVirtualBoundariesPosY[ n ] / SubHeightC − 1, yCtbC + y + j )	(8-1284)

  - Otherwise, the following applies:

vy + j = Clip3( 0, pic_height_in_luma_samples / SubHeightC − 1, yCtbC + y + j )	(8-1285)

8.8.5.5	ALF boundary position derivation process

Inputs of this process are:

  - a luma location ( xCtb, yCtb ) specifying the top-left sample of the current luma coding tree block relative to the top left sample of the current picture,

  - a luma location ( x, y ) specifying the current sample relative to the top-left sample of the current luma coding tree block.

Output of this process are:

  - the left vertical boundary position clipLeftPos,

  - the right vertical boundary position clipRightPos,

  - the above horizontal boundary position clipTopPos,

  - the below horizontal boundary position clipBottomPos.

The variables clipLeftPos, clipRightPos, clipTopPos and clipBottomPos are set equal to −128.

The variable clipTopPos is modified as follows:

  - If the bottom boundary of the current coding tree block is not the bottom boundary of the picture and y − ( CtbSizeY − 4 ) is greater than or equal to 0, the variable clipTopPos is set equal to yCtb + CtbSizeY − 4.

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is equal to 0, and yCtb + y − PpsVirtualBoundariesPosY[ n ] is greater than or equal to 0 and less than 3 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

clipTopPos =  PpsVirtualBoundariesPosY[ n ]		(8-1292)

  - Otherwise, if y is less than 3, and the top boundary of the current coding tree block is not the top boundary of the picture, and one or more of the following conditions are true, the variable clipTopPos is set equal to yCtb:

    - If the top boundary of the current coding tree block is the top boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the top boundary of the current coding tree block is the top boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If the top boundary of the current coding tree block is the top boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

The variable clipBottomPos is modified as follows:

  - If the bottom boundary of the current coding tree block is not the bottom boundary of the picture and CtbSizeY − 4 − y is greater than 0 and is less than 4, the variable clipBottomPos is set equal to yCtb + CtbSizeY − 4.

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, PpsVirtualBoundariesPosY[ n ] % CtbSizeY is equal to 0, PpsVirtualBoundariesPosY[ n ] is not equal to pic_height_in_luma_samples − 1 or 0, and PpsVirtualBoundariesPosY[ n ] − yCtb − y is greater than 0 and less than 4 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

clipBottomPos =  PpsVirtualBoundariesPosY[ n ]		(8-1293)

  - Otherwise, if CtbSizeY − y is less than 4, and the bottom boundary of the current coding tree block is not the bottom boundary of the picture, and one or more of the following conditions are true, the variable clipBottomPos is set equal to yCtb + CtbSizeY:

    - If the bottom boundary of the current coding tree block is the bottom boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the bottom boundary of the current coding tree block is the bottom boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If the bottom boundary of the current coding tree block is the bottom boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

The variable clipLeftPos is modified as follows:

  - If pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is equal to 0, and xCtb + x − PpsVirtualBoundariesPosX[ n ] is greater than or equal to 0 and less than 3 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

clipLeftPos = PpsVirtualBoundariesPosX[ n ]		(8-1294)

  - Otherwise, if x is less than 3, the left boundary of the current coding tree block is not the left boundary of the picture and one or more of the following conditions are true, the variable clipLeftPos is set equal to xCtb:

    - If the left boundary of the current coding tree block is the left boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the left boundary of the current coding tree block is the left boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If the left boundary of the current coding tree block is the left boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

The variable clipRightPos is modified as follows:

  - If pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is equal to 0, and PpsVirtualBoundariesPosX[ n ]− xCtb − x is greater than 0 and less than 4 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

clipRightPos = PpsVirtualBoundariesPosX[ n ]		(8-1295)

  - Otherwise, if CtbSizeY − x is less than 4, and the right boundary of the current coding tree block is not the right boundary of the picture, and one or more of the following conditions are true, the variable clipRightPos is set equal to xCtb + CtbSizeY:

    - If the right boundary of the current coding tree block is the right boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the right boundary of the current coding tree block is the right boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If the right boundary of the current coding tree block is the right boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

The corresponding text change of Method4 is provided and highlighted in yellow color in the following.

8.8.5.2	Coding tree block filtering process for luma samples

  - The locations ( hx + i, vy + j ) for each of the corresponding luma samples ( x, y ) inside the given array recPicture of luma samples with i, j = −3..3 are derived as follows:

  - If x is greater than or equal to 0 and less than 3, the left boundary of the current coding tree block is not the left boundary of the picture, and one or more of the following conditions are true, hx + i is derived as

hx + i = Clip3( xCtb, pic_width_in_luma_samples − 1, xCtb + x + i )

      - If the left boundary of the current coding tree block is the left boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

      - If the left boundary of the current coding tree block is the left boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

      - If the left boundary of the current coding tree block is the left boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

  - Otherwise, if CtbSizeY − x is greater than 0 and less than 4, the right boundary of the current coding tree block is not the right boundary of the picture and one or more of the following conditions are true, hx + i is derived as:

hx + i = Clip3( 0, xCtb + CtbSizeY − 1, xCtb + x + i )

      - If the right boundary of the current coding tree block is the right boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

      - If the right boundary of the current coding tree block is the right boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

      - If the right boundary of the current coding tree block is the right boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is not equal to 0, and xCtb + x − PpsVirtualBoundariesPosX[ n ] is greater than or equal to 0 and less than 3 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

hx + i = Clip3( PpsVirtualBoundariesPosX[ n ], pic_width_in_luma_samples − 1, xCtb + x + i )	(8-1229)

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is not equal to 0, and PpsVirtualBoundariesPosX[ n ]− xCtb − x is greater than 0 and less than 4 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

hx + i = Clip3( 0, PpsVirtualBoundariesPosX[ n ] − 1, xCtb + x + i )	(8-1230)

  - Otherwise, the following applies:

hx + i = Clip3( 0, pic_width_in_luma_samples − 1, xCtb + x + i )	(8-1231)

  - If y is greater than or equal to 0 and less than 3, the top boundary of the current coding tree block is not the top boundary of the picture, and one or more of the following conditions are true, vy + j is derived as:

vy + j  = Clip3( yCtb, pic_height_in_luma_samples  − 1, yCtb + y + j )

      - If the top boundary of the current coding tree block is the top boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

      - If the top boundary of the current coding tree block is the top boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

      - If the top boundary of the current coding tree block is the top boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

  - Otherwise, if yCtb − y is greater than 0 and less than 4, the bottom boundary of the current coding tree block is not the bottom boundary of the picture, and one or more of the following conditions are true, vy + j is derived as:

vy + j  = Clip3( 0, yCtb + CtbSizeY − 1, yCtb + y + j )

      - If the bottom boundary of the current coding tree block is the bottom boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

      - If the bottom boundary of the current coding tree block is the bottom boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

      - If the bottom boundary of the current coding tree block is the bottom boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is not equal to 0, and yCtb + y − PpsVirtualBoundariesPosY[ n ] is greater than or equal to 0 and less than 3 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

vy + j = Clip3( PpsVirtualBoundariesPosY[ n ], pic_height_in_luma_samples − 1, yCtb + y + j )	(8-1232)

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is not equal to 0, and PpsVirtualBoundariesPosY[ n ] − yCtb − y is greater than 0 and less than 4 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

vy + j = Clip3( 0, PpsVirtualBoundariesPosY[ n ] − 1, yCtb + y + j )	(8-1233)

  - Otherwise, the following applies:

vy + j = Clip3( 0, pic_height_in_luma_samples − 1, yCtb + y + j )	(8-1234)

8.8.5.3	Derivation process for ALF transpose and filter index for luma samples

The locations ( hx + i, vy + j ) for each of the corresponding luma samples ( x, y ) inside the given array recPicture of luma samples with i, j = −2..5 are derived as follows:

  - If x is greater than or equal to 0 and less than 2, the left boundary of the current coding tree block is not the left boundary of the picture, and one or more of the following conditions are true, hx + i is derived as

hx + i = Clip3( xCtb, pic_width_in_luma_samples − 1, xCtb + x + i )

      - If the left boundary of the current coding tree block is the left boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

      - If the left boundary of the current coding tree block is the left boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

      - If the left boundary of the current coding tree block is the left boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

  - Otherwise, if CtbSizeY−x is greater than 0 and less than 6, the right boundary of the current coding tree block is not the right boundary of the picture and one or more the following conditions are true, hx + i is derived as:

hx + i = Clip3( 0, xCtb + CtbSizeY − 1, xCtb + x + i )

      - If the right boundary of the current coding tree block is the right boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

      - If the right boundary of the current coding tree block is the right boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

      - If the right boundary of the current coding tree block is the right boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is not equal to 0, and xCtb + x − PpsVirtualBoundariesPosX[ n ] is greater than or equal to 0 and less than 2 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

hx + i = Clip3( PpsVirtualBoundariesPosX[ n ], pic_width_in_luma_samples − 1, xCtb + x + i )	(8-1239)

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is not equal to 0, and PpsVirtualBoundariesPosX[ n ] − xCtb − x is greater than 0 and less than 6 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

hx + i = Clip3( 0, PpsVirtualBoundariesPosX[ n ] − 1, xCtb + x + i )	(8-1240)

  - Otherwise, the following applies:

hx + i = Clip3( 0, pic_width_in_luma_samples − 1, xCtb + x + i )	(8-1241)

  - If y is greater than or equal to 0 and less than 2, the top boundary of the current coding tree block is not the top boundary of the picture, and one or more of the following conditions are true, vy + j is derived as:

vy + j  = Clip3( yCtb, pic_height_in_luma_samples  − 1, yCtb + y + j )

      - If the top boundary of the current coding tree block is the top boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

      - If the top boundary of the current coding tree block is the top boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

      - If the top boundary of the current coding tree block is the top boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

  - Otherwise, if yCtb − y is greater than 0 and less than 6, the bottom boundary of the current coding tree block is not the bottom boundary of the picture, and one or more of the following conditions are true, vy + j is derived as:

vy + j  = Clip3( 0, yCtb + CtbSizeY − 1, yCtb + y + j )

      - If the bottom boundary of the current coding tree block is the bottom boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

      - If the bottom boundary of the current coding tree block is the bottom boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

      - If the bottom boundary of the current coding tree block is the bottom boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is not equal to 0, and yCtb + y − PpsVirtualBoundariesPosY[ n ] is greater than or equal to 0 and less than 2 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

vy + j = Clip3( PpsVirtualBoundariesPosY[ n ], pic_height_in_luma_samples − 1, yCtb + y + j )	(8-1242)

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is not equal to 0, and PpsVirtualBoundariesPosY[ n ] − yCtb − y is greater than 0 and less than 6 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

vy + j = Clip3( 0, PpsVirtualBoundariesPosY[ n ] − 1, yCtb + y + j ) 	(8-1243)

  - Otherwise, the following applies:

vy + j = Clip3( 0, pic_height_in_luma_samples − 1, yCtb + y + j )	(8-1244)

8.8.5.4	Coding tree block filtering process for chroma samples

  - The locations ( hx + i, vy + j ) for each of the corresponding chroma samples ( x, y ) inside the given array recPicture of chroma samples with i, j = −2..2 are derived as follows:

  - If x is greater than or equal to 0 and less than 2, the left boundary of the current coding tree block is not the left boundary of the picture, and one or more of the following conditions are true, hx + i is derived as:

hx + i = Clip3( xCtbC, pic_width_in_luma_samples / SubWidthC  − 1, xCtbC + x + i )

      - If the left boundary of the current coding tree block is the left boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

      - If the left boundary of the current coding tree block is the left boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

      - If the left boundary of the current coding tree block is the left boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

  - Otherwise, if CtbSizeY/ SubWidthC −x is greater than 0 and less than 3, the right boundary of the current coding tree block is not the right boundary of the picture and one or more the following conditions are true, hx + i is derived as:

hx + i = Clip3( 0, xCtbC + CtbSizeY/ SubWidthC  − 1, xCtbC + x + i )

      - If the right boundary of the current coding tree block is the right boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

      - If the right boundary of the current coding tree block is the right boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

      - If the right boundary of the current coding tree block is the right boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is not equal to 0 and xCtbC + x − PpsVirtualBoundariesPosX[ n ] / SubWidthC is greater than or equal to 0 and less than 2 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

hx + i = Clip3( PpsVirtualBoundariesPosX[ n ] / SubWidthC,	(8-1280)
			pic_width_in_luma_samples / SubWidthC − 1, xCtbC + x + i )

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is not equal to 0 and PpsVirtualBoundariesPosX[ n ] / SubWidthC − xCtbC − x is greater than 0 and less than 3 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

hx + i = Clip3( 0, PpsVirtualBoundariesPosX[ n ] / SubWidthC − 1, xCtbC + x + i )	(8-1281)

  - Otherwise, the following applies:

hx + i = Clip3( 0, pic_width_in_luma_samples / SubWidthC − 1, xCtbC + x + i )	(8-1282)

  - If y is greater than or equal to 0 and less than 2, the top boundary of the current coding tree block is not the top boundary of the picture, and one or more of the following conditions are true, vy + j is derived as:

vy + j  = Clip3( yCtbC, pic_height_in_luma_samples / SubHeightC − 1, yCtbC/ SubHeightC + y + j )

      - If the top boundary of the current coding tree block is the top boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

      - If the top boundary of the current coding tree block is the top boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

      - If the top boundary of the current coding tree block is the top boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

  - Otherwise, if CtbSizeY/ SubHeightC −y is greater than 0 and less than 3, the bottom boundary of the current coding tree block is not the bottom boundary of the picture, and one or more of the following conditions are true, vy + j is derived as:

vy + j  = Clip3( 0, yCtbC + CtbSizeY/ SubHeightC − 1, yCtbC + y + j )

      - If the bottom boundary of the current coding tree block is the bottom boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

      - If the bottom boundary of the current coding tree block is the bottom boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

      - If the bottom boundary of the current coding tree block is the bottom boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is not equal to 0 and yCtbC + y − PpsVirtualBoundariesPosY[ n ] / SubHeightC is greater than or equal to 0 and less than 2 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

vy + j = Clip3( PpsVirtualBoundariesPosY[ n ] / SubHeightC ,	(8-1283)
			pic_height_in_luma_samples / SubHeightC − 1, yCtbC + y + j )

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is not equal to 0 and PpsVirtualBoundariesPosY[ n ] / SubHeightC − yCtbC − y is greater than 0 and less than 3 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

vy + j = Clip3( 0, PpsVirtualBoundariesPosY[ n ] / SubHeightC − 1, yCtbC + y + j )	(8-1284)

  - Otherwise, the following applies:

vy + j = Clip3( 0, pic_height_in_luma_samples / SubHeightC − 1, yCtbC + y + j )	(8-1285)

8.8.5.5	ALF boundary position derivation process

Inputs of this process are:

  - a luma location ( xCtb, yCtb ) specifying the top-left sample of the current luma coding tree block relative to the top left sample of the current picture,

  - a luma location ( x, y ) specifying the current sample relative to the top-left sample of the current luma coding tree block.

Output of this process are:

  - the left vertical boundary position clipLeftPos,

  - the right vertical boundary position clipRightPos,

  - the above horizontal boundary position clipTopPos,

  - the below horizontal boundary position clipBottomPos.

The variables clipLeftPos, clipRightPos, clipTopPos and clipBottomPos are set equal to −128.

The variable clipTopPos is modified as follows:

  - If the bottom boundary of the current coding tree block is not the bottom boundary of the picture and y − ( CtbSizeY − 4 ) is greater than or equal to 0, the variable clipTopPos is set equal to yCtb + CtbSizeY − 4.

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is equal to 0, and yCtb + y − PpsVirtualBoundariesPosY[ n ] is greater than or equal to 0 and less than 3 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

clipTopPos =  PpsVirtualBoundariesPosY[ n ]		(8-1292)

  - Otherwise, if y is less than 3, and the top boundary of the current coding tree block is not the top boundary of the picture, and one or more of the following conditions are true, the variable clipTopPos is set equal to yCtb:

    - If the top boundary of the current coding tree block is the top boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the top boundary of the current coding tree block is the top boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If the top boundary of the current coding tree block is the top boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

The variable clipBottomPos is modified as follows:

  - If the bottom boundary of the current coding tree block is not the bottom boundary of the picture and CtbSizeY − 4 − y is greater than 0 and is less than 4, the variable clipBottomPos is set equal to yCtb + CtbSizeY − 4.

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, PpsVirtualBoundariesPosY[ n ] % CtbSizeY is equal to 0, PpsVirtualBoundariesPosY[ n ] is not equal to pic_height_in_luma_samples − 1 or 0, and PpsVirtualBoundariesPosY[ n ] − yCtb − y is greater than 0 and less than 4 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

clipBottomPos =  PpsVirtualBoundariesPosY[ n ]		(8-1293)

  - Otherwise, if CtbSizeY − y is less than 4, and the bottom boundary of the current coding tree block is not the bottom boundary of the picture, and one or more of the following conditions are true, the variable clipBottomPos is set equal to yCtb + CtbSizeY:

    - If the bottom boundary of the current coding tree block is the bottom boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the bottom boundary of the current coding tree block is the bottom boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If the bottom boundary of the current coding tree block is the bottom boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

The variable clipLeftPos is modified as follows:

  - If pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is equal to 0, and xCtb + x − PpsVirtualBoundariesPosX[ n ] is greater than or equal to 0 and less than 3 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

clipLeftPos = PpsVirtualBoundariesPosX[ n ]		(8-1294)

  - Otherwise, if x is less than 3, the left boundary of the current coding tree block is not the left boundary of the picture and one or more of the following conditions are true, the variable clipLeftPos is set equal to xCtb:

    - If the left boundary of the current coding tree block is the left boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the left boundary of the current coding tree block is the left boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If the left boundary of the current coding tree block is the left boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

The variable clipRightPos is modified as follows:

  - If pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is equal to 0, and PpsVirtualBoundariesPosX[ n ]− xCtb − x is greater than 0 and less than 4 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

clipRightPos = PpsVirtualBoundariesPosX[ n ]		(8-1295)

  - Otherwise, if CtbSizeY − x is less than 4, and the right boundary of the current coding tree block is not the right boundary of the picture, and one or more of the following conditions are true, the variable clipRightPos is set equal to xCtb + CtbSizeY:

    - If the right boundary of the current coding tree block is the right boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the right boundary of the current coding tree block is the right boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If the right boundary of the current coding tree block is the right boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

# References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team, JVET-O2001

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).