| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0158-v2 |
| --- | --- |

| Title: | CE5-related: Align ALF virtual boundary processes in the bottom CTU rows of one picture and one subpicture |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai, Olena Chubach, Lulin Chen, Ching-Yeh Chen, Tzu-Der Chuang, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {jenny.lai, olena.chubach, lulin.chen, chingyeh.chen, peter.chuang, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

In VVC Draft 6, to remove the line buffers required in adaptive loop filter (ALF) hardware decoding, the ALF virtual boundary (VB) process is applied to each coding tree unit (CTU) row in a picture except for the bottom CTU row. When a picture consists of independently decodable subpictures and the subpictures may be extracted as a picture, the mismatch may occur for the bottom CTU row handling if the subpicture bottom boundary is not the picture bottom boundary. Two methods are proposed in this contribution to solve this issue.

1. Applying the ALF VB process to all CTU rows in a picture, including the bottom CTU row

2. Disabling the ALF VB process for subpicture bottom CTU rows if the subpicture can be independently decodable (subpic_treated_as_pic_flag[ i ] = 1 and loop_filter_across_subpic_enabled_flag[i]=0 )

Introduction

In VVC Draft 6 [1], ALF VB process is utilized to remove the line buffer requirement in ALF hardware decoding. The concept of ALF VB process is to create one VB for one CTU row, as shown in Fig. 1, where the picture is composed of 8x5 CTUs, one square is one CTU, and the red dash lines are ALF VBs. In order to remove the line buffers, when processing the samples above a VB, the samples below the VB cannot be used. Similarly, when processing the samples below the VB, the samples above the VB cannot be used. Hence there is no data dependency crossing each VB, so no line buffer is required. Note that ALF VB process is applied to each CTU row in one picture, except the bottom CTU row. In VVC Draft 6, luma VBs are located at positions of four luma samples above CTU row boundaries, and chroma VBs are located at positions of two chroma samples above CTU row boundaries.

Figure 1. The concept of ALF VB, where the picture is composed of 8x5 CTUs, one square is one CTU, and the red dash lines are ALF VBs.

Figure 2 shows the case of applying ALF VB to one picture with four subpictures. Since ALF VB process is applied to each CTU row in one picture except the bottom CTU row, ALF VB process is not applied to those bottom CTU rows of subpicture3 and subpicture4. However, for the bottom CTU rows of subpicture1 and subpicture2, ALF VB process is applied. When independently decoding subpicture is enabled, i.e., subpic_treated_as_pic_flag[i] is equal to 1 and and loop_filter_across_subpic_enabled_flag[i] is equal to 0, subpicture boundaries become picture boundaries. For subpicture3 and subpicture4, the subpicture bottom boundaries are also the picture bottom boundary, so ALF VB process is not applied. The same decoding results can be obtained by the independent subpicture decoding. However, for subpicture1 and subpicture2, the subpicture bottom boundaries are not the picture bottom boundary in the entire picture, so ALF VB process is applied in the original bitstream, but it should be disabled in the extracted subpicture bitstream when independently decoding one subpicture. Hence independent subpicture1 and subpicture2 decoding results are different from picture decoding results.

Figure 2. The mismatch of ALF VB process, when subpicture bottom boundary is not the picture bottom boundary and independently decoding subpicture is enabled.

Proposed methods

In order to solve the issue described in the previous section, two methods are proposed.

Method1 is always applying ALF VB process for all CTU rows in one picture. That is, no matter current CTU row is the bottom CTU row in one picture or not, ALF VB process is always applied. If the picture height is not a multiple of CTU height, the last CTU row (picWidth * N; for example: N=120, 112, 104, …, 8 for luma when CTU contains 128x128 luma samples) does not include any ALF VB, so ALF VB process will not be activated; else (the picture height is a multiple of CTU height), the last CTU row includes one ALF VB, and ALF VB process is applied to unify the ALF process between picture boundaries and subpicture boundaries.

Method2 is conditionally applying ALF VB process. When the current CTU row is the bottom CTU row of one picture or the bottom CTU row of one subpicture with enabled independently decoding for this subpicture (subpic_treated_as_pic_flag[i]=1 and and loop_filter_across_subpic_enabled_flag[i]=0), ALF VB process cannot be applied to current CTU row.

Table 1 summarizes the proposed methods and current design in VVC Draft.

| Method | Bottom CTU row in one picture | Bottom CTU row in one subpicture | Other CTU rows |
| --- | --- | --- | --- |
| VVC Draft 6 | Disable ALF VB | If the bottom CTU row is the bottom CTU row of one picture, Disable ALF VB Otherwise, Enable ALF VB | Enable ALF VB |
| Method1 | Enable ALF VB | Enable ALF VB | Enable ALF VB |
| Method2 | Disable ALF VB | If subpic_treated_as_pic_flag[i] is equal to 1 and loop_filter_across_subpic_enabled_flag[i] equal to 0, Disable ALF VB Otherwise, Enable ALF VB | Enable ALF VB |

Corresponding text changes

The corresponding text change of Method1 is provided and highlighted in yellow color in the following.

8.8.5.5	ALF boundary position derivation process

Inputs of this process are:

  - a luma location ( xCtb, yCtb ) specifying the top-left sample of the current luma coding tree block relative to the top left sample of the current picture,

  - a luma location ( x, y ) specifying the current sample relative to the top-left sample of the current luma coding tree block.

Output of this process are:

  - the left vertical boundary position clipLeftPos,

  - the right vertical boundary position clipRightPos,

  - the above horizontal boundary position clipTopPos,

  - the below horizontal boundary position clipBottomPos.

The variables clipLeftPos, clipRightPos, clipTopPos and clipBottomPos are set equal to −128.

The variable clipTopPos is modified as follows:

  - If the bottom boundary of the current coding tree block is not the bottom boundary of the picture and y − ( CtbSizeY − 4 ) is greater than or equal to 0, the variable clipTopPos is set equal to yCtb + CtbSizeY − 4.

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is equal to 0, and yCtb + y − PpsVirtualBoundariesPosY[ n ] is greater than or equal to 0 and less than 3 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

clipTopPos =  PpsVirtualBoundariesPosY[ n ]		(8-1292)

  - Otherwise, if y is less than 3, and the top boundary of the current coding tree block is not the top boundary of the picture, and one or more of the following conditions are true, the variable clipTopPos is set equal to yCtb:

    - If the top boundary of the current coding tree block is the top boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the top boundary of the current coding tree block is the top boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If the top boundary of the current coding tree block is the top boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

The variable clipBottomPos is modified as follows:

  - If the bottom boundary of the current coding tree block is not the bottom boundary of the picture and CtbSizeY − 4 − y is greater than 0 and is less than 4, the variable clipBottomPos is set equal to yCtb + CtbSizeY − 4.

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, PpsVirtualBoundariesPosY[ n ] % CtbSizeY is equal to 0, PpsVirtualBoundariesPosY[ n ] is not equal to pic_height_in_luma_samples − 1 or 0, and PpsVirtualBoundariesPosY[ n ] − yCtb − y is greater than 0 and less than 4 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

clipBottomPos =  PpsVirtualBoundariesPosY[ n ]		(8-1293)

  - Otherwise, if CtbSizeY − y is less than 4, and the bottom boundary of the current coding tree block is not the bottom boundary of the picture, and one or more of the following conditions are true, the variable clipBottomPos is set equal to yCtb + CtbSizeY:

    - If the bottom boundary of the current coding tree block is the bottom boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the bottom boundary of the current coding tree block is the bottom boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If the bottom boundary of the current coding tree block is the bottom boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

The corresponding text change of Method2 is provided and highlighted in blue color in the following.

8.8.5.5	ALF boundary position derivation process

Inputs of this process are:

  - a luma location ( xCtb, yCtb ) specifying the top-left sample of the current luma coding tree block relative to the top left sample of the current picture,

  - a luma location ( x, y ) specifying the current sample relative to the top-left sample of the current luma coding tree block.

Output of this process are:

  - the left vertical boundary position clipLeftPos,

  - the right vertical boundary position clipRightPos,

  - the above horizontal boundary position clipTopPos,

  - the below horizontal boundary position clipBottomPos.

The variables clipLeftPos, clipRightPos, clipTopPos and clipBottomPos are set equal to −128.

The variable clipTopPos is modified as follows:

  - If all of the following conditions are true the bottom boundary of the current coding tree block is not the bottom boundary of the picture, and y − ( CtbSizeY − 4 ) is greater than or equal to 0, the variable clipTopPos is set equal to yCtb + CtbSizeY − 4.

    - If the bottom boundary of the current coding tree block is not the bottom boundary of the picture.

    - If the bottom boundary of the current coding tree block is not the bottom boundary of the subpicture, loop_filter_across_subpic_enabled_flag[i] for the i-th subpicture is equal to 1, or subpic_treated_as_pic_flag[i] for the i-th subpicture is equal to FALSE.

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is equal to 0, and yCtb + y − PpsVirtualBoundariesPosY[ n ] is greater than or equal to 0 and less than 3 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

clipTopPos =  PpsVirtualBoundariesPosY[ n ]		(8-1292)

  - Otherwise, if y is less than 3, and the top boundary of the current coding tree block is not the top boundary of the picture, and one or more of the following conditions are true, the variable clipTopPos is set equal to yCtb:

    - If the top boundary of the current coding tree block is the top boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the top boundary of the current coding tree block is the top boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If the top boundary of the current coding tree block is the top boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

The variable clipBottomPos is modified as follows:

  - If all of the following conditions are true the bottom boundary of the current coding tree block is not the bottom boundary of the picture, and CtbSizeY − 4 − y is greater than 0 and is less than 4, the variable clipBottomPos is set equal to yCtb + CtbSizeY − 4.

    - If the bottom boundary of the current coding tree block is not the bottom boundary of the picture.

    - If the bottom boundary of the current coding tree block is not the bottom boundary of the subpicture, loop_filter_across_subpic_enabled_flag[i] for the i-th subpicture is equal to 1, or subpic_treated_as_pic_flag[i] for the i-th subpicture is equal to FALSE.

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, PpsVirtualBoundariesPosY[ n ] % CtbSizeY is equal to 0, PpsVirtualBoundariesPosY[ n ] is not equal to pic_height_in_luma_samples − 1 or 0, and PpsVirtualBoundariesPosY[ n ] − yCtb − y is greater than 0 and less than 4 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

clipBottomPos =  PpsVirtualBoundariesPosY[ n ]		(8-1293)

  - Otherwise, if CtbSizeY − y is less than 4, and the bottom boundary of the current coding tree block is not the bottom boundary of the picture, and one or more of the following conditions are true, the variable clipBottomPos is set equal to yCtb + CtbSizeY:

    - If the bottom boundary of the current coding tree block is the bottom boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the bottom boundary of the current coding tree block is the bottom boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If the bottom boundary of the current coding tree block is the bottom boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team, JVET-O2001

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).