| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0163-v1 |
| --- | --- |

| Title: | CE5-related: SAO encoder-only improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai, Ching-Yeh Chen, Tzu-Der Chuang, Olena Chubach, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {jenny.lai, chingyeh.chen, peter.chuang, olena.chubach, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, three encoder-only improvements for sample adaptive offset (SAO) are reported. First, the greedy SAO merge encoding algorithm, which is in VTM6.0 but disabled in common test conditions (CTC), is enabled to optimize SAO parameters for multiple coding tree units (CTUs) within one CTU row. Second, to explore more SAO merge possibilities across CTU rows, a picture-based SAO merge encoding algorithm is proposed. For each picture, the best SAO parameters, derived by either the picture-based SAO merge encoding algorithm or the greedy SAO merge encoding algorithm, are signalled. Third, the SAO picture-level on-off decision algorithm for the current picture using statistics from a previous picture, which is enabled in VTM6.0CTC, is disabled, and it is proposed to further allow changing the picture-level decision from on to off after SAO is tested for all CTUs in the picture. YCbCr BD-rates (using the AHG13 YCbCr BD-rate calculation method: (BD-Rate_Y*8+ BD-Rate_U+ BD-Rate_V)/10) under adaptive loop filter (ALF) on/off cases are summarized as follows.

Anchor: VTM6.0 + SAO off + ALF on
Test: anchor + SAO on
AI: -0.04%; RA: -0.13%; LB: -0.24%; LP: -0.26%

Anchor: VTM6.0 + SAO off + ALF on
Test: anchor + SAO on + SAO encoder-only improvements
AI: -0.15%; RA: -0.29%; LB: -0.52%; LP: -0.52%

Anchor: VTM6.0 + SAO off + ALF off
Test: anchor + SAO on
AI: -0.36%; RA: -0.78%; LB: -1.21%; LP: -2.74%

Anchor: VTM6.0 + SAO off + ALF off
Test: anchor + SAO on + SAO encoder-only improvements
AI: -0.56%; RA: -1.10%; LB: -x.xx%; LP: -3.35%

It is known that SAO and ALF overlap in coding gains. Small coding losses are observed by disabling SAO when ALF is on, while large coding losses are observed by disabling SAO when ALF is off. Given that SAO has low complexity and is deployed in one billion HEVC devices per year, it is asserted that keeping SAO is beneficial for VVC, especially for real-time low complexity encoding applications.

# SAO encoder-only improvements

## Greedy SAO merge encoding algorithm

The greedy SAO merge encoding algorithm [1], which is in VTM6.0 but disabled in CTC, is enabled.

Under VTM6.0 CTC, the SAO encoder operates one CTU by one CTU. To improve coding efficiency, the greedy SAO merge encoding algorithm can be applied. Multiple CTUs of the same CTU row are grouped together to jointly optimize SAO parameters. In a CTU group, SAO parameters are signalled at the first CTU in the group, and the rest CTUs in the group are coded with SAO merge-left mode.

## Picture-based SAO merge encoding algorithm

The greedy SAO merge encoding algorithm in VTM6.0 (disabled in CTC) considers CTU grouping within one CTU row. A picture-based SAO merge encoding algorithm is proposed to consider CTU grouping across CTU rows. A picture can be divided into regions aligned with CTU boundaries by a recursive quadtree (QT) splitting algorithm with a bottom up rate-distortion optimization (RDO) method. When a quadtree node has only one CTU in width or height, binary tree (BT) can be applied for further split. The rate estimation in the picture-based SAO merge encoding algorithm only considers context-based adaptive binary arithmetic coding (CABAC) binarization bin counts instead of precise bit counts. The picture cost of the picture-based SAO merge encoding algorithm is compared with that of the greedy SAO merge encoding algorithm. For each picture, the best SAO parameters, derived by either the picture-based SAO merge encoding algorithm or the greedy SAO merge encoding algorithm, are signalled.

## Allowing change of SAO picture-level on/off decision after testing all CTUs

The SAO picture-level on-off decision algorithm for the current picture using statistics from a previous picture, which is enabled in VTM6.0 CTC, is disabled. In addition, it is proposed to allow changing the picture-level decision from on to off after SAO is tested for all CTUs in the picture by checking the picture cost. If the picture cost indicates that SAO is not beneficial for the current picture, SAO is disabled in each slice header of the current picture.

# Experiment results

The proposed methods were implemented on VTM6.0 [2]. The simulations were performed following the JVET CTC [3]. YCbCr BD-rates (using the AHG13 YCbCr BD-rate calculation method [4]) under ALF on/off cases are summarized as follows.

Anchor: VTM6.0 + SAO off + ALF on
Test: anchor + SAO on
AI: -0.04%; RA: -0.13%; LB: -0.24%; LP: -0.26% (Table 1)

Anchor: VTM6.0 + SAO off + ALF on
Test: anchor + SAO on + SAO encoder-only improvements
AI: -0.15%; RA: -0.29%; LB: -0.52%; LP: -0.52% (Table 2)

Anchor: VTM6.0 + SAO off + ALF off
Test: anchor + SAO on
AI: -0.36%; RA: -0.78%; LB: -1.21%; LP: -2.74% (Table 3)

Anchor: VTM6.0 + SAO off + ALF off
Test: anchor + SAO on + SAO encoder-only improvements
AI: -0.56%; RA: -1.10%; LB: -x.xx%; LP: -3.35% (Table 4)

Detailed results can be found in the attached Excel files.

# References

- C.-Y. Lai, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, and S.-M. Lei, “CE2-related: Improvements of sample adaptive offset,” Document of Joint Video Experts Team, JVET-K0238

- VTM6.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

- W.-J. Chien, J. Boyce, Y.-W. Chen, R. Chernyak, K. Choi, R. Hashimoto, Y.-W. Huang, H. Jang, R.-L. Liao, S. Liu, and D. Luo, “JVET AHG report: Tool reporting procedure (AHG13),” Document of Joint Video Experts Team, JVET-O0013

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).