| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0164-v1 |
| --- | --- |

| Title: | CE5-related: Simplified ALF syntax regarding to alf_luma_coeff_signalled_flag and alf_luma_coeff_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai, Ching-Yeh Chen, Tzu-Der Chuang, Olena Chubach, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {jenny.lai, chingyeh.chen, peter.chuang, olena.chubach, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VTM6.0 adaptive loop filter (ALF), alf_luma_coeff_signalled_flag is used to indicate whether a zero filter exists in the filter set. If alf_luma_coeff_signalled_flag is true, an alf_luma_coeff_flag is signalled for each filter in the filter set to indicate whether the filter is a zero filter. In this contribution, two methods to simplify the related ALF syntax are proposed. In Method 1, alf_luma_coeff_signalled_flag is signalled before alf_luma_num_filters_signalled_minus1 with removal of alf_luma_coeff_flag. In Method 2, alf_luma_coeff_signalled_flag and alf_luma_coeff_flag are both removed. It is asserted that both methods simplify the related specification text. BD-rates are reported as follows.

Method 1:
AI: 0.00% (Y), 0.00% (Cb), 0.00% (Cr)
RA: 0.00% (Y), 0.00% (Cb), 0.00% (Cr)
LB: -0.01% (Y), 0.08% (Cb), 0.12% (Cr)

Method 2:
AI: 0.00% (Y), 0.00% (Cb), 0.00% (Cr)
RA: 0.00% (Y), 0.01% (Cb), 0.00% (Cr)
LB: -0.03% (Y), -0.03% (Cb), -0.08% (Cr)

# Introduction

In VTM6.0 ALF, alf_luma_coeff_signalled_flag is used to indicate if there is at least one zero filter in the filter set. If alf_luma_coeff_signalled_flag is true, an alf_luma_coeff_flag is signalled for each filter in the filter set to indicate whether the filter is a zero filter. Therefore, the maximum number of signalled alf_luma_coeff_flag is 25. After that, the decoder needs to check alf_luma_coeff_flag for each filter before parsing filter coefficient absolute values, filter coefficient signs, and filter coefficient clipping indexes. For a filter with alf_luma_coeff_flag equal to false, it is not required to parse filter coefficient absolute values, filter coefficient signs, and filter coefficient clipping indexes since the filter is a zero filter. ALF data syntax in VVC Draft 6 is shown as follows.

| alf_data( ) { | Descriptor |
| --- | --- |
| alf_luma_filter_signal_flag | u(1) |
| alf_chroma_filter_signal_flag | u(1) |
| if( alf_luma_filter_signal_flag ) { |  |
| alf_luma_clip_flag | u(1) |
| alf_luma_num_filters_signalled_minus1 | ue(v) |
| if( alf_luma_num_filters_signalled_minus1 > 0 ) { |  |
| for( filtIdx = 0; filtIdx < NumAlfFilters; filtIdx++ ) |  |
| alf_luma_coeff_delta_idx[ filtIdx ] | u(v) |
| } |  |
| alf_luma_coeff_signalled_flag | u(1) |
| if( alf_luma_coeff_signalled_flag ) { |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) |  |
| alf_luma_coeff_flag[ sfIdx ] | u(1) |
| } |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| alf_luma_coeff_abs[ sfIdx ][ j ] | uek(v) |
| if( alf_luma_coeff_abs[ sfIdx ][ j ] ) |  |
| alf_luma_coeff_sign[ sfIdx ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( alf_luma_clip_flag ) { |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) |  |
| alf_luma_clip_idx[ sfIdx ][ j ] | u(2) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( alf_chroma_filter_signal_flag ) { |  |
| alf_chroma_num_alt_filters_minus1 | ue(v) |
| for( altIdx = 0; altIdx <= alf_chroma_num_alt_filters_minus1; altIdx++ ) { |  |
| alf_chroma_clip_flag[ altIdx ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| alf_chroma_coeff_abs[ altIdx ][ j ] | uek(v) |
| if( alf_chroma_coeff_abs[ altIdx ][ j ] > 0 ) |  |
| alf_chroma_coeff_sign[ altIdx ][ j ] | u(1) |
| } |  |
| if( alf_chroma_clip_flag[ altIdx ] ) { |  |
| for( j = 0; j < 6; j++ ) |  |
| alf_chroma_clip_idx[ altIdx ][ j ] | u(2) |
| } |  |
| } |  |
| } |  |
| } |  |

# Simplified ALF syntax parsing

## Method 1: Signalling alf_luma_coeff_signalled_flag before alf_luma_num_filters_signalled_minus with alf_luma_coeff_flag removed

To make ALF syntax signalling more efficient, it is proposed to signal alf_luma_coeff_signalled_flag before alf_luma_num_filters_siganlled_minus1. The semantics of alf_luma_coeff_signalled_flag is as follows: 1 means the last filter in the filter set is one zero filter, and 0 means no additional filter is created to be the zero filter in the filter set. At the encoder side, all filter indexes with zero filter will be merged into one index equal to the number of signalled filters (alf_luma_num_filters_signalled_minus1 plus 1). A new bitstream conformance statement for the encoder to obey is added: If alf_luma_coeff_signalled_flag is true, the filter with the filter index equal to alf_luma_num_filters_signalled_minus1 plus 1 in the filter set shall be a zero filter. At the decoder side, the value of alf_luma_num_filters_signalled_minus1 plus alf_luma_coeff_signalled_flag is checked before parsing alf_luma_coeff_delta_idx. If the value of alf_luma_num_filters_signalled_minus1 plus alf_luma_coeff_signalled_flag is larger than 0, alf_luma_coeff_delta_idx will be signalled; otherwise, alf_luma_coeff_delta_idx will not be signalled because there is only one filter in the filter set. With this modification, alf_luma_coeff_flag is removed, and the filter coefficient absolute values, filter coefficient signs, and filter coefficient clipping indexes for zero filter are not parsed even with the removal of alf_luma_coeff_flag. If alf_luma_coeff_delta_idx is equal to the number of signalled filters, zero filter will be used.

An example is shown in Table 1. The first row is the filter index after filter merging in VTM6.0 ALF encoding process, so N<=24. The second row shows filter indicators, where 1 means selecting a nonzero filter and 0 means selecting zero filter. In this example, there are M 1s on the second row, where M=4. The third row shows the adjusted filter indexes for signalling by the proposed ALF encoder. The encoder will send alf_luma_num_filters_siganlled_minus1 equal to 3 and alf_luma_coeff_signalled_flag equal to 1 (meaning zero filter is created to be the last filter in the filter set), and alf_luma_coeff_delta_idx equal to 4 indicates a zero filter.

Table 1.  An example to help illustrate Method 1, assuming there are M 1s on the second row (M=4)

| Filter index after filter merging | 0 | 1 | 2 | 3 | 4 | 5, 6, … | N (N<=24) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Filter indicator (1: nonzero filter; 0: zero filter) (alf_luma_coeff_flag, removed by this proposal) | 0 | 1 | 0 | 1 | 1 | 0, 0, … | 1 |
| Adjusted filter index (i.e., alf_luma_coeff_delta_idx) | 4 | 0 | 4 | 1 | 2 | 4, 4, … | 3 |

The related VVC specification text modification is as follows.

| alf_data( ) { | Descriptor |
| --- | --- |
| alf_luma_filter_signal_flag | u(1) |
| alf_chroma_filter_signal_flag | u(1) |
| if( alf_luma_filter_signal_flag ) { |  |
| alf_luma_clip_flag | u(1) |
| alf_luma_coeff_signalled_flag | u(1) |
| alf_luma_num_filters_signalled_minus1 | ue(v) |
| if( alf_luma_num_filters_signalled_minus1 + alf_luma_coeff_signalled_flag > 0 ) { |  |
| for( filtIdx = 0; filtIdx < NumAlfFilters; filtIdx++ ) |  |
| alf_luma_coeff_delta_idx[ filtIdx ] | u(v) |
| } |  |
| alf_luma_coeff_signalled_flag | u(1) |
| if( alf_luma_coeff_signalled_flag ) { |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) |  |
| alf_luma_coeff_flag[ sfIdx ] | u(1) |
| } |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| alf_luma_coeff_abs[ sfIdx ][ j ] | uek(v) |
| if( alf_luma_coeff_abs[ sfIdx ][ j ] ) |  |
| alf_luma_coeff_sign[ sfIdx ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( alf_luma_clip_flag ) { |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) |  |
| alf_luma_clip_idx[ sfIdx ][ j ] | u(2) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( alf_chroma_filter_signal_flag ) { |  |
| alf_chroma_num_alt_filters_minus1 | ue(v) |
| for( altIdx = 0; altIdx <= alf_chroma_num_alt_filters_minus1; altIdx++ ) { |  |
| alf_chroma_clip_flag[ altIdx ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| alf_chroma_coeff_abs[ altIdx ][ j ] | uek(v) |
| if( alf_chroma_coeff_abs[ altIdx ][ j ] > 0 ) |  |
| alf_chroma_coeff_sign[ altIdx ][ j ] | u(1) |
| } |  |
| if( alf_chroma_clip_flag[ altIdx ] ) { |  |
| for( j = 0; j < 6; j++ ) |  |
| alf_chroma_clip_idx[ altIdx ][ j ] | u(2) |
| } |  |
| } |  |
| } |  |
| } |  |

alf_luma_coeff_signalled_flag equal to 1 indicates the last filter in the filter set is one zero filter, and 0 means no additional filter is created to be the zero filter in the filter set. alf_luma_coeff_flag[ sfIdx ] is signalled. alf_luma_coeff_signalled_flag equal to 0 indicates that alf_luma_coeff_flag[ sfIdx ] is not signalled.

alf_luma_coeff_flag[ sfIdx ] equal 1 specifies that the coefficients of the luma filter indicated by sfIdx are signalled. alf_luma_coeff_flag[ sfIdx ] equal to 0 specifies that all filter coefficients of the luma filter indicated by sfIdx are set equal to 0. When not present, alf_luma_coeff_flag[ sfIdx ] is set equal to 1.

## Method 2: Removing alf_luma_coeff_signalled_flag and alf_luma_coeff_flag

The alf_luma_coeff_signalled_flag and alf_luma_coeff_flag syntax elements are removed to simplify ALF syntax. The decoder will parse filter coefficient absolute values, filter coefficient signs, and filter coefficient clipping indexes for each filter in the filter set regardless of whether the filter is a zero filter. The related VVC specification text modification is as follows.

| alf_data( ) { | Descriptor |
| --- | --- |
| alf_luma_filter_signal_flag | u(1) |
| alf_chroma_filter_signal_flag | u(1) |
| if( alf_luma_filter_signal_flag ) { |  |
| alf_luma_clip_flag | u(1) |
| alf_luma_num_filters_signalled_minus1 | ue(v) |
| if( alf_luma_num_filters_signalled_minus1 > 0 ) { |  |
| for( filtIdx = 0; filtIdx < NumAlfFilters; filtIdx++ ) |  |
| alf_luma_coeff_delta_idx[ filtIdx ] | u(v) |
| } |  |
| alf_luma_coeff_signalled_flag | u(1) |
| if( alf_luma_coeff_signalled_flag ) { |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) |  |
| alf_luma_coeff_flag[ sfIdx ] | u(1) |
| } |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| alf_luma_coeff_abs[ sfIdx ][ j ] | uek(v) |
| if( alf_luma_coeff_abs[ sfIdx ][ j ] ) |  |
| alf_luma_coeff_sign[ sfIdx ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( alf_luma_clip_flag ) { |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) |  |
| alf_luma_clip_idx[ sfIdx ][ j ] | u(2) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( alf_chroma_filter_signal_flag ) { |  |
| alf_chroma_num_alt_filters_minus1 | ue(v) |
| for( altIdx = 0; altIdx <= alf_chroma_num_alt_filters_minus1; altIdx++ ) { |  |
| alf_chroma_clip_flag[ altIdx ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| alf_chroma_coeff_abs[ altIdx ][ j ] | uek(v) |
| if( alf_chroma_coeff_abs[ altIdx ][ j ] > 0 ) |  |
| alf_chroma_coeff_sign[ altIdx ][ j ] | u(1) |
| } |  |
| if( alf_chroma_clip_flag[ altIdx ] ) { |  |
| for( j = 0; j < 6; j++ ) |  |
| alf_chroma_clip_idx[ altIdx ][ j ] | u(2) |
| } |  |
| } |  |
| } |  |
| } |  |

alf_luma_coeff_signalled_flag equal to 1 indicates that alf_luma_coeff_flag[ sfIdx ] is signalled. alf_luma_coeff_signalled_flag equal to 0 indicates that alf_luma_coeff_flag[ sfIdx ] is not signalled.

alf_luma_coeff_flag[ sfIdx ] equal 1 specifies that the coefficients of the luma filter indicated by sfIdx are signalled. alf_luma_coeff_flag[ sfIdx ] equal to 0 specifies that all filter coefficients of the luma filter indicated by sfIdx are set equal to 0. When not present, alf_luma_coeff_flag[ sfIdx ] is set equal to 1.

## Comparison between Method 1 and Method 2

Both Method 1 and Method 2 simplify the current ALF syntax.

Although Method 1 looks more complex than Method 2 in the specification text, Method 1 removes all the syntax redundancy for supporting zero filter in the filter set.

Although Method 2 may still result in redundant bits when supporting zero filter in the filter set, it seems easier to understand Method 2 than to understand Method 1.

It is suggested to adopt either Method 1 or Method 2, and Method 1 is preferred by the proponent.

# Experimental results

Two tests were implemented on VTM6.0 [1]. The simulations were performed following the JVET common test conditions (CTC) [2]. Table 1 shows the BD-rates of signalling alf_luma_coeff_signalled_flag before alf_luma_num_filters_signalled_minus1 with alf_luma_coeff_flag removed. Table 2 shows the BD-rates of removing both alf_luma_coeff_signalled_flag and alf_luma_coeff_flag. It should be noticed that the run time data are inaccurate.

Table 1.  Results of Method 1 (inaccurate runtime data)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Class A2 | 0.00% | 0.00% | 0.00% | 102% | 104% |
| Class B | 0.00% | 0.00% | 0.00% | 103% | 104% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 96% |
| Class F | 0.00% | 0.00% | 0.00% | 97% | 97% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.03% | 105% | 102% |
| Class A2 | 0.00% | 0.00% | 0.00% | 103% | 98% |
| Class B | 0.00% | -0.01% | -0.02% | 101% | 102% |
| Class C | -0.01% | 0.00% | -0.02% | 100% | 105% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Class D | 0.02% | 0.00% | 0.00% | 99% | 108% |
| Class F | 0.00% | 0.00% | 0.01% | 100% | 104% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.02% | 0.00% | 100% | 98% |
| Class C | 0.01% | 0.03% | 0.02% | 102% | 100% |
| Class E | -0.05% | 0.27% | 0.47% | 102% | 99% |
| Overall | -0.01% | 0.08% | 0.12% | 101% | 99% |
| Class D | 0.01% | 0.04% | -0.10% | 99% | 102% |
| Class F | 0.10% | 0.15% | 0.53% | 99% | 100% |

Table 2.  Results of Method 2 (inaccurate runtime data)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Class A2 | 0.00% | 0.00% | 0.00% | 103% | 108% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class D | 0.00% | 0.00% | 0.00% | 98% | 97% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 97% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 104% | 98% |
| Class A2 | 0.00% | 0.02% | 0.01% | 104% | 98% |
| Class B | 0.00% | -0.01% | 0.00% | 102% | 98% |
| Class C | 0.00% | 0.04% | -0.02% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.00% | 102% | 98% |
| Class D | 0.01% | 0.02% | 0.00% | 100% | 104% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 102% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.01% | -0.19% | 100% | 96% |
| Class C | -0.02% | 0.02% | -0.12% | 102% | 101% |
| Class E | -0.08% | -0.15% | 0.17% | 103% | 101% |
| Overall | -0.03% | -0.03% | -0.08% | 101% | 99% |
| Class D | 0.05% | -0.06% | 0.06% | 101% | 103% |
| Class F | 0.11% | 0.23% | 0.18% | 99% | 100% |

# References

- VTM6.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).