| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0165-v2 |
| --- | --- |

| Title: | CE5-related: Simplified CCALF |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Olena Chubach, Chen-Yen Lai, Ching-Yeh Chen, Tzu-Der Chuang, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {jenny.lai, olena.chubach, chingyeh.chen, peter.chuang, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In CE5-2, the filtered result of cross-component adaptive loop filter (CCALF) is added on top of the chroma adaptive loop filter (ALF) output. To support CCALF, the number of multiplications per pixel is increased from 15 to 22 (46.7% increase compared to VTM6.0 ALF). In this contribution, two simplifications are proposed in order to reduce the computation complexity of CCALF tested in CE5-2.

- It is proposed to select CCALF, chroma ALF, or ALF-off for each chroma coding tree block (CTB), and signal the selected option explicitly. As a result, the number of multiplications per pixel is reduced from 22 to 19 (26.7% increase compared to VTM6.0 ALF).

- It is proposed to update the filter shape of the CCALF and to align to that of chroma ALF. The second simplification contains four subtests: a) all CCALF coefficients can have different values; b) top and bottom corresponding coefficients are equal, and left and right coefficients are equal; c) top, bottom, left, and right corresponding coefficients are all equal.

Simulation results with chroma gains shifted to luma are summarized as follows.

The YCbCr BD-rate (Y-BD-rate*0.8 + Cb-BD-rate*0.1 + Cb-BD-rate*0.1, according to AHG13) of the CE5-2.1 CCALF is -0.85% for RA while the number of multiplications in ALF is increased by 46.7% (comparing “VTM6.0 ALF + CCALF” to “VTM6.0 ALF”).

Method 1 preserves 78.3% of CE5-2.1 CCALF’s coding gain while saving 43% of its multiplications.

Method 2a preserves 99.9% of CE5-2.1 CCALF’s coding gain while saving 7% of its multiplications.

Method 2b preserves 95.5% of CE5-2.1 CCALF’s coding gain while saving 21% of its multiplications.

Method 2c preserves xx.x% of CE5-2.1 CCALF’s coding gain while saving 29% of its multiplications.

Method 1+2a preserves xx.x% of CE5-2.1 CCALF’s coding gain while saving 50% of its multiplications.

Method 1+2b preserves 75.4% of CE5-2.1 CCALF’s coding gain while saving 64% of its multiplications.

Method 1+2c preserves xx.x% of CE5-2.1 CCALF’s coding gain while saving 71% of its multiplications.

# Introduction

In ALF, one 7x7 diamond filter with half symmetry is applied to luma, and one 5x5 diamond filter with half symmetry is applied to chroma. Corresponding number of multiplications in the filtering process is 12 per luma sample and 6 per chroma sample. If the chroma sampling format is 4:2:0, the number of multiplications per pixel in the ALF filtering process is 15 ((4 luma samples * 12 multiplications per luma sample + 2 chroma samples * 6 multiplications per chroma sample) / 4 pixels).

The concept of CCALF [1] is to use luma samples to improve chroma samples, and the corresponding filter coefficients are derived and signalled. The filter shape of CCALF is shown in Fig. 1, where one square is one luma sample, the red square is a luma position of to-be-processed chroma sample, each number represents one filter coefficient, and different squares with the same number share one filter coefficient. In total, there are 18 filter taps with 14 filter coefficients, so we need 14 multiplications per chroma sample in CCALF.

Figure 1.  The filter shape in CCALF.

Luma samples before ALF are used in CCALF to allow performing CCALF and chroma ALF in parallel. A filtered result of CCALF is added on top of the output of chroma ALF, so the number of multiplications per pixel becomes 22 ((4 luma samples * 12 multiplications per luma sample + 2 chroma samples * 6 multiplications per chroma sample + 2 chroma samples * 14 multiplications per chroma sample) / 4 pixels). Compared to VTM6.0 ALF, the increase of multiplications is 46.7%. In order to reduce complexity, two methods are proposed in this contribution.

# Proposed methods

## Method 1

Increase of multiplications by CCALF occurs mainly due to applying CCALF and chroma ALF at the same time. In order to reduce complexity, it is proposed to avoid application of both CCALF and chroma ALF. In the suggested design, for each chroma CTB, one CCALF on/off flag is initially signalled to indicate whether CCALF is enabled. If CCALF is enabled, chroma ALF is disabled; otherwise, one chroma ALF on/off flag is signalled to indicate whether chroma ALF is enabled, where chroma ALF on/off flag equal to true means enabling chroma ALF and chroma ALF on/off flag equal to false means ALF-off. The number of multiplications becomes 19 per pixel ((4 luma samples * 12 multiplications per luma sample + max(2 chroma samples * 6 multiplications per chroma sample, 2 chroma samples * 14 multiplications per chroma sample) ) / 4 pixels). Compared to VTM6.0 ALF, the increase of multiplications is reduced from 46.7% to 26.7%.

## Method 2

In order to align CCALF and ALF designs and to further reduce the number of multiplications per pixel, it is proposed to update the filter shape of CCALF to be the same as that of chroma ALF, i.e., as it is shown in Figure 2.

0

1

2

3

4

5

6

7

8

9

10

11

12

0

1

2

3

4

5

6

7

4

8

9

10

0

0

1

2

3

0

4

5

6

0

7

8

9

0

(a)                         (b)                       (c)

Figure 2.  The proposed filter shape and filter coefficients in CCALF.

In order to find the best trade-off between coding gain and complexity, four options of filter coefficient symmetry are tested for the proposed aligned filter shape.

- All coefficients of the filter can have different values as it is shown in Figure 2a. In this case, the number of multiplications per pixel is reduced from 22 to 21.5: ((4 luma samples * 12 multiplications per luma sample + 2 chroma samples * 6 multiplications per chroma sample + 2 chroma samples * 13 multiplications per chroma sample) / 4 pixels).

- The very top coefficient is equal to the very bottom coefficient, and the very left coefficient is equal to the very right coefficient, as it is shown in Figure 2b. In this case, the number of multiplications per pixel is reduced from 22 to 20.5: ((4 luma samples * 12 multiplications per luma sample + 2 chroma samples * 6 multiplications per chroma sample + 2 chroma samples * 11 multiplications per chroma sample) / 4 pixels).

- The very top, bottom, left, and right coefficients are all equal to each other, as it is shown in Figure 2c. In this case, the number of multiplications per pixel is reduced from 22 to 20 multiplications per pixel: ((4 luma samples * 12 multiplications per luma sample + 2 chroma samples * 6 multiplications per chroma sample + 2 chroma samples * 10 multiplications per chroma sample) / 4 pixels).

## Combination of Method 1 and Method 2

Method 1 and Method 2 can be combined together. This way, the number of multiplications per pixel will depend on the filter symmetry chosen for CCALF.

If combining Method 1 and Method 2a, the number of multiplications per pixel becomes 18.5: ((4 luma samples * 12 multiplications per luma sample + max(2 chroma samples * 6 multiplications per chroma sample, 2 chroma samples * 13 multiplications per chroma sample) ) / 4 pixels).

If combining Method 1 and Method 2b, the number of multiplications per pixel becomes 17.5: ((4 luma samples * 12 multiplications per luma sample + max(2 chroma samples * 6 multiplications per chroma sample, 2 chroma samples * 11 multiplications per chroma sample) ) / 4 pixels).

If combining Method 1 and Method 2c, the number of multiplications per pixel becomes 17: ((4 luma samples * 12 multiplications per luma sample + max(2 chroma samples * 6 multiplications per chroma sample, 2 chroma samples * 10 multiplications per chroma sample) ) / 4 pixels).

# Simulation results

The proposed methods have been implemented on VTM6.0 [2]. The simulations were performed following the JVET common test conditions (CTC) [3]. Table 1 shows simulation results of CCALF in CE5-2. Table 2 shows simulation results of the proposed Method 1. Table 3, Table 4, and Table 5 show simulation results of the proposed Method 2a, Method 2b, and Method 2c, respectively. Table 6, Table 7, and Table 8 show the simulation results of a combination of Method 1 with the proposed Method 2a, Method 2b, and Method 2c, respectively. From Table 1 to Table 8, the lambda and chroma QP changes (shifting chroma gains to luma) in CE5-2 are applied. The run time data are inaccurate.

Table 1.  Results of CE5-2 CCALF

|  |  |  | AI Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.19% | 5.37% | 3.75% | 102% | 102% |
| Class A2 | -2.04% | -0.29% | -0.74% | 100% | 101% |
| Class B | -0.74% | -1.74% | 0.92% | 101% | 101% |
| Class C | -0.76% | 3.98% | 4.51% | 99% | 101% |
| Class E | -0.46% | -2.81% | 4.71% | 101% | 99% |
| Overall | -0.99% | 0.78% | 2.54% | 101% | 101% |
| Class D | -0.75% | 5.36% | 4.85% | 100% | 102% |
| Class F | -0.73% | -1.03% | -1.29% | 100% | 98% |
|  |  |  | RA Main10 |  |  |
|  |  |  | Over VTM6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.82% | 5.25% | 4.13% | 99% | 101% |
| Class A2 | -2.05% | -1.14% | -6.18% | 97% | 102% |
| Class B | -0.54% | -6.34% | -2.25% | 100% | 99% |
| Class C | -0.78% | 5.75% | 6.57% | 101% | 97% |
| Class E |  |  |  |  |  |
| Overall | -1.16% | 0.24% | 0.59% | 99% | 100% |
| Class D | -0.64% | 6.08% | 6.42% | 98% | 97% |
| Class F | -0.92% | 1.86% | 2.34% | 100% | 97% |
|  |  |  | LB Main10 |  |  |
|  |  |  | Over VTM6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.54% | -9.26% | -6.60% | 102% | 101% |
| Class C | -0.73% | 3.86% | 5.44% | 100% | 99% |
| Class E | -0.24% | -5.62% | 7.60% | 102% | 98% |
| Overall | -0.53% | -3.98% | 0.96% | 101% | 100% |
| Class D | -0.56% | 4.90% | 7.37% | 105% | 104% |
| Class F | -0.84% | -2.82% | -0.28% | 104% | 101% |

Table 2.  Results of Method 1

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.27% | 6.29% | 5.54% | 100% | 96% |
| Class A2 | -2.16% | 1.59% | 0.31% | 99% | 100% |
| Class B | -0.80% | -0.09% | 2.46% | 100% | 100% |
| Class C | -0.81% | 5.72% | 6.36% | 100% | 99% |
| Class E | -0.50% | -0.58% | 6.28% | 98% | 94% |
| Overall | -1.06% | 2.46% | 4.12% | 100% | 98% |
| Class D | -0.83% | 6.41% | 6.23% | 99% | 97% |
| Class F | -0.85% | 1.01% | 0.30% | 98% | 96% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.85% | 6.22% | 5.29% | 100% | 93% |
| Class A2 | -2.11% | 0.11% | -5.53% | 99% | 90% |
| Class B | -0.56% | -5.31% | -1.31% | 99% | 94% |
| Class C | -0.81% | 6.85% | 7.73% | 99% | 94% |
| Class E |  |  |  |  |  |
| Overall | -1.19% | 1.32% | 1.58% | 99% | 93% |
| Class D | -0.71% | 7.05% | 7.49% | 101% | 92% |
| Class F | -1.01% | 3.10% | 3.12% | 100% | 88% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.56% | -8.33% | -5.50% | 99% | 94% |
| Class C | -0.85% | 5.62% | 7.92% | 100% | 97% |
| Class E | -0.17% | -5.31% | 7.13% | 101% | 85% |
| Overall | -0.56% | -2.92% | 2.13% | 100% | 92% |
| Class D | -0.64% | 7.40% | 9.93% | 101% | 95% |
| Class F | -0.84% | -1.76% | 0.68% | 98% | 85% |

Table 3.  Results of Method 2a

|  | All IntraMain10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-6.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.19% | 5.52% | 3.76% | 101% | 103% |
| Class A2 | -2.04% | -0.12% | -0.85% | 98% | 96% |
| Class B | -0.74% | -2.19% | 0.60% | 99% | 100% |
| Class C | -0.77% | 3.83% | 4.41% | 98% | 98% |
| Class E | -0.47% | -3.61% | 4.02% | 100% | 101% |
| Overall | -0.99% | 0.54% | 2.30% | 99% | 100% |
| Class D | -0.76% | 4.72% | 4.90% | 97% | 99% |
| Class F | -0.73% | -1.43% | -1.75% | 100% | 96% |
|  | Random access Main10 |  |  |  |  |
|  |  | Over VTM-6.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.82% | 5.63% | 4.24% | 100% | 103% |
| Class A2 | -2.05% | -1.07% | -6.29% | 100% | 106% |
| Class B | -0.54% | -6.53% | -2.57% | 100% | 103% |
| Class C | -0.76% | 6.03% | 6.64% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -1.16% | 0.34% | 0.50% | 100% | 103% |
| Class D | -0.70% | 5.53% | 6.20% | 101% | 99% |
| Class F | -0.94% | 1.40% | 2.22% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  |  | Over VTM-6.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.51% | -8.67% | -6.53% | 101% | 104% |
| Class C | -0.76% | 4.65% | 6.29% | 100% | 101% |
| Class E | -0.16% | -6.57% | 6.49% | 102% | 101% |
| Overall | -0.51% | -3.71% | 1.00% | 101% | 102% |
| Class D | -0.58% | 5.39% | 7.65% | 102% | 105% |
| Class F | -0.88% | -3.09% | -0.11% | 101% | 102% |

Table 4.  Results of Method 2b

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.20% | 5.76% | 3.94% | 102% | 103% |
| Class A2 | -2.04% | 0.27% | -0.59% | 98% | 97% |
| Class B | -0.75% | -1.62% | 1.13% | 100% | 100% |
| Class C | -0.78% | 4.24% | 4.77% | 98% | 102% |
| Class E | -0.49% | -3.00% | 4.41% | 102% | 102% |
| Overall | -1.00% | 1.00% | 2.67% | 100% | 101% |
| Class D | -0.76% | 5.29% | 5.04% | 97% | 99% |
| Class F | -0.74% | -1.03% | -1.30% | 101% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.82% | 5.61% | 4.40% | 100% | 103% |
| Class A2 | -2.05% | -0.77% | -5.95% | 100% | 104% |
| Class B | -0.55% | -6.17% | -2.03% | 100% | 102% |
| Class C | -0.78% | 5.95% | 6.68% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -1.17% | 0.50% | 0.79% | 100% | 102% |
| Class D | -0.65% | 5.98% | 6.28% | 101% | 102% |
| Class F | -0.93% | 1.67% | 2.43% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.51% | -7.73% | -5.74% | 101% | 105% |
| Class C | -0.72% | 5.27% | 6.38% | 100% | 101% |
| Class E | -0.22% | -5.44% | 7.11% | 101% | 102% |
| Overall | -0.51% | -2.83% | 1.51% | 101% | 103% |
| Class D | -0.57% | 6.08% | 7.26% | 101% | 110% |
| Class F | -0.97% | -2.73% | 0.03% | 100% | 102% |

Table 5.  Results of Method 2c (to be updated)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.20% | 5.75% | 4.05% | 101% | 104% |
| Class A2 | -2.04% | 0.28% | -0.52% | 97% | 95% |
| Class B | -0.76% | -1.26% | 1.42% | 99% | 100% |
| Class C | -0.78% | 4.30% | 4.81% | 99% | 101% |
| Class E | -0.49% | -2.96% | 4.43% | 100% | 101% |
| Overall | -1.01% | 1.12% | 2.79% | 99% | 100% |
| Class D | -0.77% | 5.42% | 5.04% | 97% | 99% |
| Class F | -0.75% | -0.73% | -1.38% | 101% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.80% | 5.69% | 4.40% | 100% | 102% |
| Class A2 | -1.26% | -6.71% | -27.44% | 32% | 68% |
| Class B | -0.53% | -5.71% | -1.95% | 100% | 104% |
| Class C | -0.76% | 6.12% | 6.85% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.99% | -0.47% | -3.43% | 80% | 94% |
| Class D | -0.64% | 5.68% | 6.49% | 100% | 101% |
| Class F | -0.93% | 1.97% | 2.31% | 101% | 103% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.72% | -48.88% | -9.91% | 1% | 18% |
| Class C | -0.72% | 5.47% | 6.54% | 101% | 105% |
| Class E | -0.10% | -5.06% | 7.14% | 104% | 106% |
| Overall | 0.03% | -19.81% | -0.16% | 13% | 51% |
| Class D | -0.52% | 5.26% | 7.91% | 101% | 106% |
| Class F | -0.82% | -2.54% | 0.04% | 46% | 81% |

Table 6.  Results of Method 1 + Method 2a (to be updated)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.26% | 6.46% | 5.52% | 100% | 103% |
| Class A2 | -2.16% | 1.69% | 0.21% | 98% | 94% |
| Class B | -0.80% | -0.55% | 2.06% | 99% | 98% |
| Class C | -0.81% | 5.51% | 6.23% | 98% | 99% |
| Class E | -0.49% | -1.37% | 5.50% | 102% | 102% |
| Overall | -1.06% | 2.20% | 3.83% | 99% | 99% |
| Class D | -0.83% | 5.81% | 6.12% | 98% | 100% |
| Class F | -0.85% | 0.54% | 0.03% | 101% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.84% | 6.28% | 5.45% | 99% | #NUM! |
| Class A2 | -1.24% | -5.11% | -27.40% | 32% | #NUM! |
| Class B | -0.57% | -5.50% | -1.61% | 99% | #NUM! |
| Class C | -0.82% | 6.97% | 7.79% | 100% | #NUM! |
| Class E |  |  |  |  |  |
| Overall | -1.03% | 0.26% | -2.85% | 79% | #NUM! |
| Class D | -0.72% | 6.28% | 7.23% | 100% | #NUM! |
| Class F | -1.01% | 2.54% | 3.02% | 99% | #NUM! |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 1.00% | -50.37% | -12.16% | 1% | 18% |
| Class C | -0.83% | 6.17% | 8.32% | 100% | 102% |
| Class E | -0.25% | -5.99% | 6.58% | 102% | 106% |
| Overall | 0.08% | -20.43% | -0.65% | 12% | 50% |
| Class D | -0.65% | 7.17% | 9.74% | 101% | 102% |
| Class F | -0.98% | -1.66% | 0.41% | 46% | 79% |

Table 7.  Results of Method 1 + Method 2b (to be updated)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.27% | 6.69% | 5.59% | 100% | 102% |
| Class A2 | -2.16% | 2.09% | 0.46% | 97% | 95% |
| Class B | -0.81% | -0.04% | 2.57% | 98% | 99% |
| Class C | -0.82% | 5.90% | 6.52% | 98% | 97% |
| Class E | -0.51% | -0.85% | 5.90% | 99% | 101% |
| Overall | -1.06% | 2.62% | 4.15% | 98% | 99% |
| Class D | -0.84% | 6.24% | 6.27% | 97% | 99% |
| Class F | -0.86% | 0.89% | 0.29% | 99% | 98% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.85% | 6.51% | 5.61% | 99% | #NUM! |
| Class A2 | -2.10% | 0.52% | -5.56% | 99% | #NUM! |
| Class B | -0.55% | -5.35% | -1.21% | 99% | #NUM! |
| Class C | -0.81% | 6.87% | 7.86% | 100% | #NUM! |
| Class E |  |  |  |  |  |
| Overall | -1.19% | 1.45% | 1.70% | 99% | #NUM! |
| Class D | -0.68% | 6.49% | 7.14% | 99% | #NUM! |
| Class F | -1.00% | 2.67% | 3.20% | 100% | #NUM! |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.65% | -44.59% | -8.76% | 2% | 27% |
| Class C | -0.78% | 6.49% | 8.35% | 100% | 102% |
| Class E | -0.19% | -4.54% | 8.13% | 100% | 104% |
| Overall | -0.04% | -17.55% | 1.17% | 21% | 59% |
| Class D | -0.68% | 7.15% | 9.70% | 101% | 104% |
| Class F | -0.89% | -1.45% | 0.99% | 46% | 81% |

Table 8.  Results of Method 1 + Method 2c (to be updated)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.27% | 6.71% | 5.71% | 101% | 106% |
| Class A2 | -2.16% | 2.10% | 0.53% | 97% | 95% |
| Class B | -0.81% | 0.37% | 2.80% | 99% | 101% |
| Class C | -0.83% | 5.91% | 6.52% | 98% | 98% |
| Class E | -0.51% | -0.83% | 5.87% | 99% | 102% |
| Overall | -1.07% | 2.75% | 4.24% | 99% | 100% |
| Class D | -0.84% | 6.36% | 6.27% | 97% | 99% |
| Class F | -0.87% | 1.03% | 0.30% | 100% | 96% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.85% | 6.34% | 5.53% | 99% | 104% |
| Class A2 | -1.23% | -4.08% | -26.98% | 32% | 68% |
| Class B | -0.57% | -4.62% | -0.98% | 99% | 101% |
| Class C | -0.80% | 6.84% | 7.96% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -1.02% | 0.74% | -2.49% | 79% | 94% |
| Class D | -0.69% | 6.52% | 7.33% | 99% | 102% |
| Class F | -1.02% | 2.92% | 3.27% | 100% | 103% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.88% | -47.53% | -8.71% | 1% | 18% |
| Class C | -0.83% | 6.75% | 8.58% | 100% | 101% |
| Class E | -0.27% | -4.90% | 7.78% | 101% | 103% |
| Overall | 0.02% | -18.78% | 1.18% | 12% | 49% |
| Class D | -0.72% | 7.50% | 9.94% | 101% | 101% |
| Class F | -0.89% | -1.28% | 0.96% | 46% | 78% |

# Conclusion

In this contribution, two methods are proposed, which allow to reduce the number of multiplications per-pixel: 1) CTB-level switch between CCALF and chroma ALF; 2) align filter shape between CCALF and chroma ALF. The increase of multiplications can be reduced, depending on the filter type chosen.

# References

- K. Misra, F. Bossen, and A. Segall, “Cross-component adaptive loop filter for chroma,” Document of Joint Video Experts Team, JVET-O0636

- VTM6.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).