| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0168-v1 |
| --- | --- |

| Title: | CE7-related: Cleanup for 2x2 chroma quantization matrices |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Olena Chubach, Chen-Yen Lai, Ching-Yeh Chen, Tzu-Der Chuang, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei Hyeongmun Jang, Junghak Nam, Seethal Paluri, SeungHwan Kim, Jaehyun Lim | Email: | {olena.chubach, jenny.lai, chingyeh.chen, peter.chuang, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com {hm.jang, junghak.nam, seethal.paluri, seunghwan3.kim, jaehyun.lim}@lge.com |
| Source: | MediaTek Inc., LG Electronics Inc. |  |  |

_____________________________

# Abstract

In the current version of VVC, 2x2, 2x4, and 4x2 chroma intra coding blocks (CBs) do not exist, and the smallest intra block size is equal to 2x8 and 8x2, as well as the smallest chroma intra block copy (IBC) block size. Furthermore, in the current VVC version inter-prediction is disabled for 4x4 luma CBs. Therefore, small 2x2 chroma blocks can be created only by applying a subblock transform (SBT). Considering these essences, two methods are proposed: 1) remove 2x2 intra chroma quantization matrices (QMs) from the default QM list, and not code user-defined intra QMs for this size; 2) always apply default 2x2 chroma QMs (flat, with all values equal to 16), which is equivalent to disabling QMs for small 2x2 chroma blocks. The proposed methods are implemented on top of VTM6.1.

# Introduction

A document JVET-O0050 [1] adopted in Gothenburg introduced a smallest chroma Intra prediction unit (SCIPU) defined as a coding tree node whose chroma block size is larger than or equal to 16 chroma samples and has at least one child luma block smaller than 64 luma samples. As a result, after adopting this document, 2x2, 2x4, and 4x2 chroma intra CBs do not exist in the VVC, and the smallest intra block size is 2x8 and 8x2, as well as the smallest chroma IBC block size. Furthermore, after adoption of JVET-N0266, inter-prediction for 4x4 luma CBs is also disabled.

Yet, SBT can still be enabled for CBs with width or height larger than or equal to 8. Therefore, if SBT is applied to CBs of size equal to 8x4, 4x8, 16x4 or 4x16, a 4x4 luma block can be created. For example, when applying SBT with vertical split to a 8x4 luma CB, a 4x4 luma CB is generated, which corresponds to 2x2 chroma CB.

# Proposed methods

Two methods are proposed:

- Remove 2x2 intra chroma quantization matrices (QMs) from the default QM list, and not code user-defined Intra QMs for this size. In this case, only non-intra chroma default QMs are present and coding of user defined 2x2 chroma Inter user defined QMs is supported in the VVC

- Always apply default 2x2 chroma QMs, (flat with all values equal to 16), which is equivalent to disabling QMs for small 2x2 chroma blocks. Thus, there is no need for supporting coding of user defined 2x2 chroma QMs in the VVC.

The proposed methods are implemented on top of VTM6.1 [4]. Both proposed approaches do not affect common test condition (CTC) results.

# References

- Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, Y. Zhao, and H. Yang, “CE3-2.1.1 and CE3-2.1.2: Removing 2x2, 2x4, and 4x2 chroma CBs,” Document of Joint Video Experts Team, JVET-O0050.

- H. Liu, L. Zhang, K. Zhang, H. Liu, J. Xu, and Y. Wang, “CE2-related: Disabling bi-prediction or inter prediction for small blocks,” Document of Joint Video Experts Team, JVET-N0266.

- O. Chubach, C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, and S.-M. Lei, “CE7-related: Support of signalling default and user-defined scaling matrices,” Document of Joint Video Experts Team, JVET-N0090.

- VTM6.1, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.1

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).