| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0177-v1 |
| --- | --- |

| Title: | Non-CE3: On Constraint of CCLM and CST |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Che-Wei Kuo Jingya Li Chong Soon Lim Abe Kiyofumi | Tel: Email: | +65-6550-5423 chewei.kuo@sg.panasonic.com jingya.li@sg.panasonic.com chongsoon.lim@sg.panasoni.com abe.kiyo@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

To prevent the CCLM latency issue when CST is on, a CCLM enabling constraint is applied in VVC Draft 6. However, it allows CCLM when luma VPDU is not split while chroma VPDU is split. It is proposed to disallow this case, to reduce hardware complexity for supporting luma subpart processing.

Simulation results are summarized as follows.

|  | All Intra |  |  | Random Access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Remove luma subpart processing | 0.00% | 0.12% | 0.07% |  |  |  |  |  |  |

# Problem Statement

JVET-O1124 was adopted in the last meeting to solve the CCLM latency issue when CST is on, which an example is luma VPDU is horizontal BT split but chroma VPDU is vertical BT split. Table 1 shows the CCLM enabling constraint which reduces the latency to 32x32 luma reconstruction samples. To support luma VPDU no split with chroma VPDU QT case, a quarter VPDU luma subpart processing is assumed valid and required for luma intra prediction modules. Fig. 1 shows an illustration.

For luma angular, DC, planar, and PDPC modes, special handling for quarter VPDU subpart luma prediction and residue addition is required for CCLM chroma prediction samples. Also, the reference samples for luma subpart blocks are different from that of luma QT case, so it can not be simply reused. For MIP, a subpart matrix multiplication and interpolation process are further required. The former one needs special handling and the latter one need be cared since the prediction samples used for interpolation may across the subpart boundaries. For IBC, since luma block copy can be performed in subpart, this may not be a problem. For ISP, CCLM is not allowed in luma no split case.

Given the complexity for supporting luma subpart processing and the fact that luma blocks are usually smaller than chroma blocks, the CCLM enabling constraint for luma no split while chroma split seems not a good trade-off.

Table 1: CCLM enabling constraint in VVC Draft 6.

| Luma VPDU | Chroma VPDU |
| --- | --- |
| No split and not ISP | No split QT BT_H (no further split or followed by BT_V) |
| QT | No split QT BT_H (no further split or followed by BT_V) |

Fig. 1. Luma subpart processing is required for luma intra prediction modules.

# Proposed Method

It is proposed to remove CCLM enabling cases which require luma subpart processing. Note the ISP constraint can also be removed since the simplified condition is equivalent to the single tree case.

Table 2: Proposed CCLM enabling cases.

| Luma VPDU | Chroma VPDU |
| --- | --- |
| No split and not ISP | No split QT BT_H (no further split or followed by BT_V) |
| No split and not ISP | No split QT BT_H (no further split or followed by BT_V) |

# Simulation Results

Simulation results are reported as below. The JVET common test conditions defined in [1] are followed. The simulation was done by heterogeneous platforms and hence the runtime may not be reliable.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.31% | 0.11% | 102% | 84% |
| Class A2 | 0.02% | 0.27% | 0.31% | 115% | 94% |
| Class B | 0.01% | 0.08% | 0.00% | 112% | 97% |
| Class C | 0.00% | -0.01% | 0.02% | 105% | 96% |
| Class E | 0.00% | 0.02% | 0.00% | 99% | 90% |
| Overall | 0.00% | 0.12% | 0.07% | 107% | 93% |
| Class D | 0.00% | 0.02% | 0.01% | 99% | 91% |
| Class F | 0.01% | 0.05% | 0.07% | 108% | 95% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | 0.03% | 0.12% | 0.13% | 104% | 100% |
| Class B | 0.02% | 0.20% | 0.01% | 102% | 97% |
| Class C | -0.02% | 0.00% | 0.07% | 102% | 97% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusions

In this contribution, it is proposed to remove the CCLM enabling cases which require luma subpart processing. Negligible coding performance change is observed. It is suggested to adopt the proposed method to the next VVC working draft.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, March 2019.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed Spec Changes

8.4.4	Cross-component chroma intra prediction mode checking process

Input to this process is:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current chroma coding block relative to the top-left luma sample of the current picture.

Output to this process is:

- a variable CclmEnabled specifying if a cross-component chroma intra prediction mode is enabled (TRUE) or not enabled (FALSE) for the current chroma coding block.

The variable, CclmEnabled is derived as follows:

- If sps_cclm_enabled_flag is equal to 0, CclmEnabled is set equal to 0.

- Otherwise, if one of the following conditions is true, CclmEnabled is set equal to 1:

  - qtbtt_dual_tree_intra_flag is equal to 0.

  - slice_type is not equal to I.

  - CtbLog2SizeY is less than 6.

- Otherwise the following applies.:

  - The variables xCb64, yCb64, yCb32 are derived as follows:

xCb64 = ( xCb >> 6) << 6	(8-38)

yCb64 = ( yCb >> 6 ) << 6	(8-39)

yCb32 = ( yCb >> 5 ) << 5	(8-40)

  - If one of the following conditions is true, the variable CclmEnabled is set equal to 1:

  - CbWidth[ 1 ][ xCb64 ][ yCb64 ] is equal to 64 and CbHeight[ 1 ][ xCb64 ][ yCb64 ] is equal to 64.

  - CqtDepth[ 1 ][ xCb64 ][ yCb64 ] is equal to CtbLog2SizeY − 6, MttSplitMode[ xCb64 ][ yCb64 ][ 0 ] is equal to SPLIT_BT_HOR, CbWidth[ 1 ][ xCb64 ][ yCb32 ] is equal to 64 and CbHeight[ 1 ][ xCb64 ][ yCb32 ] is equal to 32

  - CqtDepth[ 1 ][ xCb64 ][ yCb64 ] is greater than CtbLog2SizeY − 6.

  - CqtDepth[ 1 ][ xCb64 ][ yCb64 ] is equal to CtbLog2SizeY − 6, MttSplitMode[ xCb64 ][ yCb64 ][ 0 ] is equal to SPLIT_BT_HOR, and MttSplitMode[ xCb64 ][ yCb32 ][ 1 ] is equal to SPLIT_BT_VER.

  - Otherwise, the variable CclmEnabled is set equal to 0.

When CclmEnabled is equal to 1 and one of the following conditions is true, CclmEnabled is set equal to 0:

- CbWidth[ 0 ][ xCb64 ][ yCb64 ] and CbHeight[ 0 ][ xCb64 ][ yCb64 ] are both equal to 64, and CbWidth[ 1 ][ xCb64 ][ yCb64 ] and CbHeight[ 1 ][ xCb64 ][ yCb64 ] are not both equal to 64. intra_subpartitions_mode_flag[ xCb64 ][ yCb64 ] is equal to 1.

- CbWidth[ 0 ][ xCb64 ][ yCb64 ] or CbHeight[ 0 ][ xCb64 ][ yCb64 ] is less than 64, and CqtDepth[ 0 ][ xCb64 ][ yCb64 ] is equal to CtbLog2SizeY − 6.