| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0180-v2 |
| --- | --- |

| Title: | Non-CE5: Cleanup of ALF syntax elements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu, Li Zhang, Kai Zhang, Na Zhang, Yang Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | +86 13911971613 liuhongbin.01@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In the current VVC design, when coding the ALF parameters in APS, two flags including alf_luma_filter_signal_flag and alf_luma_coeff_signalled_flag are used to indicate whether there are non-zero ALF coefficients, which is redundant. In addition, alf_luma_coeff_flag is unnecessary since it could be adjusted by number of filters to be signalled. On the other hand, two syntax elements including alf_ctb_use_first_aps_flag and alf_luma_prev_filter_idx_minus1 are used to signal index of the APS used by a CTU, which is overkilled. This contribution proposes to remove alf_luma_coeff_signalled_flag and alf_luma_coeff_flag from APS and remove alf_ctb_use_first_aps_flag from the CTU syntax table. Simulation results reportedly show that the proposed method has negligible impact on the coding performance.

# Introduction

In the current VVC design [1], when coding the ALF parameters in APS, two flags including alf_luma_filter_signal_flag and alf_luma_coeff_signalled_flag are used to indicate whether there are non-zero ALF coefficients, which is redundant and contradictary. For example, as shown in Table 1, even if alf_luma_filter_signal_flag is true which indicates that there are ALF coefficients, alf_luma_coeff_signalled_flag may still be false and ALF coefficients may not be signalled.

In addtion, for each filter, one more flag, named alf_luma_coeff_flag is furtehr signalled to indicate whether the filter has all zero coefficients or not. However, it could be not really necessary since the number of filters to be coded has been represented by alf_luma_num_filters_signalled_minus1. For example, if some of the filters are all zero coefficients, alf_luma_num_filters_signalled_minus1 could be adjusted accordingly.

On the other hand, two syntax elements including alf_ctb_use_first_aps_flag and alf_luma_prev_filter_idx_minus1 are used to signal index of the APS used by a CTU (as shown in Table 2), which is overkilled.

Table 1: Adaptive loop filter data syntax table.

| alf_data( ) { | Descriptor |
| --- | --- |
| alf_luma_filter_signal_flag | u(1) |
| alf_chroma_filter_signal_flag | u(1) |
| if( alf_luma_filter_signal_flag ) { |  |
| alf_luma_clip_flag | u(1) |
| alf_luma_num_filters_signalled_minus1 | ue(v) |
| if( alf_luma_num_filters_signalled_minus1 > 0 ) { |  |
| for( filtIdx = 0; filtIdx < NumAlfFilters; filtIdx++ ) |  |
| alf_luma_coeff_delta_idx[ filtIdx ] | u(v) |
| } |  |
| alf_luma_coeff_signalled_flag | u(1) |
| if( alf_luma_coeff_signalled_flag ) { |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) |  |
| alf_luma_coeff_flag[ sfIdx ] | u(1) |
| } |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| alf_luma_coeff_abs[ sfIdx ][ j ] | uek(v) |
| if( alf_luma_coeff_abs[ sfIdx ][ j ] ) |  |
| alf_luma_coeff_sign[ sfIdx ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( alf_luma_clip_flag ) { |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) |  |
| alf_luma_clip_idx[ sfIdx ][ j ] | u(2) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

Table 2: Coding tree unit syntax table.

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| xCtb = ( CtbAddrInRs % PicWidthInCtbsY ) << CtbLog2SizeY |  |
| yCtb = ( CtbAddrInRs / PicWidthInCtbsY ) << CtbLog2SizeY |  |
| if( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ) |  |
| sao( xCtb >> CtbLog2SizeY, yCtb >> CtbLog2SizeY ) |  |
| if( slice_alf_enabled_flag ){ |  |
| alf_ctb_flag[ 0 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| if( alf_ctb_flag[ 0 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] ) { |  |
| if( slice_num_alf_aps_ids_luma > 0 ) |  |
| alf_ctb_use_first_aps_flag | ae(v) |
| if( !alf_ctb_use_first_aps_flag ) { |  |
| if( slice_num_alf_aps_ids_luma > 1 ) |  |
| alf_use_aps_flag | ae(v) |
| if( alf_use_aps_flag ) { |  |
| if( slice_num_alf_aps_ids_luma > 2 ) |  |
| alf_luma_prev_filter_idx_minus1 | ae(v) |
| } else |  |
| alf_luma_fixed_filter_idx | ae(v) |
| } |  |
| } |  |
| … |  |
| } |  |
| … |  |
| } |  |

# Proposed method

To tackle the abovementioned problem, this contribution proposes cleanups of the ALF syntax elements.

First, it is proposed to remove the syntax elements alf_luma_coeff_signalled_flag and alf_luma_coeff_flag from APS.

Second, it is proposed to remove the syntax element alf_ctb_use_first_aps_flag from the CTU syntax table. One flag is firstly signalled to indicate whether the predictor is from ALF APS or fixed filters. Then, alf_luma_prev_filter_idx is signalled instead of alf_luma_prev_filter_idx_minus1. Meanwhile, truncated unary is used to binarize the syntax element alf_luma_prev_filter_idx.

# Experimental results

The proposed method is implemented on top of the reference software VTM-6.0 [2], and simulations were performed following the common test conditions [3]. Two tests are performed.

- Test #1: proposed first aspect.

- Test #2: proposed first aspect + proposed second aspect.

The results are shown in Table 3 and Table 4. It can be seen that proposed method has negligible impact on the coding performance.

Table 3: Coding performance of Test #1.

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 103% | 102% | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 103% | 102% | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 100% | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 103% | 0.00% | 0.00% | 0.00% | 102% | 103% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 103% |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 102% | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 106% | 0.00% | 0.00% | 0.00% | 102% | 105% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 100% | 0.00% | 0.00% | 0.00% | 102% | 103% |

Table 4: Coding performance of Test #2.

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% | -0.12% | -0.09% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.04% | 0.10% | 0.06% | 100% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 100% | 0.00% | 0.02% | -0.02% | 99% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 98% | -0.01% | -0.02% | -0.02% | 99% | 97% |
| Class E | 0.00% | 0.01% | 0.01% | 100% | 97% |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.01% | 0.00% | -0.02% | 99% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% | -0.01% | 0.04% | 0.00% | 99% | 98% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 97% | -0.01% | 0.00% | -0.04% | 99% | 98% |

# Conclusions

This contribution proposes cleanups of the ALF syntax elements. It has negligible impact on the coding performance. It is recommended to adopt it into VVC.

# Spec change

The spec change is on top of JVET-O2001-vE. The newly added parts are highlighted in yellow and the deleted parts are highlighted in yellow strikethrough.

## Suggested chagnes for Test #1

#### 7.3.2.14	Adaptive loop filter data syntax

| alf_data( ) { | Descriptor |
| --- | --- |
| alf_luma_filter_signal_flag | u(1) |
| alf_chroma_filter_signal_flag | u(1) |
| if( alf_luma_filter_signal_flag ) { |  |
| alf_luma_clip_flag | u(1) |
| alf_luma_num_filters_signalled_minus1 | ue(v) |
| if( alf_luma_num_filters_signalled_minus1 > 0 ) { |  |
| for( filtIdx = 0; filtIdx < NumAlfFilters; filtIdx++ ) |  |
| alf_luma_coeff_delta_idx[ filtIdx ] | u(v) |
| } |  |
| alf_luma_coeff_signalled_flag | u(1) |
| if( alf_luma_coeff_signalled_flag ) { |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) |  |
| alf_luma_coeff_flag[ sfIdx ] | u(1) |
| } |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| alf_luma_coeff_abs[ sfIdx ][ j ] | uek(v) |
| if( alf_luma_coeff_abs[ sfIdx ][ j ] ) |  |
| alf_luma_coeff_sign[ sfIdx ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( alf_luma_clip_flag ) { |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) |  |
| alf_luma_clip_idx[ sfIdx ][ j ] | u(2) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

#### 7.4.3.14	Adaptive loop filter data semantics

…

alf_luma_coeff_signalled_flag equal to 1 indicates that alf_luma_coeff_flag[ sfIdx ] is signalled. alf_luma_coeff_signalled_flag equal to 0 indicates that alf_luma_coeff_flag[ sfIdx ] is not signalled.

alf_luma_coeff_flag[ sfIdx ] equal 1 specifies that the coefficients of the luma filter indicated by sfIdx are signalled. alf_luma_coeff_flag[ sfIdx ] equal to 0 specifies that all filter coefficients of the luma filter indicated by sfIdx are set equal to 0. When not present, alf_luma_coeff_flag[ sfIdx ] is set equal to 1.

## Suggested chagnes for Test #2

#### 7.3.8.2	Coding tree unit syntax

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| xCtb = ( CtbAddrInRs % PicWidthInCtbsY ) << CtbLog2SizeY |  |
| yCtb = ( CtbAddrInRs / PicWidthInCtbsY ) << CtbLog2SizeY |  |
| if( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ) |  |
| sao( xCtb >> CtbLog2SizeY, yCtb >> CtbLog2SizeY ) |  |
| if( slice_alf_enabled_flag ){ |  |
| alf_ctb_flag[ 0 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| if( alf_ctb_flag[ 0 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] ) { |  |
| if( slice_num_alf_aps_ids_luma > 0 ) |  |
| alf_ctb_use_first_aps_flag | ae(v) |
| if( !alf_ctb_use_first_aps_flag ) { |  |
| if( slice_num_alf_aps_ids_luma > 1 ) |  |
| alf_use_aps_flag | ae(v) |
| if( alf_use_aps_flag ) { |  |
| if( slice_num_alf_aps_ids_luma > 1 2 ) |  |
| alf_luma_prev_filter_idx_minus1 | ae(v) |
| } else |  |
| alf_luma_fixed_filter_idx | ae(v) |
| } |  |
| } |  |
| … |  |
| } |  |
| … |  |
| } |  |

#### 7.4.9.2	Coding tree unit semantics

alf_ctb_use_first_aps_flag equal to 1 sepcifies that the filter information in APS with adaptive_parameter_set_id equal to slice_alf_aps_id_luma[ 0 ] is used. alf_ctb_use_first_aps_flag equal to 0 specifies that the luma CTB does not use the filter information in APS with adaptive_parameter_set_id equal to slice_alf_aps_id_luma[ 0 ]. When alf_ctb_use_first_aps_flag is not present, it is inferred to be equal to 0.

alf_use_aps_flag equal to 0 specifies that one of the fixed filter sets is applied to the luma CTB. alf_use_aps_flag equal to 1 specifies that a filter set from an APS is applied to the luma CTB. When alf_use_aps_flag is not present, it is inferred to be equal to 0.

alf_luma_prev_filter_idx_minus1 plus 1 specifies the previous filter that is applied to the luma CTB. The value of alf_luma_prev_filter_idx_minus1 shall be in a range of 0 to slice_num_alf_aps_ids_luma – 12, inclusive. When alf_luma_prev_filter_idx_minus1 is not present, it is inferred to be equal to 0.

The variable AlfCtbFiltSetIdxY[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] specifying the filter set index for the luma CTB at location ( xCtb, yCtb ) is derived as follows:

- If alf_ctb_use_first_aps_flag is equal to 1, AlfCtbFiltSetIdxY[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] is set equal to 16.

- Otherwise, if alf_use_aps_flag is equal to 0, AlfCtbFiltSetIdxY[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] is set equal to alf_luma_fixed_filter_idx.

- Otherwise, AlfCtbFiltSetIdxY[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] is set equal to 17 + alf_luma_prev_filter_idx_minus1 16 + alf_luma_prev_filter_idx.

#### Initialization process for context variables

| Table 9-4 – Association of ctxIdx and syntax elements for each initializationType in the initialization process |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax structure | Syntax element | ctxTable | initType |  |  |  |
|  |  |  | 0 | 1 | 2 |  |
| coding_tree_unit( ) | alf_ctb_flag[ ][ ][ ] | Table 9-5 | 0..8 | 9..17 | 18..26 |  |
|  | alf_ctb_use_first_aps_flag alf_luma_prev_filter_idx | Table 9-6 | 0 | 1 | 2 |  |
|  | alf_use_aps_flag | Table 9-7 | 0 | 1 | 2 |  |
|  | alf_ctb_filter_alt_idx[ ][ ][ ] | Table 9-8 | 0..1 | 2..3 | 4..5 |  |

Table 9-6 – Specification of initValue and shiftIdx for ctxIdx of alf_ctb_use_first_aps_flag alf_luma_prev_filter_idx

| Initialization variable | ctxIdx of alf_ctb_use_first_aps_flag alf_luma_prev_filter_idx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | EP | EP | EP |
| shiftIdx | 0 | 0 | 0 |

#### 9.3.3.1	General

#### Table 9-77 – Syntax elements and associated binarizations

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| slice_data( ) | end_of_brick_one_bit | FL | cMax = 1 |
| coding_tree_unit( ) | alf_ctb_flag[ ][ ][ ] | FL | cMax = 1 |
|  | alf_ctb_use_first_aps_flag | FL | cMax = 1 |
|  | alf_use_aps_flag | FL | cMax = 1 |
|  | alf_luma_fixed_filter_idx | TB | cMax = 15 |
|  | alf_luma_prev_filter_idx_minus1 | TBTR | cMax = slice_num_alf_aps_ids_luma − 12, cRiceParam = 0 |
|  | alf_ctb_filter_alt_idx[ ][ ][ ] | TR | cMax = alf_chroma_num_alt_filters_minus1, cRiceParam = 0 |

##### 9.3.4.2.1	General

| Table 9-82 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| end_of_brick_one_bit | terminate | na | na | na | na | na |
| alf_ctb_flag[ ][ ][ ] | 0..8 (clause 9.3.4.2.2) | na | na | na | na | na |
| alf_ctb_use_first_aps_flag | 0 | na | na | na | na | na |
| alf_use_aps_flag | 0 | na | na | na | na | na |
| alf_luma_fixed_filter_idx | bypass | bypass | bypass | bypass | bypass | bypass |
| alf_luma_prev_filter_idx_minus1 | Bypass 0 | bypass | bypass | bypass | bypass | bypass |

# References

- “Versatile Video Coding (Draft 6)”, B. Bross, J. Chen, S. Liu, Doc. JVET-O2001, July. 2019.

- VTM-6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0.

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, Doc. JVET-N1010, Mar. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).