| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0190-v2 |
| --- | --- |

| Title: | AHG8/AHG17: Scalable nesting SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang San Diego, CA, USA | Email: | ye-kui.wang@futurewei.com |
| Source: | Futurewei Technologies, Inc. |  |  |

_____________________________

# Abstract

The HEVC layered extensions use a two-level mechanism for nesting of BP, PT, and DUI SEI messages SEI messages for OLSs, by the scalable nesting SEI message and the bitstream partition nesting SEI message, the latter must be contained within the former.

This contribution proposes a design of the scalable nesting SEI message that is asserted to directly associate BP, PT, and DUI SEI messages to specific layers in the contexts of specific OLSs for HRD operations. The scalable nesting SEI message is asserted to be also able to associate non-BP/PT/DUI SEI messages for specific layers not in the context of an OLS.

In v2 of this contribution, the proposal has been updated according to earlier JVET decisions on scalability, particularly on specifying OLS-specific PTL and CPB operations instead of layer-specific PTL and CPB operations.

# Proposal

The proposed design of scalable nesting SEI message is as follows:

## General SEI message syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ) |  |
| pic_timing( payloadSize ) |  |
| else if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) |  |
| else if( payloadType = = 130 ) |  |
| decoding_unit_info( payloadSize ) |  |
| else if( payloadType = = 133 ) |  |
| scalable_nesting( payloadSize ) |  |
| else if( payloadType = = 145 ) |  |
| dependent_rap_indication( payloadSize ) // Specified in ITU-T H.SEI ¦ ISO/IEC 23002-7. |  |
| else if( payloadType = = 168 ) |  |
| frame_field_info( payloadSize ) |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) |  |
| if( payloadType = = 132 ) |  |
| decoded_picture_hash( payloadSize ) // Specified in ITU-T H.SEI ¦ ISO/IEC 23002-7. |  |
| else if( payloadType = = 133 ) |  |
| scalable_nesting( payloadSize ) |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if( more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| reserved_payload_extension_data | u(v) |
| payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

## Scalable nesting SEI message syntax

| scalable_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| nesting_ols_flag | u(1) |
| if( nesting_ols_flag ) { |  |
| nesting_num_olss_minus1 | ue(v) |
| for( i = 0; i <= nesting_num_olss_minus1; i++ ) |  |
| nesting_ols_idx_delta_minus1[ i ] | ue(v) |
| } else { |  |
| nesting_all_layers_flag | u(1) |
| if( !nesting_all_layers_flag ) { |  |
| nesting_num_layers_minus1 | ue(v) |
| for( i = 1; i <= nesting_num_layers_minus1; i++ ) |  |
| nesting_layer_id[ i ] | u(6) |
| } |  |
| } |  |
| nesting_num_seis_minus1 | ue(v) |
| while( !byte_aligned( ) ) |  |
| nesting_zero_bit /* equal to 0 */ | u(1) |
| for( i = 0; i <= nesting_num_seis_minus1; i++ ) |  |
| sei_message( ) |  |
| } |  |

## Scalable nesting SEI message semantics

The scalable nesting SEI message provides a mechanism to associate SEI messages with specific OLSs or with specific layers.

A scalable nesting SEI message contains one or more SEI messages. The SEI messages contained in the scalable nesting SEI message are also referred to as the scalable-nested SEI messages.

It is a requirement of bitstream conformance that the following restrictions apply on containing of SEI messages in a scalable nesting SEI message:

–	An SEI message that has payloadType equal to 132 (decoded picture hash) or 133 (scalable nesting) shall not be contained in a scalable nesting SEI message.

–	When a scalable nesting SEI message contains a buffering period, picture timing, or decoding unit information SEI message, the scalable nesting SEI message shall not contain any other SEI message with payloadType not equal to 0 (buffering period), 1 (picture timing), or 130 (decoding unit information).

It is a requirement of bitstream conformance that the following restrictions apply on the value of the nal_unit_type of the SEI NAL unit containing a scalable nesting SEI message:

–	When a scalable nesting SEI message contains an SEI message that has payloadType equal to 0 (buffering period), 1 (picture timing), 130 (decoding unit information), 145 (dependent RAP indication), or 168 (frame-field information), the SEI NAL unit containing the scalable nesting SEI message shall have nal_unit_type equal to PREFIX_SEI_NUT.

nesting_ols_flag equal to 1 specifies that the scalable-nested SEI messages apply to specific OLSs. nesting_ols_flag equal to 0 specifies that the scalable-nested SEI messages apply to specific layers.

It is a requirement of bitstream conformance that the following restrictions apply on the value of nesting_ols_flag:

–	When the scalable nesting SEI message contains an SEI message that has payloadType equal to 0 (buffering period), 1 (picture timing), or 130 (decoding unit information), the value of nesting_ols_flag shall be equal to 1.

–	When the scalable nesting SEI message contains an SEI message that has payloadType equal to a value in VclAssociatedSeiList, the value of nesting_ols_flag shall be equal to 0.

nesting_num_olss_minus1 plus 1 specifies the number of OLSs to which the scalable-nested SEI messages apply. The value of nesting_num_olss_minus1 shall be in the range of 0 to TotalNumOlss − 1, inclusive.

nesting_ols_idx_delta_minus1[ i ] is used to derive the variable NestingOlsIdx[ i ] that specifies the OLS index of the i-th OLS to which the scalable-nested SEI messages apply when nesting_ols_flag is equal to 1. The value of nesting_ols_idx_delta_minus1[ i ] shall be in the range of 0 to TotalNumOlss − 2, inclusive, inclusive.

The variable NestingOlsIdx[ i ] is derived as follows:

if( i  = =  0 )
	NestingOlsIdx[ i ] = nesting_ols_idx_delta_minus1[ i ]	(D-xx)
else
	NestingOlsIdx[ i ] = NestingOlsIdx[ i − 1 ] + nesting_ols_idx_delta_minus1[ i ] + 1

-

nesting_all_layers_flag equal to 1 specifies that the scalable-nested SEI messages apply to all layers that have nuh_layer_id greater than or equal to the nuh_layer_id of the current SEI NAL unit. nesting_all_layers_flag equal to 0 specifies that the scalable-nested SEI messages may or may not apply to all layers that have nuh_layer_id greater than or equal to the nuh_layer_id of the current SEI NAL unit.

nesting_num_layers_minus1 plus 1 specifies the number of layers to which the scalable-nested SEI messages apply. The value of nesting_num_layers_minus1 shall be in the range of 0 to vps_max_layers_minus1 − GeneralLayerIdx[ nuh_layer_id ], inclusive, where nuh_layer_id is the nuh_layer_id of the current SEI NAL unit.

nesting_layer_id[ i ] specifies the nuh_layer_id value of the i-th layer to which the scalable-nested SEI messages apply when nesting_all_layers_flag is equal to 0. The value of nesting_layer_id[ i ] shall be greater than nuh_layer_id, where nuh_layer_id is the nuh_layer_id of the current SEI NAL unit.

When nesting_ols_flag is equal to 0, the variable NestingNumLayers, specifying the nubmer of layer to which the scalable-nested SEI messages apply, and the list NestingLayerId[ i ] for i in the range of 0 to NestingNumLayers − 1, inclusive, specifying the list of nuh_layer_id value of the layers to which the scalable-nested SEI messages apply, are derived as follows, where nuh_layer_id is the nuh_layer_id of the current SEI NAL unit:

if( nesting_all_layers_flag ) {
	NestingNumLayers = vps_max_layers_minus1 + 1 − GeneralLayerIdx[ nuh_layer_id ] 
	for( i = 0; i < NestingNumLayers; i ++)
		NestingLayerId[ i ] = vps_layer_id[ GeneralLayerIdx[ nuh_layer_id ] + i ] 	(D-xx)
} else {
	NestingNumLayers = nesting_num_layers_minus1 + 1
	for( i = 0; i < NestingNumLayers; i ++)
		NestingLayerId[ i ] = ( i  = = 0 ) ? nuh_layer_id : nesting_layer_id[ i ]
}

nesting_num_seis_minus1 plus 1 specifies the number of scalable-nested SEI messages. The value of nesting_num_seis_minus1 shall be in the range of 0 to 63, inclusive.

nesting_zero_bit shall be equal to 0.

# Patent rights declaration(s)

Futurewei Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).