| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0195 |
| --- | --- |

| Title: | Non-CE5: Unified strong deblocking filter for luma and chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjo Ahn Donggyu Sim | Tel: Email: | +82-2-941-6470 yjahn@digitalinsights.co.kr dgsim@digitalinsights.co.kr |
| Source: | Digital Insights Inc. |  |  |

_____________________________

# Abstract

This contribution proposes unified strong deblocking filter for luma and chroma. In VTM-6.0, there are two different strong deblocking filters for luma and chroma. In this case, the cost of the hardware design can be increased by using two different filters with the same roles. Therefore, this contribution proposes to apply one unified filter in place of two different strong deblocking filters that perform the same roles. The proposed change has a negligible impact on coding performance, namely: 0.00%/-0.02%/-0.02%, 0.01%/0.01%/-0.05%, 0.00%/-0.08%/0.16%, and 0.05%/-0.03%/0.14% (Y/Cb/Cr) in AI, RA, LDB, and LDP configurations correspondingly.

# Introduction

Figure 1. Flowchart of deblocking filter for luma and chroma components in VTM-6.0

Figure 1 shows a flowchart of the deblocking filter for luma and chroma components in VTM-6.0. As shown in Figure 1, there are two different strong deblocking filters for luma and chroma components, however, the role and applied samples of strong filter are the same.

- Strong filter for luma component

p0′ = Clip3( p0 − 3 * tC, p0 + 3 * tC, ( p2 + 2 * p1 + 2 * p0 + 2 * q0 + q1 + 4 ) >> 3 ) (8-1162) p1′ = Clip3( p1 − 2 * tC, p1 + 2 * tC, ( p2 + p1 + p0 + q0 + 2 ) >> 2 ) (8-1163) p2′ = Clip3( p2 − 1 * tC, p2 + 1*tC, ( 2 * p3 + 3 * p2 + p1 + p0 + q0 + 4 ) >> 3 ) (8-1164) q0′ = Clip3( q0 − 3 * tC, q0 + 3 * tC, ( p1 + 2 * p0 + 2 * q0 + 2 * q1 + q2 + 4 ) >> 3 ) (8-1165) q1′ = Clip3( q1 − 2 * tC, q1 + 2 * tC, ( p0 + q0 + q1 + q2 + 2 ) >> 2 ) (8-1166) q2′= Clip3( q2 − 1 * tC, q2 + 1 * tC, ( p0 + q0 + q1 + 3 * q2 + 2 * q3 + 4 ) >> 3 ) (8-1167)

- Strong filter for chroma components

p0′ = Clip3( p0 − tC, p0 + tC, ( p3 + p2 + p1 + 2 * p0 + q0 + q1 + q2 + 4 ) >> 3 ) (8-1198) p1′ = Clip3( p1 − tC, p1 + tC, ( 2 * p3 + p2 + 2 * p1 + p0 + q0 + q1 + 4 ) >> 3 ) (8-1199) p2′ = Clip3( p2 − tC, p2 + tC, ( 3 * p3 + 2 * p2 + p1 + p0 + q0 + 4 ) >> 3 ) (8-1200) q0′ = Clip3( q0 − tC, q0 + tC, ( p2 + p1 + p0 + 2 * q0 + q1 + q2 + q3 + 4 ) >> 3 ) (8-1201) q1′ = Clip3( q1 − tC, q1 + tC, ( p1 + p0 + q0 + 2 * q1 + q2 + 2 * q3 + 4 ) >> 3 ) (8-1202) q2′= Clip3( q2 − tC, q2 + tC, ( p0 + q0 + q1 + 2 * q2 + 3 * q3 + 4 ) >> 3 ) (8-1203)

# Proposed method

This contribution proposes a unification of strong deblocking filter for luma and chroma components to reduce the cost of the hardware design. In this contribution, there are two tests for unified strong deblocking filter for luma and chroma.

## Test 1: Unified strong filter as conventional that of luma

If the condition for the strong deblocking filter for chroma components is true, the following strong filter the same as that of luma applies for chroma samples:

p0′ = Clip3( p0 − 3 * tC, p0 + 3 * tC, ( p2 + 2 * p1 + 2 * p0 + 2 * q0 + q1 + 4 ) >> 3 ) (8-1198) p1′ = Clip3( p1 − 2 * tC, p1 + 2 * tC, ( p2 + p1 + p0 + q0 + 2 ) >> 2 ) (8-1199) p2′ = Clip3( p2 − 1 * tC, p2 + 1*tC, ( 2 * p3 + 3 * p2 + p1 + p0 + q0 + 4 ) >> 3 ) (8-1200) q0′ = Clip3( q0 − 3 * tC, q0 + 3 * tC, ( p1 + 2 * p0 + 2 * q0 + 2 * q1 + q2 + 4 ) >> 3 ) (8-1201) q1′ = Clip3( q1 − 2 * tC, q1 + 2 * tC, ( p0 + q0 + q1 + q2 + 2 ) >> 2 ) (8-1202) q2′= Clip3( q2 − 1 * tC, q2 + 1 * tC, ( p0 + q0 + q1 + 3 * q2 + 2 * q3 + 4 ) >> 3 ) (8-1203)

## Test 2: Unified strong filter as conventional that of chroma

If the condition for the strong deblocking filter for luma component is true, the following strong filter the same as that of chroma applies for luma samples:

p0′ = Clip3( p0 − tC, p0 + tC, ( p3 + p2 + p1 + 2 * p0 + q0 + q1 + q2 + 4 ) >> 3 ) (8-1162) p1′ = Clip3( p1 − tC, p1 + tC, ( 2 * p3 + p2 + 2 * p1 + p0 + q0 + q1 + 4 ) >> 3 ) (8-1163) p2′ = Clip3( p2 − tC, p2 + tC, ( 3 * p3 + 2 * p2 + p1 + p0 + q0 + 4 ) >> 3 ) (8-1164) q0′ = Clip3( q0 − tC, q0 + tC, ( p2 + p1 + p0 + 2 * q0 + q1 + q2 + q3 + 4 ) >> 3 ) (8-1165) q1′ = Clip3( q1 − tC, q1 + tC, ( p1 + p0 + q0 + 2 * q1 + q2 + 2 * q3 + 4 ) >> 3 ) (8-1166) q2′= Clip3( q2 − tC, q2 + tC, ( p0 + q0 + q1 + 2 * q2 + 3 * q3 + 4 ) >> 3 ) (8-1167)

# Performance evaluation

The proposed unified strong deblocking filter for luma and chroma is implemented on top of VTM-6.0 and the simulation is performed under JVET common test conditions (CTC) [2]. Table 1 shows the results of Test 1 and Table 2 shows the results of Test 2.

Table 1. Test 1

|  | Over VTM-6.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra Main10 |  |  |  |  | Random access Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 98% | 100% | 0.01% | -0.05% | -0.08% | 99% | 99% |
| Class A2 | 0.00% | -0.05% | -0.01% | 99% | 100% | 0.03% | 0.05% | -0.01% | 102% | 102% |
| Class B | 0.00% | -0.04% | -0.08% | 100% | 100% | 0.00% | 0.00% | -0.11% | 97% | 97% |
| Class C | 0.00% | -0.04% | -0.01% | 100% | 102% | 0.01% | 0.02% | 0.01% | 99% | 99% |
| Class E | 0.00% | 0.03% | 0.04% | 98% | 100% |  |  |  |  |  |
| Overall | 0.00% | -0.02% | -0.02% | 99% | 100% | 0.01% | 0.01% | -0.05% | 99% | 99% |
| Class D | 0.00% | -0.02% | -0.02% | 100% | 102% | -0.01% | -0.07% | -0.04% | 99% | 100% |
| Class F | 0.00% | 0.01% | 0.02% | 102% | 102% | -0.03% | 0.06% | 0.01% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  | Low delay P Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.01% | -0.14% | 0.14% | 101% | 104% | -0.02% | -0.14% | 0.10% | 98% | 96% |
| Class C | -0.04% | -0.03% | 0.02% | 99% | 98% | 0.01% | -0.03% | -0.04% | 100% | 101% |
| Class E | 0.03% | -0.05% | 0.40% | 99% | 101% | 0.22% | 0.15% | 0.46% | 100% | 101% |
| Overall | 0.00% | -0.08% | 0.16% | 100% | 101% | 0.05% | -0.03% | 0.14% | 99% | 99% |
| Class D | 0.01% | 0.08% | 0.12% | 99% | 99% | 0.02% | 0.03% | 0.00% | 99% | 100% |
| Class F | -0.02% | -0.29% | 0.63% | 102% | 102% | -0.06% | 0.35% | 0.92% | 99% | 98% |

Table 2. Test 2

|  | Over VTM-6.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra Main10 |  |  |  |  | Random access Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.01% | -0.01% | 98% | 100% | 0.01% | -0.02% | 0.05% | 98% | 98% |
| Class A2 | 0.00% | -0.01% | 0.00% | 100% | 100% | 0.04% | 0.02% | -0.03% | 100% | 100% |
| Class B | -0.01% | 0.00% | 0.00% | 99% | 99% | 0.01% | 0.06% | 0.01% | 96% | 97% |
| Class C | 0.00% | -0.01% | 0.00% | 100% | 101% | -0.01% | -0.01% | -0.15% | 99% | 99% |
| Class E | -0.02% | 0.00% | 0.00% | 100% | 101% |  |  |  |  |  |
| Overall | -0.01% | 0.00% | 0.00% | 99% | 100% | 0.01% | 0.02% | -0.03% | 98% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 103% | -0.03% | -0.05% | -0.20% | 99% | 100% |
| Class F | -0.01% | -0.01% | -0.01% | 102% | 102% | -0.02% | -0.03% | -0.03% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  | Low delay P Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.05% | 0.00% | -0.14% | 99% | 99% | 0.02% | -0.10% | 0.28% | 92% | 84% |
| Class C | -0.05% | 0.08% | 0.07% | 99% | 99% | 0.03% | 0.12% | 0.30% | 97% | 92% |
| Class E | 0.06% | 0.17% | 0.17% | 99% | 100% | 0.11% | -0.54% | 0.22% | 100% | 100% |
| Overall | 0.02% | 0.07% | 0.01% | 99% | 99% | 0.05% | -0.14% | 0.27% | 96% | 91% |
| Class D | -0.01% | -0.18% | -0.28% | 100% | 101% | 0.08% | 0.13% | 0.03% | 97% | 95% |
| Class F | 0.04% | -0.02% | 0.45% | 100% | 101% | 0.01% | -0.13% | 0.19% | 97% | 94% |

# Conclusions

In order to reduce the cost of the hardware design of deblocking filter, unified strong deblocking filter for luma and chroma is proposed. It is recommended to adopt the proposed method 1 into VVC standard.

# References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001, 15th JVET meeting, Gothenburg, SE, July 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Shuhring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, 14th JVET meeting, Geneva, CH, March 2019.

# Patent rights declaration(s)

Digital Insights Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).