| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0201-v2 |
| --- | --- |

| Title: | Non-CE4: Simplification on BDOF offset calculation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Du Liu Ruoyang Yu Torshamnsgatan 23, 164 83 Stockholm, Sweden | Email: | du.liu@ericsson.com ruoyang.yu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In VTM-6.0, the bi-directional optical flow (BDOF) method is used to improve prediction accuracy for a bi-predicted inter CU. For each 4x4 subblock in the CU, the method derives a pair of refinement parameters (vx, vy). The method further uses the derived refinement parameters to compute a BDOF offset for each prediction sample and refines the prediction sample value using the BDOF offset.

This contribution proposes to remove the shift operations when calculating the BDOF offset for each sample and instead to perform shift operation directly on the derived (vx, vy) pair per 4x4 subblock. The proposed method was reportedly tested under VTM-6.0 random access (RA) common test configuration. The BD-rate impact is reported to be 0.00%.

The contribution also reports that there is a mismatch between the VVC specification (draft 6) and the VTM software in terms of the BDOF offset calculation. The BD-rate impact is provided in the contribution if aligning the VTM software to the VVC specification.

# Introduction

In VVC specification text (draft 6) [1], the BDOF refinement parameters vx and vy and the bdofOffset are calculated as

  - The horizontal and vertical motion offset of the current subblock are derived as:

vx = sGx2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres,	(8-817)
	−( sGxdI << 3 ) >> Floor( Log2( sGx2 ) ) )  :  0

vy = sGy2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres, ( ( sGydI << 3 ) − 	(8-818)
	( ( vx * sGxGym ) << 12 + vx * sGxGys ) >> 1 ) >> Floor( Log2( sGy2 ) ) )  :  0

  - For x =xSb − 1..xSb + 2, y = ySb − 1..ySb + 2, the prediction sample values of the current sub-block are derived as follows:

bdofOffset = ( vx * ( gradientHL0[ x + 1 ][ y + 1 ] − gradientHL1[ x + 1 ][ y + 1 ] ) ) >> 1	(8-819)
	+ ( vy * (gradientVL0[ x + 1 ][ y + 1 ] − gradientVL1[ x + 1 ][ y + 1 ] ) ) >> 1

pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ]  + offset4 +	(8-820)
	predSamplesL1[ x + 1 ][ y + 1 ] + bdofOffset )  >>  shift4 )

In VTM-6.0 software, the bdofOffset is calculated in Buffer.cpp as

void addBIOAvgCore(const Pel* src0, int src0Stride,...)

{

...

b = tmpx * (gradX0[x] - gradX1[x]) + tmpy * (gradY0[x] - gradY1[x]);

b = ((b + 1) >> 1);

...

}

As can be seen, there is a mismatch between the specification in (8-819) and the software.

However, either the method described in the VVC specification or the method in the VTM-6.0 software requires shift operation per sample for each 4x4 subblock.

# Proposal

## Method 1: Perform shift operation (with rounding) on the (Vx, Vy) per 4x4 subblock

To reduce the number of shifts, this method proposes to remove the two right shifts per location in a 4x4 subblock and instead to perform two shift operations to a pair of vx and vy only once per 4x4 subblock.

The shift operation is implemented by adding an offset 1 before right-shifting. That is,

vx = (vx +1) >>1,

vy = (vy +1) >>1.

The specification text is modified where the additional lines are marked with yellow and the removed parts are maked with red:

8.5.6 Decoding process for inter blocks 8.5.6.5 Bi-directional optical flow prediction process … - The horizontal and vertical motion offset of the current subblock are derived as: vx = sGx2 > 0 ? Clip3( −mvRefineThres, mvRefineThres, −( sGxdI << 3 ) >> Floor( Log2( sGx2 ) ) ) : 0 vy = sGy2 > 0 ? Clip3( −mvRefineThres, mvRefineThres, ( ( sGydI << 3 ) − ( ( vx * sGxGym ) << 12 + vx * sGxGys ) >> 1 ) >> Floor( Log2( sGy2 ) ) ) : 0 vx = (vx + 1) >> 1 vy = (vy + 1) >> 1 - For x =xSb − 1..xSb + 2, y = ySb − 1..ySb + 2, the prediction sample values of the current sub-block are derived as follows: bdofOffset = ( vx * ( gradientHL0[ x + 1 ][ y + 1 ] − gradientHL1[ x + 1 ][ y + 1 ] ) ) >> 1 + ( vy * (gradientVL0[ x + 1 ][ y + 1 ] − gradientVL1[ x + 1 ][ y + 1 ] ) ) >> 1 pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ] + offset4 + predSamplesL1[ x + 1 ][ y + 1 ] + bdofOffset ) >> shift4 )

## Method 2: an alternative method to the method 1

Similar to Method 1 but without adding an offset, the shift operation is implemented by directly right-shifting vx and vy by 1 bit. That is,

vx = vx >>1,

vy = vy >>1.

The modified specification text is:

8.5.6 Decoding process for inter blocks 8.5.6.5 Bi-directional optical flow prediction process … - The horizontal and vertical motion offset of the current subblock are derived as: vx = sGx2 > 0 ? Clip3( −mvRefineThres, mvRefineThres, −( sGxdI << 3 ) >> Floor( Log2( sGx2 ) ) ) : 0 vy = sGy2 > 0 ? Clip3( −mvRefineThres, mvRefineThres, ( ( sGydI << 3 ) − ( ( vx * sGxGym ) << 12 + vx * sGxGys ) >> 1 ) >> Floor( Log2( sGy2 ) ) ) : 0 vx = vx >> 1 vy = vy >> 1 - For x =xSb − 1..xSb + 2, y = ySb − 1..ySb + 2, the prediction sample values of the current sub-block are derived as follows: bdofOffset = ( vx * ( gradientHL0[ x + 1 ][ y + 1 ] − gradientHL1[ x + 1 ][ y + 1 ] ) ) >> 1 + ( vy * (gradientVL0[ x + 1 ][ y + 1 ] − gradientVL1[ x + 1 ][ y + 1 ] ) ) >> 1 pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ] + offset4 + predSamplesL1[ x + 1 ][ y + 1 ] + bdofOffset ) >> shift4 )

# Results

The proposed methods were implemented in VTM-6.0 reference software [2]. The simulations were run using the VTM common test configurations [3].

## Test 1: Results of Method 1 - shift in vx and vy with rounding

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | 0.05% | 101% | 100% |
| Class A2 | 0.01% | 0.01% | 0.05% | 100% | 97% |
| Class B | -0.02% | -0.01% | 0.02% | 101% | 102% |
| Class C | 0.01% | 0.06% | 0.00% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.02% | 101% | 101% |
| Class D | -0.01% | -0.12% | -0.01% | 100% | 100% |
| Class F | 0.01% | 0.01% | 0.01% | 101% | 99% |

## Test 2: Results of Method 2 - shift in vx and vy without rounding

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.01% | 0.06% | 103% | 100% |
| Class A2 | 0.01% | -0.02% | 0.02% | 101% | 97% |
| Class B | -0.01% | 0.03% | -0.01% | 101% | 103% |
| Class C | 0.02% | 0.03% | 0.06% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.03% | 102% | 101% |
| Class D | 0.01% | 0.02% | 0.03% | 100% | 97% |
| Class F | 0.00% | -0.01% | -0.01% | 101% | 101% |

## Test 3: Supplementary results if aligning the VTM software to the specification text

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.06% | 0.02% | 101% | 105% |
| Class A2 | 0.01% | 0.04% | 0.05% | 101% | 99% |
| Class B | -0.01% | 0.02% | -0.01% | 101% | 103% |
| Class C | 0.00% | -0.02% | 0.02% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.02% | 101% | 102% |
| Class D | -0.01% | -0.04% | -0.03% | 101% | 99% |
| Class F | 0.00% | -0.01% | -0.01% | 99% | 102% |

In v1, the PSNR numbers in the accompanying excel sheets are in 6-decimal accuracy. In v2, the PSNR numbers are corrected to be 4-decimal accuracy.

# Conclusion

The proposed method 1 removes the shift operations per location in a 4x4 subblock and instead performs two shift operations to a pair of vx and vy only once per 4x4 subblock. The modification has marginal impact on the BD-rate performance. We recommend adopting the method 1 into the next VTM software and VVC working draft.

# References

[1] B. Bross, J. Chen, S. Liu, "Versatile Video Coding (Draft 6)", Output document approved by JVET, document number JVET-O2001.

[2] VTM-6.0 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).